- Specifications BMW X1.
X1 sDrive18d, X1 sDrive20d

|  |  |  | BMW X1 sDrive18d |  | BMW X1 sDrive20d |
| --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |
| No of doors/seats |  |  | 5 / 5 |  | 5 / 5 |
| Length/width/height (unladen) | mm |  | 4 454 / 1798 / 1545 |  | 4 454 / 1798 / 1545 |
| Wheelbase | mm |  | 2 760 |  | 2 760 |
| Track, front/rear | mm |  | 1500 / 1529 |  | 1500 / 1529 |
| Turning circle | m |  | 11,3 |  | 11,3 |
| Tank capacity | app ltr |  | 61 |  | 61 |
| Cooling system incl heater | ltr |  | 7,5 |  | 7,5 |
| Engine oil1) | ltr |  | 5,2 |  | 5,2 |
| Weight, unladen, to DIN/EU | kg |  | 1470 / 1545 |  | 1490 / 1565 |
| Max load to DIN | kg |  | 550 |  | 550 |
| Max permissible | kg |  | 2030 |  | 2045 |
| Max axle load, front/rear | kg |  | 940 / 1155 |  | 940 / 1160 |
| Max trailer load (12%), braked/unbraked | kg |  | 1800 / 750 |  | 1800 / 750 |
| Max roofload/max towbar downl | kg |  | 75 / 80 |  | 75 / 80 |
| Luggage comp capacity | ltr |  | 420 – 1350 |  | 420 – 1350 |
| Air drag | Cd x A |  | 0,32 x 2,34 |  | 0,32 x 2,34 |
| Power Unit |  |  |  |  |  |
| Power Unit |  |  |  |  |  |
| Config/No of cyls/valves |  |  | R / 4 / 4 |  | R / 4 / 4 |
| Engine technology |  |  | Common-rail direct injection. Turbocharger with variable intake geometry |  |  |
| Capacity, effective | cc |  | 1995 |  | 1995 |
| Stroke / bore | mm |  | 90,0 / 84,0 |  | 90,0 / 84,0 |
| Compression ratio | :1 |  | 16,5 |  | 16,5 |
| Fuel |  |  | Diesel |  | Diesel |
| Max output | kW/hp |  | 105 / 143 |  | 130 / 177 |
| at | rpm |  | 4000 |  | 4000 |
| Max torque | Nm/lb-ft |  | 320 |  | 350 |
| at | rpm |  | 1750 – 2500 |  | 1750 – 3000 |
| Electrical System |  |  |  |  |  |
| Battery/location | Ah/– |  | 80 / luggage comp |  | 80 / luggage comp |
| Alternator | A/W |  | 180 / 2520 |  | 180 / 2520 |
| Driving Dynamics and Safety |  |  |  |  |  |
| Suspension, front |  | Aluminium double-joint tie bar axle |  |  |  |
| Suspension, rear |  | Double-wishbone five-link axle with trailing arm |  |  |  |
| Brakes, front |  | Single-piston swing-calliper disc brakes |  |  |  |
| Diameter | mm | 312 x 24 / vented |  |  | 312 x 24 / vented |
| Brakes, rear |  | Single-piston swing-calliper disc brakes |  |  |  |
| Diameter | mm | 300 x 20 / vented |  |  | 300 x 20 / vented |
| Driving stability systems |  | Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control |  |  |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, active headrests at the front, three-point inertia-reel seat belts on all seats, belt latch tensioners at the front, crash sensors, roll-over sensors, Tyre Defect Indicator |  |  |  |
| Steering |  | Rack-and-pinion power steering |  |  |  |
| Steering transmission, overall | :1 | 16,13 |  |  | 16,13 |
| Tyres, front/rear |  | 225/50 R17 94H |  |  | 225/50 R17 94H |
| Rims, front/rear |  | 7,5J x 17 steel |  |  | 7,5J x 17 steel |

- Specifications BMW X1.
X1 sDrive18d, X1 sDrive20d

|  |  | BMW X1 sDrive18d |  |  |  | BMW X1 sDrive20d |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BMW ConnectedDrive |  |  |  |  |  |  |  |
| Comfort |  |  | Optional: BMW Assist including Enquiry Service, remote-control function and V-Info+, BMW TeleServices, integration of mobile terminals |  |  |  |  |
| Infotainment |  | Optional: access to the internet, BMW Online including features such as Parkinfo, Nationalinfo, Google directory service, News, realtime weather, Office functions, BMW Routes and online music title update |  |  |  |  |  |
| Safety |  | Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant, Park Distance Control, back-up camera, Head-Up Display, Active Cruise Control with Stop & Go, Automatic/Enhanced Emergency Call |  |  |  |  |  |
| Transmission |  |  |  |  |  |  |  |
| Type of transmission |  |  | Six-speed manual |  |  |  |  |
| Gear ratios I | :1 |  | 4,002 |  |  |  | 5,140 |
| II | :1 |  | 2,108 |  |  |  | 2,830 |
| III | :1 |  | 1,380 |  |  |  | 1,804 |
| IV | :1 |  | 1,000 |  |  |  | 1,257 |
| V | :1 |  | 0,780 |  |  |  | 1,000 |
| VI | :1 |  | 0,645 |  |  |  | 0,831 |
| Reverse | :1 |  | 3,187 |  |  |  | 4,638 |
| Final drive | :1 |  | 3,231 |  |  |  | 2,643 |
| Performance |  |  |  |  |  |  |  |
| Power-to-weight ratio | kg/kW | 14,0 |  |  |  | 11,5 |  |
| Output per litre | kW/hp | 52,6 |  |  |  | 65,2 |  |
| Acceleration 0–100 km/h | sec | 9,6 |  |  |  | 8,1 |  |
| Standing-start km | sec | 30,8 |  |  |  | 28,8 |  |
| In 4th / 5th gear 80-120km/h | sec | 8,2 / 10,9 |  |  |  | 7,0 / 8,9 |  |
| Top speed | km/h | 200 |  |  |  | 205 (218)2 |  |
| Offroad characteristics |  |  |  |  |  |  |  |
| Overhang angle front / rear | ° | 18,0/22,2 |  |  |  | 18,0/22,2 |  |
| Ramp breakover angle | ° | 16,8 |  |  |  | 16,8 |  |
| Ground cleerance | mm | 194 |  |  |  | 194 |  |
| BMW EfficientDynamics |  |  |  |  |  |  |  |
| BMW EfficientDynamics standard features |  | Brake energy regeneration, Intelligent lightweight technology, on-demand fuel pump, on-demand coolant pump, Auto Start Stop function and gearshift indicator, final drive with optimised thermal management ,a/c compressor detachable |  |  |  |  |  |
| Fuel Consumption in EU Cycle |  |  |  |  |  |  |  |
| Urban | ltr/100 km | 6,1 |  |  |  | 6,4 |  |
| Extra-urban | ltr/100 km | 4,7 |  |  |  | 4,7 |  |
| Combined | ltr/100 km | 5,2 |  |  |  | 5,3 |  |
| CO2 | g/km | 136 |  |  |  | 139 |  |
| Emission rating |  | EU5 |  |  |  | EU5 |  |

Specifications applicable to ACEA markets/homologation data applicable in part only to Germany (weight),

- Oil change

- With high speed setup (depending on market)