[image: image1.png]


BMW
Media
Information

7/2009


Valid from
9/2009

BMW
Media
Information

7/2009


Valid from
9/2009


Specifications BMW X3.
X3 xDrive30d, X3 xDrive35d
	
	
	BMW X3 xDrive30d
	
	BMW X3 xDrive35d

	
	
	
	
	

	Body
	
	
	
	

	No of doors/seats
	
	5 / 5
	
	5 / 5

	Length/width/height (unladen)
	mm
	4569 / 1853 / 1674
	
	4569 / 1853 / 1674

	Wheelbase
	mm
	2795
	
	2795

	Track, front/rear
	mm
	1524 / 1542
	
	1524 / 1542

	Turning circle
	m
	11,7
	
	11,7

	Tank capacity
	app ltr
	67
	
	67

	Cooling system incl heater
	ltr
	10,3 (11,0)
	
	11,1

	Engine oil1)
	ltr
	7,5
	
	7,5

	Weight, unladen, to DIN/EU
	kg
	1805 / 1880

(1810 / 1885)
	
	1875 / 1950



	Max load to DIN
	kg
	545
	
	510

	Max permissible
	kg
	2350 (2355)
	
	2385

	Max axle load, front/rear
	kg
	1150 / 1260
	
	1177 / 1284

	Max trailer load (12%),

braked/unbraked
	kg
	2000 / 750


	
	2000 / 750



	Max roofload/max towbar downl
	kg
	75/80
	
	75/80

	Luggage comp capacity
	ltr
	480-1560
	
	480-1560

	Air drag
	Cd  x A
	0,34 x 2,50
	
	0,35 x 2,50

	Power Unit
	
	
	
	

	Power Unit
	
	
	
	

	Config/No of cyls/valves 
	
	In-line / 6 / 4
	
	In-line / 6 / 4

	Engine technology
	
	Common-rail 
direct injection with 
variable intake geometry
	
	Common-rail direct injection, 
Variable Twin Turbo

	Capacity, effective
	cc
	2993
	
	2993

	Stroke / bore
	mm
	90,0 / 84,0
	
	90,0 / 84,0

	Compression ratio
	:1
	17,0
	
	17,0

	Fuel 
	
	Diesel
	
	Diesel

	Max output
	kW/hp
	160 / 218
	
	210 / 286

	at 
	rpm
	4000
	
	4400

	Max torque
	Nm/lb-ft
	500
	
	580

	at
	rpm
	1750-2750
	
	1750-2250

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/location
	Ah/–
	90 / luggage comp
	
	90 / luggage comp

	Alternator
	A/W
	150 / 2100
	
	150 / 2100

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double-joint tie bar strut axle, small steering roll radius, anti-dive

	Suspension, rear
	
	Double-wishbone central pivot axle with trailing arms, anti-squat and anti-dive

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	325 x 25 / vented
	
	325 x 25 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	320 x 22 / vented
	
	320 x 22 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant, DSC cross-linked with xDrive, Hill Descent Control HDC

	Safety equipment
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, active headrests at the front, three-point inertia-reel seat belts on all seats, belt latch tensioners at the front, crash sensors, roll-over sensors, Tyre Defect Indicator, optional: rear sideairbags

	Steering
	
	Rack-and-pinion power steering, optional: Servotronic

	Steering transmission, overall
	:1
	18,9
	
	18,9

	Tyres, front/rear
	
	235/55 R17 99H M+S
	
	235/50 R18 97V

	Rims, front/rear
	
	8J x 17 LM
	
	8J x 18 LM


Specifications BMW X3.
X3 xDrive30d, X3 xDrive35d

	
	
	BMW X3 xDrive30d
	
	BMW X3 xDrive35d

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist including Enquiry Service, remote-control function and V-Info+, BMW TeleServices, integration of mobile terminals

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, Park Distance Control, Automatic/Enhanced Emergency Call 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	30d: Manual six-speed transmission (optional: Six-speed automatic transmission)
35d: Six-speed automatic transmission


	Gear ratios
I
	:1
	5,080 (4,171)
	
	4,171

	                                  
II
	:1
	2,804 (2,340)
	
	2,340

	                                  
III
	:1
	1,783 (1,521)
	
	1,521

	                                  
IV
	:1
	1,260 (1,143)
	
	1,143

	                                  
V
	:1
	1,000 (0,867)
	
	0,867

	                                   
VI
	:1
	0,835 (0,691)
	
	0,691

	                                   
Reverse
	:1
	4,607 (3,403)
	
	3,403

	Final drive
	:1
	2,929 (3,462)
	
	3,231

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	11,3
	
	8,9

	Output per litre
	kW/hp
	53,5
	
	70,2

	Acceleration       0–100 km/h
	sec
	7,4 (7,7)
	
	6,4

	                      Standing-start km
	sec
	28,0 (28,4)
	
	26,7

	In 4th / 5th gear 80-120km/h
	sec
	5,7 / 6,9 (-)
	
	-

	Top speed
	km/h
	210 / 2202
	
	240

	
	
	
	
	

	Offroad characteristics
	
	
	
	

	Overhang angle
front / rear



	°
	
23,9/21,8
	
	
23,9/21,8

	Ramp breakover angle
	°
	17,2
	
	17,2

	Ground cleerance
	mm
	201
	
	201

	Fording ability
	mm
	500
	
	500

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics standard features
	
	Intelligent lightweight technology, on-demand fuel pump, on-demand coolant pump, tyres with reduced roll resistance

	
	
	
	
	

	Fuel Consumption in EU Cycle
	
	
	
	

	Urban
	ltr/100 km
	9,7 (9,9)
	
	9,7

	Extra-urban
	ltr/100 km
	6,0 (6,4)
	
	6,7

	Combined
	ltr/100 km
	7,4 (7,7)
	
	7,8

	CO2
	g/km
	196 (206)
	
	208

	Emission rating
	
	EU4  
	
	EU4  

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


Specifications applicable to ACEA markets/homologation data applicable in part only to Germany (weight),

1) Oil change



















































