[image: image1.png]


BMW
Media
Information

7/2009


Valid from
9/2009

BMW
Media
Information

7/2009


Valid from
9/2009


Specifications BMW X3.
X3 xDrive20i, X3xDrive25i, X3 xDrive30i
	
	
	BMW X3 xDrive20i
	BMW X3 xDrive25i
	BMW X3 xDrive30i

	
	
	
	
	

	Body
	
	
	
	

	No of doors/seats
	
	5 / 5
	5 / 5
	5 / 5

	Length/width/height (unladen)
	mm
	4569 / 1853 / 1674
	4569 / 1853 / 1674
	4569 / 1853 / 1674

	Wheelbase
	mm
	2795
	2795
	2795

	Track, front/rear
	mm
	1538 / 1556
	1524 / 1542
	1524 / 1542

	Turning circle
	m
	11,7
	11,7
	11,7

	Tank capacity
	app ltr
	67
	67
	67

	Cooling system incl heater
	ltr
	8.7
	10,2 (10,65)
	10,2 (10,65)

	Engine oil1)
	ltr
	4,25
	6,5
	6,5

	Weight, unladen, to DIN/EU
	kg
	1655 / 1730


	1730 / 1805

(1755 / 1830)
	1740 / 1815

(1765 / 1840)

	Max load to DIN
	kg
	545
	530 (545)
	520 (535)

	Max permissible
	kg
	2200
	2260 (2300)
	2260 (2300)

	Max axle load, front/rear
	kg
	1050 / 1235
	1100 / 1260
	1100 (1150) / 1260

	Max trailer load (12%),

braked/unbraked
	kg
	1600 / 720


	1800 / 720


	2000 / 720

	Max roofload/max towbar downl
	kg
	75/80
	75/80
	75/80

	Luggage comp capacity
	ltr
	480-1560
	480-1560
	480-1560

	Air drag
	Cd  x A
	0,35 x 2,50
	0,34 x 2,50
	0,34 x 2,50

	Power Unit
	
	
	
	

	Power Unit
	
	
	
	

	Config/No of cyls/valves 
	
	R / 4 / 4
	R / 6 / 4
	R / 6 / 4

	Engine technology
	
	Fully variable valve actuation (VALVETRONIC), 
Doppel-Vanos
	Fully variable valve actuation (VALVETRONIC), 
Doppel-Vanos
	Fully variable valve actuation 
(VALVETRONIC), 
Doppel-Vanos

	Capacity, effective
	cc
	1995
	2497
	2996

	Stroke / bore
	mm
	90 / 84
	78,8 / 82,0
	88,0 / 85,0

	Compression ratio
	:1
	10,5
	11,0
	10,7

	Fuel 
	
	min ROZ 91
	min ROZ 91
	min ROZ 91

	Max output
	kW/hp
	110 / 150
	160 / 218
	200 / 272

	at 
	rpm
	6200
	6500
	6650

	Max torque
	Nm/lb-ft
	200
	250
	315

	at
	rpm
	3750
	2750-4250
	2750

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/location
	Ah/–
	55 / Luggage comp
	70 / Luggage comp
	70 / Luggage comp

	Alternator
	A/W
	110 / 1540
	150 / 2100
	150 / 2100

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double-joint tie bar strut axle, small steering roll radius, anti-dive

	Suspension, rear
	
	Double-wishbone central pivot axle with trailing arms, anti-squat and anti-dive

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	325 x 25 / vented
	325 x 25 / vented
	325 x 25 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	320 x 22 / vented
	320 x 22 / vented
	320 x 22 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant, DSC cross-linked with xDrive, Hill Descent Control HDC

	Safety equipment
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, active headrests at the front, three-point inertia-reel seat belts on all seats, belt latch tensioners at the front, crash sensors, roll-over sensors, Tyre Defect Indicator, optional: rear sideairbags

	Steering
	
	Rack-and-pinion power steering, optional: Servotronic

	Steering transmission, overall
	:1
	18,9
	18,9
	18,9

	Tyres, front/rear
	
	215/60 R17 96H M+S
	235/55 R17 99H M+S
	235/55 R17 99H M+S

	Rims, front/rear
	
	7J x 17 LM
	8J x 17 LM
	8J x 17 LM


Specifications BMW X3.
X3 xDrive20i, X3 xDrive25i, X3 xDrive30i

	
	
	BMW X3 xDrive20i
	BMW X3 xDrive25i
	BMW X3 xDrive30i

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist including Enquiry Service, remote-control function and V-Info+, BMW TeleServices, integration of mobile terminals

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, Park Distance Control, Automatic/Enhanced Emergency Call 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Manual six-speed transmission (optional: Six-speed automatic transmission)


	Gear ratios
I
	:1
	4,350
	4,350 (4,065)
	4,350 (4,065)

	                                  
II
	:1
	2,496
	2,500 (2,371)
	2,500 (2,371)

	                                  
III
	:1
	1,665
	1,660 (1,551)
	1,660 (1,551)

	                                  
IV
	:1
	1,230
	1,230 (1,157)
	1,230 (1,157)

	                                  
V
	:1
	1,000
	1,000 (0,853)
	1,000 (0,853)

	                                   
VI
	:1
	0,851
	0,851 (0,674)
	0,851 (0,674)

	                                   
Reverse
	:1
	3,926
	3,926 (3,200)
	3,926 (3,200)

	Final drive
	:1
	4,100
	3,909 (4,444)
	3,909 (4,444)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	15,0
	10,8 (11,0)
	8,7 (8,8)

	Output per litre
	kW/hp
	55,1
	64,1
	66,8

	Acceleration       0–100 km/h
	sec
	11,5)
	8,5 (8,9)
	7,2 (7,5)

	                      Standing-start km
	sec
	32,7)
	29,5 (29,9)
	27,6 (27,9)

	In 4th / 5th gear 80-120km/h
	sec
	10,9 / 13,9
	8,5 / 10,9 (-)
	7,3 / 8,9 (-)

	Top speed
	km/h
	198
	210 / 2212 
(210 / 2202)
	210 / 2322 
(210 / 2282)

	
	
	
	
	

	Offroad characteristics
	
	
	
	

	Overhang angle
front / rear



	°
	
23,9/21,8
	
23,9/21,8
	
23,9/21,8

	Ramp breakover angle
	°
	17,2
	17,2
	17,2

	Ground cleerance
	mm
	201
	201
	201

	Fording ability
	mm
	500
	500
	500

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics standard features
	
	Intelligent lightweight technology, on-demand fuel pump, on-demand coolant pump, tyres with reduced roll resistance

	
	
	
	
	

	Fuel Consumption in EU Cycle
	
	
	
	

	Urban
	ltr/100 km
	12,6
	12,8 (13,1)
	13,4 (13,3)

	Extra-urban
	ltr/100 km
	6,9
	7,3 (7,4)
	7,3 (7,6)

	Combined
	ltr/100 km
	9,0
	9,3 (9,5)
	9,5 (9,7)

	CO2
	g/km
	215
	224 (228)
	229 (233)

	Emission rating
	
	EU4  
	EU4  
	EU4  

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


Specifications applicable to ACEA markets/homologation data applicable in part only to Germany (weight),

1) Oil change
2) With optional high speed setup




















































