[image: image1.png]


BMW
Media
Information

7/2009


Valid from
9/2009

BMW
Media
Information

7/2009


Valid from
9/2009


Specifications BMW X3.
X3 xDrive18d, X3 xDrive20d
	
	
	BMW X3 xDrive18d
	
	BMW X3 xDrive20d

	
	
	
	
	

	Body
	
	
	
	

	No of doors/seats
	
	5 / 5
	
	5 / 5

	Length/width/height (unladen)
	mm
	4569 / 1853 / 1674
	
	4569 / 1853 / 1674

	Wheelbase
	mm
	2795
	
	2795

	Track, front/rear
	mm
	1538 / 1556
	
	1538 / 1556

	Turning circle
	m
	11,7
	
	11,7

	Tank capacity
	app ltr
	67
	
	67

	Cooling system incl heater
	ltr
	9,5
	
	9,5

	Engine oil1)
	ltr
	5,2
	
	5,2

	Weight, unladen, to DIN/EU
	kg
	1740 / 1815


	
	1740 / 1815

(1750 / 1825)

	Max load to DIN
	kg
	525
	
	525 (515)

	Max permissible
	kg
	2265
	
	2265

	Max axle load, front/rear
	kg
	1080 / 1260
	
	1080 (1085) / 1260

	Max trailer load (12%),

braked/unbraked
	kg
	1800 / 720


	
	1800  

(2000) / 720



	Max roofload/max towbar downl
	kg
	75/80
	
	75/80

	Luggage comp capacity
	ltr
	480-1560
	
	480-1560

	Air drag
	Cd  x A
	0,34 x 2,50
	
	0,34 x 2,50

	Power Unit
	
	
	
	

	Power Unit
	
	
	
	

	Config/No of cyls/valves 
	
	R / 4 / 4
	
	R / 4 / 4

	Engine technology
	
	Common-rail direct injection with variable intake geometry
	
	Common-rail direct injection with variable intake geometry

	Capacity, effective
	cc
	1995
	
	1995

	Stroke / bore
	mm
	90,0 / 84,0
	
	90,0 / 84,0

	Compression ratio
	:1
	16,5
	
	16,5

	Fuel 
	
	Diesel
	
	Diesel

	Max output
	kW/hp
	105 / 143
	
	130 / 177

	at 
	rpm
	4000
	
	4000

	Max torque
	Nm/lb-ft
	350
	
	350

	at
	rpm
	1750-2750
	
	1750-3000

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/location
	Ah/–
	80 / luggage comp
	
	80 / luggage comp

	Alternator
	A/W
	180 / 2520
	
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double-joint tie bar strut axle, small steering roll radius, anti-dive

	Suspension, rear
	
	Double-wishbone central pivot axle with trailing arms, anti-squat and anti-dive

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	325 x 25 / vented
	
	325 x 25 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	320 x 22 / vented
	
	320 x 22 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant, DSC cross-linked with xDrive, Hill Descent Control HDC

	Safety equipment
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, active headrests at the front, three-point inertia-reel seat belts on all seats, belt latch tensioners at the front, crash sensors, roll-over sensors, Tyre Defect Indicator, optional: rear sideairbags

	Steering
	
	Rack-and-pinion power steering, optional: Servotronic

	Steering transmission, overall
	:1
	18,9
	
	18,9

	Tyres, front/rear
	
	215/60 R17 96H M+S
	
	215/60 R17 96H M+S

	Rims, front/rear
	
	8J x 17 LM
	
	8J x 17 LM


Specifications BMW X3.
X3 xDrive18d, X3 xDrive20d

	
	
	BMW X3 xDrive18d
	
	BMW X3 xDrive20d

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist including Enquiry Service, remote-control function and V-Info+, BMW TeleServices, integration of mobile terminals

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, Park Distance Control, Automatic/Enhanced Emergency Call 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Manual six-speed transmission (optional for BMW xDrive20d: Six-speed automatic transmission)


	Gear ratios
I
	:1
	5,140
	
	5,140 (4,171)

	                                  
II
	:1
	2,830
	
	2,830 (2,340)

	                                  
III
	:1
	1,804
	
	1,804 (1,521)

	                                  
IV
	:1
	1,257
	
	1,257 (1,143)

	                                  
V
	:1
	1,000
	
	1,000 (0,867)

	                                   
VI
	:1
	0,831
	
	0,831 (0,691)

	                                   
Reverse
	:1
	4,638
	
	4,638 (3,403)

	Final drive
	:1
	3,071
	
	3,071 (3,727)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	16,6
	
	13,4 (13,5)

	Output per litre
	kW/hp
	52,6
	
	65,2

	Acceleration       0–100 km/h
	sec
	10,3
	
	8,9 (9,2)

	                      Standing-start km
	sec
	32,2
	
	30,6 (30,7)

	In 4th / 5th gear 80-120km/h
	sec
	8,7 / 10,2
	
	7,6 / 9,2 (-) 

	Top speed
	km/h
	195
	
	206 (205)

	
	
	
	
	

	Offroad characteristics
	
	
	
	

	Overhang angle
front / rear



	°
	
23,9/21,8
	

	
23,9/21,8

	Ramp breakover angle
	°
	17,2
	
	17,2

	Ground cleerance
	mm
	201
	
	201

	Fording ability
	mm
	500
	
	500

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics standard features
	
	Intelligent lightweight technology, on-demand fuel pump, on-demand coolant pump, tyres with reduced roll resistance

	
	
	
	
	

	Fuel Consumption in EU Cycle
	
	
	
	

	Urban
	ltr/100 km
	7,9
	
	8,2 (8,3)

	Extra-urban
	ltr/100 km
	5,2
	
	5,5 (5,8)

	Combined
	ltr/100 km
	6,2
	
	6,5 (6,7)

	CO2
	g/km
	165
	
	172 (178)

	Emission rating
	
	EU5  
	
	EU5  

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


Specifications applicable to ACEA markets/homologation data applicable in part only to Germany (weight),

1) Oil change



















































