PAGE  
BMW
Media
Information

1/2004
Page 61

[image: image1.png]


BMW Motorrad 2004 model range.
Contents.

1.   
Model Update of the Single-Cylinder Model Series. 
 
F650 GS, F650 Dakar, and F650 CS. 
 4

2. 
Model Update of the K 1200 LT. 
 7

3. 
Other K-Series Models in the 2004 Model Year. 
 12
 
K 1200 RS and K 1200 GT. 
4.
Boxer Series in the 2004 Model Year. 
 17
4.1
Enduro. 
 18
4.1.1
The New R 1200 GS. 
 18
 
(See separate text in the Press Kit.) 
4.1.2
R 1150 GS Adventure. 
 18

4.2 
Roadster. 
 21
4.2.1 
R 1150 R. 
 21
4.2.2 
R 850 R. 
 23
4.2.3 
R 850 R Comfort. 
 23 
4.2.4 
R 1150 R Rockster. 
 24
4.2.5 
Roadster Model Updates in the 2004 Model Year. 
 24  

4.3 
Sports Tourer and Sportsman. 
 25
4.3.1 
R 1100 S. 
 25
4.3.2 
R 1100 S BoxerCup Replica for the 2004 Season. 
 28
4.3.3 
R 1150 RS. .
 28
4.3.4 
Sports Tourer Model Updates in the 2004 Model Year. 
 29

4.4 
Tourer. 
 30
4.4.1 
R 1150 RT. 
 30
4.4.2 
Tourer Model Updates in the 2004 Model Year. 
 31

4.5 
Cruiser. 
 32
4.5.1 
R 1200 C Classic. 
 32
4.5.2 
R 1200 C Independent. 
 37
4.5.3 
R 1200 CL. 
 38
4.5.4 
R 1200 C Montauk. 
 42
4.5.5 
Cruiser Model Updates in the 2004 Model Year. 
 43 

5. 
BMW Motorrad BoxerCup 2004. 
 44 

6.
Specifications. 
 46

7.
Colours in the 2004 Model Year. 
 61

1. Model Update of the Single-Cylinder Model Series.

A major innovation in the 2004 model year is the introduction of dual 
ignition on all single-cylinder F 650 models in accordance with future emission standards and regulations. From outside the dual-ignition engines 
are distinguished by the designation “2 SPARK” on the side engine covers.

Entering their fourth model year, the extremely successful F 650 GS and 
F 650 GS Dakar enduros have been updated on a number of technical points and features. The objective in this process was to optimise rider protection from wind and weather and to improve ergonomics for even better handling and control of these highly popular all-round motorcycles. 

Specific improvements on the fairing, the windshield and instruments, as well as at the rear, give the new models an even more striking and powerful appearance, even though their basic design remains unchanged. New colours, in turn, add to this special appeal, as do a series of new features and innovations in the wide range of optional extras and special equipment.

The F 650 CS also comes with dual ignition, the engines of all models 
being virtually identical with the exception of the intake and exhaust system 
as well as various detailed modifications.

F 650 GS and F 650 GS Dakar.

Engine.

Introduction of dual ignition comes together with a new system of engine management: BMS-C II (BMW Motor System) offering a considerable enhancement in capacity as well as software with a wider range of functions. The results of this improved technology are a significant decrease in 
exhaust emissions (over-fulfilment of the Euro 2 emission standard), a further improvement of riding comfort and engine response, as well as a reduction of fuel consumption to an even lower level than before. Indeed, the improvement of fuel economy over the former model already acknowledged for its 
very low fuel consumption is between 6 per cent (with fuel consumption now reduced to 3.2 litres/100 km (88.3 mpg Imp) at a steady 90 km/h) and 
14 per cent (now 4.3 litres/100 km (65.7 mpg Imp) at a steady 120 km/h). 
Peak output and torque remain the same, but with maximum torque 
now coming as a further advantage at a somewhat lower engine speed. 

The second spark plug for dual ignition is on the right side of the combustion chamber, igniting at exactly the same time as the main plug in the 
middle. Like on the flat-twin power units, this ensures smooth and stable flame formation even under unfavourable operating conditions.

The mufflers are modified in their internal structure and the catalytic converter has been further improved to reduce emissions to an exemplary low level.

Design and body.

Refinement of the front end in its design with the front mudguard now resembling that on the “big GS”, the new air vent panel and logo plate, 
the updated windshields and the larger headlight gives both models in the 
F 650 GS range an even more distinctive, impressive look. Indeed, 
the change in design creates an even closer and more direct relationship within the “GS family”. A small but interesting point is that the air vent 
panel and logo plate generates underpressure with the wind rushing by and thus improves the flow of warm air away from the oil tank. 

The new, improved windshields enhance streamlining on both models, especially around the rider’s helmet, and provide better protection from 
wind and weather. When riding offroad, the rider may of course remove the windshields completely.

The windshield on the F 650 Dakar, as before, is larger and darker in 
colour and is now also available on the GS as a transparent windshield 
offered as special equipment.

The luggage rack is all-new in its design and structure, now allowing the rider to fit the topcase without an interim plate in between. Openings in the luggage rack serve to fasten lashing belts, the storage compartment flap is stronger and more stable than before, and rubber covers protect the rider from any kind of undue or unpleasant rubbing effect. The well-known variable cases fit perfectly also on the new models, whereas the former topcase no longer fits due to the modified connection points.

Headlight new, instrument design and electrical system modified.

The new headlight has a larger lens at the front providing even more homogeneous illumination of the road ahead. Illumination to the side has also been improved, the free-form reflector, in conjunction with the clear glass cover, providing a very modern, technical flair.

The sharper profile of the instrument cluster and the instrument faces 
now coloured grey have a more technical appearance and the numbers are even easier to read.

As an option hazard warning flashers are now also available on the F 650 GS and GS Dakar, with the switch integrated in the impact plate where the 
switch for the heated handles was accommodated before (the latter has now been moved to the right-hand control unit). A further feature now featured 
as standard is the on-board power socket allowing the rider, among other things, to charge the battery without having to remove it from the machine itself.

The new jump start point (with the plus pole beneath the seat and the 
minus pole on the gearbox cover) enables the rider to jump-start his machine, again without having to remove any major components.

Improved ergonomics on the direction indicator switch 

and clutch handle.

A critical point above all for female riders has now been eliminated on the updated model: the clutch handle with its relatively large opening. 
Operation of the clutch has therefore been modified, the narrowest position on the adjustable lever ensuring a good and safe grip for optimum 
control even for the rider with small hands (down to a glove size of just 7.5).

New colour scheme.

Three new colours – Iceberg Silver metallic, Yellow non-metallic, and Deep Black non-metallic – may be combined at random with two seat colours 
(Black and Terracotta Orange), providing very personal, individual flair ranging from sophisticated and harmonious all the way to dynamic and sporting. 
The F 650 GS Dakar comes exclusively in Blue metallic/White Aluminium matte metallic. This new twin-tone design is a modern interpretation of 
the former model’s classic Dakar look and sets this special version clearly apart from its more road-oriented F 650 GS “sister”.

F 650 CS.

With the exception of the new dual-ignition power unit (see F 650 GS), 
the F 650 CS introduced in early 2002 is entering the new model year virtually unchanged: The Scarver now also features the adjustable and ergonomic clutch handle carried over from the F 650 GS as well as the modified direction indicator switch and a power socket fitted as standard. The windshield support, luggage railing and luggage rack on the 2004 model year, in turn, are no longer translucent, but are rather made of black-tinted plastic.

A topcase is now also available on the Scarver, in compliance with numerous requests in the market. The colours, finally, are Night Blue metallic, 
Genet Yellow metallic, Titanium Silver metallic, and Stone Grey non-metallic.

2. Model Update of the K 1200 LT.

The K 1200 LT Luxury Tourer has been further enhanced in its engine and running gear for the 2004 model year, with a number of improvements 
and new, interesting features. The absolute technical highlight, of course, 
is the electrohydraulic main stand. Together with the enhanced looks 
and new colours, this outstanding machine continues to meet the greatest demands in terms of superior handling and riding behaviour, maximum 
long-distance comfort with supreme comfort and riding qualities, as well 
as a prestigious, impressive appearance.

Engine now developing 85 kW (116 bhp) and featuring high-rise gears for low running noise.

Thorough revision of the engine gives the customer even greater superiority and extra style on the road: Modification of the cylinder charge cycle with improvements on the camshaft, the throttle butterflies (increased in diameter to 36 mm/1.42´´), intake funnels and intake manifold helps to boost output 
by 15 per cent to 85 kW or 116 bhp, with maximum torque increasing by 
5 Nm to 120 Nm or 88 lb-ft.

With high-rise teeth on all gears, the improved five-speed gearbox now 
runs even more smoothly and comfortably, noise being reduced to a minimum and gear mesh improved to an optimum. 

Extra stability thanks to larger castor, enhanced comfort provided 
by the TDD spring strut.

Extended by 15 mm or 0.59´´, the longer castor gives the K 1200 LT even greater stability and smoothness on the road, completely eliminating the slight unrest on the front wheel sometimes criticised when riding at low speeds. 
A new spring strut with travel-dependent damping (TDD spring strut) now provides enhanced suspension on the rear wheel, introducing a concept which has already proven its merits on the BMW Cruiser models and the 
R 1150 GS Adventure: With the damping effect increasing as a function of spring action, the spring strut ensures superior comfort on the road but 
still offers substantial reserves preventing any undue breakthrough of forces when carrying a heavy load or riding over large bumps.

New seat for supreme comfort, better reach for the ground, 
and easier handling.  

The seat on the K 1200 LT is also a new development: Consistent re-design 
of the seat shell and seat geometry integrates the rider even better into 
the machine and allows the somewhat shorter rider to reach the ground more easily and comfortably. The design and configuration of the rider’s seat 
is indeed based on a new criterion referred to as “step length”: Covering the entire length between the two rest positions of the rider’s feet (measured 
as a function of inner leg length), this new standard considers not only the absolute, geometrical height of the seat, but also the shape and width of 
the rider’s seat at the front end relevant in this case. As a result, even relatively small riders just 1.69 metres or 5´6´´ tall acknowledge the very good qualities of the new seat giving even the not-so-tall rider a stable position with his feet on the ground and ensuring even smoother and easier handling than before.

Electrohydraulic main stand with automatic push-up effect.

The electrohydraulic main stand developed by BMW Motorrad is an absolute highlight, a brand-new feature offering unprecedented comfort as well as unique safety on a heavy motorcycle of this calibre: Even when fully laden and with a passenger at the rear, the rider is able to automatically raise up 
the motorcycle simply by pressing a button on the right handle, without even getting off his seat.

To provide this function an electric pump builds up pressure in a hydraulic system, in the process reliably moving out the main stand without any further manual intervention being required.  

Design and body – two-piece headlight, 
chrome package featured as standard. 

The sophisticated elegance and refinement of the K 1200 LT also comes out in the machine’s revised, updated design: Retaining most body components as well as its overall line, the K 1200 LT is now even more consistent, modern and refined in its looks than before thanks to just a few striking modifications as well as detailed improvements. And like on the former model, the surface finish is clear and consistent. 

From the front the special look of the modified K 1200 LT is borne out 
in particular by the new design of the headlight now split into two separate sections: Below the new free-form headlight for the low beam there is now 
a separate high-beam headlight unit harmoniously integrated into the fairing. Also featuring free-form technology, this high-beam headlight comes with 
a clear glass lens, just like its low-beam counterpart. As a result, light intensity is about 10 per cent better than on the previous model.  

Yet another new feature is the front mudguard. Larger and extending further down than before, the new mudguard takes up and continues the lines of both the side fairing and the engine spoiler.

The special design of the K 1200 LT is further enhanced and upgraded by the twin-tone concept, technical areas and components being finished in magnesium-coloured metallic paintwork to give the motorcycle a truly elegant and slender look. The various colours of the metallic paintwork harmonise 
very well with the features now coming as standard, new components being chrome-plated over and above the chrome units already boasted on the 
K 1200 LT in the past.

The following components, therefore, are now finished in high-gloss chrome:   

•
Footrests for the rider and passenger,

•
Footrest supports for the rider and passenger, 

•
Bumper strip surrounds front and rear,

•
Mirror housings,

•
Cover flaps on the windshield support,

•
Grab rail on the topcase,

•
Air vent and logo plate at the side.

The new colour range comprises Light Yellow metallic providing a particularly sporting and modern look, classic and elegant Ocean Blue metallic, as well as Dark Graphite metallic emphasizing the motorcycle’s luxurious character. 
The seat, finally, comes either in Black or “Greige”, an elegant blend of Grey and Beige. 

New instrument cluster with a flat-screen info unit. 

The instrument cluster has also been thoroughly updated and enhanced 
in its looks: 

A particular highlight of the newly designed cockpit is the new flat-screen 
info unit with its wide range of information for the rider ensuring a significantly higher standard of clarity and superior looks. The high-resolution display presents the following important data:

•
Mileage (overall) as well as two trip counters (Tripmaster),

•
Coolant temperature, air temperature,

•
Gear currently in mesh,

•
All functions on the on-board computer,

•
Radio information,

•
Tank level,

•
Time.

Optional extras and special equipment.

Despite the even wider range of standard features now offered on the 
K 1200 LT, the discerning rider still has the choice of further features available either as an option at extra cost and fitted on the production line 
(optional extras) or fitted subsequently by the dealer (special equipment).

Optional extras.

•
Anti-theft warning system*,

•
Central locking*,

•
Ground illumination*,
(*Available only in combination with one another,)

•
Railing and brake light strip on the topcase,

•
Seat heating incl backrest,

•
Windshield, high-rising,

•
CD changer,

•
Double power socket,

•
Height-adjustable, chrome-plated sideboards for the passenger,  

•
Preparation for BMW Motorcycle Navigator II.

Central locking makes the process of locking and securing the motorcycle much easier and more convenient: Simply by pressing a button, 
the rider is able to lock the topcase, the cases at the side of his machine, and the storage compartment, while intentionally leaving the tank lock open.  

Central locking is available only in conjunction with the anti-theft warning system and ground illumination activated when the rider deactivates 
the anti-theft warning system and, as an option, when switching off the ignition to illuminate the ground around the motorcycle when parked, 
thus facilitating the process of getting on to and off the machine.

The navigator support fitted at the factory is positioned above the radio control unit as a firmly integrated component. This reduces the cost of the navigator support versus its subsequent installation, the kit coming complete with 
the connection cable for the BMW Motorcycle Navigator II, a new generation of the BMW Motorcycle Navigator combining easy control with precise destination guidance. The rider is thus able to plan and calculate his routes conveniently on the PC and then call up the information he requires on 
the way.

Special equipment.

•
Inner bags for cases and topcase.

•
Railing and brake light strip on the topcase.

•
Tank bag with map holder.

•
High-rise windshield.

•
Rider/passenger backrest heated.

•
Rider/passenger backrest heated and with soft touch.

•
Height-adjustable sideboards for the passenger, chrome-plated.

•
Cupholder.

•
Anti-theft warning system with remote control.

•
Double power socket.

•
BC-V.O.I.C.E. communication system.

•
Connection cable for BMW System Helmet.

•
Connection kit for mobile phone/Nokia mobile phone.

•
Adapter cable for 433 MHz intercom systems.

•
CB radio aerial.

•
PTT communication button.

•
BMW Motorrad Navigator II.

•
Connection cable for BMW Motorrad Navigator.

3. Other K-Series Models 
in the 2004 Model Year. 
K 1200 RS and K 1200 GT.

BMW Motorrad’s K-Series stands for a water-cooled inline engine fitted lengthwise and with its cylinders in flat, horizontal arrangement. Indeed, this very special four-cylinder power unit has been the particular sign of 
distinction of the K-Series for no less than 20 years, combining superior power and performance with equally superior reliability and a long running life.

Equipped with the most advanced Digital Motor Electronics ensuring 
a high standard of environmental care in conjunction with the fully controlled 
three-way catalytic converter, this engine boasts the most advanced 
and sophisticated features. The K-Series motorcycles with their four-cylinder power unit are indeed one of the cornerstones within BMW Motorrad’s 
model range and will hold this significant position also in future. 

The K-Series comprises three distinctive models: The K 1200 RS Sports Tourer, the K 1200 GT Long-Distance Tourer, and the K 1200 LT Luxury Tourer, the latter being thoroughly updated for the 2004 model year, as already described in Section 2 of this Press Kit. 

K 1200 RS – the Sports Tourer.

Four years after the original market launch in 1997, BMW introduced 
the updated K 1200 RS in spring 2001. A new upper section on the fairing, 
a somewhat higher comfort handlebar, as well as lower footrests offered 
many customers the extra touring qualities they were looking for, without detracting from the machine’s overall sporting character. Another example is the windscreen now adjusting to two different positions with just one little 
turn of your hand. Naturally, this feature, too, has been optimised in the wind tunnel, just like the fairing itself. 

The refinement achieved in the process is borne out in particular by the improved protection of the rider’s body, hands and lower arms from the wind rushing by. Supreme dynamics and sporting performance combined 
with the right kind of weather protection for long tours are therefore the characteristic features of this outstanding tourer.

Power unit of the K 1200 RS available as an option also with 98 bhp.

Apart from the 96 kW/130 bhp power unit, another version developing 
72 kW/98 bhp is also available as an option. This latter engine is of particular interest in Germany due to the insurance classes in the market, 
with the engine developing its maximum output at a relatively low 7,000 rpm.

The power curves of the two engines rise up steeply and consistently all the way to 6,000 rpm. Then, proceeding from that point, the curve on the 
96 kW/130 bhp engine keeps on going up steadily to almost 9,000 rpm, giving the K 1200 RS mighty thrust in the process. Engine speed is not cut off electronically until 9,400 rpm.

Maximum torque.

The two engines develop maximum torque of 117 Nm (86 lb-ft) or, respect​ively, 118 Nm (87 lb-ft). The torque curve is very “beefy” on both power units, with 100 Nm or 74 lb-ft available in both cases from just 3,500 rpm.

On the 72 kW/98 bhp engine the torque curve rises up even faster from 
3,500 rpm, reaching its maximum level at just 5,500 rpm and thus offering supreme power and pulling force at moderate engine speeds.

The even more sporting and dynamic 96 kW/130 bhp version reaches not only its maximum output at higher engine speed, but also its maximum torque, achieving 117 Nm or 86 lb-ft at 6,750 rpm.

In technical terms the 130 bhp version differs from its 98 bhp counterpart 
by shorter intake manifolds with a larger diameter and a modified electronic engine control map.

Cast aluminium frame serving as a stiff backbone.
Engine resting in individual mounts to avoid the transmission 
of vibrations.

A compact cast aluminium frame resting snugly around the engine now held 
in position by separate mounts to prevent even the slightest transmission 
of vibrations forms the flexurally and torsionally stiff backbone connecting the Telelever and the Paralever. At the rear the frame accommodates the so-called silent blocks – extra-large rubber elements on the engine mounts – the rear subframe made of steel tubes, and the Paralever swinging arm. At the front the Telelever longitudinal arm is able to rotate within a floating pivot, the 
silent blocks for the front engine mounts rest on steel bars in the frame head.

Rear spring strut hydraulically adjustable.

The rear spring strut on the K 1200 RS is infinitely adjustable in its 
spring pre-tension, using a hand-wheel for the hydraulic adjustment process. Damping on the outward stroke may also be adjusted as required.

BMW Motorcycle Integral ABS fitted as standard. 

Featuring BMW’s EVO brake on the front wheel, the brake system on the 
K 1200 RS meets the highest standard and reflects the latest state of the 
art in braking technology. BMW’s unique Motorcycle Integral ABS is standard, coming in this case in its partly integral version reflecting the sporting character of this particular model. “Partly integral” means that the handbrake lever operates the front and rear wheel brakes at the same time, while the footbrake acts only on the rear wheel brake.

Light and attractive: wheels in five double-spoke design.

To match the overall looks and appearance of the K 1200 RS, the light 
17-inch cast aluminium wheels come in a highly attractive, almost delicate 
five double-spoke design.

Automatic cruise control available as an option.

The wide range of optional extras and special equipment extends from sports spring struts, white direction indicators, and case supports complete with 
a luggage rack, all the way to heated handlebars, an anti-theft warning system, a tankbag, and inner cases for the side-bags. And like the K 1200 LT and 
K 1200 GT, the K 1200 RS is available as an option with automatic cruise control setting the speed automatically to the desired level.

K 1200 GT – the Long-Distance Tourer. 

Almost from the very beginning a touring machine featuring a full fairing 
was a firm member of the K-Series, making its debut in 1984 as the K 100 RT. Its successor, the K 1100 LT, then set the standard for supreme motorcycle touring over many years through its wide range of standard equipment and supreme comfort. 

Introduction of the K 1200 LT in spring 1999 hailed a re-interpretation of the tourer philosophy: Consistently offering a supreme standard of luxury and comfort combined with a very wide range of features, this new model entered new dimensions at the time, the K 1200 LT quite appropriately being called 
the “7 Series” in the tourer segment, a motorcycle setting the standard not only in comfort and luxury, but also in terms of agility and handling in its class.

BMW Motorrad also offers the right alternative for the discerning rider not that interested in sheer luxury, but not wishing to forego the good wind and weather protection offered by a tourer as well as a comfortable, upright seating position: the K 1200 GT. 

Derived from the K 1200 RS, this is a special variant of the successful Sports Tourer placing the emphasis on the touring experience – it is a motorcycle 
for the rider wishing to combine maximum agility and dynamism with superior 

qualities for long-distance touring. Clearly, the “GT” model designation 
stands for Gran Tourismo as the synonym for refined sportiness combined with supreme touring comfort for the rider and passenger. 

Entering the market in winter 2002, the K 1200 GT rounded off the current 
K-Series range as the third model next to the K 1200 RS Sports Tourer and the K 1200 LT Luxury Tourer.

Technical highlights of the K 1200 RS: 
Drivetrain and running gear carried over from the other models.

The K 1200 RS is based on the highly advanced and proven technology of the K 1200 RS. The drivetrain has been carried over with all its features, with 
the K 1200 RS boasting the ample power of the 1,200 cc engine developing 96 kW or 130 bhp. Again, the 72 kW (98 bhp) version of this outstanding power unit is available as an option in a number of countries including Germany. 

To meet emission legislation, the oxygen sensor has been further developed and refined in the meantime, the catalyst coating and engine management 
on the four-cylinder being modified in the process in order to reduce emissions to an even lower level than before.

The suspension and running gear also comes from the K 1200 RS, without requiring any modifications: BMW’s unique Telelever on the front wheel and the BMW Paralever on the rear wheel powered by a driveshaft ensure excellent riding comfort both for a more sporting and a more luxurious and laid-back style of motorcycling. Spring travel is the same as on the K 1200 RS and the rear wheel is a 5.5-inch rim running on a wide 180 tyre. 

Body: modified fairing now with an electrically adjustable windshield. 

New features are also to be found on the fairing carried over in its basic elements from the K 1200 RS: In the interest of even better protection from wind and weather, the fairing features a wider profile at the bottom guiding 
the flow of air even more effectively around the rider’s legs. This profile piece comes in Black on all models, regardless of the paintwork colour chosen. 

Two striking features are the hand protectors finished in body colour and the new, higher windshield adjustable electrically for its optimum position. Introducing this windshield with infinite electrical adjustment, BMW Motorrad is meeting requests in the market for precise, individual adjustment 
of the windshield to riders of all sizes and with many different needs. 

Apart from functional benefits, these modifications also serve to give the 
K 1200 RS its own distinctive look.

Comfortable seats and a higher handlebar for riding in relaxed style. 

The comfort handlebar has been restyled to give the rider a really relaxed, touring-oriented seating position. Somewhat higher and modified in its curvature versus the handlebar on the K 1200 RS, the GT handlebar reduces forces acting on the rider’s wrists and thus allows a more upright seating position. The new comfort seat, in turn, comes with the same height adjustment as on the K 1200 RS, offering a seat height of either 790 or 
820 mm (31.1 or 32.3´´).

BMW Motorcycle Integral ABS fitted as standard in its partly 
integral version.

Integral ABS is standard on the K 1200 GT, in this case in its partly integral version meaning that pulling the handbrake lever the rider will apply the brakes on both the front and rear wheel. The foot-operated brake, in turn, acts 
as usual only on the rear wheel. Otherwise, the brake system has been carried over from the K 1200 RS, the front wheel thus featuring the BMW EVO 
brake offering excellent brake force and stopping power. 

Fitted as standard with a luggage rack plus cases with covers finished in the same colour as the motorcycle. 

The luggage rack and cases complete with holders come as standard on the K 1200 GT. The case covers are finished in the same colour as the motorcycle itself, creating an even more sophisticated and refined look 

Two colours and white direction indicator lenses.

The K 1200 GT comes with a choice of two colours: Orient Blue metallic 
and Grey Green metallic, in each case with a black seat. White direction indicator lenses front and rear add a touch of elegance to these attractive colours.

Optional extras and special equipment.

Heated handles in combination with seat heating for the rider and passenger are available as an option on the K 1200 GT. A further option is cruise control carried over from the K 1200 RS.

A low seat also with seat heating and adjusting to a low level of either 770 or 800 mm (30.3 and 31.5´´) is available as a no-cost option. And as already mentioned, the K 1200 GT comes in some countries – among them Germany – with BMW’s down-tuned 72 kW (98 bhp) power unit. 

Special equipment is supplied by the BMW motorcycle dealer, who naturally fits such equipment according to the rider’s requirements. And the range of special equipment is very large, comprising virtually all the items also available on the K 1200 RS inasfar as they are not fitted as standard on the GT, like the cases.

4. Boxer Series in the 2004 Model Year. 

Split up into the Enduro, Roadster, Sports Tourer/Sportsman as well as the Tourer and Cruiser segments, the BMW 2004 Boxer Series is made up of 
13 models in addition to the R 1200 GS. Widely acknowledged for its superior smoothness and high torque, the flat-twin power unit typical of BMW develops superior muscle from low speeds and has been a huge success for more 
than 80 years. Indeed, approximately 60–70 per cent of all BMW motorcycles come off the production line at the Berlin Plant with a flat-twin engine.

Dual ignition was introduced for the first time on most Boxer models in November 2002, and now all one- and two-cylinder BMW motorcycles with the exception of the R 850 R Comfort and the R 850 R are equipped with 
this technology (four-cylinder engines with their smaller cylinders do not need dual ignition). 

Instead of one spark plug, the fuel/air mixture in the combustion chamber 
is ignited in this case by two plugs, the further improvement of emission 
quality provided in this way already being applied to prepare the engines, first, for EU 2 emission legislation and then, second, for EU 3. 

The most important effects and advantages of dual ignition are higher stability and consistent, smooth combustion under all operating conditions even 
when things are not going that well. As a kind of side effect engine efficiency has been enhanced once again, allowing lower fuel consumption in the process. From outside, dual ignition is incidentally signified by the “2 SPARK” logo on the cover for the spark plug shafts.

Entering the 2004 model year, all BMW Boxers are furthermore being fitted with a high-gear six-speed transmission which, through the modified 
geometry of the gears, reduces running noise and improves the shift response.


4.1 Enduro.

4.1.1 The New R 1200 GS.

(See separate text in the Press Kit.) 

4.1.2 R 1150 GS Adventure: 

The globetrotter bike making your dreams come true.

Launching the R 80 G/S back in 1980, BMW created a brand-new market segment at the time – the large touring Enduro segment. So whether riders were out on a thrilling tour of the world, whether they were bringing home victory in the Paris-Dakar Rally, or whether they were just one of the huge number of aficionados waxing lyrical about the R 80 G/S, this flat-twin 
with the G/S logo quickly became one of the biggest success stories ever in  motorcycle history.

In spring 2002 BMW Motorrad introduced the R 1150 GS Adventure for the genuine enthusiast dreaming of the most distant and remote parts of the 
world – the ideal globetrotter machine for long distances and real adventures.

Even after introduction of the new R 1200 GS, the R1150 GS Adventure based on the technical features of the former model remains within the range. And indeed, the R 1200 GS, the R 1150 GS Adventure is able, through 
its modified range of standard equipment and a special range of optional extras and special features, to satisfy the greatest demands in offroad 
riding, as you will indeed see clearly at very first sight.

Optimised spring travel and running gear.

Whether you are out on gravel, riding on sand, making your way through rough terrain, or covering long distances, the running gear and suspension of the 
R 1150 GS Adventure is simply ideal for all these purposes. Precisely this is why spring travel is 210 mm/8.27´´ up front and 220 mm/8.66´´ at the rear.  

The rear wheel runs on a spring strut with travel-dependent damping, meaning that the damping effect increases with growing spring 
action. In particular, the spring strut excels through its sensitive response 
and superior resistance to high loads and the most severe bumps. 

The spring base is adjusted at the front by means of a hook key and at the 
rear by means of a wheel acting hydraulically on the spring mechanism. 
A small bolt turned by a screwdriver at the rear serves to vary outward-stroke damping as required. 

EVO brake and steel-clad brake hoses.

At the front the R 1150 GS Adventure comes with the brake callipers of the EVO brake introduced in spring 2001 and combining improved stopping power with enhanced brake comfort. Like on the R 1150 GS, diameter of both brake discs on the front wheel is 305 mm or 12.0´´. The brake hoses, finally, are steel-clad for extra protection.

Engine unchanged and able to run on regular fuel if necessary.

The drivetrain of the R 1150 GS Adventure was carried over from the 
R 1150 GS, that is the predecessor to the R 1200 GS. This also means that the Adventure model is able to meet an important need when travelling to distant countries, being able to run on regular grade fuel whenever necessary, thanks to the coded plug for the engine management available as special equipment. 

Sixth gear with a short transmission ratio – 
and a shorter first gear available as an option.

The sixth gear on the transmission comes with a short ratio, not with the 
long overdrive ratio to be found so often. And as an option the rider is also able to choose a short first gear for setting off more smoothly on rough terrain.

Seat and windshield built for long tours. 

The rider is able to vary his seating position on the solo seat as required, 
for example moving closer up to the tank when concentrating on rough terrain or moving further to the rear for relaxed riding on long tours. The seat is indeed long enough to accommodate two persons with appropriate comfort and 
the luggage rack offers additional lashing options through its special design and layout.

To improve protection from wind and weather, the windshield is 12 cm 
or 4.72´´ wider at the bottom and 5 cm or 1.97´´ wider at the top than on the 
R 1150 GS, just as it has been extended in length by 13 cm or 5.12´´. 
The front mudguard, in turn, is both longer and wider, and the R 1150 GS Adventure comes as standard with a handlebar impact protector as well 
as hand protectors. The extra-sturdy aluminium underfloor protector fitted beneath the engine, in turn, is even stronger and thicker, offering reliable protection from possible damage caused by stone-throw or hard bumps on rocky terrain. 

An additional 12 V socket fitted in the cockpit allows easy and convenient connection of accessories such as GPS, a roadbook, or a reading lamp.

Unique looks.

A small number of highly effective design elements gives the motorcycle 
a very unique and distinctive look. The cylinder head bolts, for example, 
are painted blue, the aluminium wheel rims come in an eloxy-blue finish.

The R 1150 GS Adventure is available in two different paintwork schemes: White Aluminium metallic is combined with innovative matte clear paint to provide a particular touch and comes with a twin-tone seat in Grey and Red. The other option is Night Black in combination with a twin-tone seat in Mandarin and Black.

Large tank and aluminium luggage system.

A 30-litre fuel tank is available as an option instead of the 22-litre tank fitted 
as standard. A further special point on the R 1150 GS Adventure is the new, robust aluminium luggage system consisting of multi-functional carriers as well as two side-cases and a topcase with its own support. Together, the side-cases offer a capacity of 75 litres (39 litres on the right, 36 litres on the left), and the topcase has a capacity of 30 litres. The covers on the side-cases are flush with the rear section of the seat and thus form an ideal area for keeping large items of luggage.

The special multifunctional supports on the aluminium cases come complete with a folding mechanism, allowing the rider to fold out the supports whenever they are not carrying a case, in this way forming an additional loading area able to accommodate all kinds of luggage lashed down as required. The aluminium cases themselves come with hard-wearing, watertight inner bags adjustable 
in their capacity by means of a roller/pin closing system. Further options are the large cylinder protection hoop, a protection grid for the headlight, foglamps with a protection grid, as well as rough-tread tyres for enhanced offroad 
driving qualities.

BMW Motorcycle Integral ABS is available as an optional extra on the 
R 1150 GS Adventure, in this case as the partly integral version with on-demand control allowing the rider to switch off the system when riding offroad. 

Model Updates in the 2004 Model Year.

Entering the 2004 model year, the R 1150 GS Adventure will be delivered 
to customers complete with a flexible steel clutch hose and comes 
with a connection point for jump-starting at the side on the starter support. 

4.2 Roadster.

The Roadster is the classic “back-to-the-roots” model added to BMW’s motorcycle model range back in autumn 1994. One of the special features of such a machine also referred to as a “naked bike”, since it does not come with any kind of fairing, is the full, unrestricted view of all the technical components. And while the Roadster guarantees sheer riding pleasure through its agile running gear, the genuine connoisseur will also choose this kind of machine with only moderate protection from wind and weather, thus braving the elements himself.

4.2.1 The R 1150 R – 
the ideal motorcycle for a pure riding experience.

Replacing the R 1100 R in spring 2001, the R1150 R roadster offers even more power and even better handling. The six-speed gearbox with high-
rise teeth, in turn, featured since 2003, improves the gearshift and reduces the noise level. The appealing design of the machine, finally, emphasizes the dynamic and agile character of the R 1150 R, while the EVO brake on the front wheel ensures superior stopping power reflecting the extra muscle 
and torque of the engine. As an option the R 1150 R is also available with BMW Motorcycle Integral ABS in its partly integrated version. 

The R 1150 R Roadster is a sporting motorcycle without making any compromises in comfort and touring qualities. And it is an outstanding blend of high-tech with truly unique, unmistakable design.

In a nutshell, therefore, the R 1150 R is the ideal motorcycle for a pure riding experience...

Lots of power and torque for extra thrust and muscle.

The R 1150 R develops maximum output of 62.5 kW (85 bhp) at 6,750 rpm and maximum torque of 98 Nm or 72 lb-ft at 5,250 rpm. A particularly remarkable point is the “beefy” torque curve throughout the entire engine speed range, above all between 3000 and 5,000 rpm. Indeed, the 
rider consistently enjoys more than 90 Nm or 66 lb-ft between 3,000 and 
6,500 rpm, benefitting from a lot more thrust and significantly improved 
pulling force in this major speed range.

Oil cooler in the side tank fairing.

The cooling system is naturally geared to the superior power and torque of the engine, with two oil coolers fitted in two aerodynamically designed air ducts within the side fairing of the fuel tank. This integrates the coolers perfectly into the overall design of the motorcycle, the air ducts being designed to guide 
the flow of warm air coming out of the oil coolers past the rider.

Transmission with a sporting sixth gear.

Power is transmitted via a hydraulically operated clutch controlled by a hand lever adjustable to four different settings. The R 1150 R benefits in this process from the six-speed transmission with high-rise gear teeth significantly improving gearshift comfort and precision over the former gearbox. Like on 
the R 1100 S, sixth gear on the R 1150 R is “shorter” and therefore more dynamic in character. An overdrive-type sixth gear for comfortable touring in style is naturally also available as an optional extra.

Longitudinal arm and spring strut.

The longitudinal BMW Telelever arm in tubular steel design is not only highly efficient, but also contributes to the attractive looks of the motorcycle 
through its very delicate but sturdy structure. The front spring strut with its single-sleeve damper adjusts infinitely on the outward stroke by means of 
a setting bolt in the longitudinal arm support, thus adapting individually to the rider’s style and the load being carried. Spring travel is 120 mm or 4.7´´.

Same wheels as the R 1100 S.

Like the R 1100 S, the R 1150 R comes on light and delicately designed 
cast aluminium wheels with five double spokes measuring 3.50 x 17 at the front and running on a 120/70 ZR 17 front tyre. Reflecting the sporting 
trend towards wider wheels, the rear wheel measures 5.00 x 17 and comes 
on a 170/60 ZR 17 tyre. 

Design bearing out the qualities of the R 1150 R.

The design of the R 1150 R bears out the excellent qualities of the engine 
and suspension at very first sight: The very sporting lines with a very powerful 
but by no means “heavy” look give the Roadster a very emotional, characteristic expression. Continuing consistently from front to rear, the lines of the machine start on the front mudguard and continue through the oil 
cooler fairing integrated perfectly as a unique design element all the way to the tank with its muscular appearance. From there the lines proceed along the attractively styled two-piece seat with the slender side panel rising up on the left side of the machine almost parallel to the rear muffler and ending in 
the rear tailpiece almost “hovering” above the rear wheel. The five-double-spoke aluminium wheels and the Telelever longitudinal arm also serve, 
through their delicate look, to give the enthusiast a genuine feeling of sheer riding pleasure right from the start when he – or she – sees the R 1150 R 
the first time. 

The overall ergonomics of the R 1150 R not only ensure a sporting, concentrated and, at the same time, comfortable and relaxed riding position, but also make a decisive contribution to the machine’s excellent riding characteristics. Important factors in providing these excellent ergonomics are the rider’s and passenger’s leaning-forward seating position, although 
the main criterion is the steel tube handlebar angled further down at the ends. 

Weight of the R 1150 R in road trim is 238 kg or 525 lb. With maximum permissible weight of 450 kg or 992 lb, the rider is able to take along 
a generous 212 kg or 467 lb.

4.2.2 R 850 R – the little sister.

The R 850 R remains an important member of the BMW motorcycle range 
in 2004. Identical in technical terms to the R 1150 R, the R 850 R comes with an 850 cc power unit developing maximum output of 52 kW or 70 bhp. 
An additional version with maximum output of 25 kW or 34 bhp is also available.

With the exception of the overdrive transmission ratio, all optional extras available on the larger R 1150 R are also available on this “smaller” model. 
And like all Boxer motorcycles with a six-speed gearbox, the R 850 R 
also comes with the updated version of this outstanding transmission.

4.2.3 R 850 R Comfort – the special version.  

Based on the R 850 R in the 2002 model year, a special version of the 
R 850 R is being introduced in the 2004 model year, taking up some of the features of the classic 2002 model with its unique design. Introducing this special model, BMW Motorrad is responding to customer requests particularly in the Italian market. 

Both the engine and the gearbox come straight from the 2004 version of the 
R 850 R, while the oil cooler as well as the oil cooler pipes are carried over from the former model in the 2002 model year. The same applies, incidentally, to the tank and seat.

The front and rear spring struts on the R 850 R Comfort come from the 
R 1150 R Rockster, the high-rise handlebar is the same as on the 
special version for the police and other authorities. And over and above the 
“regular” features of the basic model, the R 850 R Comfort comes as 
standard with white direction indicators.

The R 850 R Comfort is available exclusively in Night Black with a Black seat.

4.2.4 The extroverted BMW Roadster: 
the R 1150 R Rockster. 

An extra-sporting and dynamic version of the R 1150 R, the 
R 1150 R Rockster, has been a member of the family since March 2003, ideally supplementing the BMW Motorrad model range in the naked 
bike segment. 

With its sporting and even “aggressive” design somewhat reminiscent of 
a genuine “streetfighter”, this machine is aimed particularly at young, extroverted riders. The paintwork is in two colours with a strong contrast between the black soft paint and the lighter areas, and with the wheels 
also painted as an additional feature. Apart from Citrus metallic/Black matte, there is also a second colour variant in Orange metallic/Black matte.

The drivetrain and running gear come straight from the BMW Boxer Series, major components such as the engine, transmission, frame, swinging arm with rear wheel drive and electrics being carried over from the R 1150 R, while 
the headlights come from the R 1150 GS. Some suspension and running gear components have been taken over in slightly modified form from the 
R 1100 S, for example the slide tube on the Telelever, the wheels measuring 5.5 inches in width at the rear, and the front-wheel mudguard. The handlebar 
is wider and lower than on the R1150 R. 

Apart from the extravagant design, the R 1150 R Rockster stands out 
in particular through its unprecedented riding pleasure and an even higher standard of agility than the “regular” R 1150 R, making this machine truly unique. Indeed, the Rockster will arouse your emotions not only through its looks. 

4.2.5 Roadster Model Updates in the 2004 Model Year.   

Entering the 2004 model year, the R 1150 R and the R 1150 R Rockster 
now feature a steel flex clutch hose as well as a support point on the side starter holder for jump-starting the engine. A further feature is the power socket now coming as standard. Yet another improvement is the introduction of steel-clad brake hoses on the R 1150 R and R 850 R. 

4.3. Sports Tourer and Sportsman.

It almost goes without saying that a BMW Motorrad Sports Tourer combines superior touring qualities with the outstanding agility of a genuine athlete. 
A wide range of comfort features, excellent handling and extra safety make these models major players within the BMW motorcycle range. The sports versions, in turn, focus more on dynamic performance wherever you go.

4.3.1 Following the great tradition of the letter S: the R 1100 S.

Introducing the R 1100 S in autumn 1998, BMW Motorrad once again took up a great tradition: Back in the ’50s, ’60s and ’70s, the letter “S” in BMW motorcycle model designations already stood for that magic word “Sports”. 

Now proudly bearing the letter “S” itself, the R 1100 S has to maintain the same supreme standard, especially as it comes in the Sports Tourer segment. And although the R 1100 S naturally meets the usual touring requirements 
in terms of seating comfort for the rider and passenger as well as the use of cases, it clearly focuses primarily on sporting and dynamic performance.

The most powerful, lightest and most agile Boxer of them all.

Even from very first sight, through its very emotional look with a semi-shell fairing, the seat in solo look and the rear muffler rising up beneath the tailpiece, the R 1100 S bears testimony to its sporting genes. And this dynamic look 
is no coincidence, since the “heart” of this outstanding athlete beats with even more power and higher speeds than the “hearts” of the other Boxers.

Power comes from a two-cylinder four-valve engine displacing 1,085 cc and developing maximum output of 73 kW or 98 bhp at 7,500 rpm plus maximum torque of 97 Nm or 71 lb-ft at 5,750 rpm.

Clearly reflecting a sporting and dynamic character:
The six-speed with high-rise gear teeth.

Power is transmitted by a hydraulically operated single-plate dry clutch. 
And reflecting its sporting character, the R 1100 S comes with a six-speed gearbox featuring high-rise gear teeth for improved gearshift comfort 
and supreme precision. With the Paralever rear wheel swinging arm not being mounted in this case in the transmission housing, but rather on the newly developed main frame, the transmission housing is even more compact in its dimensions, reducing the noise-emitting surface accordingly. 

The Sports Boxer – even more agile and stable on the road.

The suspension geometry and running gear of the R 1100 S reflect the sporting style of riding this machine is built for: Wheelbase is 1,478 mm or 58.2´´ (R 1100 RS: 1,473 mm/58.0´´), camber measures 100 mm or 3.94´´ 
(111 mm/4.37´´). The steering head angle, in turn, is 65.0° (64.5°) and the maximum side angle with a rider weighing 85 kg (187 lb) is 50° (49°).

Extra-light version of the BMW Telelever.

Like all Boxers, the R 1100 S also features the BMW Telelever, a unique 
front wheel guidance system introduced in 1993 on the R 1100 RS and unparalleled in motorcycle construction to this very day.

The R 1100 S was the first machine to introduce an improved, lighter version with a special production process for the telescopic fork slide tubes made 
of individual components reducing weight by approximately by 1 kilo. Indeed, 
it is this significant reduction of unsprung masses on the front wheel that contributes to the excellent driving features and characteristics of the R 1100 S.
Front spring strut adjustable from the first time for the rider’s seat.

The front spring strut on the R 1100 S comes with a single-sleeve gas pressure damper providing 110 mm or 4.33´´ spring travel. Turning a hand wheel between the front edge of the tank and the upper fork bridge, 
the rider is able to infinitely adjust spring strut damping on the outward stroke. On the R 1100 RS, incidentally, the same adjustment is provided by a slot bolt on the spring strut. 

Light wheels in slender five-double-spoke design.

Reflecting its sporting look, the R 1100 S runs on light 17-inch cast aluminium wheels in slender five-double-spoke design. Wheel size is 3.50 x 17 at the front and 5.00 x 17 at the rear, with tyres measuring 120/70 ZR 17 at the front and 170/60 ZR 17 at the rear. A rear wheel measuring 5.50 x 17 running on 
a 180/55 ZR 17 tyre is available as an option.

Brake system optionally available with BMW Motorcycle 
Integral ABS.

The R 1100 S is naturally also available as an option with BMW Motorcycle Integral ABS for even better brake qualities. The Sports Tourer model 
comes with the partly integrated version of this ABS brake system, meaning that while the handbrake lever acts on the front and rear wheel 
brakes in unison, the footbrake lever is applied on the rear-wheel brake only.

The special look of the R 1100 S:
Sporting, emotional, dynamic and unmistakable.

Designing the R 1100 S, BMW Motorrad’s objective was to give this outstanding machine an emotional and dynamic look combining unmistakable BMW features with a level of sporting performance new to the range.

Semi-shell fairing and ellipsoid headlight.

The four-piece, sporting semi-shell fairing with its integrated direction indicators and hand protectors developed in the wind tunnel comes complete with a transparent windshield and offers comparatively good protection from wind and weather.

The highlight in the front fairing is the asymmetric dual headlight unit positioned just above the two openings for the oil cooler in that typical 
BMW kidney look. The larger of the two headlights is an ellipsoid unit with an H7 bulb serving as the low beam to provide even better illumination of the road ahead. The high beam is provided by a free-form headlight with an H1 bulb and makes a significant contribution to the unique looks of the R 1100 S. 

High-rise handlebar and extra-high windshield available as an option.

The handlebar consists of two halves separated from one another and made of forged aluminium clamped on to the support tube directly beneath the 
fork bridge. Grab width of the handlebar is 680 mm or 2.68´´ (complete with handlebar weights 760 mm/2.99´´). A somewhat higher comfort handlebar is available as an option in conjunction with a higher windshield. The handbrake lever as well as the hand lever for the hydraulic clutch can be set to four different positions.

Finished in black, the cockpit also offers a very sporting touch. It is dominated by two instruments – the speedometer and rev counter – both featuring 
yellow numbers and indicators on a black background. The telltales are on the left-hand side of the cockpit, a digital clock is fitted in the middle.

Weighing in at just 229 kg or 505 lb.

Ready-to-go and in road trim, the Sports Boxer weighs a mere 229 kg or 
505 lb.

Sporting but comfortable seating position.

The R 1100 S comes with one key for the ignition and steering locks, 
the tank cover and seat. Two hooks beneath the seat serve to hold two helmets securely on the motorcycle.

Reflecting the more sporting design and look of the R 1100 S, the rider footrests are somewhat further back and moved up slightly. Exchangeable domes are bolted on to the rider footrests from beneath to give the rider appropriate feedback as soon as he leans over to the maximum angle in a bend. With a rider weighing 85 kg or 187 lb, this angle is a maximum of 50°. Acting together, the special arrangement of the footrests and the handlebar handles ensure a sporting but nevertheless comfortable seating position.

4.3.2 New BMW R 1100 S BoxerCup Replica for the 2004 Season.

On the race track the racing version of the R 1100 S makes the heart of many a two-cylinder fan skip a beat in the international BMW Motorrad BoxerCup. And now, since the end of 2002, a derivative of this racing machine has also been available for road use: the R 1100 S BoxerCup Replica. Indeed, 
this outstanding machine has already proved so popular with aficionados 
of the Sports Boxer that the whole year’s production was virtually sold out 
by the end of summer.

No surprise, therefore, that BMW Motorrad decided to repeat production of the Replica for the 2004 season, the new R 1100 S BoxerCup Replica 
being presented to the international press and the public for the first time 
at the 2003 Tokyo Motor Show. 

Three-colour paintwork in brand-new also boasted by the racing machines in 2004, dual ignition and technical features even more similar to the racing machines – these are the main characteristics of this most sporting Boxer. Inter alia, the 2004 Replica comes with the higher suspension and running gear 
of the racing models, the racing spoiler on the engine, a cylinder head cover made of carbon-fibre reinforced plastic, and a sound-optimised sports muffler specially modified for road use to meet legal sound emission limits. 
Engine output is the same as before, without any changes versus the R 1100 S.
Further new features are the white instrument faces, the high seat and the mirrors carried over from the R 1150 GS, which are easier to remove than 
the standard mirrors fitted on the fairing and therefore offer an advantage in track racing.

Like its predecessor, the new R 1100 S BoxerCup Replica runs on Metzeler tyres. And to reflect the character of a genuine sports machine, this special motorcycle does not come with any further extras or special equipment, 
not even ABS.

4.3.3 A Sports Tourer of the highest calibre: the R 1150 RS.

The R 1150 RS has been in the market since autumn 2001, replacing the 
R 1100 RS after 8 years of success. Powered by a 1,130-cc engine complete with a fully controlled catalytic converter and a six-speed gearbox with high-rise gear teeth, this model also comes with other important features such as the EVO front-wheel brake and, as an option, BMW Motorcycle Integral ABS – in this case the partly integrated version reflecting the sporting character of this machine.

The R 1150 RS is a dynamic and agile, comfortable and safe motorcycle – 
the ideal Sports Tourer with the emphasis on “tourer”.

Displacement, power and torque.

Maximum output of the engine is 70 kW (95 bhp) at 7,250 rpm, 
maximum torque is 100 Nm or 74 lb-ft at 5,500 rpm. The “beefy” torque curve throughout the entire speed range ensures superior pulling force and acceleration, with more than 90 Nm or 66 lb-ft being consistently available between 3,000 and 6,500 rpm.

Gearbox with sixth overdrive gear.

Power on the R 1150 RS is transmitted via a hydraulically operated clutch with a hand lever adjustable to three different settings.

The six-speed gearbox with its high-rise gear teeth is designed for comfortable touring with a “longer” overdrive-type sixth gear to reduce engine speeds and noise, as well as vibrations and fuel consumption.

Suspension and running gear: agile and safe.

BMW’s exclusive front-wheel Telelever and the BMW Paralever, a widely-proven double-joint swinging arm on the shaft-driven rear wheel, guarantee superior riding safety and excellent comfort on the R 1150 RS.

Spring travel at the front on the R 1150 RS is 120 mm or 4.7´´ and the spring strut is infinitely adjustable on its outward stroke by means of a slotted bolt. Spring strut at the rear is 135 mm or 5.3´´, with the spring strut hydraulically adjustable for spring pre-tension by means of a hand wheel and with adjustment of the outward stroke with the help of an adjustment bolt.

Cast aluminium wheels for excellent handling.

The R 1150 RS comes on delicately-designed, extra-light cast aluminium wheels with five double spokes. With the front wheel measuring 3.50 x 17, 
the rear wheel is smaller and wider, measuring 5.00 x 17 as opposed to 
4.50 x 18 in the past. The benefits are superior riding stability and excellent handling. 

4.3.4 Sports Tourer Updates in the 2004 Model Year. 

Starting in the 2004 model year, the R 1100 S is also available with 
a higher seat (820 mm/32.3´´) as special equipment at no extra cost, 
and comes as standard with a power socket.

Like various other models, the R 1100 S and the R 1150 RS also feature 
a support point on the side starter holder for jump-starting the engine.

4.4 Tourer.

Many competitors have copied BMW Motorrad’s Tourer concept over the years. And this is no surprise, considering the outstanding features of these full-fairing machines such as excellent protection from wind and weather, 
a high level of comfort for the rider and passenger even on long journeys with luggage, as well as good handling on all kinds of roads. 

4.4.1 The R 1150 RT – the perfect Tourer.

The R 1150 RT entered the in spring 2001 as the successor to the 
R 1100 RT. The progress in technology offered by the new model, however, comes not only from the drivetrain, but also from the brake system in 
the interest of active safety on the road. Precisely this is why the R 1150 RT features BMW’s EVO brake on the front wheel and is equipped as standard with BMW Motorcycle Integral ABS, in this case the fully integrated version.

The “face” of the R 1150 RT is dominated by a tandem headlight with 
two foglamps and the upper fairing section. 

Ample displacement, superior power and torque.

The engine of the R 1150 RT offers lots of power and torque, the four-valve two-cylinder power unit developing maximum output of 70 kW (95 PS) at  
7,250 rpm and maximum torque of 100 Nm (74 lb-ft) at 5,500 rpm. A strong torque curve throughout the entire speed range ensures powerful acceleration and pulling force at all times, with more than 90 Nm or 66 lb-ft constantly available between 3,000 and 6,500 rpm.

Gearbox with sixth gear acting as overdrive.

Power is transmitted on the R 1150 RT via a hydraulically operated clutch with a hand lever adjustable to three different settings. A further feature also to be found on this model is the six-speed gearbox with high-rise gear teeth offering a significant improvement in terms of both gearshift precision and shift comfort. Conceived for comfortable touring in superior style, the gearbox comes with an extra-long sixth gear serving as an overdrive and thus reducing engine speed. Noise, vibrations and fuel consumption are all significantly lower.

Suspension and running gear: more agile and safer in the interest 
of enhanced riding pleasure.

The BMW Telelever, a most exclusive front-wheel technology of the highest standard, and the BMW Paralever, BMW’s proven double-pivot swinging arm on the shaft-driven rear wheel, combine superior riding safety with outstanding comfort at all times. Spring travel on the front spring strut is 120 mm or 
4.72´´, spring travel at the rear is 135 mm or 5.31´´. Spring pre-tension can be adjusted by means of a hand wheel, outward stroke damping is set by 
an adjustment bolt, in both cases with infinite action. This ensures optimum adjustment to all kinds of load and road conditions.

Cast aluminium wheels for excellent handling. 

The R 1150 RT comes on extra-slender and light cast aluminium wheels 
with five double spokes. Front wheel dimensions are 3.50 x 17 running 
on 120/70 ZR 17 tyres, the rear wheel measures 5.00 x 17. This ensures superior riding stability and exceptional handling at all times. 

EVO brake and BMW Integral ABS featured as standard – 
in this case as the fully integrated version.

The front wheel is fitted with BMW’s EVO brake, the front and rear brake linings are made of a sintered metal combining superior brake behaviour in the wet with equally superior brake power and a long service life. 

A further important point is that the R 1150 RT comes as standard with 
BMW Motorcycle Integral ABS, in this case the fully integrated version more appropriate for the particular character of a touring motorcycle. This means that the handbrake and footbrake levers both act simultaneously on the front- and rear-wheel brakes, making the entire process of brake application even more comfortable, effective, and safer.

Rider’s seat for maximum comfort. 

The highly comfortable, two-piece, height-adjustable seat offers optimum thigh support, a high standard of seat comfort, and very good ergonomics. The seat itself is adjustable to a height of 805, 825, and 845 mm (31.7, 
32.5, and 33.3´´), respectively. Smaller riders also have the choice of a lower seat, with seat height measuring 780, 800, and 820 mm (30.7, 31.5, and 32.3´´), respectively.

As befitting a genuine Tourer, the R 1150 RT comes as standard with case supports featuring an integrated luggage rack as well as two system cases. Together with the radio compartment, the central ignition and handlebar lock, as well as the seat, the cases open and close with one single key.

4.4.2 Tourer Model Updates in the 2004 Model Year.

Starting in the 2004 model year, the R 1150 RT features a support point 
on the side starter cover for jump-starting the engine when required.

4.5 Cruiser.

In September 1997 BMW Motorrad entered the Cruiser motorcycle segment for the first time with the introduction of the R 1200 C Classic, thus setting 
a milestone in the history of BMW motorcycles. Sometimes also referred to in Germany and a number of others countries as the Chopper, the Cruiser 
is simply perfect for a relaxed, laid-back style of riding absolutely free of aggression: This is the ideal machine, as the name indicates, for superior cruising, a machine particularly popular in the States.

In 1997, the year of its launch, the BMW Motorrad Cruiser also made a spectacular appearance with James Bond on the saddle in the 007 thriller “Tomorrow Never Dies”, stuntman Jean-Pierre Goy performing stunts 
never seen before.

Winning the most outstanding design awards as well as many reader’s choices, inter alia as the “Cruiser of the Year”, the BMW Cruiser has certainly justified its presence in this segment. And this also explains why today the BMW cruiser family is made up of four different models particularly successful in the US market.

4.5.1 R 1200 C Classic.

The R 1200 C Classic is a genuine Cruiser through and through, offering 
the rider that unique experience of relaxed cruising with supreme power and performance reserves. The flat fork, high-rise handlebar, long wheelbase 
as well as the rider’s seat in traditional saddle design with low seat height of just 740 mm or 29.1´´ are typical features of a genuine Cruiser.

Engine.

Facing the challenge to give the Cruiser the right kind of engine, the engineers at BMW Motorrad opted for BMW’s flat-twin Boxer basically offering all 
the essential characteristics of a Cruiser engine right from the start: Energetic development of power for cruising in style, the throaty rumble of the 
two-cylinder as an acoustic message of superior muscle, and the ideal mass balance of the Boxer in the interest of supreme motoring refinement. 
A further significant point is that through its looks alone, the Boxer is typical 
of BMW right from the start.

BMW Motorrad’s engine specialists also realised, however, that in its 
existing format the Boxer was certainly a very well-trained athlete, but a few thousand revs off the mark for an ideal Cruiser in the actual sense of the word.

So the engineers decided to put the Boxer through a significant process 
of internal change, converting this nimble sprinter into a more laid-back power athlete.

Increase in engine size.

The first and most important step in providing the right kind of power and riding behaviour was to increase engine displacement, especially as a “real” Cruiser always needs a big engine, anyway. So by increasing engine bore 
from 99 mm (3.90´´) to 101 mm (3.98´´) and extending stroke from 70.5 mm (2.78´´) to 73 mm (2.87´´), the engineers beefed up engine size from the original 1,085 cc to 1,170 cc for the BMW R 1200 C.

Same torque curve as the back of a buffalo.

The result of this increase in engine size is supreme power at all speeds. 
But even that was not enough for BMW’s engine specialists in their definition of the perfect Cruiser. They also wanted to give this supreme engine the 
right kind of torque curve literally quite similar in its appearance to the back of a buffalo. And the result of their work is a machine consistently doing without high engine speeds, making a rev counter the most superfluous feature in the world.

The gentle giant.

Fine-tuned in this sophisticated style, the sporting Boxer becomes a well-mannered Cruiser for gliding along free of stress on the highway. So wherever you go, ample 45 kW (61 bhp) at a moderate 5,000 rpm are just right 
when acting in harmony with the Boxer engine in the R 1200 C. For what counts when cruising is powerful torque at low speeds provided simply 
by turning your right wrist: Maximum torque of 98 Nm or 72 lb-ft comes at just 3,000 rpm, with torque remaining consistently above 90 Nm or 66 lb-ft all the way between 2,500 and 4,500 rpm.

Indeed, the BMW Cruiser offers an even much wider range of powerful torque and lots of practical engine output between 1,500 rpm and, if really necessary, 6,500 rpm. In fifth and top gear, the BMW R 1200 C is therefore able to 
cover the entire speed range from 50 km/h in town all the way to its top speed 160 km/h or 100 mph plus – all this in one single gear. So cruising BMW-style means that you are never forced to shift gears to enjoy your Cruiser. But if 
you feel like shifting gears nevertheless, you can count on equally outstanding riding pleasure.

At muscular, throaty Boxer sound.

Carefully tuned, the exhaust system naturally also makes its contribution to the desired torque curve, combining classic style and modern technology in perfect unison. Indeed, the classy look of the system with two short tailpipes in shining chrome comes up front of a design concept beneficial to the environment in two ways: The combination of a central pre-silencer and the two compact tail silencers at the side reduces riding noise to the strict European standard of just 80 dB (A). But the sound is nevertheless still typical of BMW and a genuine Cruiser, that is throaty and muscular.

Hydraulic clutch and six-speed gearbox with high-rise gears.

Power on the BMW R 1200 C is transmitted by a hydraulically operated single-plate dry clutch leading to the five-speed gearbox updated in autumn 2003 
to receive all the improvements introduced step-by-step on all Boxer machines since the middle of 2002. Typical features are the change in gearshift kinematics, optimised gearshift operation, and high-rise teeth on the fourth and fifth gear. The result of this update is enhanced shift comfort with 
equally superior precision and much lower running noise.

Finding the right gear increments on the five-speed unit was a very careful and painstaking process in order to meet the demands made of a Cruiser: 
The lower gears come with a “short” transmission ratio and close increments for quick acceleration on winding country roads. The last gear, in turn, has 
a far “longer” transmission ratio for ideal cruising between 80 and 120 km/h with maximum torque and optimum power reserves.

Like on all BMW Boxers rear-wheel drive is provided by a drive shaft running 
in this case in the hollow Monolever swinging arm and featuring not just two universal joints, but also a torsion damper. 

Long wheelbase for a genuine Cruiser-style riding position.

A long wheelbase is simply ideal for a Cruiser in order to bring out the qualities so typical of this motorcycle. After all, cruising means relaxed and laid-
back riding both in a straight line and on winding roads. So unerring directional stability is more important in this case than a spontaneous change from one daring angle in bends to another.

A further advantage of the long wheelbase is that it provides ample space 
for the rider taking on a pleasant position. Precisely this is why the engineers creating this BMW stretched into a Cruiser have moved the rider’s saddle-shaped seat further to the rear but shifted the footrests much further up front, providing a pleasant seating position between the two wheels for supreme pleasure even on long tours. 

Wheelbase is up by almost 200 mm or 7.9´´ from the relatively short 1,473 mm or 58.0´´ on the sporting and dynamic R 1100 RS to 1,650 mm or 65.0´´ 
on the R 1200 C. Together with this increase in wheelbase, overall length of the motorcycle has also increased from 2,175 mm or 85.6´´ on the R 1100 RS 
to 2,340 mm or 92.1´´ on the R 1200 C.

The Telelever remains an important design feature.                          

The upper Telelever mount rests in the top section of the front frame, precisely where it belongs. Indeed, it was clear from the start that the R 1200 C, just 
like all models in the current Boxer generation, was to have this innovative and exclusive front wheel suspension. And since this longitudinal lever is fully visible on the Cruiser, it also a particularly important and attractive design feature, which is why it is made of polished aluminium for the first time 
on the R 1200 C. 

The immersion tubes on the Telelever fork with a shallow steering of 60.5° typical of a Cruiser come in the same glossy finish. Front wheel castor, taking the superior directional stability and long wheelbase into account, is relatively short at 86 mm or 3.39´´ in the interest of good handling.

Spring action is provided by a centrally arranged spring strut with coil springs and a single-sleeve gas pressure damper. Spring travel measures 144 mm or 5.67´´.

Extra-long Monolever instead of the BMW Paralever.

Apart from the shallower angle of the Telelever, the rear wheel swinging arm extended in length by 90 mm or 3.53´´ also contributed to the much longer wheelbase of the BMW R 1200 C. And since a long swinging arm is able to set off the reaction of the drive shaft just about as well as the shorter, but more complex double-joint single swinging arm BMW Paralever, the engineers at BMW Motorrad have given the BMW R 1200 C a longer version of the original single swinging arm for reasons of function and design: the BMW Monolever. Here, too, suspension and spring action is provided by a central spring 
strut with a single-sleeve gas pressure damper resting on the swinging arm. Pre-tension of the coil spring may be adjusted mechanically as a function 
of load, with overall spring travel of 100 mm or 3.94´´.

Stylish and stable: BMW cross-spoke wheels.

With the wheels being crucial to the character and looks of a motorcycle, BMW’s designers and suspension experts have done their best to find the right wheel design also for the Cruiser. And indeed, the solution they have found is a wonderful combination of traditional looks and modern technology, the wheels of the BMW R 1200 C coming in classic wire-spoke design as 
their basic motif. This is brought out particularly in this case by the modern, high-gloss chrome-plated version of the BMW cross-spoke wheel. 

Highly efficient brake system – as an option with
BMW Motorcycle Integral ABS in its fully integrated version.

Although highly dynamic performance is not that essential on a motorcycle of this kind, a genuine BMW in Cruiser trim must still provide optimum stopping power with minimum stopping distances. And this is precisely why the 
brakes come in more than ample dimensions, the two cross-drilled stainless-
steel discs on the front wheel measuring 300 mm or 11.81´´ in diameter. 
Four-piston fixed callipers by Brembo hold the discs powerfully in place whenever required. The rear brake comes with a single disc measuring 
285 mm or 11.22´´ in diameter and resting in a two-piston floating calliper.

It almost goes without saying that the R 1200 C is available as an option 
with BMW Motorcycle Integral ABS in its fully integrated version.

The bottom line, therefore, is clear: Enjoying the R 1200 C, you are not only able to cruise through nature in genuine style, but can also make the best of winding roads with all the dynamic power you would like to experience. 
Or, put it in a nutshell, the BMW R 1200 C is a Cruiser also built for serious motorcycling.

Design and features.

The underlying design philosophy is naturally crucial to a BMW Cruiser. 
This means that the design of the machine must reflect the basic values of the brand and give the motorcycle its very own, particular identity. And precisely this is why the design of the R 1200 C interprets all the traditional values 
of BMW such as innovation, clarity in contents and looks, sensible 
and purposeful function, as well as reliability. Added to which, of course, 
there is also a big shot of emotion as the most important ingredient.

Passenger seat also serving as the rider’s backrest.

A small, leather-coated seat cushion accommodates the passenger behind the rider’s seat on short trips and excursions. When riding solo, in turn, you can move up the seat cushion to a vertical position serving as a backrest for the rider and adjustable in its angle by hand to three different settings. Whenever the seat cushion is tilted up in this way, it reveals a cast aluminium unit 
finished in pearl gloss chrome and serving as a luggage rack.

High-quality materials and surfaces.

The sophisticated but casual elegance of the BMW R 1200 C is further accentuated by the careful use of high-quality materials and surfaces perfectly reflecting that typical style of a Cruiser. Consistent use of hot-galvanised steel plate on the tank, wheel covers and side panels is more than “just” back to the roots. For steel plate is very robust and easy to handle in everyday use and provides the ideal background for high-quality paintwork retaining its perfect gloss on this material for many years of motorcycling.

Lots of chrome and polished aluminium.

Again reflecting that typical style of a Cruiser, the BMW R 1200 C self-confidently boasts a lot of chrome wherever appropriate. Indeed, this high-gloss metal finish is applied to the most demanding quality standards in 
three layers of nickel, copper and chrome, thus creating a highly attractive 
but very rugged surface.

Polished aluminium, in turn, is featured on the Telelever to give additional 
gloss and a particularly stylish look to the BMW Cruiser.

Optional extras and special equipment for the R 1200 C.

The typical rider of a Cruiser attaches a lot more significance than other motorcyclists to customising his machine. And this is precisely why 
BMW Motorrad offers a wide range of optional extras and special equipment for the Cruiser.

The Cruiser luggage system:
From saddle bags all the way to the tankbag.

The Cruiser’s outstanding qualities for long tours are underlined by a luggage set in stylish design perfectly matching this unique machine. Instead of hard-shell plastic cases, the R 1200 C comes with saddle bags made of water-repellent, impregnated leather and fitting at the appropriate point on integrated supports next to the rear wheel. The stable rear section made of plastic ensures a firm connection free of vibration, offering a functional and attractive solution for your luggage in conjunction with the leather bags. In addition, water-tight inner bags made of a special hard-wearing material and with a canvas-look surface keep everything tidy inside and ensure convenient 
use of the lockable saddle bags.

4.5.2 R 1200 C Independent with even more glossy chrome.

Even more glossy chrome, exclusive aluminium wheels, a solo seat, oval mirrors, an additional headlight, a speedster windshield and two choices of extravagant twin-tone paintwork – these are the features that characterise 
the R 1200 C Independent. Indeed, this model is BMW Motorrad’s answer 
to customer requests for an even higher level of customisation.

Launched in autumn 2000 and offering an even higher standard of elegance and sophisticated looks, the R 1200 C almost identical in technical terms 
with its sister models comes in twin-tone paintwork with a choice of Piemont Red metallic/Titanium Silver or Orient Blue metallic/Ivory.

The unique character of this twin-tone paintwork is further accentuated by 
the tinted speedster windshield, two additional small-lens foglamps, and new wheels: Instead of cross-spoke wheels, the R 1200 C Independent comes with two-piece aluminium wheels with a three-spoke inner star. The wheel rim and the star are connected to one another by titanium bolts.

In standard trim the R 1200 C Independent comes with a solo seat placing the focus on the rider. And should the rider not be quite as “independent” as 
the model name suggests, the R 1200 C Independent, like the other Cruisers, 
is available with either a small or a large passenger seat complete with footrests. Another option is the choice of the high or low Classic handlebar instead of the touring handlebar featured as standard on the Independent.

The direction indicators feature white lenses and yellow bulbs, the rear-view mirror housings are oval and chrome-plated, and not round. Indeed, the even larger amount of chrome than on the R 1200 C Classic gives the Independent an even more glamorous appearance, even the cover on the alternator at the front end of the engine housing, the newly designed air intake grids in front of the oil coolers, the clutch and handbrake levers as well as the covers on the brake and clutch fluid equalisation tanks coming in high-gloss chrome. And to provide the final touch, as it were, two additional BMW brand logos 
highlight the alternator cover at the front and the wheel bolt cover at the rear.

4.5.3 R 1200 CL: the Luxury Cruiser for touring in genuine style.

To round off the range and reach additional purchasers, BMW Motorrad introduced the R 1200 CL Luxury Cruiser in autumn 2002 as a further member of the Cruiser family. 

The basic idea was to carry over features from a touring motorcycle to 
a Cruiser, thus developing a machine with the best of both worlds. 
The result is a truly unique model, a Cruiser for touring in genuine style, leaving nothing to be desired in terms of comfort and equipment. 

The technical foundation for the new model was provided by the 
R 1200 C Classic, although basically only the engine, the rear-wheel drive system, the front frame, fuel tank and a few other features were actually 
taken over. Otherwise the R 1200 CL is a completely unique design and 
a brand-new development in many of its features.

Body and design: 
Unique face, powerful presence, fully integrated cases.

Presence, a powerful look and luxurious character – these words describe the convincing impression of the BMW R 1200 CL in short and sweet terms. Perhaps the most eye-catching highlight is the Tourer fairing fitted firmly to the handlebar and extending back in its lines along the separate fairing covers at the side of the tank, thus almost conveying the impression of a fully integrated fairing when viewed from the side. Clearly, a fairing of this kind gives the rider 
a high level of comfort through good protection in wind and weather.

Four headlights integrated in the fairing – two for the low, two for the high beam – give the R 1200 CL an unmistakable, unique face and impressive optical presence never seen before on a motorcycle. Clearly, these four headlights also ensure excellent illumination of the road ahead.

A particularly imaginative feature is the aerodynamic design of the windshield with its upper edge shaped almost like a wave. This special contour guides 
the flow of air in exactly the right direction to protect the rider. At the same time the recess in the middle allows the rider to look over the windshield without any kind of obstruction, giving him clear visibility of the road ahead regardless of any dirt or moisture on the windshield.

The front mudguard wrapped around at the side all the way to the wheel 
rim adds to the motorcycle’s powerful look. This design not only offers good protection from splashwater, but also, together with the large front 
tyre, accentuates the powerful styling of the front end while nevertheless emanating a feeling of casual, laid-back elegance. The Telelever at a 
shallower angle than on the other models gives even greater emphasis 
to the typical style of a Cruiser.

The rear section is dominated by the hard-shell cases integrated into and firmly connected with the motorcycle as well as the removable topcase on the curved luggage rack serving at the same time as a handle for the passenger. Both the cases as such as well as the topcase are finished in the same colour as the motorcycle itself, thus helping to form a harmonious entity.

The wide comfort seats in two-level arrangement for the rider and front passenger, together with their characteristic backrests, again add a further touch of distinction to the R 1200 CL.

Luxury guaranteed by exclusive colours, 
sophisticated surfaces and materials.

The R 1200 CL is available in three exclusive colours: Pearl Silver metallic and Sapphire Black metallic, each with Black seats, and Mojave Brown metallic with Brown seats (as an option also in Black). The elegance of these colours is enhanced by careful choice of materials and the perfect finish of surfaces 
and joints. The luggage rack, for example, is made of pressure-cast aluminium and painted in White aluminium, the handlebar is chrome-plated and the upper instrument cover also comes in White aluminium paint. The front fairing comes with an all-round inner lining and the kneepads with side covers are finished in the same material as the seats. All this underlines the commitment to luxury and perfection.
Drivetrain now featuring new, extra-quiet six-speed gearbox – 
flat-twin power unit unchanged.

While the 1,170 cc flat-twin power unit has been carried over without any modifications from the other Cruisers – performance data also remaining the same at 45 kW (61 bhp) and 98 Nm (72 lb-ft) torque at 3,000 rpm ( the transmission on the R 1200 CL has been modified to provide an even higher standard of excellence: It comes with six gears and was indeed the first gearbox with high-rise configuration of the gear teeth then carried over to all other Boxers.

In the meantime the improved gearshift kinematics revised to an even higher standard is an important feature on all Boxer transmissions.

With its “long”, overdrive-like transmission ratio in sixth gear, the transmission allows smooth riding at low engine speeds on long tours in flat regions, reducing fuel consumption and the noise level accordingly. A shift arm replaces the usual gearshift lever, enabling the rider to change gears with a casual flip of his foot. Gearshift comfort, low noise, low engine speeds 
but ample power at all times – these are the features which contribute to the unique pleasure the R 1200 CL has to offer.

A point that hardly needs emphasising is that the R 1200 CL, like every new BMW motorcycle worldwide launched since 1997, comes as standard with the most advanced emission management technology featuring a fully-controlled three-way catalytic converter. Because this standard of refined environmental protection has become a natural BMW asset in the meantime.

Suspension elements ensuring even greater comfort – new Telelever and spring strut at the rear with travel-dependent damping. 

Front wheel suspension stretched out to the front in a shallow angle to the road and with extra-large castor is another feature typical of a Cruiser. 
To achieve this configuration the still unique BMW Telelever has been re-configured for the R 1200 CL, the fork struts being placed further apart 
in order to accommodate the muscular, 150 mm (5.91´´) wide front wheel.

Rear suspension is provided by a spring strut with travel-dependent damping for excellent riding comfort at all times. Overall spring travel is up by 20 mm 
or 0.79´´ versus the other Cruisers to 120 mm or 4.72´´. The spring base itself is adjusted hydraulically by an easily accessible hand wheel in order to vary spring action as a function of load.

Rear wheel swinging arm optimised and rear frame new. 

The rear wheel swinging arm complete with the final drive housing – 
the BMW Monolever – has been reinforced and suitably re-designed to accommodate a larger rear wheel brake. The reinforced rear subframe is brand-new, accommodating the sideboards, case supports, luggage 
rack and the new seats as well as the modified side support. The front frame made of cast aluminium has been carried over with only minor modifications from the former R 1200 C. 

Wheels made of cast aluminium, seats, sideboards and handlebar. 

The visual impression of a motorcycle depends to a great extent on the wheels. So to create the right look, the R 1200 CL comes on avantgarde-style cast aluminium wheels with rims measuring 16 inches in diameter at the 
front and 15 inches at the rear to accommodate extra-large tyres: 150/80 at the front and 170/80 on the rear wheel. 

The seats are separate for the rider and passenger in order to meet their different needs. The wide comfort saddle for the rider, for example, comes complete with integrated hip contours for excellent support at all times. 
Seat height is 745 mm or 29.3´´. The passenger seat is also designed consistently for comfort and a superior riding position and is a bit higher up than the rider’s seat to give the passenger a better view, allowing him or 
her to really enjoy the landscape while cruising by.

Generously designed Cruiser-like sideboards for the rider again help to provide relaxed seating conditions. The passenger’s footrests likewise ensure very good support and, together with the appropriate knee angle, allow fatigue-free touring also on the rear seat. 

The wide, chrome-plated handlebar not only provides that unique feeling so typical of a Cruiser, but also comes in the right height and at the right angle 
to avoid even the slightest fatigue also on long trips. The hand levers and switches in BMW’s proven, unique control logics come straight from the other models, without any modifications.

High technology also on the brakes – 
BMW EVO brake and Integral ABS as an option.

By tradition, BMW has always given top priority to superior safety – which is precisely why the R 1200 CL comes with the BMW EVO brake on the front wheel, a brake technology with improved brake power that has already proven its qualities on other BMW motorcycles. BMW’s unique Motorcycle 
Integral ABS is available as an option, in this case in its fully integrated version reflecting the character of the machine: Regardless of whether the rider 
pulls the handbrake or footbrake lever, brake power is always applied perfectly to both wheels.

The front wheel features a double disc brake measuring 305 mm or 12.00´´ 
in diameter, the rear wheel features the single-disc brake system carried over from the K 1200 LT, with disc diameter measuring 285 mm or 11.22´´.

Wide range of optional extras for safety, 
comfort and individual luxury.

The R 1200 CL boasts a very wide range of optional extras extending from BMW Motorcycle Integral ABS for safe application of the brakes at all times through comfort features such as cruise control, heated handlebars and seat heating, all the way to luxurious customisation features with soft touch seats, 
a chrome package and a remote-controlled radio complete with CD player. Further options are the high windshield and anti-theft warning system as well as the choice of Black instead of Brown seats on models finished in Mojave Brown metallic.

Special equipment – customisation also after delivery.

The wide range of special equipment offered and fitted by the 
BMW motorcycle dealer allows further customisation of the R 1200 CL, 
also a long time after taking delivery of the motorcycle.

Typical features in this range of special equipment are the backrest for 
the passenger, sideboards also for the passenger, cylinder protection hoops, protection hoops at the rear, inner bags within the cases, cupholders, 
on-board communication (V.O.I.C.E.), and a CB aerial.

4.5.4 R 1200 C Montauk: masculine, striking, full of strength.

Marking the introduction of yet another unique model, the R 1200 C Montauk – a purist Cruiser in classic design, with innovative technology and superior riding behaviour – entered the market in autumn 2003.

The R 1200 C Montauk perfectly rounds off the range between the R 1200 CL Luxury Cruiser and the “basic” Cruisers. This is a “semi-naked” bike featuring only a windshield and boasting very powerful, masculine looks. At the front the Montauk is characterised by its double headlight in vertical arrangement and 
the extra-wide Telelever carried over from the R 1200 CL. The wheels and the extra-wide 150/80 16 (front) and 170/80 15 (rear) tyres also come from the 
R 1200 CL.

The other suspension and running gear features have been taken from the 
R 1200 C Classic, new features being the side covers and the somewhat more slender, but higher passenger seat, both of which give the Montauk 
a powerful, striking look also from the side.

In terms of brakes, the R 1200 C Montauk features the EVO front-wheel brake now to be admired on all Boxer and K-models, offering maximum brake power at all times. Incorporating two 305-mm (12.00”) brake discs and four-piston fixed callipers at the front and one 285-mm (11.22”) brake disc with a double floating calliper at the rear, the brake technology as such has been carried over from the R 1200 CL. The brake lines, finally, are steel-clad.

BMW’s unique Motorcycle Integral ABS is available as option, in this case in the fully integrated version reflecting the particular character of this special machine. So regardless of whether the rider operates the hand or footbrake lever, brake power is always applied perfectly on both wheels.

The drivetrain is the same as on the other Cruisers, dual ignition now being featured in general on all 1,200-cc Boxer engines. As a result, all Cruisers as of the 2004 model year come with the same significant benefits in terms of emission management, fuel economy and running smoothness. Output and torque remain the same, that is a standard 45 kW (61 bhp) and 98 Nm 
(72 lb-ft) at 3,000 rpm on all models. 

Performance of the R 1200 C Montauk on the road is the same as with the 
R 1200 C Classic.

4.5.5 Cruiser Model Updates in the 2004 Model Year. 

Together with the R 1200 C Montauk, all versions of the R 1200 C (with the exception of the R 1200 CL with a six-speed gearbox) feature BMW Motorrad’s updated five-speed gearbox and EVO brake as of the 2004 model year. Progressive BMW Motorcycle Integral ABS (fully integrated version) is available as an option at extra cost. Dual ignition within the engine is standard on all Cruisers.

An attractive innovation is the rear spring strut with travel-dependent damping (TDD spring strut) now also featured on the Classic and Independent 
models and providing an even higher standard of all-round suspension comfort.

All models come with a contact point on the side starter holder for jump-starting the engine. An extended range of chrome features is also available 
as an option.

The mufflers on all Cruisers are modified inside, with the features initially introduced on the R 1200 C Montauk. This optimisation ensures an even better sound pattern typical of a Cruiser, without increasing the noise level 
as such.

5. BMW Motorrad BoxerCup 2004.
Motorsport and outstanding success in racing have been part and parcel of BMW motorcycle history ever since the very beginning, making 
BMW motorcycles world-famous and contributing significantly to the great reputation of the brand for superiority and reliability wherever you go. 

Motorcycles with the white-and-blue logo have been writing racing history 
for more than 80 years, scoring countless unforgettable racing wins and setting up numerous world records. These outstanding achievements start with Georg (“Schorsch”) Meier’s legendary triumph in the 1938 Isle of 
Man TT, which he won as the first non-British rider, extending through the world speed records set up by Ernst Henne (with a maximum speed in 
1937 of 279.5 km/h or 173.29 mph) as well as a total of 19 victories in the Side Car World Championship featuring machines powered by the BMW 
side-shaft Boxer engine, all the way to six wins in the Paris-Dakar Rally in 1981, 1983, 1984, 1985, 1999 and 2000 – throughout history, the name BMW 
has been linked inseparably with outstanding success in racing.

In 1999 that unique sound of the Boxer returned to the race track, spectators being able for the first time in decades to admire Boxers in racing trim in the BMW Motorrad BoxerCup, to quote the official name of this newly established brand trophy. With production-based machines derived from the R 1100 S racing on a road track, the objective is to show the motorsport aficionado the potential the BMW Sports Boxer is able to develop under track conditions. First held as a national event, this racing series has become international in the meantime, receiving the official status of the European Motorsport Federation UEM (Union Européenne de Motocyclisme) in 2002.

Forming part of the World Championship programme at MotoGP and, respectively, International Endurance and Superbike Championships, these races draw large crowds and receive great attention all over Europe. Particularly at the “classic” Grand Prix events such as the legendary Dutch TT in Assen/Netherlands, the BMW BoxerCup races are held in front of an impressive crowd of up to 100,000 spectators. The German Grand Prix at Sachsenring, held in a region where people have always been thrilled by motorcycles, also attracts huge crowds. 

In 2003 the BMW Motorrad BoxerCup finally became intercontinental with 
the first race taking place in America on the occasion of the famous 200 Miles of Daytona and thus taking up the great heritage of the ’70s (in 1976 BMW scored a one-two win in Daytona with racing machines based on the R 90 S).

In 2004 the BMW Motorrad BoxerCup is entering its fourth international year. And with substantial prize money as well as other attractive prizes for the first three riders overall (outstanding BMW Group vehicles for the prizewinners), 
the BMW Motorrad BoxerCup is the most valuable motorcycle brand trophy in the world.

The particular appeal of the BoxerCup races lies in the fact that all the motorcycles and their components are absolutely identical. 
Only BMW R 1100 S motorcycles prepared for racing according to strict regulations without any “personal” variations or deviations qualify for the Cup. So apart from just a few exactly prescribed features, the motorcycles must 
be exactly the same as their production counterparts and must be made up entirely of original components. All relevant dimensions must remain within the usual production tolerances, and no modifications for extra power are allowed.

This means that all drivers ride identical machines in terms of power and suspension, the racing machines being very similar to the production models and maintaining all the basic data.

The main modifications allowed for racing are as follows:
•
Removal of all glass, headlights, mirrors, the horn, 
 
stands and passenger footrests.
•
Installation of wire fastenings holding down all important bolts.
•
Integration of a sports package primarily comprising a high-rise suspension 
 
for extra ground clearance when leaning to the side.
•
Installation of the BMW Motorrad Racing Kit (lighter front fairing, 
 
engine spoiler, carbon covers for the cylinder head bolts, sports mufflers, 
 
gearshift with opposite shift pattern).

The only factors that count, therefore, are the rider’s skill, his courage and clever tactics in the race. And clearly, this guarantees fascinating races, exciting overtaking manoeuvres, and tough fights for the leading places up to the last second. In the 2003 season, for example, the overall winner was only decided in the very last, extremely exciting race.

Spectators and BMW fans find it particularly attractive to identify with the motorcycles. And this is certainly understandable, since they are admiring the BMW R 1100 S in basically standard trim, but with professional riders on 
the saddle demonstrating a level of power and performance rightly regarded as almost incredible.

6. Specifications BMW Motorcycles.

F 650 GS, F 650 GS Dakar.

	
	
	


F 650 GS
	


F 650 GS Dakar
	F 650 GS 
25 kW/34 bhp/
F 650 GS Dakar
25 kW/34 bhp
	

	Engine
	
	
	
	
	

	Capacity
	cc
	652
	
	
	

	Bore/stroke
	mm
	100/83
	
	
	

	Max output
	kW/bhp
	37/50
	
	25/34
	

	at
	rpm
	6,500
	
	5,500
	

	Max torque
	Nm
	60
	
	51
	

	at
	rpm
	4,800
	
	4,000
	

	No of cylinders
	
	1
	
	
	

	Compression ratio/fuel grade
	
	11.5:1/reg
	
	
	

	Valve/gas management
	
	DOHC (double overhead  camshaft) 
	
	
	

	Valves per cylinder
	
	4
	
	
	

	Intake/outlet dia
	mm
	36/31
	
	
	

	Fuel supply
	
	BMS-C II
	
	
	

	
	
	
	
	
	

	Electrical system
	
	
	
	
	

	Alternator
	W
	400
	
	
	

	Battery
	V/Ah
	12/12 maintenance-free
	
	
	

	Headlight
	W
	H 4 60/55
	
	
	

	Starter
	kW
	0.9
	
	
	

	
	
	
	
	
	

	Power transmission
	
	
	
	
	

	Clutch
	
	Multiple-plate clutch in oil bath
	
	

	Transmission
	
	
Five-speed gearbox with dog-type shift
	
	

	Primary ratio
	
	1.946
	
	
	

	Transmission ratios
I
	
	2.750
	
	
	

	
II
	
	1.750
	
	
	

	
III
	
	1.310
	
	
	

	
IV
	
	1.050
	
	
	

	
V
	
	0.880
	
	
	

	Final drive
	
	        O-ring chain 5/8 x ¼ 
	
	

	Transmission ratio
	
	                                 2.937
	
	

	
	
	
	

	Suspension and running gear
	
	
	

	Frame
	
	
Steel bridge tubular frame
	
	

	Wheel suspension, front
	
	Telescopic fork
	
	
	

	Wheel suspension, rear
	
	Swinging arm
	
	
	

	Spring travel, front/rear
	mm
	170/165
	210/210
	
	

	Castor
	mm
	113.3
	123.3
	
	

	Wheelbase
	mm
	1,479
	1, 489
	
	

	Steering head angle
	°
	60.8
	
	
	

	Brakes
	front
	
Single-disc brake, dia 300 mm
	
	

	
	rear
	
Single-disc brake, dia 240 mm 
	
	

	
	
	Optional BMW Motorcycle ABS
	
	
	

	Wheels
	
	Wire spoke wheels
	Wire spoke wheels
	
	

	
	front
	2.5 x 19
	1.6 x 21
	
	

	
	rear
	3.00 x 17
	
	
	

	Tyres
	front
	100/90–19 57 S
	90/90–21 54 S
	
	

	
	rear
	130/80–17 65 S
	
	
	

	
	
	
	
	
	

	Dimensions and weight
	
	
	
	
	

	Length, overall
	mm
	2, 175
	2,189
	
	

	Width, overall, with mirrors
	mm
	910
	
	
	

	Width, overall, without mirrors
	mm
	785
	901
	
	

	Seat height
	mm
	780
	870
	
	

	Weight, unladen, with full tank
	kg
	193
	192
	
	

	Max permissible weight, overall
	kg
	380
	
	
	

	Tank capacity
	ltr
	17.3
	
	
	

	
	
	
	
	
	

	Performance data
	
	
	
	
	

	Fuel consumption
	
	
	
	
	

	90 km/h
	ltr/100 km
	3.2
	
	3.2
	

	120 km/h
	ltr/100 km
	4.3
	
	4.3
	

	Acceleration
	
	
	
	
	

	0–100 km/h
	sec
	5.7
	
	–
	

	Standing-start km
	sec
	26.8
	
	–
	

	Top speed
	km/h
	170
	
	145
	


Specifications BMW Motorcycles.
F 650 CS.
	
	
	F 650 CS
	F 650 CS

25 kW/34 bhp
	

	Engine
	
	
	
	

	Capacity
	cc
	652
	
	

	Bore/stroke
	mm
	100/83
	
	

	Max output
	kW/bhp
	37/50
	25/34
	

	at 
	rpm
	6,500
	6,000
	

	Max torque
	Nm
	62
	50
	

	at
	rpm
	5,500
	3,000
	

	No of cylinders
	
	1
	
	

	Compression ratio/fuel grade
	
	11.5:1/reg
	
	

	Valve/gas management
	
	DOHC (double overhead camshaft)
	
	

	Valves per cylinder
	
	4
	
	

	Intake/outlet dia
	mm
	36/31
	
	

	Fuel supply
	
	BMS-C II
	
	

	
	
	
	
	

	Electrical system
	
	
	
	

	Alternator
	W
	400
	
	

	Battery
	V/Ah
	12/12 maintenance-free
	
	

	Headlight
	W
	H 1/H 3 55/55
	
	

	Starter
	kW
	0.9
	
	

	
	
	
	
	

	Power transmission
	
	
	
	

	Clutch
	
	Multiple-plate clutch in oil bath
	
	

	Transmission
	
	Five-speed gearbox with dog-type shift
	

	Primary ratio
	
	1.946
	
	

	Transmission ratios
I
	
	2.750
	
	

	
II
	
	1.750
	
	

	
III
	
	1.310
	
	

	
IV
	
	1.050
	
	

	
V
	
	0.840
	
	

	Final drive
	
	                      Tooth belt
	

	Transmission ratio
	
	                             2.929
	

	
	
	

	Suspension and running gear
	
	

	Frame
	
	Steel bridge tubular frame 
with integrated oil tank
	

	Wheel suspension, front
	
	Telescopic fork
	
	

	Wheel suspension, rear
	
	Swinging arm
	
	

	Spring travel, front/rear
	mm
	125/120
	
	

	Castor
	mm
	86
	
	

	Wheelbase
	mm
	1 493
	
	

	Steering head angle
	°
	62.1
	
	

	Brakes
	front
	
Single-disc brake, dia 300 mm
	

	
	rear
	
Single-disc brake, dia 240 mm 
	

	
	
	Optional BMW Motorcycle ABS
	

	Wheels
	
	    Light-alloy cast wheels
	

	
	front
	30 x 17
	
	

	
	rear
	4.5 x 17
	
	

	Tyres
	front
	110/70 ZR 17
	
	

	
	rear
	160/60 ZR 17
	
	

	
	
	
	
	

	Dimensions and weight
	
	
	
	

	Length, overall
	mm
	2,142
	
	

	Width, overall, with mirrors
	mm
	915
	
	

	Width, overall, without mirrors
	mm
	824
	
	

	Seat height
	mm
	780
	
	

	Weight, unladen, with full tank
	kg
	189
	
	

	Max permissible weight, overall
	kg
	370
	
	

	Tank capacity
	ltr
	15
	
	

	
	
	
	
	

	Performance data
	
	
	
	

	Fuel consumption
	
	
	
	

	90 km/h
	ltr/100 km
	3.0
	
	

	120 km/h
	ltr/100 km
	4.1
	
	

	Acceleration
	
	
	
	

	0–100 km/h
	sec
	5.1
	7.4
	

	Standing-start km
	sec
	27.3
	32
	

	Top speed
	km/h
	178
	147
	


Specifications BMW Motorcycles.

K 1200 LT.

	
	
	
	
	
K 1200 LT
	K 1200 LT
72 kW/98 bhp*
	

	Engine
	
	
	
	
	
	

	Capacity
	cc
	
	
	1,171
	
	

	Bore/stroke
	mm
	
	
	70.5/75
	
	

	Max output
	kW/bhp
	
	
	85/116
	72/98
	

	at
	rpm
	
	
	8,000
	8,000
	

	Max torque
	Nm
	
	
	120
	118
	

	at
	rpm
	
	
	5,250
	5,250
	

	Engine configuration
	
	
	
	Inline
	
	

	No of cylinders
	
	
	
	4
	
	

	Compression ratio/fuel grade
	
	
	
	11.5/premium
	
	

	Valve/gas management
	
	
	
	DOHC (double overhead  camshaft)
	
	

	Valves per cylinder
	
	
	
	4
	
	

	Intake/outlet dia
	mm
	
	
	26.5/23
	
	

	Fuel supply
	
	
	
	Motronic MA 2.4
	
	

	
	
	
	
	
	
	

	Electrical system
	
	
	
	
	
	

	Alternator
	W
	
	
	840
	
	

	Battery
	V/Ah
	
	
	12/19 maintenance-free
	
	

	Headlight
	W
	
	
	H 7/55 W
	
	

	
	
	
	
	H 9/65 W
	
	

	Starter
	kW
	
	
	0.7
	
	

	
	
	
	
	
	
	

	Power transmission
	
	
	
	
	
	

	Clutch
	
	Single-plate dry clutch, dia 180 mm
	

	Transmission
	
	Five-speed gearbox with dog-type shift
	
	

	Primary ratio
	
	
	
	1.889
	
	

	Transmission ratios  
I
	
	
	
	2.045
	
	

	
II
	
	
	
	1.600
	
	

	
III
	
	
	
	1.267
	
	

	
IV
	
	
	
	0.970
	
	

	
V
	
	
	
	0.806
	
	

	Final drive
	
	
	
	Driveshaft
	
	

	Transmission ratio
	
	
	
	2.62
	
	

	
	
	
	
	
	
	

	Suspension and running gear
	
	
	
	
	
	

	Frame
	
	
Cast aluminium frame
	

	Wheel suspension, front
	
	
	
	BMW Telelever
	
	

	Wheel suspension, rear
	
	
	
	BMW Paralever
	
	

	Spring travel, front/rear
	mm
	
	
	102/130
	

	Castor
	mm
	
	
	117
	

	Wheelbase
	mm
	
	
	1,627
	

	Steering head angle
	°
	
	
	63
	

	Brakes
	front
	
Double-disc brake, dia 320 mm
	

	
	rear
	Single-disc brake, dia 285 mm 
	

	
	
	
	
	BMW Motorcycle Integral ABS fitted as standard (fully integral)
	

	Wheels
	
	
	
	Light alloy
	

	
	front
	
	
	3.50 x 17 MTH 2
	

	
	rear
	
	
	5.00 x 17 MTH 2
	

	Tyres
	front
	
	
	120/70 ZR 17
	

	
	rear
	
	
	160/70 ZR 17
	

	
	
	
	
	
	

	Dimensions and weight
	
	
	
	
	

	Length, overall
	mm
	
	
	2 502
	

	Width, overall, with mirrors
	mm
	
	
	1 080
	

	Width, overall, without mirrors
	mm
	
	
	826
	

	Seat height
	mm
	
	
	770/790
	

	Weight, unladen, with full tank
	kg
	
	
	387
	

	Max permissible weight, overall
	kg
	
	
	600
	

	Tank capacity
	ltr
	
	
	23.4
	

	
	
	
	
	
	

	Performance data
	
	
	
	
	

	Fuel consumption
	
	
	
	
	

	90 km/h
	ltr/100 km
	
	
	4.3
	

	120 km/h
	ltr/100 km
	
	
	5.7
	

	Acceleration
	
	
	
	
	

	0–100 km/h
	sec
	
	
	5.0
	

	Standing-start km
	sec
	
	
	–
	

	Top speed
	km/h
	
	
	210
	


*Available in France only.
Specifications BMW Motorcycles.

K 1200 GT.

	
	
	
K 1200 GT
	K 1200 GT
72 kW/98 bhp
	

	Engine
	
	
	

	Capacity
	cc
	1,171
	

	Bore/stroke
	mm
	70.5/75
	

	Max output
	kW/bhp
	96/130
	72/98
	

	at
	rpm
	8,750
	7,000
	

	Max torque
	Nm
	117
	118
	

	at
	rpm
	6,750
	5,500
	

	Engine configuration
	
	Inline
	

	No of cylinders
	
	4
	

	Compression ratio/fuel grade
	
	11.5/premium
	

	Valve/gas management
	
	DOHC (double overhead camshaft)
	

	Valves per cylinder
	
	4
	

	Intake/outlet dia
	mm
	26.5/23
	

	Fuel supply
	
	Motronic MA 2.4
	

	
	
	
	

	Electrical system
	
	
	

	Alternator
	W
	840
	

	Battery
	V/Ah
	12/19 maintenance-free
	

	Headlight
	W
	H 7/55 W
	

	
	
	H 3/55 W
	

	Starter
	kW
	0.7
	

	
	
	
	

	Power transmission
	
	
	

	Clutch
	
	Single-plate dry clutch, dia 180 mm 
	

	Transmission
	
	Six-speed gearbox with dog-type shift
	

	Primary ratio
	
	1.889
	

	Transmission ratios
I
	
	2.045
	

	
II
	
	1.600
	

	
III
	
	1.267
	

	
IV
	
	1.038
	

	
V
	
	0.900
	

	
VI
	
	0.806
	

	Final drive
	
	Driveshaft
	

	Transmission ratio 
	
	2.75
	

	
	
	
	

	Suspension and running gear
	
	
	

	Frame
	
	Cast aluminium frame
	

	Wheel suspension, front
	
	BMW Telelever
	

	Wheel suspension, rear
	
	BMW Paralever 
	

	Spring travel, front/rear
	mm
	115/150
	

	Castor in normal trim
	mm
	124
	

	Wheelbase in normal trim
	mm
	1,555
	

	Steering head angle in normal trim
	°
	62.8
	

	Brakes
	front
	Double-disc brake, dia 320 mm 
	

	
	rear
	Single-disc brake, dia Ø 285 mm
	

	
	
	BMW Motorcycle Integral ABS fitted as standard (partly integral)
	

	Wheels
	
	Light alloy
	

	
	front
	3.50 x 17 MTH 2
	

	
	rear
	5.5 x 17 MTH 2
	

	Tyres
	front
	120/70 ZR 17
	

	
	rear
	180/55 ZR 17
	

	
	
	
	

	Dimensions and weight
	
	
	

	Length, overall
	mm
	2,250
	

	Width, overall, with mirrors
	mm
	920
	

	Width, overall, without mirrors
	mm
	715
	

	Seat height
	mm
	790/820
	

	Weight, unladen, in road trim
	kg
	300
	

	Max permissible weight, overall
	kg
	500
	

	Tank capacity
	ltr
	20.5
	

	
	
	
	

	Performance data
	
	
	

	Fuel consumption
	
	
	

	90 km/h
	ltr/100 km
	5.0
	

	120 km/h
	ltr/100 km
	5.9
	

	Acceleration
	
	
	

	0–100 km/h
	sec
	3.7
	4.1
	

	Top speed
	km/h
	245
	225
	


Specifications BMW Motorcycles.

K 1200 RS.

	
	
	
K 1200 RS
	K 1200 RS
72 kW/98 bhp
	
	

	Engine
	
	
	
	
	

	Capacity
	cc
	1,171
	
	
	

	Bore/stroke
	mm
	70.5/75
	
	
	

	Max output
	KW/bhp
	96/130
	72/98
	
	

	At
	rpm
	8,750
	7,000
	
	

	Max torque
	Nm
	117
	118
	
	

	At
	rpm
	6,750
	5,500
	
	

	Engine configuration
	
	Inline
	
	
	

	No of cylinders
	
	4
	
	
	

	Compression ratio/fuel grade
	
	11.5/premium
	
	
	

	Valve/gas management
	
	DOHC (double overhead camshaft)
	
	
	

	Valves per cylinder
	
	4
	
	
	

	Intake/outlet dia
	mm
	26.5/23
	
	
	

	Fuel supply
	
	Motronic MA 2.4
	
	
	

	
	
	
	
	
	

	Electrical system
	
	
	
	
	

	Alternator
	W
	840
	
	
	

	Battery
	V/Ah
	12/19 maintenance-free
	
	
	

	Headlight
	W
	H 7/55 W
	
	
	

	
	
	H 3/55 W
	
	
	

	Starter
	kW
	0.7
	
	
	

	
	
	
	
	
	

	Power transmission
	
	
	
	
	

	Clutch
	
	Single-plate dry clutch, dia 180 mm               
	
	

	Transmission
	
	Six-speed gearbox with dog-type shift
	
	
	

	Primary ratio
	
	1.889
	
	
	

	Transmission ratios
I
	
	2.045
	
	
	

	
II
	
	1.600
	
	
	

	
III
	
	1.267
	
	
	

	
IV
	
	1.038
	
	
	

	
V
	
	0.900
	
	
	

	
VI
	
	0.806
	
	
	

	Final drive
	
	Driveshaft 
	
	
	

	Transmission ratio
	
	2.75
	
	
	

	
	
	
	
	
	

	Suspension and running gear
	
	
	
	
	

	Frame
	
	Cast aluminium frame
	
	

	Wheel suspension, front
	
	BMW Telelever
	
	
	

	Wheel suspension, rear 
	
	BMW Paralever
	
	
	

	Spring travel, front/rear
	mm
	115/150
	
	
	

	Castor
	mm
	124
	
	
	

	Wheelbase
	mm
	1555
	
	
	

	Steering head angle
	°
	62.75
	
	
	

	Brakes
	front
	Double-disc brake, dia 320 mm
	
	
	

	
	rear
	Single-disc brake, dia 285 mm 
	
	
	

	
	
	BMW Motorcycle Integral ABS fitted as standard (partly integral)
	
	
	

	Wheels
	
	Light alloy
	
	
	

	
	front
	3.50 x 17 MTH 2
	
	
	

	
	rear
	5.00 x 17 MTH 2
	
	
	

	Tyres
	front
	120/70 ZR 17
	
	
	

	
	rear
	180/55 ZR 17
	
	
	

	
	
	
	
	
	

	Dimensions and weight
	
	
	
	
	

	Length, overall
	mm
	2,250
	
	
	

	Width, overall, with mirrors
	mm
	850
	
	
	

	Width, overall, without mirrors
	mm
	680
	
	
	

	Seat height
	mm
	770/800
	
	
	

	Weight, unladen, with full tank
	kg
	285
	
	
	

	Max permissible weight, overall
	kg
	500
	
	
	

	Tank capacity
	ltr
	20.5
	
	
	

	
	
	
	
	
	

	Performance data
	
	
	
	
	

	Fuel consumption
	
	
	
	
	

	90 km/h
	ltr/100 km
	4.9
	
	
	

	120 km/h
	ltr/100 km
	5.8
	
	
	

	Acceleration
	
	
	
	
	

	0–100 km/h
	sec
	3.7
	4.1
	
	

	Standing-start km
	sec
	–
	–
	
	

	Top speed
	km/h
	245
	225
	
	


Specifications BMW Motorcycles.
R 1200 GS.
	
	
	R 1200 GS (72 kW)*
	
	R 1200 GS (74 kW)
	

	Engine
	
	
	
	
	

	Capacity
	cc
	1,170
	
	
	

	Bore/stroke
	mm
	101/73
	
	
	

	Max output
	kW/bhp
	72/98 
	
	74/100
	

	at
	rpm
	7,000
	
	
	

	Max torque
	Nm
	115
	
	
	

	at
	rpm
	5,500
	
	
	

	Engine configuration
	
	Flat-twin
	
	
	

	No of cylinders
	
	2
	
	
	

	Compression ratio/fuel grade
	
	11.0/premium
	
	
	

	Valve/gas management
	
	HC (high camshaft)
	
	
	

	Valves per cylinder
	
	4
	
	
	

	Intake/outlet dia
	mm
	36/31
	
	
	

	Fuel supply
	
	BMS-K
	
	
	

	
	
	
	
	
	

	Electrical system
	
	
	
	
	

	Alternator
	W
	600
	
	
	

	Battery
	V/Ah
	12/14 maintenance-free 
	 
	
	

	Headlight
	W
	H7
	
	
	

	Starter
	kW
	1.1
	
	
	

	Power transmission
	
	
	
	
	

	Clutch
	
	
         Single-plate dry clutch, dia 180 mm         
	
	

	Transmission
	
	Six-speed gearbox with dog-type shift
	
	
	

	Primary ratio
	
	1.823
	
	
	

	Transmission ratios
I
	
	2.277
	
	
	

	
II
	
	1.583
	
	
	

	
III
	
	1.259
	
	
	

	
IV
	
	1.033
	
	
	

	
V
	
	0.903
	
	
	

	
VI
	
	0.805
	
	
	

	Final drive
	
	Driveshaft
	
	
	

	Transmission ratio
	
	                                                               2.82
	
	

	
	
	
	
	
	

	Suspension and running gear
	
	
	
	
	

	Frame
	
	     Steel tubular frame, load-bearing engine
	
	
	

	Wheel suspension, front
	
	BMW Telelever
	
	

	Wheel suspension, rear
	
	BMW Paralever
	
	

	Spring travel, front/rear
	mm
	190/200
	
	
	

	Castor
	mm
	110
	
	
	

	Wheelbase
	mm
	1,519
	
	
	

	Steering head angle
	°
	62.9
	
	
	

	Brakes
	front
	
Double-disc brake, dia 305 mm 
	
	
	

	
	rear
	Single-disc brake, dia 265 mm  
	
	

	
	
	Optional BMW Motorcycle Integral ABS (partly integral, on-demand)
	
	
	

	Wheels
	
	Cast wheel,  optional cross-spoke wheels
	
	
	

	
	front
	2.5 x 19 
	
	
	

	
	rear
	4.0 x 17
	
	
	

	Tyres
	front
	110/80 H 19 TL
	
	
	

	
	rear
	150/70 H 17 TL
	
	
	

	
	
	
	
	
	

	Dimensions and weight
	
	
	
	
	

	Length, overall
	mm
	2,210
	
	
	

	Width, overall, with mirrors
	mm
	915
	
	
	

	Width, overall, without mirrors
	mm
	871
	
	
	

	Seat height
	mm
	840/860
	
	
	

	Weight, unladen, with full tank
	kg
	225
	
	
	

	Max permissible weight, overall
	kg
	425
	
	
	

	Tank capacity
	ltr
	20
	
	
	

	
	
	
	
	
	

	Performance data
	
	
	
	
	

	Fuel consumption
	
	
	
	
	

	90 km/h
	ltr/100 km
	4.3
	
	
	

	120 km/h
	ltr/100 km
	5,5
	
	
	

	Acceleration
	
	
	
	
	

	0–100 km/h
	sec
	3.4
	
	
	

	Standing-start km
	sec
	24.7
	
	
	

	Top speed
	km/h
	205 
	
	208
	


* Available in Germany only.

Specifications BMW Motorcycles.
R 1150 GS Adventure.
	
	
	R 1150 GS Adventure
	
	

	Engine
	
	
	
	

	Capacity
	cc
	1,130
	
	

	Bore/stroke
	mm
	101/70,5
	
	

	Max output
	kW/bhp
	62.5/85
	
	

	at
	rpm
	
	
	

	Max torque
	Nm
	98
	
	

	at
	rpm
	5,250
	
	

	Engine configuration
	
	Flat-twin
	
	

	No of cylinders
	
	2
	
	

	Compression ratio/fuel grade
	
	10.3/premium
	
	

	Valve/gas management
	
	HC (high camshaft)
	
	

	Valves per cylinder
	
	4
	
	

	Intake/outlet dia
	mm
	34/29
	
	

	Fuel supply
	
	Motronic MA 2.4
	
	

	
	
	
	
	

	Electrical system
	
	
	
	

	Alternator
	W
	700
	
	

	Battery
	V/Ah
	12/19 maintenance-free
	
	

	Headlight
	W
	H 1/H7
	
	

	Starter
	KW
	1.1
	
	

	
	
	
	
	

	Power transmission
	
	
	
	

	Clutch
	
	   Single-plate dry clutch, dia 165 mm
	

	Transmission
	
	Six-speed gearbox with dog-type shift
	
	

	Primary ratio
	
	1.889
	
	

	Transmission ratios
I
	
	2.038
	 Short optional ratio 2.375

	
II
	
	1.600
	
	

	
III
	
	1.267
	
	

	
IV
	
	1.034
	
	

	
V
	
	0.903
	
	

	
VI
	
	0.806
	
	

	Final drive
	
	Driveshaft
	
	

	Transmission ratio
	
	2.82
	
	

	
	
	
	
	

	Suspension and running gear
	
	
	
	

	Frame
	
	Composite frame, front section in cast aluminium, load-bearing engine
	
	

	Wheel suspension, front
	
	BMW Telelever 
	
	

	Wheel suspension, rear
	
	BMW Paralever
	
	

	Spring travel, front/rear
	mm
	210/220
	
	

	Castor in normal trim
	mm
	121
	
	

	Wheelbase in normal trim
	mm
	1,501
	
	

	Steering head angle in normal trim
	°
	64
	
	

	Brakes
	front
	                                   dia 305 mm
	
	

	
	rear
	                                 dia 276 mm
	

	
	
	Optional BMW Motorcycle Integral ABS (partly integral)
	
	

	Wheels
	
	Cross-spoke wheels
	
	

	
	front
	2.5 x 19 
	
	

	
	rear
	4.0 x 17
	
	

	Tyres
	front
	110/80 B 19 TL
	
	

	
	rear
	150/70 H 17 TL
	
	

	
	
	
	
	

	Dimensions and weight
	
	
	
	

	Length, overall
	mm
	2,180
	
	

	Width, overall, with mirrors
	mm
	980
	
	

	Width, overall, without mirrors
	mm
	903
	
	

	Seat height
	mm
	900
	
	

	Weight, unladen, with full tank
	kg
	253
	
	

	Max permissible weight, overall
	kg
	460
	
	

	Tank capacity
	ltr
	22.1
	(optional 30)

	
	
	
	
	

	Performance data
	
	
	
	

	Fuel consumption
	
	
	
	

	90 km/h
	ltr/100 km
	4.5
	
	

	120 km/h
	ltr/100 km
	5.7
	
	

	Acceleration
	
	
	
	

	0–100 km/h
	sec
	4.3
	
	

	Standing-start km
	sec
	24.9
	
	

	Top speed
	km/h
	192
	
	


Specifications BMW Motorcycles.

R 1150 R.
	
	
	
	
	R 1150 R
	

	Engine
	
	
	 
	
	

	Capacity
	cc
	
	
	1,130
	

	Bore/stroke
	mm
	
	
	101/70.5
	

	Max output
	kW/bhp
	
	
	62.5/85
	

	at
	rpm
	
	
	6,750
	

	Max torque
	Nm
	
	
	98
	

	at
	rpm
	
	
	5 250
	

	Engine configuration
	
	
	
	Flat-twin
	

	No of cylinders
	
	
	
	2
	

	Compression ratio/fuel grade
	
	
	
	10.3/premium
	

	Valve/gas management
	
	
	
	HC (high camshaft)
	

	Valves per cylinder
	
	
	
	4
	

	Intake/outlet dia
	mm
	
	
	34/29
	

	Fuel supply
	
	
	
	Motronic MA 2.4
	

	
	
	
	
	
	

	Electrical system
	
	
	
	
	

	Alternator
	W
	
	
	700
	

	Battery
	V/Ah
	
	
	12/19 maintenance-free
	

	Headlight
	W
	
	
	H 4
	

	Starter
	kW
	
	
	1.1
	

	
	
	
	
	
	

	Power transmission
	
	
	
	
	

	Clutch
	
	 Single-plate dry clutch, dia 165 mm
	

	Transmission
	
	
Six-speed gearbox with dog-type shift
	

	Primary ratio
	
	
	
	1.889
	

	Transmission ratios
I
	
	
	
	2.038
	

	
II
	
	
	
	1.600
	

	
III
	
	
	
	1.267
	

	
IV
	
	
	
	1.034
	

	
V
	
	
	
	0.903
	

	

     VI
	
	
	
	0.806
	

	Final drive
	
	
	
	Driveshaft
	

	Transmission ratio
	
	
	
	2.82
	

	
	
	
	
	
	

	Suspension and running gear
	
	
	
	
	

	Frame
	
	Composite frame, front section in aluminium load-bearing engine 
	

	Wheel suspension, front
	
	
	
	BMW Telelever
	

	Wheel suspension, rear
	
	
	
	BMW Paralever 
	

	Spring travel, front/rear
	mm
	
	
	120/135
	

	Castor
	mm
	
	
	127
	

	Wheelbase
	mm
	
	
	1 487
	

	Steering head angle
	°
	
	
	63
	

	Brakes
	front
	 Double-disc brake, dia 320 mm
	

	
	rear
	 Single-disc brake, dia 276 mm
	

	
	
	Optional BMW Motorcycle Integral ABS (partly integral)
	

	Wheels
	
	                                                                                        Light alloy
	

	
	front
	
	
	3.50 x 17
	

	
	rear
	
	
	5.00 x 17
	

	Tyres
	front
	
	
	120/70 ZR 17
	

	
	rear
	
	
	170/60 ZR 17
	

	
	
	
	
	
	

	Dimensions and weight
	
	
	
	
	

	Length, overall
	mm
	
	
	2,170
	

	Width, overall, with mirrors
	mm
	
	
	970
	

	Width, overall, without mirrors
	mm
	
	
	825
	

	Seat height
	mm
	
	
	800
	

	Weight, unladen, with full tank
	kg
	
	
	238
	

	Max permissible weight, overall
	kg
	
	
	450
	

	Tank capacity
	ltr
	
	
	20,4
	

	
	
	
	
	
	

	Performance data
	
	
	
	
	

	Fuel consumption
	
	
	
	
	

	90 km/h
	ltr/100 km
	
	
	4.6
	

	120 km/h
	ltr/100 km
	
	
	5.7
	

	Acceleration
	
	
	
	
	

	0–100 km/h
	sec
	
	
	4.23
	

	Standing-start km
	sec
	
	
	24.86
	

	Top speed
	km/h
	
	
	197
	


Specifications BMW Motorcycles.

R 850 R, R 850 R Comfort.

	
	
	R 850 R
	R 850 R
25 kW/34 bhp
	R 850 R Comfort



	Engine
	
	
	 
	

	Capacity
	cc
	848
	
	

	Bore/stroke
	mm
	87.5/70.5
	
	

	Max output
	kW/bhp
	52/70
	25/34
	53/70

	at
	rpm
	7,000
	5,000
	7,000

	Max torque
	Nm
	77
	60
	78 

	at
	rpm
	5,600
	3,000
	5,600

	Engine configuration
	
	Flat-twin
	
	

	No of cylinders
	
	2
	
	

	Compression ratio/fuel grade
	
	10.3/premium
	
	

	Valve/gas management
	
	HC (high camshaft)
	
	

	Valves per cylinder
	
	4
	
	

	Intake/outlet dia
	mm
	32/27
	
	

	Fuel supply
	
	Motronic MA 2.4
	
	

	
	
	
	
	

	Electrical system
	
	
	
	

	Alternator
	W
	700
	
	

	Battery
	V/Ah
	12/19 maintenance-free 
	
	

	Headlight
	W
	H 4 
	
	

	Starter
	kW
	1.1
	
	

	
	
	
	
	

	Power transmission
	
	
	
	

	Clutch
	
	Single-plate dry clutch, dia 165 mm            
	
	

	Transmission
	
	Six-speed gearbox with dog-type shift
	
	

	Primary ratio
	
	1.889
	
	

	Transmission ratios
I
	
	2.038
	
	

	
II
	
	1.600
	
	

	
III
	
	1.267
	
	

	
IV
	
	1.034
	
	

	
V
	
	0.903
	
	

	
VI
	
	0.806
	
	

	Final drive
	
	Driveshaft
	
	

	Transmission ratio
	
	3.09
	
	

	
	
	
	
	

	Suspension and running gear
	
	
	
	

	Frame
	
	Composite frame, front section in aluminium load-bearing engine
	
	

	Wheel suspension, front
	
	                                                BMW Telelever 
	

	Wheel suspension, rear
	
	BMW  Paralever
	
	

	Spring travel, front/rear
	mm
	120/135
	
	

	Castor in normal trim
	mm
	127
	
	

	Wheelbase in normal trim
	mm
	1,487
	
	1,486

	Steering head angle in normal trim
	°
	61.9
	
	63.1

	Brakes
	front
	Double-disc brake, dia 320 mm                     
	

	
	rear
	Single-disc brake, dia 276 mm                           
	

	Wheels
	
	Light alloy

	
	front
	3.50 x 17
	
	3.50 x 18

	
	rear
	5.00 x 17
	
	5.00 x 18

	Tyres
	front
	120/70 ZR 17
	
	120/70 ZR 18

	
	rear
	170/60 ZR 17
	
	170/60 ZR 18

	
	
	
	
	

	Dimensions and weight
	
	
	
	

	Length, overall
	mm
	2,170
	
	

	Width, overall, with mirrors
	mm
	940
	
	870

	
	
	
	
	

	Seat height
	mm
	800
	
	780

	Weight, unladen, in road trim
	kg
	238
	
	

	Max permissible weight, overall
	kg
	450
	
	

	Tank capacity
	ltr
	20.5
	
	17.6

	
	
	
	
	

	Performance data
	
	
	
	

	Fuel consumption
	
	
	
	

	90 km/h
	ltr/100 km
	4.6
	
	

	120 km/h
	ltr/100 km
	5.7
	
	

	Acceleration
	
	
	
	

	0–100 km/h
	sec
	5.0
	
	

	Standing-start km
	sec
	26.0
	
	

	Top speed
	km/h
	187
	
	185


Specifications BMW Motorcycles.

R 1150 R Rockster.

	
	
	R 1150 R Rockster
	

	Engine
	
	
	

	Capacity
	cc
	1,130
	

	Bore/stroke
	mm
	101/70.5
	

	Max output
	kW/bhp
	62.5/85
	

	at
	rpm
	6,750
	

	Max torque
	Nm
	98
	

	at
	rpm
	5,250
	

	Engine configuration
	
	Flat-twin
	

	No of cylinders
	
	2
	

	Compression ratio/fuel grade
	
	10.3/premium
	

	Valve/gas management
	
	HC (high camshaft)
	

	Valves per cylinder
	
	4
	

	Intake/outlet dia
	mm
	34/29
	

	Fuel supply
	
	Motronic MA 2.4
	

	
	
	
	

	Electrical system
	
	
	

	Alternator
	W
	700
	

	Battery
	V/Ah
	12/19 maintenance-free
	

	Headlight
	W
	H 1/H 7
	

	Starter
	kW
	1.1
	

	
	
	
	

	Power transmission
	
	
	

	Clutch
	
	                   Single-plate dry clutch, dia  165 mm
	

	Transmission
	
	Six-speed gearbox with dog-type shift
	

	Primary ratio
	
	1.889
	

	Transmission ratios
I
	
	2.038
	

	
II
	
	1.600
	

	
III
	
	1.267
	

	
IV
	
	1.034
	

	
V
	
	0.903
	

	                                             VI
	 
	0.806
	

	Final drive
	
	Driveshaft
	

	Transmission ratio
	
	2.82
	

	
	
	
	

	Suspension and running gear
	
	
	

	Frame
	
	Composite frame, front section in aluminium load-bearing engine
	

	Wheel suspension, front
	
	BMW Telelever
	

	Wheel suspension, rear
	
	BMW Paralever
	

	Spring travel, front/rear
	mm
	120/135
	

	Castor
	mm
	127
	

	Wheelbase
	mm
	1,487
	

	Steering head angle
	°
	63.1
	

	Brakes
	front
	             Double-disc brake, dia                       
	320 mm
	

	
	rear
	      Single-disc brake, dia                             
	276 mm
	

	
	
	Optional BMW Motorcycle Integral ABS 
(partly integral)
	

	Wheels
	
	

	
	front
	3.50 x 17
	

	
	rear
	5.5 x 17
	

	Tyres
	front
	120/70 ZR 17
	

	
	rear
	      180/55 ZR 17
	

	
	
	
	

	Dimensions and weight
	
	
	

	Length, overall
	mm
	2,170
	

	Width, overall, with mirrors
	mm
	920
	

	Width, overall, without mirrors
	mm
	825
	

	Seat height
	mm
	835
	

	Weight, unladen, with full tank
	kg
	239
	

	Max permissible weight, overall
	kg
	450
	

	Tank capacity
	ltr
	20.4
	

	
	
	
	

	Performance data
	
	
	

	Fuel consumption
	
	
	

	90 km/h
	ltr/100 km
	4.4
	

	120 km/h
	ltr/100 km
	5.6
	

	Acceleration
	
	
	

	0–100 km/h
	sec
	4.23
	

	Standing-start km
	sec
	24.86
	

	Top speed
	km/h
	197
	


Specifications BMW Motorcycles.

R 1100 S.
	
	
	
	R 1100 S
	
	

	Engine
	
	
	 
	
	

	Capacity
	cc
	
	1,085
	
	

	Bore/stroke
	mm
	
	99/70.5
	
	

	Max output
	kW/bhp
	
	72/98
	
	

	at
	rpm
	
	7,500
	
	

	Max torque
	Nm
	
	97
	
	

	at
	rpm
	
	5,750
	
	

	Engine configuration
	
	
	Flat-twin
	
	

	No of cylinders
	
	
	2
	
	

	Compression ratio/fuel grade
	
	
	11.3/premium
	
	

	Valve/gas management
	
	
	HC (high camshaft)
	
	

	Valves per cylinder
	
	
	4
	
	

	Intake/outlet dia
	mm
	
	34/29
	
	

	Fuel supply
	
	
	Motronic MA 2.4
	
	

	
	
	
	
	
	

	Electrical system
	
	
	
	
	

	Alternator
	W
	
	600
	
	

	Battery
	V/Ah
	
	12/15 maintenance- free
	
	

	Headlight
	W
	
	H1/H 7/55
	
	

	Starter
	kW
	
	1.1
	
	

	
	
	
	
	
	

	Power transmission
	
	
	
	
	

	Clutch
	
	Single-plate dry clutch, dia 165 mm
	
	

	Transmission
	
	Six-speed gearbox with dog-type shift
	
	

	Primary ratio
	
	
	1.889
             
	
	

	Transmission ratios
I
	
	
	2.038
	
	

	
II
	
	
	1.600
	
	

	
III
	
	
	1.267
	
	

	
IV
	
	
	1.034
	
	

	
V
	
	
	0.930
	
	

	
VI
	
	
	0.806
	
	

	Final drive
	
	
	Driveshaft
	
	

	Transmission ratio 
	
	
	2.75
	
	

	
	
	
	
	
	

	Suspension and running gear
	
	
	
	
	

	Frame
	
	Aluminium and cast aluminium frame, 
load-bearing engine
	
	

	Wheel suspension, front
	
	
	BMW Telelever
	
	

	Wheel suspension, rear
	
	
	BMW Paralever
	
	

	Spring travel, front/rear
	mm
	
	110/130
	
	

	Castor
	mm
	
	100
	
	

	Wheelbase
	mm
	
	1 478
	
	

	Steering head angle
	°
	
	65
	
	

	Brakes
	front
	
              Double-disc brake, dia 320 mm
	
	

	
	rear
	
                        Single-disc brake, dia  276 mm 
	
	

	
	
	Optional BMW Motorcycle ABS (partly integral)
	
	

	Wheels
	
	
	Light alloy
	
	

	
	front
	
	3.50 x 17
	
	

	
	rear
	
	5.00 x 17
	
	

	Tyres
	front
	
	120/70 ZR 17
	
	

	
	rear
	
	170/60 ZR 17
	
	

	
	
	
	
	
	

	Dimensions and weight
	
	
	
	
	

	Length, overall
	mm
	
	2,180
	
	

	Width, overall, with mirrors
	mm
	
	880
	
	

	Width, overall, without mirrors
	mm
	
	760
	
	

	Seat height
	mm
	
	800
	
	

	Weight, unladen, with full tank
	kg
	
	229
	
	

	Max permissible weight, overall
	kg
	
	450
	
	

	Tank capacity
	ltr
	
	18
	
	

	
	
	
	
	
	

	Performance data
	
	
	
	
	

	Fuel consumption
	
	
	
	
	

	90 km/h
	ltr/100 km
	
	4.3
	
	

	120 km/h
	ltr/100 km
	
	5.2
	
	

	Acceleration
	
	
	
	
	

	0–100 km/h
	sec
	
	4.0
	
	

	Standing-start km
	sec
	
	22.8
	
	

	Top speed
	km/h
	
	226
	
	


Specifications BMW Motorcycles.

R 1100 S BoxerCup Replica.

	
	
	R 1100 S BoxerCup Replica
	
	

	Engine
	
	
	 
	

	Capacity
	cc
	1,085
	
	

	Bore/stroke
	mm
	99/70.5
	
	

	Max output
	kW/bhp
	72/98
	
	

	at
	rpm
	7,500
	
	

	Max torque
	Nm
	97
	
	

	at
	rpm
	5,750
	
	

	Engine configuration
	
	Flat-twin
	
	

	No of cylinders
	
	2
	
	

	Compression ratio/fuel grade
	
	11.3/premium
	
	

	Valve/gas management
	
	HC (high camshaft)
	
	

	Valves per cylinder
	
	4
	
	

	Intake/outlet dia
	mm
	34/29
	
	

	Fuel supply
	
	Motronic MA 2.4
	
	

	
	
	
	
	

	Electrical system
	
	
	
	

	Alternator
	W
	600
	
	

	Battery
	V/Ah
	12/15 maintenance-free
	
	

	Headlight
	W
	H1/H 7/55
	
	

	Starter
	kW
	1.1
	
	

	
	
	
	
	

	Power transmission
	
	
	
	

	Clutch
	
	Single-plate dry clutch, dia 165 mm
	
	

	Transmission
	
	Six-speed gearbox with dog-type shift
	
	

	Primary ratio
	
	1.889
	
	

	Transmission ratios
I
	
	2.038
	
	

	
II
	
	1.600
	
	

	
III
	
	1.267
	
	

	
IV
	
	1.034
	
	

	
V
	
	0.903
	
	

	
VI
	
	0.806
	
	

	Final drive
	
	Driveshaft                                                                     
	
	

	Transmission ratio 
	
	2.75
	
	

	
	
	
	
	

	Suspension and running gear
	
	
	
	

	Frame
	
	Aluminium and cast aluminium frame,  
load-bearing engine
	
	

	Wheel suspension, front   
	
	BMW Telelever
	

	Wheel suspension, rear
	
	BMW Paralever
	

	Spring travel, front/rear
	mm
	110/130
	
	

	Castor in normal trim
	mm
	100
	
	

	Wheelbase in normal trim
	mm
	1,478
	
	

	Steering head angle in normal trim
	°
	65
	
	

	Brakes
	front
	Double-disc brake, dia 320 mm
	
	

	
	rear
	Single-disc brake, dia  276 mm 
	

	Wheels
	
	Light alloy
	
	

	
	front
	3.5 x 17
	
	

	
	rear
	5.5 x 17
	
	

	Tyres
	front
	120/70 ZR 17
	
	

	
	rear
	180/55 ZR 17
	
	

	
	
	
	
	

	Dimensions and weight
	
	
	
	

	Length, overall
	mm
	2,180
	
	

	Width, overall, with mirrors
	mm
	895
	
	

	Width, overall, without mirrors
	mm
	760
	
	

	Seat height
	mm
	860
	
	

	Weight unladen, in road trim
	kg
	229
	
	

	Max permissible weight, overall
	kg
	450
	
	

	Tank capacity
	ltr
	18
	
	

	
	
	
	
	

	Performance data
	
	
	
	

	Fuel consumption
	
	
	
	

	90 km/h
	ltr/100 km
	4.3
	
	

	120 km/h
	ltr/100 km
	5.2
	
	

	Acceleration
	
	
	
	

	0–100 km/h
	sec
	4.0
	
	

	Standing-start km
	sec
	22.8
	
	

	Top speed
	km/h
	226
	
	


Specifications BMW Motorcycles.

R 1150 RS, R 1150 RT.
	
	
	R 1150 RS
	R 1150 RT
	
	

	Engine
	
	
	 
	
	

	Capacity
	cc
	1,130
	1,130
	
	

	Bore/stroke
	mm
	101/70.5
	101/70.5
	
	

	Max output
	kW/bhp
	70/95
	70/95
	
	

	at
	rpm
	7,250
	7,250
	
	

	Max torque
	Nm
	100
	100
	
	

	at
	rpm
	5,500
	5,500
	
	

	Engine configuration
	
	Flat-twin
	Flat-twin
	
	

	No of cylinders
	
	2
	2
	
	

	Compression ratio/fuel grade
	
	11.3/premium
	11.3/premium
	
	

	Valve/gas management
	
	HC (high camshaft)
	HC (high camshaft)
	
	

	Valves per cylinder
	
	4
	4
	
	

	Intake/outlet dia
	mm
	34/29
	34/29
	
	

	Fuel supply
	
	Motronic MA 2.4
	Motronic MA 2.4
	
	

	
	
	
	
	
	

	Electrical system
	
	
	
	
	

	Alternator
	W
	700
	700
	
	

	Battery
	V/Ah
	12/19 maintenance-free
	12/19 maintenance-free
	
	

	Headlight
	W
	H 4 
	H 7/H 1/55 
	
	

	Starter
	kW
	1.1
	1.1
	
	

	
	
	
	
	
	

	Power transmission
	
	
	
	
	

	Clutch
	
	Single-plate dry clutch, dia 165 mm
	
	

	Transmission
	
	Six-speed gearbox with dog-type shift
	
	
	

	Primary ratio
	
	1.889
	1.889
	
	

	Transmission ratios
I
	
	2.038
	2.038
	
	

	
II
	
	1.600
	1.600
	
	

	
III
	
	1.267
	1.267
	
	

	
IV
	
	1.034
	1.034
	
	

	
V
	
	0.903
	0.903
	
	

	

     VI
	 
	0.698
	0.698
	
	

	Final drive
	
	Driveshaft 
	Driveshaft
	
	

	Transmission ratio 
	
	2.82
	2.91
	
	

	
	
	
	
	
	

	Suspension and running gear
	
	
	
	
	

	Frame
	
	Composite frame, cast aluminium front section, load-bearing engine
	
	

	Wheel suspension, front
	
	BMW Telelever
	BMW Telelever
	
	

	Wheel suspension, rear
	
	BMW Paralever
	BMW Paralever
	
	

	Spring travel, front/rear
	mm
	120/135
	120/135
	
	

	Castor
	mm
	111
	122
	
	

	Wheelbase
	mm
	1 469
	1 485
	
	

	Steering head angle
	°
	64.5
	62.9
	
	

	Brakes
	front
	
Double-disc brake, dia 320 mm
	
	

	
	rear
	Single-disc brake, dia  276 mm  
	
	
	

	
	
	Optional BMW Motorcycle Integral ABS (partly integral) 
	Standard BMW Motorcycle Integral ABS (fully integral)     
	   
	

	Wheels
	
	Light alloy
	Light alloy
	
	

	
	front
	3.50 x 17
	3.50 x 17
	
	

	
	rear
	5.00 x 17
	5.00 x 17
	
	

	Tyres
	front
	120/70 ZR 17
	120/70 ZR 17
	
	

	
	rear
	170/60 ZR 17
	170/60 ZR 17
	
	

	
	
	Tubeless
	Tubeless
	
	

	
	
	
	
	
	

	Dimensions and weight
	
	
	
	
	

	Length, overall
	mm
	2,170
	2,230
	
	

	Width, overall, with mirrors
	mm
	920
	898
	
	

	Width, overall, without mirrors
	mm
	758
	898
	
	

	Seat height
	mm
	790/810/830
	805/825/845
	
	

	Weight, unladen, with full tank
	kg
	246
	279
	
	

	Max permissible weight, overall
	kg
	450
	495
	
	

	Tank capacity
	ltr
	23
	25,2
	
	

	
	
	
	
	
	

	Performance data
	
	
	
	
	

	Fuel consumption
	
	
	
	
	

	90 km/h
	ltr/100 km
	4.3
	4.7
	
	

	120 km/h
	ltr/100 km
	5.2
	5.6
	
	

	Acceleration
	
	
	
	
	

	0–100 km/h
	sec
	4.1
	4.3
	
	

	Standing-start km
	sec
	23.3
	24.4
	
	

	Top speed
	km/h
	215
	200
	
	


Specifications BMW Motorcycles.

R 1200 C Classic, R 1200 C Independent.

	
	
	R 1200 C Classic   

R 1200 C Independent
	
	
	

	Engine
	
	
	 
	
	

	Capacity
	cc
	1,170
	
	
	

	Bore/stroke
	mm
	101/73
	
	
	

	Max output
	kW/bhp
	45/61
	
	
	

	at
	rpm
	5,000
	
	
	

	Max torque
	Nm
	98
	
	
	

	at
	rpm
	3,000
	
	
	

	Engine configuration
	
	Flat-twin
	
	
	

	No of cylinders
	
	2
	
	
	

	Compression ratio/fuel grade
	
	10.0/premium
	
	
	

	Valve/gas management
	
	HC (high camshaft)
	
	
	

	Valves per cylinder
	
	4
	
	
	

	Intake/outlet dia
	mm
	34/29
	
	
	

	Fuel supply
	
	Motronic MA 2.4
	
	
	

	
	
	
	
	
	

	Electrical system
	
	
	
	
	

	Alternator
	W
	700
	
	
	

	Battery
	V/Ah
	12/19 maintenance-free
	
	
	

	Headlight
	W
	H 4 55/60
	
	
	

	Starter
	kW
	1.1
	
	
	

	
	
	
	
	
	

	Power transmission
	
	
	
	
	

	Clutch
	
	Single-plate dry clutch, dia 165 mm
	
	

	Transmission
	
	
Five-speed gearbox with dog-type shift
	
	

	Primary ratio
	
	1.889
	
	
	

	Transmission ratios
I
	
	2.038
	
	
	

	
II
	
	1.600
	
	
	

	
III
	
	1.267
	
	
	

	
IV
	
	1.034
	
	
	

	
V
	
	0.806
	
	
	

	Final drive
	
	Driveshaft 
	
	
	

	Transmission ratio
	
	2.54
	
	
	

	
	
	
	
	
	

	Suspension and running gear
	
	
	
	
	

	Frame
	
	Composite frame, front section in aluminium load-bearing engine
	
	

	Wheel suspension, front
	
	BMW Telelever
	
	
	

	Wheel suspension, rear
	
	BMW Monolever
	
	
	

	Spring travel, front/rear
	mm
	144/100
	
	
	

	Castor
	mm
	86
	
	
	

	Wheelbase
	mm
	1, 650
	
	
	

	Steering head angle
	°
	60.5
	
	
	

	Brakes
	front
	
Double-disc brake, dia 305 mm
	
	

	
	rear
	
Single-disc brake, dia  285 mm 
	
	

	
	
	Optional BMW Motorcycle Integral ABS (fully integral)
	
	
	

	Wheels
	
	Cross-spoke wheels1
	
	
	

	
	front
	2.5 x 18
	
	
	

	
	rear
	4.0 x 15
	
	
	

	Tyres
	front
	100/90 ZR 18
	
	
	

	
	rear
	170/80 ZR 15
	
	
	

	
	
	
	
	
	

	Dimensions and weight
	
	
	
	
	

	Length, overall
	mm
	2,340
	
	
	

	Width, overall, with mirrors
	mm
	1,050
	
	
	

	Width, overall, without mirrors
	mm
	7752
	
	
	

	Seat height
	mm
	740
	
	
	

	Weight, unladen, with full tank
	kg
	256
	
	
	

	Max permissible weight, overall
	kg
	450
	
	
	

	Tank capacity
	ltr
	17.5
	
	
	

	
	
	
	
	
	

	Performance data
	
	
	
	
	

	Fuel consumption
	
	
	
	
	

	90 km/h
	ltr/100 km
	4.8
	
	
	

	120 km/h
	ltr/100 km
	5.9
	
	
	

	Acceleration
	
	
	
	
	

	0–100 km/h
	sec
	5.8
	
	
	

	Standing-start km
	sec
	27.9
	
	
	

	Top speed
	km/h
	168
	
	
	


1
R 1200 C Independent, aluminium wheels.

2
R 1200 C Independent 980 mm.

Specifications BMW Motorcycles.
R 1200 CL, R 1200 C Montauk.

	
	
	R 1200 CL
	R 1200 C Montauk

	Engine
	
	
	

	Capacity
	cc
	1,170
	

	Bore/stroke
	mm
	101/73
	

	Max output
	kW/bhp
	45/61
	

	at
	rpm
	5,000
	

	Max torque
	Nm
	98
	

	at
	rpm
	3,000
	

	Engine configuration
	
	Flat-twin
	

	No of cylinders
	
	2
	

	Compression ratio/fuel grade
	
	10.0/premium (RON 95)
	

	Valve/gas management
	
	HC (high camshaft)
	

	Valves per cylinder
	
	4
	

	Intake/outlet dia
	mm
	34/29
	

	Fuel supply
	
	Motronic MA 2.4
	

	
	
	
	

	Electrical system
	
	
	

	Alternator
	W
	800
	700

	Battery
	V/Ah
	12/19 maintenance-free
	

	Headlight
	W
	2xH1/2xH4
	 1xH4 55/60    1xH1 55

	Starter
	KW
	1.1
	

	
	
	
	

	Power transmission
	
	
	

	Clutch
	
	Single-plate dry clutch, dia 165 mm
	

	Transmission
	
	Six-speed gearbox with dog-type shift
	Five-speed gearbox with dog-type shift

	Primary ratio
	
	1.889
	1.889

	Transmission ratios                      I
	
	2.038
	2.038

	
          II
	
	1.600
	1.600

	
          III
	
	1.267
	1.267

	
          IV
	
	1.034
	1.034

	
          V
	
	0.903
	0.806

	
          VI
	
	0.698
	–

	Final drive
	
	Driveshaft
	Driveshaft

	Transmission ratio 
	
	2.62
	2.54

	
	
	

	Suspension and running gear
	
	

	Frame
	
	Composite frame, cast aluminium front section, load-bearing engine
	

	Wheel suspension, front
	
	BMW Telelever
	

	Wheel suspension, rear
	
	BMW Monolever
	

	Spring travel, front/rear
	mm
	144/120
	

	Castor in normal trim
	mm
	184
	

	Wheelbase in normal trim
	mm
	1,641
	

	Steering head angle in normal trim
	°
	56.5
	

	Brakes
	front
	dia 305 mm
	

	
	rear
	dia 285 mm
	

	
	
	Optional BMW Motorcycle Integral ABS 
(fully integral) 
	

	Wheels
	
	Aluminium cast wheels
	

	
	front
	3.5 x 16 
	

	
	rear
	4.0 x 15
	

	Tyres
	front
	150/80-16 
	

	
	rear
	170/80-15 
	

	
	
	

	Dimensions and weight
	
	

	Length, overall
	mm
	2,415
	2,370

	Width, overall, with mirrors
	mm
	1,075
	1,120

	Width, overall, without mirrors
	mm
	853
	980

	Seat height
	mm
	745
	790

	Weight, unladen, in road trim
	Kg
	308
	265

	Max permissible weight, overall
	Kg
	530
	460

	Tank capacity
	L
	17.5
	

	
	
	

	Performance data
	
	

	Fuel consumption
	
	

	90 km/h
	ltr/100 km
	5.1
	4.6

	120 km/h
	ltr/100 km
	6.2
	5.7

	Acceleration
	
	

	0–100 km/h
	sec
	6.4
	5.8

	Standing-start km
	sec
	28.8
	27.9

	Top speed
	km/h
	165
	168


7. Colours in the 2004 Model Year.

	F-Models
	Colours
	Seat
	Side covers

	F 650 CS 
	Night Blue metallic
	Black or Grey or Mandarin
	Graphitane metallic or White Aluminium

	
	Genet Yellow metallic
	Black or Grey or Mandarin
	Graphitane metallic or White Aluminium

	
	Titanium Silver metallic
	Black or Grey or Mandarin
	Graphitane Metallic or White Aluminium

	
	Stone Grey non-metallic
	Black or Grey or Mandarin
	Graphitane metallic or White Aluminium

	F 650 GS
	Iceberg Silver metallic
	Terracotta Orange or Black
	

	
	Yellow non-metallic
	Black
	

	
	Deep Black non-metallic
	Terracotta Orange or Black
	

	F 650 GS Dakar 
	Blue metallic/White Aluminium 
metallic matte 
	Black
	

	
	
	
	

	R-Models
	Colour
	Seat
	

	R 850 R/R 1150 R
	Titanium Silver metallic
	Bronzite Beige or Black
	

	
	Night Blue non-metallic
	Black
	

	
	Piemont Red non-metallic
	Black
	

	R 1150 R
	Dark Ferro metallic/Ferro metallic
	Black
	

	R 1150 R Rockster
	Citrus metallic/black matte
	Black
	

	
	Orange metallic/black matte
	Schwarz
	

	R 1200 GS
	Desert Yellow non-metallic, metallisée
	Light Grey or Black
	Granite Grey metallic 
or Black soft paint

	
	Rock Red non-metallic, metallisée
	Light Grey or Black
	Granite Grey metallic 
or Black soft paint

	
	Ocean Blue metallic
	Light Grey or Black
	Granite Grey metallic 
or Black soft paint

	R 1150 GS Adventure
	White Aluminium metallic
	Grey/Red
	

	
	Night Black non-metallic
	Yellow/Black or Black
	

	R 1100 S
	Pacific Blue metallic
	Black
	

	
	Titanium Silver metallic
	Black
	

	
	Piemont Red metallic
	Black
	

	
	Titanium Grey metallic/Mandarin non-metallic
	Black
	

	R 1100 S BoxerCup Replica
	Pacific Blue metallic/Alpine White/Japan Red 
	Black
	

	R 1200 C
	Night Black non-metallic
	Black or Canyon Brown
	

	
	Sorrent Blue metallic
	Black
	

	
	Mocca Brown metallic
	Saddle Brown or Black
	

	
	White Aluminium metallic matte
	Black
	

	R 1200 C Independent
	Piemont Red metallic/Titanium Silver metallic
	Black
	

	
	Orient Blue metallic/Ivory non-metallic
	Dark Blue
	

	R 1200 CL
	Pearl Silver metallic
	Black
	

	
	Sapphire Black metallic
	Black
	

	
	Mojave Brown metallic
	Saddle Brown or Black
	

	R 1200 C Montauk
	Sapphire Black metallic
	Black
	

	
	Champagne metallic
	Black
	

	
	Arctic Blue metallic
	Black
	

	R 1150 RS
	Pacific Blue metallic
	Black
	

	
	Dark Blue metallic/Frost Blue metallic
	Black
	

	
	Phoenix Grey metallic/Ivory non-metallic
	Black
	

	
	Titanium Silver metallic
	Black
	

	R 1150 RT
	Titanium Silver metallic
	Black
	

	
	Titanium Grey metallic
	Black
	

	
	Biarritz Blue metallic
	Black
	

	
	Piemont Red metallic
	Black 
	

	
	
	
	

	K-Models
	Colour
	Seat
	

	K 1200 RS
	Piemont Red metallic
	Black
	

	
	Alpine White non-metallic/Orient Blue metallic
	Black
	

	
	Capri Blue metallic
	Black
	

	K 1200 GT
	Orient Blue metallic
	Black
	

	
	Grey Green metallic
	Black
	

	K 1200 LT 
	Ocean Blue metallic
	Greige or Black
	

	
	Dark Graphite metallic
	Greige or Black
	

	
	Light Yellow metallic
	Greige or Black
	






































