[image: image1.png]


BMW
Media
Information

1/2010

Valid from 3/2010
BMW
Media
Information

1/2010

Valid from 3/2010

Specifications BMW 3 Series Convertible.
320d, 325d, 330d
	
	
	BMW 320d Convertible
	BMW 325d Convertible
	BMW 330d Convertible

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	2 / 4
	2 / 4
	2 / 4

	Length/width/height (unladen)
	mm
	4612 / 1782 / 1384
	4612 / 1782 / 1384
	4612 / 1782 / 1384

	Wheelbase
	mm
	2760
	2760
	2760

	Track, front/rear
	mm
	1500 / 1513
	1500 / 1513
	1500 / 1513

	Ground clearance
	
	138
	138
	138

	Turning circle
	m
	11.0
	11.0
	11.0

	Tank capacity
     appr ltr
	61
	61
	61

	Cooling system incl heater
	ltr
	7.2 (7.5)
	7.9 (8.2)
	7.9 (8.2)

	Engine oil1)
	ltr
	5.5
	7.5
	7.5

	Weight, unladen, to DIN/EU
	kg
	1655 / 1730

(1670 / 1745)
	1740 / 1815

(1755 / 1830)
	1750 / 1825

(1765 / 1840)

	Max load to DIN
	kg
	430
	430
	430

	Max permissible
	kg
	2085 (2100)
	2170 (2180)
	2180 (2195)

	Max axle load, front / rear
	kg
	920 / 1185
	1005 / 1200
	1005 / 1200

	Max trailer load (12%),

braked / unbraked
	kg
	1600 / 745
	1800 / 750
	1800 / 750

	Max roofload / max download
	kg
	- / 75
	- / 75
	- / 75 

	Luggage comp
	ltr
	210-350
	210-350
	210-350

	Air drag
	cd x A
	0.28 x 2.08
	0.28 x 2.08
	0.28 x 2.08

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	 Straight-4 / 4
	Straight-6 / 4
	Straight-6 / 4

	Engine technology
	
	Common-rail direct fuel injection, turbocharger

with variable intake geometry
	Common-rail direct fuel injection, turbocharger

with variable intake geometry
	Common-rail direct fuel injection, turbocharger

with variable intake geometry

	Capacity, effective
	cc
	1995
	2993
	2993

	Stroke / bore
	mm
	90.0 / 84.0
	90.0 / 84.0
	90.0 / 84.0

	Compression ratio
	:1
	16.5
	17.0
	16.5 

	Fuel
	
	Diesel
	Diesel
	Diesel

	Max output
	kW/hp
	135 / 184
	150 / 204
	180 / 245

	at
	rpm
	4000
	3750
	4000

	Max torque
	Nm/lb-ft
	380 / 280
	430 / 317
	520 / 383

	at 
	rpm
	1900-2750
	1750
	1750-3000

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery / installation
	Ah/–
	90 (80) / luggage comp
	90 / luggage comp
	90 / luggage comp

	Alternator
	A/W
	180 / 2520
	150 / 2100
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double-joint spring strut axle in aluminium with displaced castor, small positive steering roll radius, compensation of lateral forces, anti-dive

	Suspension, rear
	
	Five-arm axle in lightweight steel

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	312 x 24 / vented
	330 x 24 / vented
	348 x 30 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	300 x 20 / vented
	336 x 22 / vented
	336 x 22 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering 

Brake Control, DBC Brake Assistant, BMW 325d and BMW 330d also with 

Dry Braking, Fading Compensation, Start-Off Assistant 



	Safety
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger with combined head airbags, three-point inertia-reel belts on all seats, integrated on front seats with belt retainer, belt latch tensioner and belt force limiter, crash-activated headrests at the front, crash sensors, rollover safety system, 

Tyre Defect Indicator

	Steering
	
	Electric Power Steering

	Steering ratio, overall
	:1
	16.0
	16.0
	16.0

	Tyres, front/rear
	
	225/45 R17 91W RSC
	225/45 R17 91W RSC
	225/45 R17 91Y RSC

	Wheels, front/rear
	
	8J x 17 light-alloy
	8J x 17 light-alloy
	8J x 17 light-alloy


Specifications BMW 3 Series Convertible.
	
	
	BMW 320d Convertible
	BMW 325d Convertible 

Convertible Cabrio
	BMW 330d Convertible

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist incl Enquiry Service, remote control functions and V-

Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google directory search, news, realtime weather, office functions, BMW Routes

and music title update online

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant,

Park Distance Control, cruise control, automatic/enhanced emergency call function 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Six-speed manual (optional: six-speed automatic)

	Gear ratios
I
	:1
	4.110 (4.171)
	5.080 (4.171) 
	5.080 (4.171) 

	                                  
II
	:1
	2.248 (2.340)
	2.804 (2.340)
	2.804 (2.340)

	                                  
III
	:1
	1.403 (1.521)
	1.783 (1.521)
	1.783 (1.521)

	                                  
IV
	:1
	1.000 (1.143)
	1.260 (1.143)
	1.260 (1.143)

	                                  
V
	:1
	0.802 (0.867)
	1.000 (0.867)
	1.000 (0.867)

	                                   
VI
	:1
	0.659 (0.691)
	0.835 (0.691)
	0.835 (0.691)

	                                   
R
	:1
	3.727 (3.403)
	4.607 (3.403)
	4.607 (3.403)

	Final drive
	:1
	3.385 (3.154)
	2.471 (2.929)
	2.471 (2.929)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	12.3 (12.4)
	11.6 (11.7)
	9.7 (9.8)

	Output per litre
	kW/hp
	67.7 / 92.1
	50.1 / 68.1
	60.1 / 81.7

	Acceleration
0–100 km/h
	sec
	8.3 (8.4)
	7.5 (7.6)
	6.4 (6.5)

	
standing-start
	sec


	29.1 (29.3)
	28.0 (28.1)
	26.5 (26.7)

	In 4th/5th gear
80–120 km/h 
	sec
	6.6 / 8.4 (-)
	6.3 / 7.9 (-)
	5.2 / 6.6 (-) 

	Top speed
	km/h
	228 (226)
	238 (236)
	250

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, Auto Start Stop (on BMW 320d manual), 

gearshift point indicator (manual gearbox models), Electric Power Steering, 

on-demand management of ancillary units, intelligent lightweight construction, 

tyres with reduced roll resistance, a/c compressor detachable

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	ltr/100km
	6.3 (7.1) 
	7.7 (8.2) 
	7.7 (8.2)

	Extra-urban
	ltr/100km
	4.4 (4.7)
	5.2 (5.4)
	5.2 (5.4) 

	Combined
	ltr/100km
	5.1 (5.6)
	6.1 (6.4)
	6.1 (6.4) 

	CO2
	g/km
	135 (149)
	160 (168)
	162 (170)

	Emission rating
	
	EU5
	EU5
	EU5  

	
	
	
	
	


Specifications apply to ACEA markets/data relevant to homologation apply in part only to Germany (weight) 

Figures in brackets apply to models with automatic transmission

1)  Oil change




















































