[image: image1.png]


BMW
Media
Information

1/2010

Valid from 3/2010
BMW
Media
Information

1/2010

Valid from 3/2010

Specifications BMW 3 Series Coupé.
325i, 330i, 335i
	
	
	BMW 325i Coupé
	BMW 330i Coupé
	BMW 335i Coupé

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	2 / 4
	2 / 4
	2 / 4

	Length/width/height (unladen)
	mm
	4612 / 1782 / 1395
	4612 / 1782 / 1395
	4612 / 1782 / 1395

	Wheelbase
	mm
	2760
	2760
	2760

	Track, front/rear
	mm
	1500 / 1513
	1500 / 1513
	1500 / 1507

	Ground clearance
	
	143
	143
	125

	Turning circle
	m
	11.0
	11.0
	11.0

	Tank capacity
   appr ltr
	63
	63
	63

	Cooling system incl heater
	ltr
	8.2 (8.5)
	8.2 (8.5)
	8.2

	Engine oil1)
	ltr
	6.5
	6.5
	6.5

	Weight, unladen, to DIN/EU
	kg
	1430 / 1505

(1460 / 1535)
	1465 / 1540

(1480 / 1555)
	1525 / 1600

(1540 / 1615)

	Max load to DIN
	kg
	480
	480
	480

	Max permissible
	kg
	1910 (1940)
	1945 (1960)
	2005 (2020)

	Max axle load, front / rear
	kg
	905 / 1045
	930 / 1070
	960 / 1080

	Max trailer load (12%),

braked / unbraked
	kg
	1600 / 745
	1700 / 750
	1700 / 750

	Max roofload / max download
	kg
	75 / 75
	75 / 75
	75 / 75 

	Luggage comp
	ltr
	440
	440
	430

	Air drag
	cd x A
	0.27 x 2.08
	0.27 x 2.08
	0.30 x 2.08

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	Straight-6 / 4
	Straight-6 / 4
	Straight-6 / 4

	Engine technology
	
	Second-generation

direct fuel injection

 (High Precision Injection) with lean burn
	Second-generation

direct fuel injection

 (High Precision Injection) with lean burn
	BMW TwinPower Turbo, Second-generation

direct fuel injection

 (High Precision Injection)  fully variable valve

 management (VALVETRONIC)

	Capacity, effective
	cc
	2996
	2996
	2979

	Stroke / bore
	mm
	88.0 / 85.0
	88.0 / 85.0
	89.6 / 84.0

	Compression ratio
	:1
	12.0
	12.0
	10.2 

	Fuel
	
	min 91 RON
	min 91 RON
	min 95 RON

	Max output
	kW/hp
	160 / 218
	200 / 272
	225 / 306

	at
	rpm
	6100
	6700
	5800

	Max torque
	Nm/lb-ft
	270 / 200
	320 / 236
	400 / 295

	at 
	rpm
	2400-4200
	2750-3000
	1200-5000

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery / installation
	Ah/–
	80 / luggage compartment
	80 / luggage comp
	80 / luggage comp

	Alternator
	A/W
	155 / 2170 (180 / 2520)
	155 / 2170 (180 / 2520)
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double-joint spring strut axle in aluminium with displaced castor, small positive steering roll radius, compensation of lateral forces, anti-dive

	Suspension, rear
	
	Five-arm axle in lightweight steel

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	330 x 24 / vented
	348 x 30 / vented
	348 x 30 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	300 x 20 / vented
	336 x 22 / vented
	336 x 22 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, 

CBC Cornering Brake Control, DBC Brake Assistant,

Dry Braking, Fading Compensation, Start-Off Assistant

	Safety
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel belts on all seats, with belt retainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator

	Steering
	
	Electric Power Steering
	Rack-and-pinion

power steering

	Steering ratio, overall
	:1
	16.0
	16.0
	16.0

	Tyres, front/rear
	
	225/45 R17 91W RSC
	225/45 R17 91W RSC
	225/45 R17 91W RSC / 

255/40 R17 94W RSC

	Wheels, front/rear
	
	8J x 17 light-alloy
	8J x 17 light-alloy
	8J/8.5J x 17 light-alloy


Specifications BMW 3 Series Coupé.
	
	
	BMW 325i Coupé
	BMW 330i Coupé
	BMW 335i Coupé

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist incl Enquiry Service, remote control functions and V-

Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google directory search, news, realtime weather, office functions, BMW Routes

and music title update online

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant,

Park Distance Control, cruise control, automatic/enhanced emergency call function 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Six-speed manual (optional: six-speed automatic, 
BMW 335i: seven-speed double-clutch)

	Gear ratios
I
	:1
	4.323 (4.171)
	4.350 (4.171)
	4.110 (4.780) 

	                                  
II
	:1
	2.456 (2.340)
	2.496 (2.340)
	2.315 (3.056)

	                                  
III
	:1
	1.659 (1.521)
	1.665 (1.521)
	1.542 (2.153)

	                                  
IV
	:1
	1.230 (1.143)
	1.230 (1.143)
	1.179 (1.678)

	                                  
V
	:1
	1.000 (0.867)
	1.000 (0.867)
	1.000 (1.390)

	                                   
VI
	:1
	0.848 (0.691)
	0.851 (0.691)
	0.846 (1.203)

	
VII
	:1
	
	
	(1.000)

	                                   
R
	:1
	3.938 (3.403)
	3.926 (3.403)
	3.727 (4.454)

	Final drive
	:1
	3.071 (3.462)
	3.154 (3.636)
	3.077 (2.563)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	8.9 (9.1)
	7.3 (7.4)
	6.8

	Output per litre
	kW/hp
	53.4 / 72.6
	66.8 / 90.8
	75.5 / 102.7

	Acceleration
0–100 km/h
	sec
	6.7 (7.1)
	6.0 (6.2)
	5.5 (5.4)

	
standing-start
	sec


	26.7 (27.2)
	25.4 (25.6)
	24.5 (24.4)

	In 4th/5th gear
80–120 km/h 
	sec
	7.3 / 9.1 (-)
	6.1 / 7.6 (-)
	5.2 / 6.2 (-) 

	Top speed
	km/h
	250
	250
	250

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics,

standard features
	
	Brake Energy Regeneration, gearshift point indicator (manual gearbox models), Electric Power Steering (BMW 325i and 330i), on-demand management of ancillary units, intelligent lightweight construction, tyres with reduced roll resistance, 
oil pump with map control, a/c compressor detachable

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	ltr/100km
	
	
	

	Extra-urban
	ltr/100km
	9.8 (10.0) 
	10.0 (10.2)
	12.0 (11.8)

	Combined
	ltr/100km
	5.7 (5.9)
	5.9 (5.9)
	6.3 (6.4)

	CO2
	g/km
	7.2 (7.4)
	7.4 (7.5)
	8.4 (8.4) 

	Emission rating
	
	168 (174)
	173 (175)
	196 (196)

	
	
	EU5
	EU5
	EU5  

	
	
	
	
	


Specifications apply to ACEA markets/data relevant to homologation apply in part only to Germany (weight) 

Figures in brackets apply to models with automatic transmission

1)  Oil change




















































