[image: image1.png]


BMW
Media
Information

1/2010

Valid from 3/2010
BMW
Media
Information

1/2010

Valid from 3/2010

Specifications BMW 3 Series Saloon.
318i, 320i
	
	
	BMW 318i Saloon
	
	BMW 320i Saloon

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	4 / 5
	
	4 / 5

	Length/width/height (unladen)
	mm
	4531 / 1817 / 1421
	
	4531 / 1817 / 1421

	Wheelbase
	mm
	2760
	
	2760

	Track, front/rear
	mm
	1506 / 1535
	
	1506 / 1535

	Ground clearance
	
	144
	
	144

	Turning circle
	m
	11.0
	
	11.0

	Tank capacity
   appr ltr
	63
	
	63

	Cooling system incl heater
	ltr
	8.4 (9.2)
	
	8.4 (9.2)

	Engine oil1)
	ltr
	4.25
	
	4.25

	Weight, unladen, to DIN/EU
	kg
	1360 / 1435

(1390 / 1465)
	
	1370 / 1445

(1400 / 1475)

	Max load to DIN
	kg
	520
	
	520

	Max permissible
	kg
	1880 (1910)
	
	1890 (1920)

	Max axle load, front / rear
	kg
	895 / 1060
	
	895 / 1060

	Max trailer load (12%),

braked / unbraked
	kg
	1250 (1400) / 695
	
	1400 (1500) / 695

	Max roofload / max download
	kg
	75 / 75
	
	75 / 75 

	Luggage comp
	ltr
	460
	
	460

	Air drag
	cd x A
	0.26 x 2.17
	
	0.26 x 2.17

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	Straight-4 / 4
	
	Straight-4 / 4

	Engine technology
	
	Second-generation 

direct fuel injection 

(High Precision Injection), lean burn
	
	Second-generation 

direct fuel injection 

(High Precision Injection), lean burn

	Capacity, effective
	Cc
	1995
	
	1995

	Stroke / bore
	mm
	90.0 / 84.0
	
	90.0 / 84.0

	Compression ratio
	:1
	12.0
	
	12.0 

	Fuel
	
	min 91 RON
	
	min 91 RON

	Max output
	kW/hp
	105 / 143
	
	125 / 170

	at
	rpm
	6000
	
	6700

	Max torque
	Nm/lb-ft
	190 / 140
	
	210 / 155

	at 
	rpm
	4250
	
	4250

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery / installation
	Ah/–
	90 (70) / luggage comp
	
	90 (70) / luggage comp

	Alternator
	A/W
	180 / 2520
	
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double-joint spring strut axle in aluminium with displaced castor, small positive steering roll radius, compensation of lateral forces, anti-dive

	Suspension, rear
	
	Five-arm axle in lightweight steel

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	300 x 24 / vented
	
	300 x 24 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	300 x 20 / vented
	
	300 x 20 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, 

CBC Cornering Brake Control, DBC Brake Assistant

	Safety
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel belts on all seats, with belt retainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator

	Steering
	
	Electric Power Steering

	Steering ratio, overall
	:1
	16.0
	
	16.0

	Tyres, front/rear
	
	205/55 R16 91H
	
	205/55 R16 91H

	Wheels, front/rear
	
	7J x 16 steel
	
	7J x 16 steel


Specifications BMW 3 Series Saloon.
	
	
	BMW 318i Saloon
	
	BMW 320i Saloon

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist incl Enquiry Service, remote control functions and V-

Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google directory search, news, realtime weather, office functions, BMW Routes

and music title update online

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant,

Park Distance Control, cruise control, automatic/enhanced emergency call function 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Six-speed manual (optional: six-speed automatic)

	Gear ratios
I
	:1
	4.323 (4.171)
	
	4.323 (4.171)

	                                  
II
	:1
	2.456 (2.340)
	
	2.456 (2.340)

	                                  
III
	:1
	1.659 (1.521)
	
	1.659 (1.521)

	                                  
IV
	:1
	1.230 (1.143)
	
	1.230 (1.143)

	                                  
V
	:1
	1.000 (0.867)
	
	1.000 (0.867)

	                                   
VI
	:1
	0.848 (0.691)
	
	0.848 (0.691)

	                                   
R
	:1
	3.938 (3.403)
	
	3.938 (3.403)

	Final drive
	:1
	3.385 (3.909)
	
	3.455 (3.909)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	13.0 (13.2)
	
	11.0 (11.2)

	Output per litre
	kW/hp
	52.6 / 71.5
	
	62.7 / 85.3

	Acceleration
0–100 km/h
	sec
	9.1 (9.9)
	
	8.2 (8.9)

	
standing-start
	sec


	30.1 (30.7)
	
	28.9 (29.4)

	In 4th/5th gear
80–120 km/h 
	sec
	9.4 / 12.4 (-)
	
	8.3 / 11.0 (-) 

	Top speed
	km/h
	210
	
	228 (226)

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, Electric Power Steering,

Auto Start Stop and gearshift point indicator (manual gearbox models), on-demand management of ancillary units, intelligent lightweight construction, 
a/c compressor detachable

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	ltr/100km
	8.1 (8.7) 
	
	8.3 (9.3)

	Extra-urban
	ltr/100km
	5.3 (5.4)
	
	5.3 

	Combined
	ltr/100km
	6.3 (6.6)
	
	6.4 (6.8) 

	CO2
	g/km
	146 (155)
	
	148 (159)

	Emission rating
	
	EU5
	
	EU5  

	
	
	
	
	


Specifications apply to ACEA markets/data relevant to homologation apply in part only to Germany (weight) 

Figures in brackets apply to models with automatic transmission

1)  Oil change




















































