16
BMW
Media
Information

10/2005
Page 2

The new BMW K 1200 R.
Contents.
1.
Concept and Claim. 
 3

2.
Technical Features and Design. 
 4

3.
Range of Equipment. 
 9

4.
Engine Output and Torque. 
 11

5.
Specifications. 
 12

6.
The K 1200 R Colour Range. 
 13

The new BMW K 1200 R. 
1.
Concept and Claim.

BMW Motorrad proudly presents the most powerful Roadster in the world: the new K 1200 R. Featuring top-end technology, carried over from the K 1200 S, this high-end muscle bike will hit the headlines in the motorcycle market. And with its ultra-modern, 120 kW (163 bhp) four-cyl​inder engine, and running gear designed and built without compro​mise for supreme and absolutely thrilling riding dynamics, outstanding innovations such as the BMW Duolever and ESA, as well as a highly ad​vanced on-board electrical and electronic network, the K 1200 R sets a truly unique benchmark in the segment of high-performance sports roadsters. 

Through its extroverted design alone, the K 1200 R marks a new approach far beyond the conventional. Following the quantum leap in all areas borne out by the K 1200 S, the K 1200 R is now adding further emotions to the BMW motorcycle range. Combining the thrill of high technology and superior power and performance with a unique touch of purism and “naked” technol​ogy clearly visible from every angle, the new K 1200 R is truly unique – a typical product of BMW Motorrad in every respect.

As you may expect of BMW Motorrad, however, the focus with the K 1200 R, as with all other BMW motorcycles, is not on a short-lived “show-off” effect. Rather, like all motorcycles with the white-and-blue logo, the ob​jec​tive is to give the rider useful added value provided by future-oriented technologies. This philosophy is rounded off, as usual, by a truly perfect range of options and accessories enhancing the practical value of this powerful Roadster.

Optimum environmental compatibility ensured by ultra-modern exhaust gas management incorporating a fully controlled three-way catalytic converter, as well as supreme active safety provided by BMW’s advanced Integral ABS in conjunction with the high-performance brake system are further, cutting-edge features of this unique BMW – features, naturally, to be found on all other models.
2.
Technical Features and Design.

In technical terms the K 1200 R is derived from the K 1200 S high-per​formance machine. The drive-train, running gear, the brakes and the on-board network have all been carried over from the K 1200 S, modified only in a few features. The new body and equipment, as well as the truly exceptional, 
ex​tro​verted design, however, make the K 1200 R a brand-new and quite different motorcycle. Weighing just 237 kilos  (DIN unladen weight on a full tank, without ABS), the K 1200 R keeps its pledge of offering supe​rior handling and light manoeuvrability especially at low speeds on country roads and in town.

Drive-train.

Slender and compact, the straight-four power unit displaces 1,157 cc, 
the same as the engine of the K 1200 S. The other engine data is also similar, the new Roadster developing its somewhat lower peak output of 120 kW 
(163 bhp) at the same speed of 10,250 rpm, with maximum torque of 127 Nm coming 8,250 rpm.            

In mechanical terms the engine of the K 1200 R is identical to that of 
the K 1200 S, with the same configuration of the camshafts and the same valve timing. The flow of intake air, on the other hand, has been modified 
for the new machine, the K 1200 R featuring just one snorkel leading to the airbox on the right-hand side. This modified flow of intake air is the reason 
for the somewhat lower level of maximum power and torque. The silencer and ex​haust manifold systems both come from the K 1200 S.

The transmission has also been carried over unchanged from the K 1200 S. 
At 2.91:1 (instead of 2.82:1 on the K 1200 S), the final drive ratio on the 
drive-​train is adjusted to the lower top speed of this “naked” motorcycle with its higher drag coefficient.            

Suspension and running gear.

The suspension and running gear only had to be modified slightly for the new model: The Duolever is slightly steeper (0.4°), changing the steering head angle to 61°, with camber 11 mm or 0.43´´ shorter at 101 millimetres or 3.98´´. This change in suspension geometry is achieved by a minor modi​fication in machining the bearing points for the joints in the Duolever. 

The positive effect of this modified suspension geometry is even better 
handl​ing particularly at low speeds. Any theoretical repercussions on direc​tional stability at high speeds versus the K 1200 S are barely noticeable 
in practice and are irrelevant, anyway, on a motorcycle without a fairing and, accordingly, with somewhat lower top speed.                                      

The K 1200 R comes as standard with different wheels in the same 
basic design as the wheels on the new R 1200 ST. Wheel dimensions are 
3.5´´ x 17 at the front and 5.5´´ x 17 at the rear, with tyres measuring 
120/70-ZR17 and, respectively, 180/55-ZR17. As an option, the customer also has the choice of the K 1200 S sports wheels measuring 3.5´´ x 17 at 
the front and 6.0´´ x 17 at the rear, in which case the rear tyre measures 190/50-ZR17.
BMW Motorrad’s Electronic Suspension Adjustment (ESA) is also available 
as an option. Introduced for the first time on the K 1200 S, 
this world-first achievement enables the rider to set spring tension and damping from the handlebar with utmost convenience merely at 
the touch of a button – an operation he can perform even while riding. 

ESA varies inbound damping on the front wheel and both inbound and 
re​bound damping on the rear wheel. The rider also has the option to 
adjust the rear-wheel spring base electrohydraulically while the motorcycle is at a standstill as a function of load conditions. 

An exact description of ESA with all its technical features is to be found in the press release on the K 1200 S. Spring travel on the K 1200 R is the same 
as on the K 1200 S sports machine, measuring 115 millimetres or 4.52´´ up front and 135 millimetres or 5.31´´ at the rear.
Brakes – high-performance EVO brake system and Integral ABS 
as an option.

The K 1200 R comes with BMW’s EVO brake system also featured in all other K-Series and Boxer models. In its configuration and dimensions, the brake system with disc brake diameter of 320 millimetres or 12.60´´ at the front and 265 millimetres or 10.43´´ at the rear is the same as on the K 1200 S. 
The brake hoses, in turn, are steel-clad for extra safety and reliability. 

BMW’s EVO brake system – with EVO standing for evolution – is one of the safest and most effective brake systems in the world.

As another option, the EVO brake system is available with BMW Integral ABS anti-lock brakes in the sporting, partly integrated version. “Partly integrated” means that both brakes (front and rear wheel) are acti​vated by the handbrake lever, while the footbrake lever acts only on the rear wheel.

Even when applying the brakes all-out, the rider of the K 1200 R runs virtu​ally no risk of taking a fall or tumbling forward thanks to the motorcycle’s low centre of gravity and the special suspension geometry interacting with the kinematic configuration of the Duolever. In other words, the rider benefits 
un​der virtually all conditions from maximum tyre friction when applying the brakes all-out with ABS control, enjoying optimum safety even in an extreme situation.

Controls and instruments.           

The handlebar and handlebar bridge on the K 1200 R are both new. 
Com​pared with the K 1200 S, the handlebar is both wider and higher. 
As befits a motorcycle of this type, the rider’s seating position is more upright and moved further to the front. Oval rear-view mirrors fastened to the handlebar appropriately reflect the concept of reducing all features to the essential.

In terms of both its technical features and looks, the instrument cluster with its speedometer and rev counter is based on the same concept as the in​stru​ment cluster of the R 1200 GS, while the instrument faces are new. As on the 
K 1200 S, the displays in the info-flatscreen provide ongoing infor​mation on coolant temperature, the fuel level, the time, as well as the gear currently 
in mesh. On machines equipped with ESA as an option, the info-flatscreen provides additional information on the current set-up of the sus​pension. 
Total mileage, trip mileage, and the range remaining on the fuel in the tank are all available on demand.

Electrics and electronics.

Boasting a wide range of innovations, the on-board network in CAN-bus 
tech​nology (Controller Area Network) has been carried over directly from the 
K 1200 S sports motorcycle without any modifications. Inter alia, the big 
ad​vantages of this technology are the far wider range of functions compared 
with a conventional on-board network coming with far simpler and more straightforward cable connections, the absence of any conventional melt-down fuses, as well as full-scale diagnostic management and examination options. The fully integrated electronic immobiliser comes as standard, 
con​sistently exchanging coded and constantly changing data between the key 

to the motorcycle (by means of a transponder) and the on-board electronic system. In technical terms this is the best and safest immobiliser currently available, ensuring that only the authorised rider really gets to experience this unique machine with all its sheer riding pleasure.

Design and body.       

In its design, the K 1200 R is both athletic and extroverted. The underlying philosophy in creating this machine was to reduce all features to the essen​tial and achieve high emotional value. Such minimalist design places the focus 
on the visible technical features and components of both the drivetrain and the suspension, highlighting BMW’s claim to leadership in technology and innovation. With its muscular and, indeed, masculine look, the K 1200 R clearly bears testimony even at a standstill to the performance and dynam​ics of what is currently the most powerful Roadster in the world.

It is no surprise that this design evokes contradictory discussions and 
opin​ions, especially as it would not have been appropriate, given 
the demanding standards of BMW Motorrad, to give a motorcycle of this calibre anything but classically beautiful and really attractive looks.

While the heritage of the K 1200 R with its close ties to the K 1200 S is ob​vious – both motorcycles, for example, sharing the same exciting contour lines on the fuel tank and seat – the K 1200 R goes much further in its styl​ing features: The machine’s contours and shapes are even more distinc​tive and, indeed, organic, making the K 1200 R truly striking, unmistakable and, 
in fact, quite unique. This design language is continued in the side covers at the front of the fuel tank, taking up and extending the consistent line to 
the front end with the oil cooler fairing beneath the striking double headlight. 

The small windshield above the headlight, despite its compact dimensions, significantly reduces wind pressure at high speeds.

The reflectors in the dual headlight system (2 x H7) come in free form sur​face technology and are housed beneath extra-clear, impact- and scratch-proof, lightweight polycarbonate covers. Finished in matt chrome, the head​light housing made of a special synthetic material accentuates the straight-forward technical character of the motorcycle through its colour alone.

The Duolever comes with plastic covers at the side emulating the basic 
de​sign of the wheel bearings. At the same time these covers serve to guide the flow of cool air to the radiator with its minimalist fairing.

The K 1200 R comes in a range of powerful-looking colours full of character and style. Dark Graphite metallic, for example, forms a strong contrast to the drive-train and running gear finished in silver, highlighting the technical and aesthetic qualities of the new machine. White Aluminium metallic, on the other hand, comes together with the drive-train and running gear in black and, therefore, turns around the contrasts to be admired on the K 1200 R, giving greater emphasis to the contours of the motorcycle and its muscular shape. With this combination of colours, the wheels come in Slate Grey metallic matt. A particularly striking colour scheme is Sun Yellow non-metallic to​geth​er 
with the drive-train and running gear in black and the wheels in Slate Grey metallic matt. 

The seats come in black on all models and with all paintwork options.

Fuel tank and seat.

The fuel tank of the K 1200 R is made of an extra-light, impact-resistant 
syn​thetic material and offers a capacity of 19 litres or almost 4.2 Imp gals 
(in​clud​ing 4 litres reserve). The overall package of the K 1200 R, just like 
the package on BMW Motorrad’s other machines, serves to keep the tank par​tic​ularly slender beneath and in front of the rider, thus ensuring optimum 
knee contact. Production of the tank in a rotational process ensures maxi​mum freedom in the exterior design of the tank and allows the distinctive 
contours on the tank fairing so characteristic of this special machine.
Similar in its basic design at the front, the seat is even more slender than on the K 1200 S in the interest of an even more forward-oriented, active riding position. The rider’s absolute geometric seat height is 820 millimetres 
(with step arch length of 1,810 mm). 

3.
Range of Equipment.

Optional extras and special equipment – customisation is the name 
of the game.

Although the K 1200 R is most at home in an urban environment and on nearby routes not far from the rider’s home, it is naturally – like all 
BMW motorcycles – quite suited for long distances and tours. And there are indeed enthusiasts who, wishing to enjoy a direct experience on their motorcycle, are happy to do without a fairing. Even on a Roadster, however, they need appropriate solutions for their holiday baggage and other items 
they wish to take along.

Precisely this is why the K 1200 R is available with the sports cases carried over from the K 1200 S combining superior function with sporting design and thus perfectly reflecting the specific character of BMW’s new high-per​formance Roadster. Combining textile and firm materials, this luggage sys​tem, benefiting from the carriers fitted almost invisibly on the motorcycle, can be mounted and removed within a matter of seconds. 

Loading space is variable, extending from 17 litres per case in normal configuration up to 25 litres in each case folded fully open. The fully inte​grated inner bags are naturally watertight.

The Sports windshield is ideal for the rider wishing to enjoy somewhat more protection from wind and weather, the windshield itself extending up a bit higher and coming in wider dimensions than the small, standard windshield.

The usual range of features is available for further customisation of the 
K 1200 R. And again as usual, optional extras come straight from the factory, being integrated during production at the Berlin Plant. Special equipment, 
in turn, can be fitted subsequently either by the customer himself or by the BMW motorcycle dealer, thus allowing subsequent enhancement of the machine at any time

Optional extras:    

•
Sports windshield.

•
Low rider’s seat (seat height 790 mm) 

•
Luggage rack.

Special equipment

•
Luggage supports.

•
Set of sports cases.

•
Watertight tank rucksack.

•
Watertight tank bag.    

•
Watertight sports softcase, small (for the luggage rack).

•
Luggage rack

Pricing:

K 1200 R




R118 000 (Incl. VAT)

ABS and heated grips



R8 400 (Incl. VAT)

Electronic Suspension Adjustment (ESA)
R5 500 (Incl. VAT)

Specifications.
K 1200 R.
	
	
	K 1200 R
	
	
	

	Engine
	
	
	
	
	

	Capacity
	cc
	1,157
	
	
	

	Bore/stroke
	mm
	79/59
	
	
	

	Output  
	kW/bhp
	120/163
	
	
	

	at
	rpm
	10,250
	
	
	

	Torque
	Nm/lb-ft
	127/94
	
	
	

	at
	rpm
	8,250
	
	
	

	Configuration 
	
	Inline
	
	
	

	No of cylinders
	
	4
	
	
	

	Compression ratio/fuel grade
	
	13:1/premium plus (98 octane)
	
	
	

	Valve/gas control  
	
	DOHC (double overhead  camshaft)
	
	
	

	Valves per cylinder 
	
	4
	
	
	

	Intake/outlet valve diameter
	mm
	32/27.5
	
	
	

	Throttle butterfly diameter   
	mm
	46
	
	
	

	Fuel supply/management
	
	BMSK
	
	
	

	
	
	
	
	
	

	Electrical system 
	
	
	
	
	

	Alternator   
	W
	580
	
	
	

	Battery
	V/Ah
	12/14 maintenance-free
	
	
	

	Headlights  
	W
	Low beam 1x H 7/55 W
	
	
	

	
	
	High beam 1x H 7/55 W
	
	
	

	Starter
	kW
	0.7
	
	
	

	
	
	
	
	
	

	Power transmission/gearbox
	
	
	
	
	

	Clutch   
	
	Multi-plate oil bath clutch, disc dia 151 mm
	
	

	Gearbox  
	
	Dog-type six-speed gearbox
	
	
	

	Primary transmission ratio
	
	1.559
	
	
	

	Gear ratios
	
	2.521
	
	
	

	
II
	
	1.842
	
	
	

	
III
	
	1.455
	
	
	

	
IV
	
	1.287
	
	
	

	
V
	
	1.143
	
	
	

	
VI
	
	1.015
	
	
	

	Final drive     
	
	Driveshaft
	
	
	

	Final drive ratio
	
	2.91
	
	
	

	
	
	
	
	
	

	Suspension and running gear
	
	
	
	
	

	Frame      
	
	Composite aluminium frame with IHU/extrusion-pressed profiles and die-castings
	
	

	Wheel guidance, front
	
	BMW Duolever
	
	
	

	Wheel guidance, rear 
	
	BMW Paralever
	
	
	

	Spring travel, front/rear
	mm
	115/135
	
	
	

	Camber  
	mm
	101
	
	
	

	Wheelbase
	mm
	1,571
	
	
	

	Steering head angle
	°
	61
	
	
	

	Brakes 
	front
	Double disc brake, dia 320 mm
	
	
	

	
	rear 
	Single disc brake, dia 265 mm
	
	
	

	
	
	Optional BMW Motorrad Integral ABS (partly integral)
	
	
	

	Wheels
	
	Light-alloy cast wheels
	
	
	

	
	front
	3.50 x 17 MTH 2
	
	
	

	
	rear
	5.50 x 17 MTH 2 

(optional 6.00 x 17 MTH 2)
	
	
	

	Tyres 
	front
	120/70 ZR 17
	
	
	

	
	rear
	180/55 ZR 17

(190/50 ZR 17)
	
	
	

	
	
	
	
	
	

	Dimensions and weight
	
	
	
	
	

	Length, overall
	mm
	2228
	
	
	

	Width, overall, with mirrors
	mm
	856
	
	
	

	Handlebar width, without mirrors
	mm
	785
	
	
	

	Seat height
	mm
	820 (790)
	
	
	

	Weight, unladen, with full tank
	kg
	237
	
	
	

	Weight, max permissible
	kg
	450
	
	
	

	Tank capacity
	Ltr
	19
	
	
	

	
	
	
	
	
	

	Performance data
	
	
	
	
	

	Fuel consumption   
	
	
	
	
	

	90 km/h
	ltr/100 km
	4.7
	
	
	

	120 km/h
	ltr/100 km
	5.5
	
	
	

	Acceleration  
	
	
	
	
	

	0–100 km/h
	sec
	2.8
	
	
	

	Standing-start km
	sec
	–
	
	
	

	Top speed            
	km/h
	200 plus
	
	
	


6.
The K 1200 R Colour Range.

	
	Colour
	Drivetrain and suspension
	Wheels

	K 1200 R
	Dark Graphite metallic
	Silver
	Silver

	
	White Aluminium metallic
	Black  
	Slate Grey metallic, matt 

	
	Sun Yellow non-metallic
	Black  
	Slate Grey metallic, matt 






































PAGE  
2

