TECHNICAL SPECIFICATIONS

No. of doors/seats
4 / 5

Vehicle length
5834 mm / 229.7 ins

Vehicle width
1990 mm / 78.3 ins


Vehicle height, unladen
1632 mm / 64.3 ins


Wheelbase
3570 mm / 140.6 ins

Turning circle
13.8 m / 45.3 ft

Track, front
1685 mm / 66.3 ins


Track, rear
1670 mm / 65.7 ins


Width at shoulder height, front
1509 mm / 59.4 ins


Width at shoulder height, rear
1431 mm / 56,3 ins

Leg room, front 
1028 mm / 40.5 ins

Leg room, rear  
947 mm / 37.3 ins

Head room, front
1020 mm / 40.2 ins

Head room, rear
979 mm / 38.5 ins

Boot volume (DIN)
460 ltr / 16.2 cu ft

Fuel tank capacity
100 ltr / 22 imp gal

Unladen weight (DIN)
2,485 kg / 5,478 lb

Axle load ratio, rear (unladen)
49.5 %

Gross vehicle weight
3,030 kg / 6,680 lb

Payload
500 kg / 1,102 lb

Axle load limit, front
1,425 kg / 3,141 lb

Axle load limit, rear
1,800 kg / 3,968 lb

Engine / valves
60°V12 / 48

Fuel management
Direct injection

Displacement
6,749 cc / 411.8 cu in

Stroke
84.6 mm / 3.33 ins

Bore
92.0 mm / 3.62 ins

Power output 
453 bhp / 460 PS / 338 kW

@ engine speed
5,350 rpm

Max. torque @ engine speed
531 lb ft / 720 Nm @ 3,500 rpm

Compression ratio
11:1

Transmission type
ZF 6HP32

Gear ratios

 1st 
4.171

 2nd
2.340

 3rd
1.521

 4th 
1.143

 5th
0.867

 6th
0.691

Reverse 
3.403

Final drive ratio
3.460

Steering type
Rack and pinion, speed-sensitive


variable rate power assistance

Brakes

Front / diameter
Disc vent. 374 mm / 14.7 ins

Rear / diameter
Disc vent. 370 mm / 14.6 ins

Drag  cd
0.383

Drag  cd x A
1,071 sq. m / 11,5 sq. ft

Top speed
149 mph / 240 km/h

Acceleration 0-60 mph
5.7 sec

Acceleration 0-100 km/h
5.9 sec

Acceleration 0-1,000 m
25.6 sec

EU test cycle

Fuel consumption urban
24.6 ltr/100 km / 11.5 mpg

Fuel consumption extra urban
11.0 ltr/100 km / 25.7 mpg

Fuel consumption combined
15.9 ltr/100 km / 17.8 mpg

CO2 emissions 
385 g/km

Tyre size 
PAX 265 x 790 R540 A 111W

Wheel size 
PAX 265 x 540 A

Radiator incl. heater / Engine oil
14 ltr / 8.5 ltr

Battery capacity / installed position
90+70Ah / boot

Alternator output rating
2 x 180A / 2520W

All figures: manufacturer’s test data

TECHNICAL SPECIFICATIONS (NORTH AMERICA)

No. of doors/seats
4 / 5

Vehicle length
229.7 ins

Vehicle width
78.3 ins

Vehicle height, unladen
64.3 ins

Wheelbase
140.6 ins

Turning circle
45.3 ft

Track, front
66.3 ins

Track, rear
65.7 ins

Width at shoulder height, front
59.4 ins

Width at shoulder height, rear
56,3 ins

Leg room, front 
40.5 ins

Leg room, rear  
37.3 ins

Head room, front
40.2 ins

Head room, rear
38.5 ins

Boot volume (DIN)
16.2 cu ft

Fuel tank capacity
26.4 US gal

Curb weight
5,577 lb

Axle load ratio, rear (unladen)
49.5 %

Gross vehicle weight
6,680 lb

Payload
1,102 lb

Axle load limit, front
3,141 lb

Axle load limit, rear
3,968 lb

Engine / valves
60° V12 / 48

Fuel management
Direct injection

Displacement
411.8 cu in

Stroke
3.33 ins

Bore
3.62 ins

Power output
453 hp (SAE)

@ engine speed
5,350 rpm

Max. torque @ engine speed
531 lb ft @ 3,500 rpm

Compression ratio
11:1

Transmission type
ZF 6HP32

Gear ratios

 1st 
4.171

 2nd
2.340

 3rd
1.521

 4th 
1.143

 5th
0.867

 6th
0.691

Reverse 
3.403

Final drive ratio
3.460

Steering type
Rack and pinion, speed-sensitive 
variable rate power assistance

Brakes

Front / diameter
Disc vent. 374 mm / 14.7 ins

Rear / diameter
Disc vent. 370 mm / 14.6 ins

Drag  cd
0.383

Drag  cd x A
11.5 sq ft

Top speed
130 mph / 208 km/h

Acceleration 0 - 60 mph
5.7 sec

Acceleration 0-100 km/h
5.9 sec

Acceleration 0 - 1/4 mile
14.3 sec

EPA fuel cycle

Fuel consumption city
13.9 mpg

Fuel consumption highway
23.8 mpg

Fuel consumption combined
17.1 mpg

Tyre size 
PAX 265 x 790 R540 A 111V

Wheel size 
PAX 265 x 540 A

Radiator incl. heater / Engine oil
14.8 quarts / 9.0 quarts

Battery capacity / installed position
90+70Ah / boot

Alternator output rating
2 x 180A / 2520W

All figures: manufacturer’s test data

