64
BMW
Media
Information

1/2007
Page 3

[image: image1.png]


The new BMW 5 Series.
Contents.
The new BMW 5 Series.
Description in Brief. 
 2

Perfection. Perfected:
The new BMW 5 Series.
(Short Version) 
 4
Specifications. 
 12
Exterior and Interior Dimensions. 
 14
[image: image2.png][::%(§§§\ P /6§§,§::3
00 J E%\ - /Qé D
3 —\l py=—
- T
OON —
S . J
- 1581 -
- 1846 _
/
P >
\
|

00
O
N
Y
A

2888
484

L




The new BMW 5 Series.
Description in Brief.
•
All-round enhancement of the dynamic, innovative and sophisticated character of the BMW 5 Series, attractive modifications in design, enhanced style and value within the interior, best-of-class fuel economy in all model variants, trendsetting, future-oriented enhancement of comfort and safety features, enhancement of the model’s leading position in the segment: BMW 5 Series Saloon as the epitome of sheer driving pleasure in the Business Class. 

•
Sporting and elegant accentuations of exterior design: headlights and rear light clusters in technically outstanding clear glass look, chrome headlight surrounds with position light rings at the front, LED direction indicators and five light conductor rods at the rear, newly contoured air intake with upward-facing design, BMW kidney grille with individual surrounds flush to the front air dam, greater horizontal subdivision and additional light contours at the rear providing an even more powerful look, striking light contour in the side-sills accentuating the car’s elegance from 
the side, attractive new light-alloy wheels, additional exterior colours. 

•
Enhancement of driving and operating comfort by particularly  sophisticated materials and refined style within the interior, clear lines, colours and materials providing a calm ambience combining quality 
and style, two-tone door panels with smooth, flowing transition to the dashboard, integrated map pockets with enlarged capacity, padded armrests and door closing handles, electronic gear selector lever for the automatic transmission with unique visual, ergonomic and functional qualities, iDrive control system enhanced by eight favourite buttons for free choice of functions.

•
Four interior colour worlds, six upholstery materials, six trim variants, wide range of options serving to customise the individual, multi-faceted 
style and appearance of the BMW 5 Series, new combination of cloth and leather in particularly sporting style, leather upholstery with new colours and seam patterns. 

•
Wide range of innovations for enhanced driving dynamics, safety and comfort, only car in the segment with cruise control incorporating a separate brake function, Active Cruise Control complete with Stop & Go function for keeping a safe distance in flowing traffic and when driving dead slow, Lane Departure Warning with vibration signal on the steering wheel, Head-Up Display, Night Vision, High-Beam Assistant and 
Active Steering, navigation systems with optimised graphics and full-word voice entry for 80,000 towns and cities, daytime driving lights, Adaptive Headlights, Bending Lights. 

•
Lightweight chassis with aluminium double-joint tiebar axle at the front and aluminium Integral-IV axle at the rear, DSC Dynamic Stability Control with additional functions, electrohydraulic brake system.
•
Engine variants:
BMW 550i: Eight-cylinder petrol engine with VALVETRONIC and double-VANOS, capacity 4,799 cc, max output 270 kW, max torque 490 Nm, acceleration 0–100 km/h in 5.2 seconds, top speed 250 km/h (limited electronically), average fuel consumption to the EU standard 11.1 litres/100 kilometres (automatic transmission 10.8 litres/100km).

BMW 530i: Straight-six petrol engine with VALVETRONIC and double-VANOS, capacity 2,996 cc, max output 200 kW, max torque 315 Nm, acceleration 0–100 km/h in 6.3 seconds, top speed 250 km/h (limited electronically), average fuel consumption to the EU standard 8.8 litres/100 kilometres (automatic transmission 9.3 litres).


BMW 525i: Straight-six petrol engine with with VALVETRONIC and double-VANOS, capacity 2,497 cc, max output 160 kW, max torque 250 Nm, acceleration 0–100 km/h in 7.5 seconds, top speed 245 km/h, 
average fuel consumption to the EU standard 8.7 litres/100 kilometres.


BMW 523i: Straight-six petrol engine with with VALVETRONIC and double-VANOS, capacity 2,497 cc, max output 140 kW, max torque 230 Nm, acceleration 0–100 km/h in 8.3 seconds, top speed 237 km/h, 
average fuel consumption to the EU standard 8.5 litres/100 kilometres.


BMW 530d: Straight-six diesel with common-rail fuel injection and 
VTG turbocharger, capacity 2,993 cc, max output 160 kW, max torque 480 Nm, acceleration 0–100 km/h in 7.1 seconds, top speed 250 km/h, 
average fuel consumption to the EU standard 6.7 litres/100 kilometres.
[image: image3.png]"
(SS—
W,

RS S S S e e e o T o R B TS T ST ST T
OLILITIRICIRECERELEELELILILIHRIZSCSESE LSO ICIE LIS SSC S S SESESE ST
S SSEEZEL LSRRI EEIREREL IR ILIR S ETITETIRITIL S ELIRELILILILELELELEIES
ROLSLELILERELERELERELIRILIELIRILIIELIRELEIRILEIIIRELEIIRILELTRELE2LS2S
RS TS TSt SRST IR ST ST ST ST ST ST STST ST ST ST STSTSTSTSTSTSTSTSTSTSTS
SRS ERELELEIELELELEREEEREEELSTIEILILICITECSEETSL IS STIRSISEELEIESETY
B o S R R e R ool
_—_—

e

r

A
N
O
98
O

Y

1581

v

A

1846 -

A

28388
- 4841

00
Ol
N
y
A

V
A

1101 ——

Y

1452
14906
]




Perfection. Perfected:
The new BMW 5 Series.
(Short Version)
More convincingly than any other car in its class, the BMW 5 Series combines supreme style and presence, on the one hand, with sporting power and performance, on the other. And now the looks, driving experience and superior comfort offered by the 5 Series have reached a new level of perfection. Highly attractive modifications to the exterior and significant, visible and tangible enhancement of style and sophistication in the interior reconfirm the sporting, elegant and classy BMW 5 Series as the epitome of Sheer Driving Pleasure in the business class. 

The premium ambience within the car’s interior is borne out by particularly sophisticated materials, attractive colours and re-designed interior elements and controls. 

Whilst three of the engines remain unchanged, two of the six-cylinder power units are even more muscular than before with the exceptional all-round efficiency BMW is recognised for producing. In fact, the company’s petrol and diesel engines are already widely lauded for their superior power and supreme refinement but, in the new 5 Series, they further stretch their advantage in driveability and economy in all performance classes. 

The new BMW 5 Series also offers a wide range of innovations in driver assistance and comfort features. As an example, it is the only car in its segment available with Active Steering, a High-Beam Assistant, BMW Night Vision, a Head-Up Display and, from now on, also with Active Cruise Control featuring a Stop & Go function as well as Lane Change Warning. 

Featured as standard, iDrive control – now with eight freely programmable favourite buttons as well as an optionally available navigation system with voice control – enhance the long-distance touring comfort of the BMW 5 Series to a new, unprecedented standard. 

Exterior design: sporting character with new elegance.

New highlights in design and ongoing refinement ensure even greater harmony of sporting character and sophisticated elegance in the BMW 5 Series. The headlights in their characteristic look, for example, now come with new clear glass technology, transparent glass panels covering the direction indicators, chrome surrounds highlighting the technically sophisticated character of the round headlights. 

The surrounds on the BMW double kidney grille, in turn, are now flush with the surfaces on the front air dam, and the air intake sweeps up at the side along the same dynamic line as the headlight contours contributing to the smooth, harmonious and friendly look of the front end. And last but not least, the horizontal trim bar in the middle highlights the wide track of the car and its muscular stance on the road. 

An additional light contour along the side-sills gives the body an even sleeker, more stretched look from the side. The rear end, in turn, is dominated by horizontal lines highlighting the look of superior power. Like the headlights at the front, the rear lights also come in a new, sophisticated clear-glass look and now feature LED direction indicators. Finally, the trim bar around the rear numberplate is now defined even more smoothly and precisely. 

Refined and sophisticated style within the interior.

Enhanced control comfort, new design and particularly sophisticated materials on the surfaces give the interior of the new BMW 5 Series a new standard of quality both the driver and his passengers will see and feel at first sight, ensuring a wonderful experience of driving pleasure at all times. High-class Pearl Gloss Chrome, the very pleasant rubber finish on the controls for smooth and easy handling, comfortable padding on the door armrests and panels as well as on the centre console all contribute to the particular style and class of the interior. At the same time additional storage options give the new BMW 5 Series an even higher standard of practical function. 

The most striking feature, clearly bearing out the new refinement of the car’s interior design, is the twin-tone finish on the door panels: Black door trim bars at the top and the lower panel section in the respective interior colour, together with elegantly sweeping decal bars, take up and continue the look and the materials used on the dashboard. The buttons for the window lifts and mirror adjustment have been integrated in the armrest, just as the handles for 
closing the doors have been optimised in their ergonomic design. The door pockets, now integrated even more effectively than before in the door panels, provide greater storage capacity for enhanced convenience on the road.

In its appealing look and ideal ergonomics, the new, electrical gear selector lever for the automatic transmission merges perfectly into the stylish interior. Control of the iDrive vehicle management system has also been optimised 
to an even higher standard, the Controller positioned on the centre console and the central Control Display in the dashboard serving to mastermind the Climate Control, Navigation, Entertainment and Communication functions. 

Apart from enhanced menu guidance and refined graphics, particularly the new favourite buttons ensure even more intuitive control and operation of the car. The eight freely available buttons with their favourite functions allow the driver and front passenger to access important functions very quickly, smoothly and easily: All they do is press one single button to activate, say, a specific destination, a telephone number dialled frequently, or a preferred 
radio station saved in advance. 

Power units: more power for 523i and 530i.

The new BMW 5 Series is entering the market with three unchanged and two enhanced power units. All engines rank right at the top of the segment in terms of all-round economy and performance within their respective category.

Depending on the specific engine involved, the various models in the 
new BMW 5 Series come with a range of enhaned and carried over technologies and features that serve to minimise fuel consumption and emissions, as well as make maximum power available when it’s needed most. 

Operating on-demand, the electrical coolant pump on all new BMW 5 Series engines consumes much less power than a conventional pump and helps the engine reach its normal operating temperature at an earlier point. The Varioserv steering assistance pump, varying the curve ring as a function of pressure and volume flow, avoids the usual increase in drive losses as a function of higher engine speeds. And when the driver and passengers do not require the air conditioning, the a/c compressor is disconnected to reduce drag forces to a minimum. 

The eight-cylinder in the range-topping BMW 550i is unchanged. This means it benefits from the same enhanced fuel economy innovations from the previous 5 Series, ensuring that it offers a standard of efficiency unheard of in this performance class. Powered by a 4.8-litre, 270 kW V8, the BMW 550i accelerates to 100 km/h in just 5.2 seconds. Top speed is limited electronically to 250 km/h. Average fuel consumption in the EU test cycle is just 11.1 litres/100 kilometres, while the automatic transmission models is even more economical and fuel-efficient, reducing fuel consumption to just 10.8 litres/ 100 kilomteres.

530i and 523i Straight-six Power Units with more power.

The range of straight-six power units available for the BMW 5 Series comprises no less than three different power and performance versions. 
Two of these engines displace 2.5 litres, while the third is a 3.0-litre. 
In the BMW 530i, this 3.0-litre straight-six petrol engine develops a dynamic 200 kW – an increase of 10kW over the previous version. At the same time, maximum torque has increased from 300Nm to 315Nm. Acceleration to 100 km/h comes in 6.3 seconds while top speed, in turn, is limited electronically to 250 km/h. 

The 2.5-litre six-cylinder power unit in the BMW 523i now develops 140 kW – which again makes this engine 10 kW and, in this case, 5 Newton-metres more powerful than its predecessor. On the road, this kind of power enables the BMW 523i Saloon to accelerate to 100 km/h in 8.3 seconds and reach a top speed of 237 km/h. Average fuel consumption also remains unchanged at 8.5 litres/100 km.

The power unit of the BMW 525i remains unchanged. Maximum power output remains 160 kW with 250 Nm of maximum torque on tap. This accelerates the BMW 525i to 100 km/h in 7.5 seconds and provides a top speed of 245 km/h. 

3.0-litre diesel with variable turbine geometry.

Engine output in the new BMW 530d remains 160 kW. This version of the 3.0-litre power unit boasts a turbocharger system with variable turbine geometry (VTG) for muscular and, at the same time, harmonious development of power and performance under all conditions. And again, this power unit develops its maximum torque – in this case 480 Newton-metres – at just 1,750 rpm. The result is powerful acceleration to 100 km/h in just 
7.1 seconds. Top speed of the new BMW 530d, in turn, is 245 km/h. At 6.7 litres/100 km, average fuel consumption remains unchanged.
Optional: six-speed automatic transmission and automatic 
sports mode.

As an option and, indeed, as a highly interesting alternative to the six-speed manual gearbox featured as standard, all model variants in the BMW 5 Series are available with a six-speed Steptronic automatic transmission.

Apart from the direct connection to the engine, this transmission stands 
out in particular through its upgraded hydraulics, innovative torque converter and even more powerful management software, ensuring an even more spontaneous response to even the slightest movement of the gas pedal and an extremely fast and efficient gearshift. On the road, this means reaction times now some 40 per cent faster than on former automatic transmission models, with the actual gearshift time being almost halved. Shifting 
back a gear is now just as fast, too, thanks to the direct gear path finder. 

The automatic transmission is controlled by an electronic gear selector 
lever following the usual gearshift pattern. The new gear selector lever moves back to its initial position once the gear selected is in mesh, with the transmission itself masterminded not in a mechanical process, but rather via electrical signals. 

Active Steering for unique driving comfort.

Featured as standard in the BMW 5 Series, Servotronic serves to vary the degree of steering assistance as a function of road speed. At the same time the BMW 5 Series is the only car in its segment optionally available with Active Steering varying the steering angle in accordance with the speed at which the car is currently travelling: With the same movement of the steering wheel, the front wheels will move further to the left and right at low speeds than at high speeds. This allows the driver to manoeuvre the car with less effort and force on the steering when parking, while at high speeds it is easier to remain precisely on track. 

The power and effectiveness of the high-performance swing-calliper brakes with inner-vented brake discs is now controlled electrohydraulically on the BMW 5 Series, brake pressure being modified individually as required on all four wheels. So should the electronic control unit masterminding the electrohydraulic brake system notice that the driver is applying the brakes all-out, the system will build up brake pressure very quickly indeed, making the entire process of applying the brakes more comfortable and enhancing driving safety thanks to the shorter stopping distances.  

Yet a further point is that the BMW 5 Series is equipped with an ongoing, continuous brake wear indicator providing information at all times on the condition of the brake pads. 

DSC Dynamic Stability Control with new functions. 

DSC Dynamic Stability Control now featuring new, innovative functions helps to make the brake system even more effective. The first and foremost DSC function is to apply the brakes on individual wheels or intervene in engine management in order to counteract any over- or under-steer in bends. Additional features incorporated within DSC are ABS brake control, ASC Automatic Stability Control, a Brake Assistant and CBC Cornering Brake Control. Then there is also Fading Compensation and the Start-Off Assistant likewise incorporated in the DSC system on the new BMW 5 Series. Regular Dry Braking helps to ensure optimum stopping power in wet weather, and pre-loading of the brake pads enhances the brake standby function when the driver requires a particularly short stopping distance. 

Cruise control with brake function.

As an option the BMW 5 Series is available with cruise control complete with 
a special brake function. The system is activated in order to maintain the speed set by the driver in advance – at least 30 km/h – regard​less of the route the driver is taking or its specific profile. So depending on the current requirements, the system either increases engine power or slows down the car. 
Contrary to conventional cruise control, this intelligent new system is able to control the road speed of the car not just via the drag force generated by the engine and by shifting down on models with automatic transmission, but also by activating the brakes, thus keeping the car at a consistent speed when, say, driving downhill for a longer distance or towing a trailer. 
And using the Curve Speed Limiter, the system is also able to re​duce 
road speed in bends whenever required for reasons of driving dynam​ics. 

Active Cruise Control complete with Stop & Go.

Even more sophisticated and refined, Active Cruise Control with Stop & Go offers the driver even greater support and ease of motoring. This innovative system available for the first time as an option on automatic transmission versions of the new BMW 5 Series comes complete with automatic distance control enabling the driver to cruise along smoothly and conveniently on 
the motorway or country roads, with the system applying the brakes where ne​cessary until the car reaches a standstill and keeping a consistent distance from the vehicle ahead even in stop-and-go traffic at very low speeds. 

To provide this superior convenience, the system uses radar sensors of the latest generation with an enlarged area of vision, offering the driver a choice of four distance settings. Once the distance the driver has chosen to the vehicle ahead is no longer maintained, the system will intervene in engine management, building up brake pressure to control the car accordingly. 
Then, once the road ahead is free again, Active Cruise Control with Stop & Go will re-accelerate the car to the desired speed. 

This sophisticated control system is also able to reduce speed auto​matically to a standstill as a function of traffic conditions. In this case Active Cruise Control with Stop & Go slows down the car to a stand​still and holds it in position. 

The maximum brake force applied by Active Cruise Control with Stop & Go 
is 4 metres/sec2. So should the driver be required to intervene be​cause 
the vehicle ahead is being decelerated very hard under ex​treme conditions, the driver is urged to intervene by visual and acous​tic signals.

In stop-and-go traffic the system helps the driver by automatically maintaining the appropriate distance from the vehicle ahead, reliev​ing the driver of the usual chore of constantly having to apply the brakes in slow and unsmooth traffic. However, the driver still retains his responsibility, since after, say, 
a short stop of more than three seconds he is required to briefly press down the accelerator pedal or press the Resume button in order to re-accelerate 
the car. And even when Active Cruise Control with Stop & Go is activated, 
the driver is able to control the speed of his car at any time simply by accelerating or applying the brakes. Pressing down the brake pedal while driving, finally, the driver is able to deactivate the system altogether. 

Keeping reliably on course: Lane Change Warning.

To enhance driver awareness, the new BMW 5 Series is available with a newly developed and quite unique assistance system helping to keep the car exactly on course, that is in the lane chosen by the driver. Referred to as “Lane Change Warning”, this highly innovative system identifies any deviations from the correct course of the car and gives the driver a discreet but tangible signal where he really feels the steering – on the steering wheel itself. 

The Lane Change Warning system consists of a camera fitted near the interior mirror on the windscreen, a control unit serving to compare data, and a signalling device. 

Head-Up Display and driver support in poor weather with bad visibility.

The BMW 5 Series is the only car in its segment available as an option with a Head-Up Display. Activating this function, the driver has the benefit of receiving information and data particularly relevant for motoring directly in front of his eyes on the windscreen. So, as an example, he is able to check out 
the current speed of the car or navigation data in his direct line of vision with maximum convenience at all times. 

In combination with the optional xenon headlights, the new BMW 5 Series comes with a daytime lights function using the corona rings in the dual round headlights. Another feature also available as an option on the new 
BMW 5 Series is Adaptive Headlight technology illuminating the road ahead in exactly the right way and in the right direction following the curves taken by the driver. In this case, the headlights swivel in accordance with the position of the steering wheel, the yaw rate of the car, and its road speed. The Bending Lights function, in turn, adjusts the direction of the headlights when taking bends at a speed of less than 40 km/h.

Yet another option available in the new BMW 5 Series as the only car in its segment is the High-Beam Assistant. This sophisticated lights system automatically switches off the high-beam headlights as soon as oncoming traffic is approaching, the distance to a vehicle ahead drops below a certain level, or the road ahead is sufficiently illuminated, for example in a built-up area or on a brightly lit road. 

BMW Night Vision, also available as an option, ensures an even higher standard of all-round safety when driving at night. Unique to the BMW 5 Series in this segment, BMW Night Vision is based on a thermal imaging camera able to detect people, animals and objects on and next to the road up to a distance of 300 metres, transmitting a clear, high-contrast image to the Control Display also used by the navigation system. 

Audio and navigation systems of the highest calibre. 

Top-end audio systems and telematics functions serve to enhance motoring comfort and driving pleasure in the new BMW 5 Series to an even higher level of enjoyment. Particularly the newly developed BMW Individual High End Audio System offers exceptional sound quality at all times. The heart of the system is BMW’s trendsetting DIRAC (Dual Input Room Acoustics Calculator) signal processing technology. 
The digital nine-channel amplifier with Digital Signal Processing (DSP) used in this case develops maximum output of 825 watts. The AUX-In connector, in turn, offers additional options for connecting external audio sources. This means that in practice the driver or passenger is able to integrate all versions of the Apple iPod (except the Shuffle) fully and conveniently into the audio system of the BMW 5 Series. 

A particular highlight in motoring comfort in the BMW 5 Series is the new generation of navigation systems available as an option. New menu guidance and optimised graphic presentation in the Control Display enhance both the appeal and function of these systems controlled and masterminded via BMW iDrive. 

Comfortable, generous, practical: the interior.

Comfortable seats and ample roominess inside the car make both the Saloon and Touring version of the BMW 5 Series ideal vehicles for grand touring 
and long distances. A further feature adding to the joy of driving the car 
is the electrical glass sliding roof available as an option.

Offering luggage compartment capacity of 520 litres, the BMW 5 Series gives the driver, and his passengers, superior space for taking along lots of luggage. 
A further important feature is the electrically retractable trailer hook and again makes the 5 Series quite unique. Since this new trailer hook no longer has to be fastened manually after moving out, operation and use of the trailer hook is significantly easier than before. 

###

For media enquiries
Guy Kilfoil

Manager: Automotive Communications

Email: guy.kilfoil@bmw.co.za

Tel: +2712 522 2227

Fax: +2711 805 2775

Mobile: +2782 583 6262

www.press.bmwgroup.com

www.quickpic.co.za

www.bmw.co.za

Specifications BMW 5 Series Saloon.
523i, 525i, 530i. 

	Body     

fsdfd

d

Anzahl Türen / s
	
	523i
	525i
	530i

	No of doors/seats     

l
	
	4/5
	4/5
	4/5

	Length/width/height (unladen)

)
	mm
	4,841/1,846/1,468
	4,841/1,846/1,468
	4,841/1,846/1,468

	Wheelbase
	mm
	2,888
	2,888
	2,888

	Track, front/rear      

wgsdfg


	mm
	1,558/1,581
	1,558/1,581
	1,558/1,581

	Turning circle
	m
	11.4
	11.4
	11.4

	Tank capacity
	approx ltr
	70
	70
	70

	Cooling system incl heating    

l
	ltr
	10.0 (10.5)
	10.0 (10.5)
	10.0 (10.5)

	Engine oil

Getriebeöl


	ltr
	6.5
	6.5
	6.5

	Trans fluid/final drive       
	ltr
	Lifetime
	Lifetime
	Lifetime

	Weight, unladen, to EU1
	kg
	1,575 (1,600)
	1,585 (1,605)
	1,605 (1,615)

	Max load to DIN
	kg
	560
	560
	560

	Max permissible to DIN
	kg
	2,010 (2,030)
	2,035 (2,055)
	2,050 (2,060)

	Max axle load, front/rear    
	kg
	980/1,170
	980/1,180
	980/1,180

	Max trailer load2
	             
	
	
	

	braked (12%)/unbraked  


	kg
	1,600 (1,800)/750
	1,800 (2,000)/750
	2,000/750

	Max roof load/trailer dn load
	kg
	100/90
	100/90
	100/90

	Luggage comp cap DIN70020
	l
	520
	520
	520

	Air drag     
	cd x A
	0.27 (0.26) x 2.26
	0.27 (0.26) x 2.26
	0.27 (0.26) x 2.26

	Power Unit
	
	
	
	

	Config/No of cyls/valves   Zylinder/Ventile


	
	Inline/6/4
	Inline/6/4
	Inline/6/4

	Fuel supply           
	
	           Siemens MSV70
	           Siemens MSV70
	           Siemens MSV70

	Capacity, effective
	cc 
	2,497
	2,497
	2,996 

	Stroke/bore
	mm
	78.8/82.0
	78.8/82.0
	88.0/85.0

	Compression ratio


	: 1
	11.0
	11.0
	10.7

	Fuel    
	
	
	
	

	Max output
	kW/hp
	140
	160
	200

	at  
	rpm
	5,800
	6,500
	6,600

	Max torque
	Nm/lb-ft
	235
	250
	315

	at          
	rpm  
	3,500
	2,750
	3,500

	Electrical System
	
	
	
	

	Battery/installation 
	Ah/–
	80/luggage comp
	80/luggage comp
	80/luggage comp

	Alternator   
	A/W
	155/2,170
	155/2,170
	155/2,170

	Chassis and Suspension
	
	
	
	

	Suspension, front   
	
	Double-joint tiebar spring strut axle, aluminium;
compensation of lateral forces, anti-dive

	Suspension, rear  
	
	Integral-IV multi-arm axle, aluminium, multi-dimensional suspension 
with anti-squat and anti-dive

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter   
	mm
	324 x 30, vented
	324 x 30, vented
	348 x 30, vented



	Brakes, rear  
	
	Single-piston swing-calliper disc brakes

	Diameter   
	mm
	320 x 20, vented
	320 x 20, vented
	345 x 24, vented



	Driving stability systems
	
	ABS, CBC, ASC, DSC, DTC, DBC; optional: Dynamic Drive

	Steering 


	
	Rack-and-pinion Servotronic steering; option: Active Steering

	Overall steering ratio   
	: 1
	13.7
	13.7
	13.7

	Gearbox   
	
	6-speed manual (6-speed automatic with Steptronic)

	Gear ratios
I

  
	: 1
	4.323 (4.171)
	4.323 (4.171)
	4.350 (4.171)

	
II
	: 1
	2.456 (2.340)
	2.456 (2.340)
	2.496 (2.340)

	
 III
	: 1
	1.659 (1.521)
	1.659 (1.521)
	1.665 (1.521)

	
IV
	: 1
	1.230 (1.143)
	1.230 (1.143)
	1.230 (1.143)

	  
V
	: 1
	1.000 (0.867)
	1.000 (0.867)
	1.000 (0.867)

	 
VI
	              : 1
	0.848 (0.691)
	0.848 (0.691)                        0,848 (0,691)
	0.851 (0.691)

	
R
	: 1
	3.938 (3.403)
	3.938 (3.403)
	3.926 (3.403)

	Final drive          
	: 1
	3.23 (3.73)
	3.15 (3.64)
	3.23 (3.64)

	Tyres 
	
	225/55 R16 95 V
	225/55 R16 95 W
	225/50 R17 94 W

	Rims   
	
	7J x 16 light-alloy
	7 J x 16 light-alloy
	7.5J x17 light-alloy

	Performance   
	
	
	
	

	Power-to-weight, DIN
	kg/kW
	10.7 (10.9)
	9.4 (9.6)
	7.7

	Output per litre 
	kW/hp
	56.1/76.3
	53.4/72.6
	66.8/90.8

	Acceleration 0–100 km/h
	sec
	8.3 (8.8)
	7.5 (7.7)
	6.3 (6.5)

	Standing-start km
	sec
	28.6 (29.2)
	27.2 (27.8)
	25.9 (26.2)

	80–120 km/h in 4th/5th gear
	sec
	9.0/– (–)
	7.9/– (–)
	6.6 (–)

	Top speed
	km/h
	235 
	245
	2504

	Fuel Consumption in EU Cycle
	
	
	
	

	Urban    
	ltr/100 km
	12.1 (13.4)
	12.4 (13.5)
	12.6 (13.6)

	Extra urban   
	ltr/100 km
	6.4 (6.9)
	6.5 (7.0)
	6.6 (6.8)

	Composite 
	ltr/100 km
	8.5 (9.3)
	8.7 (9.4)
	8.8 (9.3)

	CO2
	g/km
	205 (224)
	210 (227)
	212 (224)

	Miscellaneous
	
	
	
	

	Emission rating
	
	EU3
	EU3
	EU3


Figures in brackets apply to models with automatic transmission.


Weight of vehicle in road trim (DIN) plus 75 kg for driver and baggage.



2May be increased under certain conditions.                       


3Figures for performance and fuel consumption relate to RON 98.



4Electronically limited.

Specifications BMW 5 Series Saloon.
530d, 550i.

	Body     

fsdfd

d

Anzahl Türen / s
	
	530d
	550i

	No of doors/seats      

l
	
	4/5
	4/5

	Length/width/height (unladen)

)
	mm
	4,841/1,846/1,468
	4,841/1,846/1,468

	Wheelbase
	mm
	2,888
	2,888

	Track, front/rear       

wgsdfg


	mm
	1,558/1,581
	1,558/1,581

	Turning circle
	m
	11.4
	11.4

	Tank capacity
	approx ltr
	70
	70

	Cooling system incl heating

l
	ltr
	8.0 (8.2)
	13.8 (14.2)

	Engine oil

Getriebeöl


	ltr
	7.7
	8,0

	Trans fluid/final drive
	ltr
	Lifetime
	Lifetime

	Weight, unladen, to EU1
	kg
	1,665 (1,675)
	1,735

	Max load to DIN
	kg
	560
	560

	Max permissible to DIN
	kg
	2,140 (2,160)
	2,220

	Max axle load, front/rear
	kg
	1,050/1,200
	1,070/1,235

	Max trailer load2
	
	
	

	braked (12%)/unbraked   


	kg
	2,000/750
	2,000/750

	Max roof load/trailer dn load
	kg
	100/90
	100/90

	Luggage comp cap DIN70020
	ltr
	520
	520

	Air drag     
	cd x A
	0.28 (0.27) x 2.26
	0.28 (0.27) x 2.26

	Power Unit
	
	
	

	Config/No of cyls/valves    Zylinder/Ventile


	
	Inline/6/4
	V90/8/4

	Fuel supply          
	
	DDE 6.2.6
	ME9.2.3.

	Capacity, effective
	cc
	2,993
	4,799

	Stroke/bore
	mm
	90.0/84.0
	88.3/93.0

	Compression ratio


	:1
	17.0
	10.5

	Fuel     
	
	Diesel
	ROM 91–983

	Max output
	kW/hp
	160
	270/367

	at 
	rpm
	4,000
	6,300

	Max torque 
	Nm/lb-ft
	480
	490

	at         
	rpm
	1,750–3,000
	3,400

	Electrical System
	
	
	

	Battery/installation
	Ah/–
	90/luggage comp
	90/luggage comp

	Alternator  
	A/W
	180/2,520
	1,735

	Chassis and Suspension
	
	
	

	Suspension, front   
	
	Double-joint tiebar spring strut axle, aluminium; compensation of lateral forces, anti-dive

	Suspension, rear  
	
	Integral-IV multi-arm axle, aluminium, 
multi-dimensional suspension with anti-squat and anti-dive

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter    
	mm
	Vented, 324 x 30
	348 x 36, vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter   
	mm
	Vented, 320 x 20
	345 x 24, vented

	Driving stability systems
	
	ABS, CBC, ASC, DSC, DTC, DBC; optional: Dynamic Drive

ABS+CBC, ASC

ABS+CBC, ASC

	Steering


	
	Rack-and-pinion Servotronic steering; option: Active Steering

	Overall steering ratio   
	: 1
	13.7
	13.7

	Gearbox   
	
	Six-speed manual (six-speed automatic with Steptronic)

	Gear ratios
I

  
	: 1
	5.080 (4.171)
	4.055 (4.171)

	
II
	: 1
	2.804 (2.340)
	2.396 (2.340)

	
III
	: 1
	1.783 (1.521)
	1.582 (1.521)

	 
IV
	: 1
	1.260 (1.143)
	1.192 (1.143)

	
V
	: 1
	1.0 (0.867)
	1.0 (0.867)

	
VI
	: 1
	0.835 (0.691)
	0.872 (0.691)

	 
R
	: 1
	4.607 (3.403)
	3.677 (3.403)

	Final drive         
	: 1
	2.47 (2.65)
	2.93 (3.38)

	Tyres 
	
	225/50 R 17 94W
	225/50R17 94W

	Rims  
	
	7.5 J x 17 light-alloy
	8 J x 17 light-alloy

	Performance  
	
	
	

	Power-to-weight, DIN
	kg/kW
	9.94
	6.15

	Output per litre
	kW/hp
	53.46
	56.26

	Acceleration  0–100 km/h
	sec
	7.1 (7.2)
	5.5 (5.6)

	Standing-start km
	sec
	27.3 (27.7)
	24.3 (24.5)

	80–120 km/h in 4th/5th gear
	sec
	5.2
	5.2

	Top speed            
	km/h
	244
	250

	Fuel Consump in EU Cycle
	
	
	

	Urban   
	ltr/100 km
	9.4 (10.3)
	16.4 (15.9)

	Extra urban  
	ltr/100 km
	5.2 (5.9)
	8.0 (7.8)

	Composite
	ltr/100 km
	6.7 (7.5)
	11.1 (10.8)

	CO2
	g/km
	179 (200)
	267 (260)

	Miscellaneous
	
	
	

	Emission rating     
	
	EU3
	EU3


Figures in brackets apply to models with automatic transmission.


Weight of vehicle in road trim (DIN) plus 75 kg for driver and baggage.

2May be increased under certain conditions.

3Electronically limited.

Exterior and Interior Dimensions. 

BMW 5 Series Saloon.






































