1
BMW
Media

Information

07/2009
Page 7

[image: image1.png]


The new BMW X6 xDrive50i.
Contents.
The new BMW X6 xDrive50i.
Description. 
 2

Specifications. 
 5

Output and Torque Diagrams. 
 7
[image: image2.jpg]Torgue [Nm]

850 340

800 320
300 kW @ 5,500 min-'

750 ~ 300

700 / 280

650 / 260

600 Nm @ 1,750 min-' /
600 \ 240
550 / / N\ 220
500 / / \ 200
450 7 / \\ 180
400 / 160
350 // 140
300 / 120
250 / 100
200 / 80

150 / 60
/

100 40
50 20
0 0

0 1,000 2,000 3,000 4,000 5,000 6,000 7,000

Engine speed [min']

Output [kW]



The new BMW X6 xDrive50i.
A Coupé Above.
Midrand – BMW South Africa has expanded its dynamic and sophisticated BMW X6 Sport Activity Coupé range with a new flagship, the X6 xDrive50i.  With its new V8 Twin Turbo petrol engine, the third engine derivative following the introduction of the xDrive35i and xDrive35d models last year, the new top-of-the range has an unprecedented combination of on-road dynamic performance so typical of the BMW brand, and the unique supremacy typical of all BMW X models on all tracks and terrain. The new flagship to the BMW X6 range enters the market with the commitment to introduce a radical change in mentality, particularly with regards to the definition of performance.

The new eight-cylinder with Twin Turbo Technology.

The new BMW X6 xDrive50i adds an incomparably dynamic variant to the line-up of BMW X Models, boasting a particularly sporting and dynamic eight-cylinder.  Displacing 4.4 litres, the new V8 power unit in the BMW X6 xDrive50i features Twin Turbo technology and direct petrol injection (High Precision Injection) allowing a maximum output of 300kW throughout a broad range of engine speed from 5,500–6,400 rpm.  Engine torque reaches its peak of 600 Newton-metres throughout an unusually large speed range from 1,750–4,500 rpm.
The X6 xDrive50i is endowed with true sportscar performance and dynamism on the road.  The Sports Activity Coupé will accelerate from standstill to 100 km/h in a scintillating 5,4 seconds, with top speed restricted to 250km/h.

The new V8 power unit is also the most efficient engine in its class.  Combined with a number of further technologies developed in the overall context of BMW EfficientDynamics, the engine gives the BMW X6 xDrive50i fuel consumption and emission figures significantly better than other eight-cylinder vehicles competing with the BMW X Models.  Average fuel consumption of the BMW X6 xDrive50i is 12.5 litres/100 km, while the CO2 rating is 329 grams per kilometre.
Automatic transmission designed for sports motoring.

Power is transmitted to all four wheels by a six-speed automatic transmission with a particularly sporting gearshift enhanced to an even higher level.
The automatic transmission chooses the appropriate gear with incomparable spontaneity, precision and efficiency, benefiting not only the superior comfort, but also the supreme driving dynamics of the BMW X6.  Being directly connected to the engine, incorporating a converter clutch operating largely without slip, and allowing particularly precise choice of gears, the six-speed automatic transmission also helps to optimise the overall efficiency of the power unit in practice.

The automatic transmission shifts gears either in response to the electronic gear selector lever or the gearshift paddles on the steering wheel.  The shift pattern on the gear selector lever is the same as on a conventional automatic transmission, but with the selector level returning to its starting point after each gearshift.

Elegant exterior design:
The world’s first Sports Activity Coupé, the BMW X6 with its low side window areas and a roofline dropping smoothly to the rear, clearly offers all the proportions of a genuine, fully-fledged Coupé.  Yet, at the same time, enhanced ground clearance, flared wheel arches in striking design, four doors, a large rear tailgate as well as the high waistline all bear out a clear styling resemblance to the other models in the BMW X range.  This design is indeed an authentic expression of the dynamic driving potential offered by the BMW X6 through its drivetrain and suspension technology.  The larger pair of exhausts on the xDrive50i distinguishes the flagship model from its siblings.
Dynamic Performance Control for superior agility:
Featuring BMW’s intelligent xDrive all-wheel-drive technology with electronic control for variable distribution of drive power between the front and rear axles, the chassis and drivetrain of the BMW X6 consistently adjusts to driving conditions at all times and in all situations. At the same time, BMW’s ICM Integrated Chassis Management high-performance electronic control ensures perfect harmony of drivetrain and suspension functions within fractions of a second, spelling out maximum stability and supreme performance wherever you go and at all times.
Standard on the BMW X6, and featured for the first time on any BMW model in the world, Dynamic Performance Control ensures variable distribution of drive forces between the two rear wheels.  A world-first, this cutting-edge technology significantly enhances steering precision and, as a result, the agility of the vehicle from low speeds whenever required.  In fast bends with abrupt steering manoeuvres or when the driver suddenly decelerates, Dynamic Performance Control serves to further improve the stability of the entire vehicle, allowing the driver to safely enjoy new dimensions of driving dynamics.  On slippery or uneven surfaces, finally, Dynamic Performance Control provides the further advantage of even safer and more stable traction.
Enhanced driving comfort:

Inside, the new model – again in typical Coupé style – provides ample space for the driver and front passengers as well as two further occupants at the rear, travelling on generously dimensioned rear seats in sporting design, with individual seat contours and integrated headrests.
The level of standard specifications is out of the top drawer.  Features include self-levelling rear axle, reversing camera, glass sunroof, Professional Navigation system, Hi-Fi system with Digital Sound Processor and 6-DVD changer, Park Distance Control, Xenon headlights with headlight washer system, Automatic tailgate operation and cruise control with brake function.

Pricing:

BMW X6 xDrive50i Standard model

R890 500

BMW X6 xDrive50i Sport model

R906 000

BMW X6 xDrive50i Exclusive model

R926 800

BMW X6 xDrive50i Innovation model

R939 100

BMW X6 xDrive50i Individual model

R967 500

Specifications BMW X6 xDrive50i.

	Body     

d


	
	X6 xDrive50i
	

	No of doors/seats      

l
	
	5/4
	

	Length/width/height (unladen)

)
	mm
	4,877/1,983/1,6901)
	

	Wheelbase
	mm
	2,933


	

	Track, front/rear       


	mm
	1,644/1,706
	

	Turning circle
	m
	12.8
	

	Tank capacity
	approx ltr
	85
	

	Cooling system incl heating

l
	ltr
	17.2
	

	Engine oil


	ltr
	8.0
	

	Trans fluid/final drive
	ltr
	Lifetime
	

	Weight, unladen, to EU1
	kg
	2,265 (2,190)
	

	Max load to DIN
	kg
	650
	

	Max permissible to DIN
	kg
	2,840
	

	Max axle load, front/rear
	kg
	1,400/1,520
	

	Max trailer load2
	
	
	

	braked (12%)/unbraked   


	kg
	3,500/750
	

	Max roof load/trailer dn load
	kg
	100/140
	

	Luggage comp cap DIN70020
	ltr
	570–1,450
	

	Air drag     
	cd x A
	0.36 x 2.82
	

	Power Unit
	
	
	

	Config/No of cyls/valves    Zylinder/Ventile


	
	V/8/4
	

	Engine management
	
	MSD85
	

	Capacity, effective
	cc
	4,395
	

	Stroke/bore
	mm
	89.0/88.3
	

	Compression ratio


	:1
	10.0
	

	Fuel grade    
	
	RON 95–98
	

	Max output
	kW
	300
	

	at 
	rpm
	5,500–6,400
	

	Max torque 
	Nm
	600
	

	at         
	rpm
	1,750–4,500
	

	Electrical System
	
	
	

	Battery/installation
	Ah/–
	90/luggage comp
	

	Alternator  
	A/W
	210/2,940
	

	Chassis and Suspension
	
	
	

	Suspension, front   
	
	Double track arm axle; small, negative steering roll radius; anti-dive

	Suspension, rear  
	
	Integral axle; space-effect suspension with anti-squat and anti dive

	Brakes, front
	
	Singe piston frame calliper

	Diameter    
	mm
	385 x 36, vented

	Brakes, rear
	
	Single-piston swing calliper

	Diameter   
	mm
	385 x 24, vented

	Driving stability systems
	
	DSC III (HDC, DBC, ABS, ASC-X, ADB-X, DTC, Trailer Stability)



	Steering


	
	Rack-and-pinion Servotronic steering; option: Active Steering

	Overall steering ratio   
	: 1
	13.7

	Gearbox   
	
	Six-speed automatic with Steptronic, electronic gearlever and shift paddles on steering wheel

	Gear ratios
I

  
	: 1
	4.171

	
II
	: 1
	2.340

	
III
	: 1
	1.521

	 
IV
	: 1
	1.143

	
V
	: 1
	0.867

	
VI
	: 1
	0.691

	 
R
	: 1
	3.403

	Final drive         
	: 1
	3.636

	Tyres 
	
	255/50 R19 107W XL RSC 

	Rims
	
	9J x 19 light-alloy

	Performance  
	
	
	

	Power-to-weight, DIN
	kg/kW
	7.3

	Output per litre
	kW
	68.3

	Acceleration  0–100 km/h
	sec
	5.4

	Standing-start km
	sec
	24.6

	80–120 km/h in 4th/5th gear
	sec
	–/–

	Top speed            
	km/h
	250

	Fuel Consump in EU Cycle
	
	
	

	Urban   
	ltr/100 km
	18.2

	Extra urban  
	ltr/100 km
	10.1

	
	
	

	CO2
	g/km
	329

	Miscellaneous 
	
	
	
	

	Emission rating     
	
	EU4
	 

	Embankment angle, front/rear

°
	25.1/25.5


	 

	Ramp angle
	°
	19.1


	 

	Ground clearance, unladen
	mm
	212


	 

	Perm. all-wheel drive
	
	Variable


	variable


Figures in brackets apply to models with automatic transmission.


Weight of vehicle in road trim (DIN) plus 75 kg for driver and baggage.

2May be increased under certain conditions.

3Electronically limited.

BMW X6 xDrive50i.


###

For media enquiries:
Benedict Maaga
Manager: Group Automotive Communications
Email: benedict.maaga@bmw.co.za

Tel: +2712 522 2227

Fax: +2711 805 2775

Mobile: +2782 877 8998

www.press.bmwgroup.com

www.quickpic.co.za
www.bmw.co.za

