MINI
Media Information
11/2010
Page 4
[image: image1.jpg]



MINI
Media Information
11/2010
Page 39

The MINI Countryman.
Contents.

1.
Fun has no limits.
The MINI Countryman. 
 2


2.
Description in brief. 
 4

3.
MINI on a growth path.
The concept. 
8

4.
The first of its kind in the premium segment – 
and unmistakably a MINI.
The design. 
 10

5.
Lively and economical 
The engines. 
 15

6.
New routes to unbeatable agility.
Power transfer and chassis. 
 19

7.
Inspiringly versatile, imposingly safe.
Body and occupant protection. 
 22

8.
New customisation options for individualists.

Equipment and accessories.
 26

9.
Specifications. 
 32

10.
Output and torque diagrams. 
 38

11.
Exterior and interior dimensions. 
 40
[image: image2.jpg]


1.
Fun has no limits.

The MINI Countryman.

The MINI Countryman takes the hallmark driving fun of the brand into a new dimension. This is the first MINI to come with four doors and a large tailgate 
as well as a versatile interior offering space for up to five people. 
The MINI Countryman is the first member of the MINI family with a body measuring more than four metres in length and which can be ordered as an option with all-wheel drive. With these features, in addition to the Countryman’s raised seating position and optimised ride comfort, the world’s only premium vehicle of its kind crosses the boundaries of urban mobility to take the trademark MINI feeling into new dimensions and win over additional target groups.
The latest generation of petrol engines.
Two petrol engines are available in South Africa. The selection 
of petrol variants available in South Africa ranges from the MINI Cooper Countryman, which develops 90 kW while the MINI Cooper S Countryman, whose 1.6-litre four-cylinder engine is the most efficient unit in its displacement class, produces 135 kW. All variants of the MINI Countryman meet the EU5 exhaust gas standard in Europe and the ULEV II standard in the USA. 
MINI ALL4: Breaking new ground in driving fun.
Optional MINI ALL4 all-wheel drive allows the driving fun you expect from the brand to be experienced off the beaten track as well. The permanent all-wheel-drive system’s quick and precise responses to changing conditions provide 
a new, traction-led expression of the handling talents for which MINI is famed. An electromagnetic centre differential positioned directly on the final drive varies the distribution of power seamlessly between the front and rear axles. The ALL4 system is available for the MINI Cooper S Countryman. 
Newly developed suspension combines quintessential MINI 
agility with outstanding ride comfort.
With its precise handling and captivating agility, the MINI Countryman transplants the driving characteristics associated with the brand into a new vehicle concept. These attributes are complemented by outstanding ride comfort, equipping the Countryman perfectly for a wider range of usage. Underpinning this is sophisticated new suspension technology, which includes a front axle with MacPherson spring struts and forged track control arms, 
multi-link rear suspension and the electromechanical power steering system EPS. The MINI Countryman comes as standard with the Dynamic Stability Control (DSC) system. 
Design: Undeniably an individualist, unmistakably a MINI.
The MINI Countryman exudes an air of originality, robustness and versatility. Four doors and a large tailgate, a striking roof shape and an upright front end reinforce its status as an individualist within the MINI ranks. Other distinctive features are its MINI proportions and design cues. Short overhangs, a high window line, the wrap-around look of the windows and the car’s powerful 
over-the-wheel stance create a familiar appearance transplanted onto larger exterior dimensions. Elsewhere, the Countryman provides a fresh interpretation of characteristic MINI features, such as the hexagonal radiator grille, large headlights set into the bonnet, side indicator surrounds, voluminous wheel arches and vertical rear light clusters.  
Versatility and innovative details deliver cutting-edge functionality.
The MINI Countryman comes standard with a three-seater bench in the rear or – as a no-cost option – two individual seats for passengers. The rear 
seats slide fore-and-aft individually or in a 60 : 40 split (three-seat bench), 
the angle of the backrests can be adjusted, and the backrests can be 
folded down either individually or in a 40 : 20 : 40 split (three-seat bench) – 
all of which allows luggage capacity to be increased from 350 litres to a maximum 1,170 litres. The MINI Countryman is also fitted as standard with the innovative Centre Rail storage and fastening system. 
Top-class standard equipment, premiere for MINI Connected.
The list of standard equipment for the MINI Countryman includes air conditioning and a CD audio system. A wide range of customisation options are also available, such as exterior paint finishes and interior colours designed exclusively for the Countryman, a host of upholstery variants, trim elements and model-specific equipment items, and high-quality audio and navigation systems. A further in-car entertainment highlight is the unique MINI Connected system, which includes the news (via RSS feeds), Twitter and web radiofunctions. 
2.
Description in brief.
•
With the addition of the MINI Countryman as the fourth variant in its model family, MINI is taking the inimitable MINI feeling into a new dimension. 
The MINI Countryman crossover bridges the gap between the classic MINI concept and a contemporary Sports Activity Vehicle. Its broader range of abilities in both urban use and beyond paves the way for larger and evolving target groups with a greater need for space and flexibility 
to experience the driving fun that comes with a MINI. The brand’s first model to feature four doors and a wide-opening tailgate, the Countryman offers more space – which can be used in a wide variety of ways – to go with its raised seating position and optimised ride comfort. Plus, the 
“go-kart” feeling for which MINI is a byword has been preserved and takes on a new dimension with the optional MINI ALL4 all-wheel-drive system. 
The MINI Countryman expresses the defining virtues of the brand in terms of design, premium quality, handling, efficiency and the scope for customisation – and does so in an absorbingly individual way.
•
The MINI Countryman manifests an effective and assured development 
of the brand’s design language. The new model combines larger body dimensions, increased ground clearance and four side doors with the hallmark features of MINI design. Short overhangs, a high window line, 
a commanding over-the-wheel stance and the wrap-around look of the windows – extending all the way around the vehicle – create typical 
MINI proportions. The Countryman provides a fresh interpretation of classic MINI features, such as the roof line, hexagonal radiator grille, large headlights set into the bonnet, side indicator surrounds, capacious wheel arches and vertical rear light clusters. 
•
Using the MINI design language as a starting point, the stand-out features of the MINI Countryman are showcased in fitting style. The upright front end exudes presence and helps to optimise pedestrian protection. The increase in interior space is emphasised by large glass surfaces, the 
four-door layout by the distinctive contours of the roof. The extra-wide border around the lower part of the body and powerfully bolstered wheel arches are a nod to the robustness of the vehicle and its all-wheel drive. The precise transposition of the MINI style into a new vehicle segment ensures that the MINI Countryman comes across as a totally new type 
of vehicle and yet is instantly recognisable as a MINI.
•
The interior of the MINI Countryman is very much in keeping with the brand’s familiar style but boasts a selection of new design and functional elements. The slightly raised seating position allows easy entry, optimises the driver’s view over the road and reinforces the powerful character of 
the new model. The Centre Speedo and air vents are bordered by coloured rings. The unique MINI Centre Rail, meanwhile, runs lengthwise through the middle of the interior in place of a conventional centre console and opens up new ways of integrating storage boxes, cup holders, external audio devices, mobile phones and other comfort-enhancing elements just as you want them. Clip-in fixtures have a flexible range of positioning that allows the storage area to be divided up as desired and items to be kept close at hand at all times. Added to which, the Centre Rail creates an unconventional visual and functional connection between the front and rear compartments (when specified with two individual seats in the rear).
•
The MINI Countryman comes as standard with five seats, while two individual seats in the rear is available as a no-cost option. Its interior enhances comfort over long journeys with generous legroom, headroom and shoulder room. The rear seats can be moved forwards and backwards individually or in a 60 : 40 split (three-seat bench), the angle of the backrests can be adjusted and the backrests can be folded down either individually or in a 40 : 20 : 40 split (three-seat bench), all of which allows luggage capacity to be increased from 350 litres to a maximum 1,170 litres.
•
The MINI Countryman will be available with two petrol engines in South Africa. All the drive units are sourced from a new generation 
of engines optimised using the BMW Group’s development expertise. 
And they all meet the EU5 and ULEV II exhaust emission standards. Output comprises of 135 kW in the MINI Cooper S Countryman. With its twin-scroll turbocharger and direct injection now complemented by fully variable valve management, the 1.6-litre four-cylinder petrol engine in the range-topping model offers by far the best balance between output and fuel consumption in its displacement class. For the petrol engine variants a six-speed automatic transmission with Steptronic is available as an alternative to the standard six-speed manual.
•
The MINI Cooper S Countryman can be ordered as an option with MINI ALL4 permanent all-wheel drive. Here, an electromagnetic centre differential positioned directly on the final drive varies the distribution of power seamlessly between the front and rear axles. In normal driving conditions up to 50 per cent of the drive is sent to the rear wheels, in extreme situations as much as 100 per cent. The result is a new, traction-led expression of the agile handling for which MINI is famed. This ability is underpinned by sophisticated new suspension technology, which includes a front axle with MacPherson spring struts and forged track control arms, multi-link rear suspension and the electromechanical power steering system EPS. The MINI Countryman comes as standard with the Dynamic Stability Control (DSC) system. Available as an option are Dynamic Traction Control (DTC, standard on the MINI Cooper S Countryman – both with front-wheel drive and ALL4 all-wheel) and an electronic locking function for the front axle differential.
•
A crash-optimised body structure with precisely defined load paths and deformation zones ensures exceptional passive safety. Standard safety features include front and side airbags, plus side curtain airbags for the front and rear seats, three-point inertia-reel seat belts on all seats, belt tensioners and belt force limiters at the front and ISOFIX child seat attachments in the rear. A Tyre Defect Indicator with runflat tyres (across all models) also comes as standard.

•
New model-specific equipment features have been added to the usual extensive selection of exterior and interior customisation options available for the MINI. A newly configured range of interior colours, trim elements and upholstery materials allows for particularly harmonious combinations. Air conditioning, the MINI Centre Rail and an audio system with CD player are all fitted as standard, while the model-specific array of optional equipment items and accessories available for the MINI Countryman includes high-quality audio and navigation systems as well as mobile phone interfaces with full integration capability for the Apple iPhone or other smartphones. MINI Connected, meanwhile, delivers a unique form 
of in-car entertainment with the addition of news (via RSS feeds), Twitter and web radio functions, among others. Customers can also specify a large panoramic sunroof, Adaptive Headlights with xenon lights, a heated windscreen, 17- to 19-inch light-alloy wheels, sports suspension (with the vehicle height lowered by 10 mm ) and John Cooper Works performance components.
•
Engine variants:
MINI Cooper S Countryman: Four-cylinder petrol engine with 
twin-scroll turbocharger, direct injection and fully variable valve management based on the BMW Group’s VALVETRONIC technology.
 
Displacement: 1,598 cc, output: 135 kW at 5,500 rpm, max. torque: 240 Nm at 1,600 – 5,000 rpm (260 Nm with Overboost). Acceleration 0–100 km/h: 7.6 seconds, top speed: 215 km/h. Average fuel consumption according to EU standard: 6.6 litres/100 km, CO2 emissions: 154 g/km.

MINI Cooper Countryman: Four-cylinder petrol engine with fully variable valve management based on the BMW Group’s 
VALVETRONIC technology. Displacement: 1,598 cc, output: 90 kW at 6,000 rpm, max. torque: 160 Nm at 4,250 rpm. Acceleration 0–100 km/h: 10.5 seconds, top speed: 190 km/h.  Average fuel consumption according to EU standard: 6.5 litres/100 km, CO2 emissions: 152 g/km.

•
Exterior dimensions:
Length: 4,097 millimetres (MINI Cooper S Countryman: 4,110 millimetres) 
Width: 1,789 millimetres
Height: 1,561 millimetres Wheelbase: 2,595 millimetres 
[image: image3.png]=@



3.
MINI on a growth path.

The concept. 
The distinctive appearance of a MINI and the driving fun inextricably linked with the brand are now packaged in a totally new and extremely versatile vehicle displaying a multifaceted spread of excellence. The MINI Countryman crossover represents a whole new combination of the absorbing handling characteristics of a MINI with the versatile interior of a four-door car and the optimised traction of a Sports Activity Vehicle (in the shape of the optional 
all-wheel-drive system). At the same time, it provides a fresh and distinctive interpretation of established MINI features as far as design, premium quality, efficiency and customisation are concerned. The brand’s fourth model – joining the classic two-door MINI Hatch, MINI Clubman and MINI Convertible – meets the challenges of urban mobility with impressive flexibility, captures the imagination beyond traditional MINI territory with its broader range of abilities, and paves the way for an expanding band of prospective customers with varying mobility-based requirements to experience the trademark driving fun of a MINI. 
With the long-distance comfort of four individual seats or space for up 
to five people on board, plus a raised seating position, versatile interior, 
new generation of powerful and efficient engines and optional 
MINI ALL4 all-wheel-drive system, the Countryman adds another sprinkling 
of attractive features to the emotional driving experience of a MINI. Its body and interior concept invite you to share the MINI feeling with friends and in a variety of different situations. This propels the Countryman onto the radar of broader target groups, whose family or leisure pursuits have created an increased need for space and flexibility and who are therefore keen to blend the distinctive style of the brand with innovative functionality and a higher 
level of interior versatility.
Classic principle, innovative concept, hallmark MINI style.

The unmistakable MINI design language showcases the innovative vehicle concept of the Countryman in fitting style. The first MINI to measure more than four metres in length also follows the principle established for 
the classic Mini of creating maximum interior space and functionality on 
a compact footprint. In addition to the classic MINI proportions, characteristic design features at the front, side and rear of the car help to ensure that 
the MINI Countryman is immediately recognisable as a member of the brand family. 
The choice of name for the fourth model in the brand's range also reflects the historic roots and British origins of the MINI. Like the MINI Clubman, the 
MINI Countryman also builds on the heritage of a forefather based on the brilliant small car concept devised by Alec Issigonis. Indeed, a particularly versatile variant of the classic Mini bearing the Countryman name came onto the market as early as 1960. The Austin Seven Countryman and its technically identical Morris Mini Traveller sister model provided an extra dose of variability, their larger interior offering flexible usage potential. Available for purchase up to 1969, this variant of the Mini – not least the "woody" version with its wooden frame trim on the body flanks and rear doors – achieved a cult status unrivalled to this day.
The MINI Countryman will be built at the BMW Group’s Austrian partner 
firm Magna Steyr Fahrzeugtechnik in Graz. Production will take place on a separate assembly line set up and operated according to the exacting standards of the BMW Group. As with all the brand's models, the petrol engines for the MINI Countryman will be sourced from the Hams Hall engine plant, one corner of the MINI Production Triangle in England. 
MINI Countryman: The first premium model of its kind brings 
fresh variety to the MINI model family.

As the brand’s fourth model and the first premium car of its kind, the 
MINI Countryman is a pioneer in many respects. The new model is defined by classic MINI features, including its evocative design embodying the unmistakable style of the brand, the agile handling – a.k.a. “go-kart” feeling – which makes a MINI such fun to drive, and its premium quality, in keeping with the high standards of the BMW Group. The MINI Countryman paves the way for new target groups to experience these traits and talents. 
The characteristics mentioned above also open up new perspectives for the MINI brand itself. Expanding its model family allows the premium small car manufacturer to bring an unmatched variety to this segment of the market. 
In so doing, the MINI brand has demonstrated its ability once again – and more forcefully than ever – to grow with the demands of its fan base and yet remain true to its own identity. The MINI Countryman adds new and intriguing facets to the brand, and presents them in an innovative new body concept.
4. [image: image4.jpg]2000 3000 4000 5000 6000 7000 8000

1000



The first of its kind in the premium segment – 
and unmistakably a MINI.
The design.

The concept of the MINI Countryman puts clear water between the new model and the brand’s existing line-up. This new direction is expressed in 
its confident body design. Measuring 4,097 millimetres in length (MINI Cooper S Countryman: 4,110 mm), 1,789 in width and 1,561 millimetres in height, the four-door body combines the customary brand proportions with increased ground clearance and a raised seating position. The MINI Countryman therefore bridges the gap between the classic MINI concept and a state-of-the-art Sports Activity Vehicle. 
Like the wider spectrum of possibilities opened up by the innovative body concept of the MINI Countryman, its design embodies the further development of the MINI brand’s distinctive design language. The new model showcases fresh interpretations of fundamental MINI design elements. 
These include the striking three-level split of body, greenhouse and roof, the front end look (sculpted by large headlights, the hexagonal radiator grille and upright windscreen), the vertically stacked rear lights and the “cascading” contours of the rear, which broaden as they head from roof to wheel. Characteristic MINI design details like the headlights and rear lights integrated like “islands” into the body, the side scuttle between the front wing and door, and the greenhouse tapering back towards the rear of the car also clearly betray the identity of the Countryman as a MINI family member. At the same time their game-altering design gives them a special appeal.
These ingredients combine to highlight both the distinctive character of the new model and its family roots to unmistakable effect. The design of the 
MINI Countryman exudes originality, performance, a dependable robustness and versatility. And yet the customary MINI charm and individual allure 
remain resoundingly intact.
Front end with characteristic look and mature presence.
The clear structure of its front end gives the MINI Countryman a mature and stylishly imposing appearance. The upright radiator grille, strikingly contoured bonnet and large headlight units are all characteristic features of the front end. Rising up high, it meets the stipulations of current pedestrian protection legislation and lends the crossover model a powerful stance. The powerdome tapering forward to the front of the bonnet betrays the performance of the engine lurking below. Together with the main headlights, which occupy 
the traditional MINI position on the outer edges of the bonnet, the powerdome gives the surface of the car an unmistakable look. The newly designed headlights have chrome surrounds and extend conspicuously far into the shoulders of the car, lending extra emphasis to its width. The shape of 
the headlights is a marked departure from the classic circular form of the other MINI models in the range. The upper edge slants to the outside, adding further presence to the wheel arches. On the inner edge, meanwhile, they appear to be shrinking away from the broad-spread radiator grille.
The modified form of the radiator grille complements the stand-alone styling of the MINI Countryman: the enclosed hexagonal unit stands more upright than the grilles on the brand’s existing models. The three horizontal 
grille slats of the MINI Cooper Countryman reach a long way back, creating a free-floating impression. The slats on the MINI Cooper Countryman grille is painted silver. The front end of the MINI Cooper S Countryman has its very own hexagonal grille design in black and can also be identified by an additional air intake integrated into the grille and ventilation inlets for the brakes.
Larger stature and raised ground clearance define the side view. From the side the impeccable harmony of the metal body and greenhouse is particularly clear. The familiar MINI three-level split of body, greenhouse and roof is also striking on the Countryman. Despite its height and greater ground clearance, the keynote MINI proportions remain intact. The short front and rear overhangs highlight the brand’s trusted philosophy of creating maximum interior space within a compact footprint.
The distinctive contours of the MINI Countryman’s roof underline the 
four-door layout of the body and make the car easily recognisable. Functional roof rails are integrated into the roof as standard, lending extra visual effect to a silhouette reminiscent of a helmet. The greenhouse wraps itself like a glass ribbon around the car and exudes an inviting allure with its powerful transparency. The rising shoulder line of the body lends the greenhouse 
a “V” shape, which in turn gives the car the appearance of surging forward dynamically – even when at a standstill.
A precise character line, which arcs between the wheels along the lower third of the car’s flank, lowers its visual centre of gravity and stretches out the side view. This stylistic bridge between the front and rear axle also serves as 
a reference to the optional all-wheel drive. The imposing wheel arches grow powerfully out of the body of the car and symbolise its solid roadholding.
The black border around the lower part of the body is more clearly defined 
on the MINI Countryman than on the brand’s other models and underlines 
the raised ground clearance and clear emphasis on the wheels. The 
extra-long suspension travel of the wheels is also noticeable and hints at 
the car’s broader range of possible uses. The fresh design of the side scuttle at the transition from front wing to door also helps to set the MINI Countryman apart within the brand family. This element – another new interpretation 
of the welding seams in a similar position on the classic Mini – houses both the side direction indicators and an air outlet on the MINI Countryman. 
The side scuttle has a broad, extremely sturdy-looking design and leads 
the eye from the A-pillar down towards the front wheel. Like the path of the 
C-pillar heading towards the rear axle, this line also heightens the impression of the body’s powerful over-the-wheels stance.
Rear view: Attractive contrasts created by horizontal lines and
upright rear light clusters.
The proportions and horizontal structuring of the rear of the car underline 
the sporting capability of the MINI Countryman and create a strong sense 
of purpose. The body spreads out in stages from roof to wheels and accentuates the car’s solid stance, while the muscular shoulder line reinforces its masculine character. The roof, with its integrated, aerodynamically efficient spoiler lip, appears to be “planted” on top of the greenhouse. Another horizontal line is provided by the narrow chrome strip which wraps around 
the car as a border between the greenhouse and body.
As with all MINI models, the rear lights on the MINI Countryman are arranged in upright clusters, contrasting with the otherwise horizontal lines of the rear. These clusters have chrome surrounds and protrude from the body in 
three-dimensional form. Positioned on the outer edges of the rear end, 
they emphasise the width of the car and, in so doing, offer a clear indication 
of the generously sized interior. All the light functions are arranged within these “islands”. The inner structure of the lights is defined – in customary MINI style – by full and “sawn-off” circles, which give the rear of the car its distinctive appearance during night driving.
For the first time, the MINI logo at the rear of the car is also assigned 
a function. Pressing the inner circle of the brand emblem releases the upward-opening tailgate. The licence plate recess echoes the form of the 
air intake in the front bumper, forging a stylistic link between the front and 
rear of the car.
The rear apron of the MINI Cooper S Countryman takes the form of 
a diffuser, which channels the airflow under the rear of the car to optimise its aerodynamic properties. The most powerful variant in the Countryman range can also be identified by its model-specific roof spoiler, the twin-tailpipe exhaust system and the eye-catching cut-outs in the outer edges of the 
rear bumper. 
A selection of 11 body colours is available for the MINI Countryman – 
five non-metallic and six metallic paint finishes. Customers choosing a MINI Countryman can customise their car by specifying the roof and mirror caps in body colour or opt for the choice of contrast paint shades of Black or Light White as an alternative.
Standard-fitted 17-inch wheels on the MINI Cooper Countryman and MINI Cooper S Countryman (18-inch for the MINI Cooper S ALL4 Countryman) also contribute to the visual impact of the MINI Countryman. In addition, optionally available light-alloy wheels – some designed exclusively for the 
MINI Countryman – emphasise both the sporting and robust character of 
the new model. They are available in various designs and in sizes between 
17 and 18 inches. 
The optional Chrome Line Exterior, meanwhile, lends the outward appearance of the MINI Countryman a particularly exclusive edge. This package adds 
a model-specific sparkle to elements including the front bumper and side indicator surrounds. On the MINI Cooper S Countryman the front brake ventilation ducts also get a chrome surround and the rear bumper features a chrome strip. The radiator grille slats and rear licence plate surround of all other variants likewise benefit from a touch of chrome.
Interior: High-quality materials and a wide variety of design options.
The interior of the MINI Countryman shows some similarities with the established MINI design language, but adds a refreshing new identity of its own. All the interior surfaces have a new and eye-catching 
grain, which merges perfectly with the powerful styling. Thus the interior, 
too, reflects the innovative overall concept of the MINI Countryman 
and its distinctive character.
The dashboard of the MINI Countryman is striking with its pronounced, concave forms and a sporty yet functional feel. Particularly large side air vents underline an interior character defined by cutting-edge functionality. Like the central vents, Centre Speedo and gearshift lever, they are ringed in a broad surround sporting a contrasting colour. The side edging of the centre console adds another dash of colour and has the visual effect of supporting the dashboard. The centre console exudes an extremely high-quality feel with 
its matt black finish and chrome ring encircling the controls for the 
standard-fitted air conditioning. 
Newly designed interior door trim highlights the distinctive body shape and extra space of the MINI Countryman. The trademark MINI elliptical door ring around the armrests and door pulls is centrally positioned for the first time 
in the new model. This design element also takes in the B-pillars and emphasises the length of the interior with its extended contours.
The MINI Countryman is fitted with model-specific seats. The MINI Cooper S Countryman, for example, has sports seats as standard. All model variants can be specified as an option with sports seats in a choice of two cloth/leather combinations or two full leather options. Another highlight 
of the seat range is the Lounge variant, whose exceptionally high-quality hide is reminiscent of the piped leather seats of a classic British car.
The optional Colour Line offers a selection of additional customisation options which add colour emphasis to the door centre panel inside the elliptical door ring, the lower section of the B-pillar trim and the Centre Rail insert. 
The selection of colours comprises Carbon Black, Polar Beige, Pure Red, Dark Tobacco and – exclusively for the MINI Cooper S Countryman – the powerful Surf Blue. Meanwhile, customers can tailor the interior even more closely to their personal style with trim elements for the cockpit and Chrome Line Interior, which imbues the air vents, Centre Speedo, rev counter and gearshift lever surround with a subtle lustre.
[image: image5.jpg]1000 2000 3000 4000 5000 6000 7000 8000

0



5.
Lively and economical engines.

The latest generation of petrol engines have been recruited to power the MINI Countryman. Their sporting power development and optimised smoothness help to ensure that the Countryman displays both trademark MINI agility and excellent long-distance comfort. Plus, the remarkable ability of all the units to extract maximum benefit from the energy they consume enables the brand’s fourth model to boast a stand-out level of efficiency over its direct competitors. 
The MINI Countryman is available in South Africa with a choice of two petrol engines, whose technology is rooted in the superlative level of development know-how available within the BMW Group. All the engine variants available for the Countryman meet the EU5 emissions standard in Europe 
and the ULEV II standard in the USA.
New generation of petrol engines.

The two engines which can be ordered for the MINI Countryman boast both high output and exceptional efficiency. The petrol engines both belong to a new generation of drive units designed for MINI models. 
The technology they use is the product of the BMW Group’s impressive expertise in engine development. Instantaneous power delivery, free-revving characteristics, a high level of refinement and extraordinary efficiency are 
the central features which set the new MINI engines apart from the crowd.
All the petrol engines available for the MINI Countryman enjoy the benefit 
of fully variable valve management. This throttle-free load control technology – based on the VALVETRONIC system used in BMW engines and unique in 
the new MINI model’s segment – optimises the engine’s responses and 
at the same time makes for a significant reduction in fuel consumption and emissions. Within fractions of a second, this valve management adjusts 
the stroke and opening period of the intake valves to the amount of output required, the camshaft acting on the valves through an additional intermediate arm, and not directly through the cam follower. The pivot point of this additional intermediate arm is infinitely adjustable by an eccentric shaft controlled by an electric motor.
The petrol engines in the new MINI Countryman do have a throttle butterfly, but it is not used – as in conventional engines – to control load. Instead, under normal circumstances it remains fully open at all times to minimise the flow losses in the intake manifold; it is fitted solely as an emergency backup and 
for diagnostic purposes. This is true of both the naturally aspirated and the turbocharged variants of the four-cylinder petrol engine, which differ from each other principally in their mixture preparation.
MINI Cooper S Countryman: 
Bringing together variable valve management with 
turbocharging and direct injection for the first time.

The MINI Cooper S Countryman is the brand’s first model in which fully variable valve management has been combined with turbocharging and 
petrol direct injection. This impressive blend of technology helps the 1.6-litre four-cylinder unit with twin-scroll turbocharger to achieve a balance between output and fuel consumption that is unsurpassed in this displacement class. The fuel economy of this new engine in the EU test cycle is around 9 per cent better than its predecessor’s.
In a twin-scroll turbocharger the ducts of two cylinders are combined with 
one another in the exhaust manifold. This design enables the turbine to reach its maximum speed of 210,000 rpm at low engine revs. The full charge effect (approx. 0.8 bar overpressure) is on tap from as low down the engine speed range as 1,600 rpm. Instead of the familiar “turbo lag” delay in response characteristic of engines with conventional turbocharging, with this technology the driver can enjoy instantaneous power delivery. The compressed air is brought down to a low temperature in an intercooler before entering the combustion chamber. Increasing the specific oxygen content in this way optimises power output. 
Petrol is fed into the turbocharged engine by a direct injection system. 
A mechanically driven two-piston high-pressure pump delivers fuel to the injection valves via a stainless-steel distributor rail. The high-pressure valves then spray the fuel directly into the combustion chambers from the side at 
a maximum 120 bar of pressure, producing a homogeneous fuel/air mixture. The engine in the MINI Cooper S Countryman is not reliant on sulphur-free fuel and so can be used around the world.
The four-cylinder engine of the MINI Cooper S Countryman develops its maximum output of 135 kW at an engine speed of 5,500 rpm. 
Peak torque of 240 Nm is available between 1,600 and 5,500 rpm 
and can be raised to 260 Nm for a short time using the turbocharger’s Overboost function (approx. 1.0 bar overpressure). This serves up additional pulling power in the 1,700 to 4,500 rpm range when particularly dynamic acceleration is required. The MINI Cooper S Countryman sprints from 0 to 100 km/h in just 7.6 seconds (automatic: 7.9 seconds) on the way to a top speed of 215 km/h (automatic: 210 km/h). Average fuel consumption in the EU test cycle comes to 6.1 litres (7.1 litres) per 100 kilometres and CO2 emissions are 143 grams (166 grams) per km.  
The MINI Cooper S Countryman with ALL4 accelerates to 100 km/h (62 mph) in 7.9 seconds (automatic: 8.3 seconds) and has a top speed of 210 km/h (205 km/h). The average fuel consumption of the all-wheel-drive MINI Cooper S Countryman stands at 7.3 litres (automatic: 8.1 litres) per 100 km, its CO2 emissions at 171 grams (189 grams) per km.
MINI Cooper Countryman: 
Sporting character and exceptional efficiency.

The 1.6-litre naturally aspirated engines in the MINI Cooper Countryman also use fully variable valve management to support sporty power delivery and efficient fuel consumption. The power units are fed by a multi-point fuel injection system. Controlled individually by the engine electronics, single-spark ignition coils supply each spark plug with the optimum ignition voltage. In addition to fully variable valve management, other measures such as the map-controlled oil pump, thermostat-controlled water pump and a reduction in friction losses in the base engine also help these four-cylinder units to achieve an extremely effective balance between output and fuel consumption. The new units can claim an improvement of up to 9 per cent in fuel economy in the EU test cycle over their respective predecessor engines. 
The engine under the bonnet of the MINI Cooper Countryman generates 
its peak output of 90 kW at 6,000 rpm. Maximum torque of 160 Nm comes on stream at 4,250 rpm. The MINI Cooper Countryman dashes from 0 to 100 km/h in 10.5 seconds (automatic: 11.6 seconds) and is capable of a maximum 190 km/h (automatic: 182 km/h). Its fuel consumptionin the EU test cycle is 6.5 litres (7.6 litres) per 100 km, its CO2 emissions 152 grams (177 grams) per kilometre.
Six-speed manual gearbox fitted as standard, 
six-speed automatic optional.

All model variants of the MINI Countryman are fitted as standard with 
a six-speed manual gearbox matched perfectly to the engine’s performance characteristics. With its precise action and short shift travel it ticks all the right boxes for a premium vehicle in this segment. MINI Cooper Countryman and MINI Cooper S Countryman can both be ordered as an option with a six-speed automatic transmission with Steptronic and steering wheel shift paddles.
6.
New routes 
to unbeatable agility.
Power transfer and chassis.

While the state-of-the-art engines deliver the power, it is the newly developed chassis and the optional all-wheel-drive system designed and set up specifically for the MINI Countryman that ensure this power is harnessed 
into a new and therefore memorable driving experience. The fourth model in the brand’s line-up follows its stablemates in delivering a captivating level of agility unmatched by any of its direct rivals, immediately underscoring its identity as a chip off the MINI block. The sophisticated chassis technology enables this “go-kart” feeling – a distinguishing feature of every MINI – to 
be combined with ride comfort which meets the demands of both everyday driving and long-distance touring. 
Customers looking for an increased level of traction can specify the all-wheel-drive system MINI ALL4 for the MINI Cooper S Countryman model. Marking the debut of all-wheel drive in a MINI, ALL4 distributes power variably between the font and rear wheels. In so doing it paves the way for superlative traction on loose ground and an extra dose of driving fun when the driver decides to explore the sporting tendencies of the MINI Countryman. 
The advanced chassis technology of the MINI Countryman comprises a front axle with MacPherson spring struts and forged track control arms, multi-link rear suspension and the electromechanical power steering system EPS with speed-sensitive power assistance. In the interests of optimising active safety all models are equipped as standard with the DSC (Dynamic Stability Control) system. Available as an option are Dynamic Traction Control (DTC, standard on the MINI Cooper S Countryman) and an electronic locking function for the front axle differential.
MINI ALL4: More driving fun on or off the beaten track.

The MINI Cooper S Countryman can be specified as an option with the permanent all-wheel-drive system MINI ALL4. Based on an electromagnetic centre differential, this system distributes drive seamlessly between the front and rear axle. The result is a traction-led expression of the handling attributes for which MINI is famed, allowing the driver to experience the driving fun associated with the brand on rough surfaces as well as asphalt. Added to which, MINI ALL4 significantly enhances the car’s dynamic potential and imbues it with even more assured handling particularly in critical situations.
In normal situations, up to 50 per cent of the engine’s power is sent to 
the rear wheels; in extreme situations this can be 
as much as 100 per cent. This need-based distribution of drive between 
the front and rear axle optimises the efficiency of the MINI crossover. Integrating MINI ALL4 control electronics directly into the DSC control unit for the first time achieves reaction times of under a tenth of a second. This means the MINI Countryman can react to changing dynamic requirements with the right distribution of drive for the situation at hand. This gives the car unbeatable agility combined with supremely safe driving characteristics, as one has come to expect of MINI. The proactive intervention of the MINI ALL4 system to distribute drive as required by the situation at hand allows it to prevent spinning wheels. All of which means the driver enjoys both the dynamic benefits of the system and considerably enhanced comfort.
In addition, models equipped with ALL4 have a very specific chassis set-up. Together with the sophisticated final drive and drive shaft mounting, which includes an inner vibration damper, this set-up gives the all-wheel-drive variants of the MINI Countryman the same high standard of acoustic and dynamic comfort as the models with front-wheel drive.
Newly developed chassis gives the MINI Countryman 
the best handling characteristics in its segment.

The unique position held by the MINI Countryman in its segment is based, among other things, on the “go-kart” handling with which the brand is synonymous. The MINI chassis technology, which has been further developed for the new crossover model, comfortably harnesses the potential of the impressively powerful engines and converts it with great authority into captivating agility. Precise, emphatically sporty handling and excellent ride comfort lift the MINI Countryman head and shoulders above its competitors with comparable vehicle concepts.
The front axle of the Countryman has MacPherson spring struts and forged track control arms. This construction provides excellent wheel guidance and ensures that drive forces cannot be felt through the steering wheel, even under rapid acceleration or when cornering at high speed. In addition, the difference in length between the drive shafts to the left and right front wheels – the result of the transverse engine layout – is evened out by an intermediate shaft. This design element, normally only found on front and all-wheel-drive vehicles in higher segments, prevents contortion under acceleration and deceleration.
The multi-link rear axle of the MINI Countryman, which benefited from the BMW Group’s chassis expertise in its development, is designed to fit into either a front-wheel-drive or an all-wheel-drive construction. Anti-roll bars 
on the front and rear axle minimise body sway, further boosting the car’s safe 
and agile handling. The suspension of the MINI Cooper S Countryman has 
an even sportier set-up than the other model variants. In addition, sports suspension, which also lowers the vehicle’s ride height by 10 millimetres, can be specified as an option for all variants of the MINI Countryman.
The powerful brakes of the MINI Countryman allow precise inputs and maintain their performance even under heavy loads. The front brakes 
have inner-vented discs with a diameter of 294 mm (MINI Cooper S Countryman: 307 mm), the rear items are fitted with solid 280 mm discs.
Electromechanical power steering: 
Trademark MINI handling with unbeatable efficiency.
A large slice of the credit for the trademark MINI agility of the Countryman goes to EPS (Electric Power Steering), offered as standard in all model variants. The precise set-up of the system delivers remarkable accuracy at 
all speeds, as well as the clear feedback which has become a MINI hallmark. 
The power assistance provided by EPS is graded according to the speed of the car. Parking and manoeuvring at low speeds demand very little effort on behalf of the driver, but the degree of assistance is reduced at higher speeds to aid steering precision. Added to which, EPS also smoothes out jolts and other unwanted vibrations from the steering to impressive effect.
The agile handling of the Countryman – a characteristic feature of all 
MINI cars – is achieved with remarkable efficiency. In contrast to conventional mechanical/hydraulic systems, the steering’s power assistance is generated by an electric motor. This motor is only activated when power assistance 
is required or desired by the driver. Which means that on straight sections 
of road, or when driving around corners with a constant steering angle, 
no energy is used. EPS is therefore another element in the package of MINIMALISM measures designed to reduce fuel consumption and emissions.
Also on the list of standard equipment for the MINI Countryman is the DSC (Dynamic Stability Control) system. DSC comprises ABS anti-lock brakes, Electronic Brake Force Distribution (EBD), Cornering Brake Control (CBC), Brake Assist and Hill Assist. DTC mode allows controlled slip through the driven wheels to ease moving off on loose sand or deep snow. DTC is included as a standard feature on MINI Cooper S variants and is available as an option for other models. When the stability system is deactivated (in DSC Off mode) an electronic locking function for the front axle differential comes into play. In tight corners it brakes a spinning wheel as required to enhance handling. Known as Electronic Differential Lock Control (EDLC), this system enhances the car’s traction without adversely affecting its steering properties.
Both the MINI Cooper Countryman and MINI Cooper S Countryman have 17-inch light-alloy wheels fitted as standard. The MINI Cooper S ALL4 Countryman meanwhile, comes with standard 18-inch light-alloy rims. A wide choice of 17 and 18-inch rims are available as an option for all model variants and 19-inch wheels can be ordered from the accessories range to lend the Countryman an even sportier appearance.
7.
Inspiringly versatile, 

imposingly safe. 

Body and occupant protection.

Although the design of the MINI Countryman interior displays clear similarities with the familiar MINI design language, its larger dimensions alone give it a refreshingly original look. Indeed, the Countryman wheelbase measures 2,595 millimetres and the car stands 1,561 millimetres tall, allowing the interior to offer a degree of space and functionality that outstrips any model the brand has built before.
The safety concept of the MINI Countryman is likewise based on the principles developed for the brand’s cars and has been carried over in detail 
to the new vehicle concept. The body structure, member structures and the dimensions and arrangement of deformation zones are all designed to keep the forces generated in a collision away from the passenger compartment 
to the greatest possible degree. The restraint systems inside the 
MINI Countryman provide maximum protection in the widest possible range 
of accident scenarios.
Characteristic features of the MINI Countryman include a raised seating position, more space and sliding rear seats.
One defining feature of the MINI Countryman is its raised seating position. This enables comfortable entry into the car, optimises the driver’s overview 
of the road and provides a more intense driving experience along the lines 
of a Sports Activity Vehicle.
The standard three–seater rear bench can slide back or forwards separately through 130 millimetres in a 60 :  40 split. A pram, for example, can be accommodated in the luggage area of the MINI Countryman without moving the seats from their standard position.
The rear seat backrests can be moved into a cargo position should extra transportation capacity be required, increasing the size of the luggage area from 350 to 450 litres. Plus, the rear seat backrests can be folded down in part or in full to aid the flexible use of space in the rear. To this end, the backrests of the three-seat rear bench allow a 40 :  20 :  40 split, increasing the potential capacity of the luggage area to a maximum 1,170 litres. That creates enough space for two mountain bikes with detachable front wheels. Available as an option to supplement the standard roof rails is a rear carrier system, which can be prepared for fitting at the factory.
MINI Centre Rail: Unique storage concept with wide range 
of customisation options.
In place of a conventional centre console between the seats, the 
MINI Countryman is fitted with an innovative rail system. The unique 
MINI Centre Rail allows personal items to be kept within easy reach at all times. Added to which, it creates a visual and functional link between the front and rear compartments. If the MINI Countryman is fitted with individual seats in the rear, the U-profile Centre Rail runs through the middle of the passenger compartment back as far as the rear seat backrests. If the three-seat rear bench is specified, however, the MINI Centre Rail stops at the front seat backrests.
A specially developed and user-friendly clip-in attachment system 
allows the MINI Centre Rail to be divided up as desired and fitted with various storage elements. Everything from brackets for entertainment and telecommunications devices to cup holders, from a glasses case in 
MINI design to centre armrests, can be fitted to the rail safely and easily. 
The storage elements and holders can also slide backwards or 
forwards continuously along the Centre Rail. 
Comprehensive safety concept – including measures to optimise pedestrian protection.

The high level of passive safety achieved by the MINI Countryman is 
reflected in its solid and robust appearance. Rising up high, the front end not only lends the crossover model a particularly powerful stance, it meets the stipulations of current pedestrian protection legislation. Large deformation zones are designed to reduce the risk of injury in the event of a collision with 
a pedestrian or cyclist.
Inside the MINI Countryman the driver and passengers are protected by 
highly effective restraint systems. What’s more, its body structure is also designed to ensure maximum occupant protection in the event of a crash. Highly durable member structures, precisely defined deformation zones and an extremely strong passenger cell help to keep impact energy away from those inside the car. The external forces generated in a collision are diverted away from the passenger compartment to good effect via member structures in the floorpan and side frame, on the firewall and in the front and rear end of the car. This ensures that the impact energy is distributed between several different body components and transferred to the deformation zones. 
All the restraint systems are controlled by central safety electronics to ensure they are activated as required in different types of collision. The standard equipment on board the MINI Countryman includes front and side airbags, 
as well as side curtain airbags for the front and rear seats. All seats are fitted with three-point inertia-reel seat belts, while the front seats have belt tensioners and belt force limiters, and ISOFIX child seat attachments are provided in the rear.
The MINI Cooper S Countryman with the ALL4 drive system comes as standard with 225/45 R18 tyres featuring runflat technology. These tyres 
are available as an option for all other models. All MINI Countryman variants are fitted with the Tyre Defect Indicator and runflat technology as standard. This system monitors the tyres and alerts the driver to damage with a visible signal in the on-board computer display.
8.
New customisation options 

for individualists.

Equipment and accessories.

The MINI Countryman underlines its premium character with a high-quality array of standard equipment and a large number of options previously only available in higher vehicle classes. In addition, the Countryman can be ordered with a range of communications and entertainment systems which are unique and therefore break new ground. The range has been carefully tailored to 
a target group of customers who frequently use communications devices and maintain an interest in advanced entertainment technology. Here again, 
the MINI Countryman is a country mile ahead of its competitors. MINI is 
the world’s first car manufacturer to offer, as part of its MINI Connected concept, a newly developed USB interface which uses its unparalleled range of functionality to maximise the integration of the Apple iPhone into the car’s audio and infotainment system. This technology is exclusive to MINI and opens up a whole new level of integration in the use of smartphone functions.
The range of standard equipment fitted in the MINI Countryman includes air conditioning, the MINI Centre Rail and a high-spec radio system with MP3-compatible CD player, AUX IN connection and five loudspeakers. Added to this, customers can choose from a model-specific selection of optional extras and accessories to help them configure the new model in detail – and in 
true MINI style – to their personal requirements. 
Optimum visibility with Adaptive Headlights, foglamps and a rain sensor.

Customers can order their MINI Countryman with Adaptive Headlights in conjunction with the optional xenon units. This technology adjusts the beam angle of the headlights to the path of each corner by tracking the steering angle of the car. In addition, it offers variable light distribution, which adapts the beam of light to the speed of the car. The MINI Countryman is fitted with foglamps, while the rain sensor –optional – adjusts the frequency of the windscreen wipers to the intensity of the rainfall. The heated windscreen also helps to make adverse weather conditions less problematic. This option has long proved its value in MINI cars and is also available for the brand’s new model.
The MINI Countryman can be specified with a panoramic sunroof to ensure the perfect view skywards. The two-piece glass roof has a pop-up function at both the front and the rear, and comes with a sliding roof liner. The the rear-view mirror and exterior mirrors all have an automatic dimming function, while the three-spoke leather steering wheel and sports leather steering wheel form part of a package with a leather-covered gearshift lever or selector lever. All MINI Countryman variants are equipped with multifunction buttons on the steering wheel to operate the telephone (optional) and audio functions, and activate and deactivate the cruise control system. 
The ambient lighting system (part of the optional light package) adds a special allure to the interior of the MINI Countryman. This system uses indirect illumination to give a colour accent to the door centre panel during night driving, lending the interior a unique ambience. The colour of the light can 
be varied – using a toggle switch – between a warm orange and a sporty blue, just as the mood takes you. Optical fibres also bathe the top surface of the MINI Centre Rail in the desired colour tone along its full length. 
Comfort, functionality and sportiness, as desired.

Other features available to enhance the levels of on-board comfort include automatic climate control, heated seats, an armrest for the driver and front passenger, and a height-adjustable front passenger seat. The optional Comfort Access system allows the driver’s door to be opened and the engine started without taking the car key out of your pocket. The MINI Countryman can also be ordered with Park Distance Control, which makes reversing manoeuvres that much easier. The optional Storage Package includes features such as a retaining net in the passenger-side footwell, a 12-volt power socket in the luggage area, a high-quality glasses case, bag holders and a universal adapter for mobile devices – plus, if individual seats are specified for the rear, a further 12-volt power socket and two additional cup holders. A luggage compartment net is also available to complement the Storage Package. 
Customers can also select from an array of other equipment options which help to lend a particularly sporty touch to both the appearance of the 
MINI Countryman and the driving experience behind the wheel. For example, the rear spoiler fitted as standard on the MINI Cooper S Countryman is also available for the other model variants as an option, and a Sport Button can 
be added to the centre console of any variant of the new model. A press of this button gives the accelerator and steering particularly direct responses. 
In model variants fitted with an automatic gearbox, pressing the Sport Button also shortens gearchange times. The likewise optional sports suspension 
set-up also drops the car’s ride height by 10 millimetres.
Radio and navigation systems with new functions.

The radio systems available for the MINI Countryman offer a wide variety of options to allow customers to tailor the on-board entertainment and innovative communications functions to their personal preferences. The standard-fitted Radio MINI Boost CD includes an MP3-compatible CD player, AUX IN connection to hook up external music sources with the car’s own audio system and an on-board computer with two-line display in the Centre Speedo.
The MINI Visual Boost radio and MINI navigation system come with a 6.5-inch high-resolution display in the Centre Speedo and a Bluetooth hands-free facility with USB audio interface. The maps for the MINI navigation systems are stored on an on-board Flash memory device and can be updated via a USB interface. The map display can be viewed in a day and night mode. 
Also, when the car is stationary, video files stored on a compatible Apple iPod/iPhone can be viewed on the on-board monitor. And to optimise the audio experience, the options list also includes a Harman Kardon hi-fi loudspeaker system. With ten loudspeakers and a 480-watt amplifier, this system treats passengers to high-quality and dynamic music playback. 
In conjunction with the Bluetooth mobile phone preparation with USB audio interface, which also includes a snap-in adapter in the centre console with charging function and a roof aerial, both the MINI Visual Boost radio and 
MINI navigation system allow the use of add-on functions supported by a connected mobile device. These include audio streaming via Bluetooth, album cover artwork display on the on-board monitor and innovative office functions. For example, caller lists stored on a mobile phone and business cards from contacts can be viewed on the on-board monitor. In addition, 
the optional voice output function allows calendar entries recorded in a smartphone to be read out.
Innovative and ground-breaking: MINI Connected.
Both the MINI Visual Boost radio and MINI navigation system allow owners 
of an Apple iPhone access to MINI Connected. This development opens the door to a globally unique form of in-car entertainment, using new technology specially developed for the MINI to integrate the telecommunications, entertainment and online functions of the Apple iPhone into the car. MINI-specific functions can be integrated using an application for the Apple iPhone and operated using the joystick, steering wheel buttons and on-board monitor. Using the familiar MINI display and operating logic, MINI Connected allows all functions to be used comfortably, simply, safely and intuitively with minimal distraction from the road ahead. The mobile phone is hooked up to the car either via a cable supplied through the MINI Parts range for the USB socket or via a snap-in adapter.
Exclusive MINI technology for the integration of an Apple iPhone forms the basis for extensive update and add-on options. The functional repertoire of MINI Connected can be easily extended by dipping into the Apple App Store. This will allow MINI Connected customers to continue to benefit from technical advances and creative developments in the field of in-car entertainment into the future.
The application for MINI Connected, a full version of which will be available from the fourth quarter of 2010, includes a web radio function to allow users to pick up their preferred radio stations irrespective of their location. The station database available through the application contains thousands of radio stations whose programmes can be accessed online. The other functions of MINI Connected – such as access to your personal Twitter account and reception of user-definable RSS news feeds, the content of which is displayed on the on-board monitor and can be read out using the optional voice output function – are also unique in the small car segment. 
In addition, this application will gives MINI owners the platform to receive Facebook posts in their car, view them on the on-board monitor and use MINI Connected voice output to have them read out. Standardised text messages can also be sent out directly from the car via both services. This function enables MINI drivers to warn each other about traffic congestion or other problems on the road, for example.  
A further exclusive MINI innovation is the Dynamic Music function included 
in the Apple iPhone application. This comprises a selection of exclusively composed music which can be played on the audio system of the MINI and whose rhythm and dynamics change according to the driving style. This function allows MINI drivers to use their accelerator and steering wheel to create their own personal soundtrack to that trademark “go-kart” feeling.
The range of accessories: Functional, sporty and typically MINI.

The multi-faceted character of the MINI Countryman is also reflected in the selection of accessories available for the fourth model in the brand’s line-up. The range contains a host of high-quality products designed to allow even more thorough exploration of the new car’s broader range of abilities. What’s more, model-specific exterior components can be specified which open the door to even greater individualisation of the MINI Countryman. In addition, 
the sporting potential of the new model can be accentuated to impressive effect with John Cooper Works components.
The robust appearance of the MINI Countryman is given extra emphasis by underguard protection-style design elements for the front and rear ends. 
The silver-coloured plastic attachments can be fixed to the standard bumpers. For the interior, meanwhile, customers can order hard-wearing all-weather floor mats, and a luggage area mat in matching design is also available. Additional storage potential is provided by items such as a survival kit and a universal box, which can be fixed to the MINI Centre Rail.
The youngest of MINI fans can look forward to safe and comfortable journeys thanks to the new generation of MINI child seats. These seats, boasting a typically MINI design and orange/black colour scheme, can be secured safely in the rear of the car using the ISOFIX attachments provided. In addition, customers can order the Tablet DVD system as part of a customised entertainment package for the rear passengers. 
The accessories range also offers a variety of options which create additional transportation capacity beyond the boundaries of the flexible-usage interior. For example, a roof carrier system can be mounted to the standard-fitted roof rails of the MINI Countryman. Additional attachments can then allow the Countryman driver to transport large items of sports equipment or extra luggage. Among the accessories available are a bicycle rack, ski or snowboard holder, surfboard mount, universal carrier, and a roof box. A rear carrier for up to two bicycles can also be purchased for the MINI Countryman. This system can be mounted quickly and easily to the rear of the car with the help of the rear carrier preparation available as an option.
The exterior design accessories available for the MINI Countryman also lend 
it an unmistakable identity. The wide variety of items which customers can specify to achieve a detailed level of personalisation include mirror caps in Union Jack and Checkered Flag design, the bonnet Sport Stripes also available for other MINI models, and the Union Jack roof flag. 
The sporting character of the MINI Countryman is given an extra edge by the John Cooper Works performance components. The aerodynamics package available for the MINI Cooper S Countryman includes flaps for the front bumper, eye-catching side skirts (painted in the body colour and featuring an additional air intake), a diffuser for the rear apron, which generates extra downforce, and extra-large embellishers for the exhaust tailpipes. 19-inch double-spoke light-alloy wheels are also available exclusively from the range of John Cooper Works accessories. These can be ordered in matt black or in gloss black with a machined surface. For the interior, customers may be tempted by a three-spoke sports steering wheel with Alcantara cover and carbon clasp and a gearshift lever in the same material combination.
9.
Specifications.
MINI Cooper S Countryman,
MINI Cooper S Countryman Automatic.

	Body  
	
	          MINI Cooper S Countryman
	    MINI Cooper S Countryman
                                    Automatic
	

	No of doors/seats
	
	5 / 5 (4)
	5 / 5 (4)
	

	Length/width/height (unladen)
	mm
	4110 / 1789 / 1561
	4110 / 1789 / 1561
	

	Wheelbase
	mm
	2595
	2595
	

	Track, front/rear       
	mm
	1525 / 1551
	1525 / 1551
	

	Turning circle
	m
	11.6
	11.6
	

	Tank capacity
	ca. l
	47
	47
	

	Cooling system incl. heater 
	l
	5.5
	6.0
	

	Engine oil
	l
	4.2
	4.2
	

	Transmission oil incl. drive train


	l
	lifetime
	lifetime
	

	Weight, unladen to EU (DIN)1

	kg
	1310 / 1385
	1335 / 1410
	

	Max load to DIN
	kg
	470
	470
	

	Max permissible load
	kg
	1780
	1805
	

	Max axle load, front/rear     
	kg
	960 / 855
	980 / 855
	

	
	

	

	

	

	Max roofload/max download
	kg
	75 / 75
	75 / 75
	

	Luggage comp 
	l
	350 / 450 1170
	350 / 450 1170
	

	Air drag cx / A / cx x A
	 / m2 / m2
	0.36 / 2.35 / 0.85
	0.36 / 2.35 / 0.85
	

	Engine
	
	
	

	Config/No of cyls/valves 
	
	Inline / 4 / 4
	Inline / 4 / 4
	

	Engine management
	
	MEVD 17.2.2
	MEVD 17.2.2
	

	Capacity
	cm³
	1598
	1598
	

	Bore/stroke   
	mm
	77.0 / 85.8
	77.0 / 85.8
	

	Compression ratio
	:1 
	10.5
	10.5
	

	Fuel grade
	ROZ
	91-98
	91-98
	

	Max output
	kW / PS
	135 / 184
	135 / 184
	

	at
	min–1
	5500
	5500
	

	Max torque (with overboost)
	Nm
	240 (260)
	240 (260)
	

	at
	min–1
	1600 – 5000 (1700 – 4500)
	1600 – 5000 (1700 – 4500)
	

	Electrical System
	
	
	

	Battery/installation 
	Ah / –
	60 / Engine compartment


	55 / Engine compartment


	

	Alternator   
	A
	150 
	120 
	

	Chassis 
	
	
	

	Suspension, front   
	
	Single-joint MacPherson spring strut axle with anti-dive control


	

	Suspension, rear    
	
	Multi-link axle with trailing arms with aluminium lightweight construction


	

	Front brakes
	
	Disc vented


	Disc vented


	

	Diameter   
	mm
	307 x 24 
	307 x 24 
	

	Rear brakes
	
	Disc
	Disc
	

	Diameter   
	mm
	280 x 10
	280 x 10
	

	Driving stability systems
	Hydraulic 2-circuit brake system with Antilock Brake System (ABS), 
electronic brake force distribution (EBD) and Cornering Brake Control (CBC), Dynamic Stability Control (DSC) with Brake Assist and Hill Assist, 
Dynamic Traction Control (DTC) and Electronic Differential Lock Control (EDLC). Parking brake acts mechanically on rear wheels
	

	Steering
	Electric power steering (EPS); 2.4 rotations in total
	

	Steering transmission, overall
	:1 
	14.1
	14.1
	

	Tires


	
	205/55 R17 91V RSC
	205/55 R17 91V RSC
	

	Wheels
	
	7J x 17 LM
	7J x 17 LM
	

	Transmission
	
	
	

	Type of gearbox 
	
	Six-speed manual transmission 
	Six-speed automatic
	

	Gear ratios
I


	:1 
	3.308 
	4.044
	

	
II
	:1 
	2.130
	2.371
	

	
III
	:1 
	1.483 
	1.556
	

	
IV
	:1 
	1.139 
	1.159
	

	
V
	:1 
	0.949 
	0.852
	

	
VI
	:1 
	0.816 
	0.672
	

	Reverse gear


	:1
	3.231 
	3.193
	

	Final drive ratio


	:1
	3.706 
	3.683
	


	Performance 
	
	
	

	Power-to-weight ratio to DIN
	kg / kW
	9.7 
	9.9
	

	Output per litre
	kW / l
	84.5
	84.5
	

	Acceleration
0–100 km/h
	s
	7.6 
	7.9
	

	
0–1000 m
	s
	28.2 
	28.5
	

	In 4th/5th gear 
80–120 km/h
	s
	7.1 / 8.6
	– / –
	

	Top speed            
	km / h
	215
	210
	

	Fuel Consumption in EU Cycle
	
	
	

	Urban    
	l / 100 km
	8.3
	10.1
	

	Extra-urban   
	l / 100 km
	5.6
	6.0
	

	Composite 
	l / 100 km
	6.6 
	7.5
	

	CO2
	g / km
	154 
	175
	

	Miscellaneous 
	
	
	

	Emission rating     
	
	EU5
	EU5
	

	Ground clearance (empty)


	mm
	149
	149
	


1 Weight of the car in road trim (DIN) plus 75 kg for driver and luggage.

2 Deviations are possible under certain circumstances. 

Specifications.
MINI Cooper S Countryman ALL4,
MINI Cooper S Countryman ALL4 Automatic.

	Body  
	
	                                     MINI Cooper S 
                             Countryman ALL4
	                              MINI Cooper S 
                                  Countryman 
                          ALL4 Automatic
	

	No of doors/seats
	
	5 / 5 (4)
	5 / 5 (4)
	

	Length/width/height (unladen)
	mm
	4110 / 1789 / 1561
	4110 / 1789 / 1561
	

	Wheelbase
	mm
	2595
	2595
	

	Track, front/rear       
	mm
	1525 / 1551
	1525 / 1551
	

	Turning circle
	m
	11.6
	11.6
	

	Tank capacity
	ca. l
	47
	47
	

	Cooling system incl. heater 
	l
	5.5
	6.0
	

	Engine oil
	l
	4.2
	4.2
	

	Transmission oil incl. drive train


	l
	lifetime
	lifetime
	

	Weight, unladen to EU (DIN)1

	kg
	1380 / 1455 
	1405 / 1480
	

	Max load as peDIN
	kg
	460
	460
	

	Max permissible load
	kg
	1840 
	1865
	

	Max axle load, front/rear     
	kg
	980 / 895
	1000 / 895
	

	
	
	

	

	

	Max roofload/max download
	kg
	75 / 75
	75 / 75
	

	Luggage comp 
	l
	350 / 450 / 1170
	350 / 450 / 1170
	

	Air drag cx / A / cx x A
	- / m2 / m2 m2
	0.36 / 2.36 / 0.85
	0.36 / 2.36 / 0.85
	

	Engine
	
	
	

	Config/No of cyls/valves 
	
	Inline / 4 / 4
	Inline / 4 / 4
	

	Engine management
	
	MEVD 17.2.2
	MEVD 17.2.2
	

	Capacity
	cm³
	1598
	1598
	

	Bore/stroke   
	mm
	77.0 / 85.8
	77.0 / 85.8
	

	Compression ratio
	:1 
	10.5
	10.5
	

	Fuel grade
	ROZ
	91-98
	91-98
	

	Max output
	kW / PS
	135 / 184
	135 / 184
	

	at
	min–1
	5500
	5500
	

	Max torque (with overboost)
	Nm
	240 (260) 
	240 (260)
	

	at
	min–1
	1600 – 5000 (1700 – 4500)
	1600 – 5000 (1700 – 4500)
	

	Electrical System
	
	
	

	Battery/installation 
	Ah / –
	70 / Engine compartment


	55 / Engine compartment


	

	Alternator   
	A
	150 
	120 
	

	Chassis 
	
	
	

	Suspension, front   
	
	Single-joint MacPherson spring strut axle with anti-dive control
	

	Suspension, rear    
	
	Multi-link axle with trailing arms with aluminium lightweight construction
	

	Front brakes
	
	Disc vented
	Disc vented
	

	Diameter   
	mm
	307 x 24 
	307 x 24 
	

	Rear brakes


	
	Disc
	Disc
	

	Diameter   
	mm
	280 x 10
	280 x 10
	

	Driving stability systems
	Hydraulic 2-circuit brake system with Antilock Brake System (ABS), 
electronic brake force distribution (EBD) and Cornering Brake Control (CBC), Dynamic Stability Control (DSC) with Brake Assist and Hill Assist, 
Dynamic Traction Control (DTC) and Electronic Differential Lock Control (EDLC). Parking brake acts mechanically on rear wheels
	

	Steering
	Electric power steering (EPS); 2.4 rotations in total
	

	Steering transmission, overall
	:1 
	14.1
	14.1
	

	Tires


	
	225/45 R18 RSC
	225/45 R18 RSC
	

	Wheels


	
	7.5J x 18 LM
	7.5J x 18 LM
	

	Transmission
	
	
	

	Type of gearbox 
	
	Six-speed manual transmission
	

Six-speed automatic
	

	Gear ratios
I


	:1 
	3.308 
	4.044
	

	
II
	:1 
	2.130 
	2.371
	

	
III
	:1 
	1.483 
	1.556
	

	
IV
	:1 
	1.139 
	1.159
	

	
V
	:1 
	0.949
	0.852
	

	
VI
	:1 
	0.816 
	0.672
	

	Reverse gear


	:1
	3.231 
	3.193
	

	Final drive ratio


	:1
	3.706
	3.683
	


	Performance 
	
	
	

	Power-to-weight ratio to DIN
	kg / kW
	10.2 
	10.3
	

	Output per litre
	kW / l
	84.5
	84.5
	

	Acceleration
0–100 km/h
	s
	7.9
	8.3
	

	
0–1000 m
	s
	28.4 
	29.0
	

	In 4th/5th gear 
80–120 km/h
	s
	7.2 / 9.4
	– / –
	

	Top speed            
	km / h
	210
	205
	

	Fuel Consumption in EU Cycle
	
	
	

	Urban    
	l / 100 km
	9.2
	10.9

	

	Extra-urban   
	l / 100 km
	6.2
	6.5
	

	Composite 
	l / 100 km
	7.3 
	8.1
	

	CO2
	g / km
	171
	189
	

	Miscellaneous 
	
	
	

	Emission rating     
	
	EU5
	EU5
	

	Ground clearance (empty)


	mm
	149
	149
	


1 Weight of the car in road trim (DIN) plus 75 kg for driver and luggage.

2 Deviations are possible under certain circumstances. 

Specifications.
MINI Cooper Countryman,
MINI Cooper Countryman Automatic.

	Body 
	
	                   MINI Cooper Countryman
	     MINI Cooper Countryman
                                   Automatic
	

	No of doors/seats
	
	5 / 5 (4)
	5 / 5 (4)
	

	Length/width/height (unladen)
	mm
	4097 / 1789 / 1561
	4097 / 1789 / 1561
	

	Wheelbase
	mm
	2595
	2595
	

	Track, front/rear       
	mm
	1534 / 1559
	1534 / 1559
	

	Turning circle
	m
	11.6
	11.6
	

	Tank capacity
	ca. l
	47
	47
	

	Cooling system incl. heater 
	l
	5.5
	6.0
	

	Engine oil
	l
	4.2
	4.2
	

	Transmission oil incl. drive train


	l
	lifetime 
	lifetime 
	

	Weight, unladen to EU (DIN)1

	kg
	1265 / 1340
	1295 / 1370
	

	Max load to DIN
	kg
	470
	470
	

	Max permissible load
	kg
	1735 
	1765
	

	Max axle load, front/rear     
	kg
	930 / 855 
	960 / 855
	

	
	

	

	

	

	Max roofload/max download
	kg
	75 / - 
	75 / 75
	

	Luggage comp 
	l
	350 / 450 / 1170
	350 / 450 / 1170
	

	Air drag cx / A / cx x A
	- / m2 / m2
	0.35/ 2.36 / 0.83
	0.35/ 2.36 / 0.83
	

	Engine
	
	
	

	Config/No of cyls/valves 
	
	Inline / 4 / 4
	Inline / 4 / 4
	

	Engine management
	
	MEV 17.2.2
	MEV 17.2.2
	

	Capacity
	cm³
	1598
	1598
	

	Bore/stroke   
	mm
	77/ 85.8
	77/ 85.8
	

	Compression ratio
	:1 
	11.0
	11.0
	

	Fuel grade
	ROZ
	91-98
	91-98
	

	Max output
	kW / PS
	90 / 122
	90 / 122
	

	at
	min–1
	6000
	6000
	

	Max torque (with overboost)
	Nm
	160
	160
	

	at
	min–1
	4250
	4250
	

	Electrical System
	
	
	

	Battery/installation 
	Ah / –
	60 / Engine compartment


	55 / Engine compartment


	

	Alternator   
	A
	150 
	120
	

	Chassis 
	
	
	

	Suspension, front   
	
	Single-joint MacPherson spring strut axle with anti-dive control


	

	Suspension, rear    
	
	Multi-link axle with trailing arms with aluminium lightweight construction


	

	Front brakes 
	
	Disc vented
	Disc vented
	

	Diameter   
	mm
	294 x 22
	294 x 22
	

	Rear brakes 
	
	Disc
	Disc
	

	Diameter   
	mm
	280 x 10
	280 x 10
	

	Driving stability systems
	Hydraulic 2-circuit brake system with Antilock Brake System (ABS), 
electronic brake force distribution (EBD) and Cornering Brake Control (CBC), Dynamic Stability Control (DSC) with Brake Assist and Hill Assist, 
optional: Dynamic Traction Control (DTC) and Electronic Differential Lock Control (EDLC). Parking brake acts mechanically on rear wheels
	

	Steering
	Electric power steering (EPS); 2.4 rotations in total
	

	Steering transmission, overall
	:1 
	14.1
	14.1
	

	Tires


	
	205/55 R17 91V
	205/55 R17 91V
	

	Wheels


	
	7J x 17 LM
	7J x 17 LM
	

	Transmission
	
	
	

	Type of gearbox 
	
	Six-speed manual transmission
	Six-speed automatic
	

	Gear ratios
I


	:1 
	3.214
	4.148
	

	
II
	:1 
	1.792
	2.370
	

	
III
	:1 
	1.194
	1.556
	

	
IV
	:1 
	0.914
	1.155
	

	
V
	:1 
	0.784 
	0.859
	

	
VI
	:1 
	0.683 
	0.686
	

	Reverse gear


	:1
	3.143 
	3.394
	

	Final drive ratio


	:1
	4.722 
	4.643
	


	Performance
	
	
	

	Power-to-weight ratio to DIN
	kg / kW
	14.1
	14.4
	

	Output per litre
	kW / l
	56.3
	56.3
	

	Acceleration
0–100 km/h
	s
	10.5 
	11.6
	

	
0–1000 m
	s
	32.2
	33.3
	

	In 4th/5th gear 
80–120 km/h
	s
	11.6 / 14.9
	– / –
	

	Top speed            
	km / h
	190
	182
	

	Fuel Consumption in EU Cycle
	
	
	

	Urban    
	l / 100 km
	8.3
	9.9

	

	Extra-urban   
	l / 100 km
	5.5
	6.3
	

	Composite 
	l / 100 km
	6.5
	7.6
	

	CO2
	g / km
	152
	177
	

	Miscellaneous 
	
	
	

	Emission rating     
	
	EU5
	EU5
	

	Ground clearance (empty)
	mm
	149
	149
	


1 Weight of the car in road trim (DIN) plus 75 kg for driver and luggage.

2 Deviations are possible under certain circumstances. 


10.
Output and torque

diagrams.

MINI Cooper S Countryman.



MINI Cooper Countryman.


11.
Exterior and 

interior dimensions.

MINI Cooper S Countryman.

MINI Cooper Countryman.




135 kW @ 5500 min-1








240 Nm @ 1600-5000 min-1





Engine speed [min-1]





Output [kW]








Torque [Nm]





160 Nm @ 4250 min-1





90 kW @ 6000 min-1





Torque [Nm]





Engine speed [min-1]





Output [kW]

























































