Technical specifications.

The new BMW X5.

X5 xDrive30d.

|  |  | BMW X5 xDrive30d |  |
| --- | --- | --- | --- |
| Body |  |  |  |
| No of doors/seats |  | 5 / 5 (7) 1) |  |
| Length/width/height (unladen) | mm | 4922 / 2004 / 1745 |  |
| Wheelbase | mm | 2975 |  |
| Track, front/rear | mm | 1666 / 1685 |  |
| Turning circle | m | 12.6 |  |
| Fuel tank capacity | approx. l | 80 |  |
| Engine oil 2) | l | 6.5 |  |
| Weight, unladen, to DIN/EU | kg | 2110 / 2185 (2185 / 2260) 1) |  |
| Max load to DIN | kg | 770 (905) 1) |  |
| Max permissible weight | kg | 2860 (3090) 1) |  |
| Max axle load, front/rear | kg | 1320 / 1615 (1340 / 1870) 1) |  |
| Max trailer load, braked (12%)/unbraked | kg | 2700 / 750 |  |
| Max roofload/max towbar download | kg | 100 / 140 |  |
| Luggage comp capacity | l | 645 – 1860 |  |
| Air resistance | cd x A | 0.31 x 2.90 |  |
| Power Unit |  |  |  |
| Config/No of cyls/valves |  | In-line / 6 / 4 |  |
| Engine technology |  | BMW TwinPower Turbo technology: turbocharger with variable inlet geometry, common-rail direct injection with piezo injectors (max. injection pressure: 2500 bar) |  |
| Effective capacity | cc | 2993 |  |
| Stroke/bore | mm | 90.0 / 84.0 |  |
| Compression ratio | :1 | 16.5 |  |
| Fuel |  | Diesel |  |
| Max output | kW/hp | 195 / 265 |  |
| at | rpm | 4000 |  |
| Max torque | Nm | 620 |  |
| at | rpm | 2000 – 2500 |  |
| Electrical System |  |  |  |
| Battery/installation | Ah/– | 92 / luggage compartment |  |
| Driving Dynamics and Safety |  |  |  |
| Suspension, front |  | Double-wishbone axle in aluminium construction; optional: air suspension with automatic self-levelling |  |
| Suspension, rear |  | Five-link axle in lightweight steel construction; optional: air suspension with automatic self-levelling |  |
| Brakes, front |  | Single-piston fixed-calliper disc brakes, vented |  |
| Brakes, rear |  | Single-piston floating-calliper disc brakes, vented |  |
| Driving stability systems |  | Standard: DSC incl. ABS and DTC (Dynamic Traction Control), CBC (Cornering Brake Control), DBC (Dynamic Brake Control), Dry Braking function, Fading Compensation, Start-Off Assistant, DSC networked with xDrive all-wheel-drive system, ADB-X (Active Differential Brake), HDC (Hill Descent Control), Dynamic Damper Control; optional: active roll stabilisation, electronically controlled rear differential lock |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, three-point inertia-reel seatbelts on all seats with belt stopper, belt latch tensioner and belt force limiter in the front, crash sensors, tyre pressure indicator |  |
| Steering |  | Electric Power Steering (EPS); optional: Integral Active Steering |  |
| Steering ratio, overall | :1 | 18.7 |  |
| Tyres, front/rear |  | 255/55 R18 109W XL |  |
| Rims, front/rear |  | 8.5J x 18 light-alloy |  |

|  |  | BMW X5 xDrive30d |  |
| --- | --- | --- | --- |
| Transmission |  |  |  |
| Type of transmission |  | Eight-speed Steptronic transmission |  |
| Gear ratios I | :1 | 5.500 |  |
| II | :1 | 3.520 |  |
| III | :1 | 2.200 |  |
| IV | :1 | 1.720 |  |
| V | :1 | 1.317 |  |
| VI | :1 | 1.000 |  |
| VII | :1 | 0.823 |  |
| VIII | :1 | 0.640 |  |
| R | :1 | 3.993 |  |
| Final drive | :1 | 2.929 |  |
| Performance |  |  |  |
| Power-to-weight ratio (DIN) | kg/kW | 10.8 (11.2) 1) |  |
| Output per litre | kW/l | 65.2 |  |
| Acceleration 0–100 km/h | s | 6.5 |  |
| Top speed | km/h | 230 |  |
| Off-road characteristics |  |  |  |
| Angle of approach/departure | ° | 25.2 / 22.3 |  |
| Breakover angle | ° | 20.2 |  |
| Ground clearance when unladen | mm | 214 |  |
| Fording depth (at 7 km/h) | mm | 500 |  |
| BMW EfficientDynamics |  |  |  |
| BMW EfficientDynamics standard features |  | Brake Energy Regeneration with recuperation display, Electric Power Steering (EPS), Auto Start Stop function, Optimum Shift Indicator, ECO PRO mode with coasting function, BMW EfficientLightweight, optimised aerodynamic attributes, on-demand operation of ancillary units, map-regulated oil pump, efficiency- and weight-optimised all-wheel drive, differential and transfer case with optimised warm-up behaviour, tyres with reduced rolling resistance, BMW Blue Performance technology |  |
| Fuel Consumption ECE 3) |  |  |  |
| Urban | l/100 km | 7.7 – 6.8 |  |
| Extra-urban | l/100 km | 6.3 – 5.6 |  |
| Combined | l/100 km | 6.8 – 6.0 |  |
| CO2 | g/km | 179 – 158 |  |
| Emission rating |  | EU6d-TEMP |  |

Specifications apply to ACEA markets/data relevant to homologation applies in part only to Germany (weight)

All technical specifications are provisional

1) With optional third seat row

2) Oil change

3) Fuel consumption and CO2 emissions depend on the selected tyre format

The fuel consumption, CO2 emissions and electric power consumption figures were determined according to the European Regulation (EC) 715/2007 in the version applicable. The figures refer to a vehicle with basic configuration in Germany. The range shown considers the different sizes of the selected wheels/tyres and the selected items of optional equipment, and may vary during configuration.

The values for the vehicles marked * are already based on the new WLTP test cycle and have been translated back into NEDC-equivalent values in order to ensure comparability between the vehicles. With respect to these vehicles, for vehicle-related taxes or other duties based (at least inter alia) on CO2 emissions, the CO2 values may differ from the values stated here (depending on national legislation).

The CO2 efficiency specifications are determined according to Directive 1999/94/EC and the latest version of the Pkw-EnVKV, and based (for classification) on the fuel consumption and CO2 values as per the NEDC cycle.

Further information on official fuel consumption figures and specific CO2 emission values of new passenger cars is included in the following guideline: ‘Leitfaden über den Kraftstoffverbrauch, die CO2-Emissionen und den Stromverbrauch neuer Personenkraftwagen’ (Guideline for fuel consumption, CO2 emissions and electric power consumption of new passenger cars), which can be obtained free of charge from all dealerships and at https://www.dat.de/en/offers/publications/guideline-for-fuel-consumption.html.

The new BMW X5.
X5 M50d.

|  |  | BMW X5 M50d |  |
| --- | --- | --- | --- |
| Body |  |  |  |
| No of doors/seats |  | 5 / 5 (7) 1) |  |
| Length/width/height (unladen) | mm | 4922 / 2004 / 1745 |  |
| Wheelbase | mm | 2975 |  |
| Track, front/rear | mm | 1680 / 1687 |  |
| Turning circle | m | 12.6 |  |
| Fuel tank capacity | approx. l | 80 |  |
| Engine oil 2) | l | 6.5 |  |
| Weight, unladen, to DIN/EU | kg | 2275 / 2350 (2350 / 2425) 1) |  |
| Max load to DIN | kg | 820 (880) 1) |  |
| Max permissible weight | kg | 3010 (3160) 1) |  |
| Max axle load, front/rear | kg | 1395 / 1670 (1405 / 1865) 1) |  |
| Max trailer load, braked (12%)/unbraked | kg | 2700 / 750 |  |
| Max roofload/max towbar download | kg | 100 / 140 |  |
| Luggage comp capacity | l | 645 – 1860 |  |
| Air resistance | cd x A | 0.32 x 2.90 |  |
| Power Unit |  |  |  |
| Config/No of cyls/valves |  | In-line / 6 / 4 |  |
| Engine technology |  | M Performance TwinPower Turbo technology: multi-stage turbocharging with four turbochargers (two turbochargers for the high-pressure stage and two for the low-pressure stage), high-pressure turbochargers with variable inlet geometry, common-rail direct injection with piezo injectors (max. injection pressure: >2500 bar) |  |
| Effective capacity | cc | 2993 |  |
| Stroke/bore | mm | 90.0 / 84.0 |  |
| Compression ratio | :1 | 16.0 |  |
| Fuel |  | Diesel |  |
| Max output | kW/hp | 294 / 400 |  |
| at | rpm | 4400 |  |
| Max torque | Nm | 760 |  |
| at | rpm | 2000 – 3000 |  |
| Electrical System |  |  |  |
| Battery/installation | Ah/– | 92 / luggage compartment |  |
| Driving Dynamics and Safety |  |  |  |
| Suspension, front |  | Double-wishbone axle in aluminium construction; optional: air suspension with automatic self-levelling |  |
| Suspension, rear |  | Five-link axle in lightweight steel construction; optional: air suspension with automatic self-levelling |  |
| Brakes, front |  | Single-piston fixed-calliper disc brakes, vented |  |
| Brakes, rear |  | Single-piston floating-calliper disc brakes, vented |  |
| Driving stability systems |  | Standard: DSC incl. ABS and DTC (Dynamic Traction Control), CBC (Cornering Brake Control), DBC (Dynamic Brake Control), Dry Braking function, Fading Compensation, Start-Off Assistant, DSC networked with xDrive all-wheel-drive system, ADB-X (Active Differential Brake), HDC (Hill Descent Control), Dynamic Damper Control; optional: active roll stabilisation, electronically controlled rear differential lock |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, three-point inertia-reel seatbelts on all seats with belt stopper, belt latch tensioner and belt force limiter in the front, crash sensors, tyre pressure indicator |  |
| Steering |  | Electric Power Steering (EPS); optional: Integral Active Steering |  |
| Steering ratio, overall | :1 | 18.7 |  |
| Tyres, front/rear |  | 275/40 R21 107Y XL/ 315/35 R21 111Y XL |  |
| Rims, front/rear |  | 9.5J x 21 light-alloy/ 10.5J x 21 light-alloy |  |

|  |  | BMW X5 M50d |  |
| --- | --- | --- | --- |
| Transmission |  |  |  |
| Type of transmission |  | Eight-speed Steptronic transmission |  |
| Gear ratios I | :1 | 5.500 |  |
| II | :1 | 3.520 |  |
| III | :1 | 2.200 |  |
| IV | :1 | 1.720 |  |
| V | :1 | 1.317 |  |
| VI | :1 | 1.000 |  |
| VII | :1 | 0.823 |  |
| VIII | :1 | 0.640 |  |
| R | :1 | 3.993 |  |
| Final drive | :1 | 3.154 |  |
| Performance |  |  |  |
| Power-to-weight ratio (DIN) | kg/kW | 7.7 (8.0) 1) |  |
| Output per litre | kW/l | 98.2 |  |
| Acceleration 0–100 km/h | s | 5.2 |  |
| Top speed | km/h | 250 |  |
| Off-road characteristics |  |  |  |
| Angle of approach/departure | ° | 25.2 / 22.3 |  |
| Breakover angle | ° | 20.2 |  |
| Ground clearance when unladen | mm | 214 |  |
| Fording depth (at 7 km/h) | mm | 500 |  |
| BMW EfficientDynamics |  |  |  |
| BMW EfficientDynamics standard features |  | Brake Energy Regeneration with recuperation display, Electric Power Steering (EPS), Auto Start Stop function, Optimum Shift Indicator, ECO PRO mode with coasting function, BMW EfficientLightweight, optimised aerodynamic attributes, on-demand operation of ancillary units, map-regulated oil pump, efficiency- and weight-optimised all-wheel drive, differential and transfer case with optimised warm-up behaviour, BMW Blue Performance technology |  |
| Fuel Consumption ECE 3) |  |  |  |
| Urban | l/100 km | 8.1 – 7.6 |  |
| Extra-urban | l/100 km | 6.7 – 6.3 |  |
| Combined | l/100 km | 7.2 – 6.8 |  |
| CO2 | g/km | 190 – 179 |  |
| Emission rating |  | EU6d-TEMP |  |

Specifications apply to ACEA markets/data relevant to homologation applies in part only to Germany (weight)

All technical specifications are provisional

1) With optional third seat row

2) Oil change

3) Fuel consumption and CO2 emissions depend on the selected tyre format

The fuel consumption, CO2 emissions and electric power consumption figures were determined according to the European Regulation (EC) 715/2007 in the version applicable. The figures refer to a vehicle with basic configuration in Germany. The range shown considers the different sizes of the selected wheels/tyres and the selected items of optional equipment, and may vary during configuration.

The values for the vehicles marked * are already based on the new WLTP test cycle and have been translated back into NEDC-equivalent values in order to ensure comparability between the vehicles. With respect to these vehicles, for vehicle-related taxes or other duties based (at least inter alia) on CO2 emissions, the CO2 values may differ from the values stated here (depending on national legislation).

The CO2 efficiency specifications are determined according to Directive 1999/94/EC and the latest version of the Pkw-EnVKV, and based (for classification) on the fuel consumption and CO2 values as per the NEDC cycle.

Further information on official fuel consumption figures and specific CO2 emission values of new passenger cars is included in the following guideline: ‘Leitfaden über den Kraftstoffverbrauch, die CO2-Emissionen und den Stromverbrauch neuer Personenkraftwagen’ (Guideline for fuel consumption, CO2 emissions and electric power consumption of new passenger cars), which can be obtained free of charge from all dealerships and at https://www.dat.de/en/offers/publications/guideline-for-fuel-consumption.html.

Output and torque diagrams.

The new BMW X5.

X5 xDrive30d.

The new BMW X5.

X5 M50d.

Exterior and interior dimensions.

The new BMW X5.

* cm

*dimensions in cm