Specifications.

The new BMW 7 Series.

750Li xDrive.

|  |  | BMW 750Li xDrive |  |
| --- | --- | --- | --- |
| Body |  |  |  |
| No of doors/seats |  | 4 / 5 |  |
| Length/width/height 1) (unladen) | mm | 5260 / 1902 / 1479 |  |
| Wheelbase | mm | 3210 |  |
| Track, front/rear | mm | 1621 / 1650 |  |
| Ground clearance | mm | 135 |  |
| Turning circle | m | 12.9 |  |
| Fuel tank capacity | approx. l | 78 |  |
| Engine oil 2) | l | 10.5 |  |
| Weight, unladen, to DIN/EU | kg | 2000 / 2075 |  |
| Max load to DIN | kg | 675 |  |
| Max permissible weight | kg | 2675 |  |
| Max axle load, front/rear | kg | 1340 / 1395 |  |
| Max trailer load, braked (12%)/unbraked | kg | 2300 / 750 |  |
| Max roofload/max towbar download | kg | 100 / 100 |  |
| Luggage comp capacity | l | 515 |  |
| Air resistance | cd x A | 0.27 x 2.42 |  |
| Power Unit |  |  |  |
| Config/No of cyls/valves |  | V / 8 / 4 |  |
| Engine technology |  | BMW TwinPower Turbo technology: two TwinScroll turbochargers, High Precision Direct Injection, VALVETRONIC fully variable valve timing, Double-VANOS variable camshaft timing |  |
| Effective capacity | cc | 4395 |  |
| Stroke/bore | mm | 88.3 / 89.0 |  |
| Compression ratio | :1 | 10.5 |  |
| Fuel |  | min. RON 91 |  |
| Max output | kW/hp | 390 |  |
| at | rpm | 5500 – 6000 |  |
| Max torque | Nm | 750 |  |
| at | rpm | 1800 – 4600 |  |
| Electrical System |  |  |  |
| Battery/installation | Ah/– | 105 / luggage compartment |  |
| Driving Dynamics and Safety |  |  |  |
| Suspension, front |  | Double-wishbone axle in aluminium construction with separate lower track arm, small steering roll radius, anti-dive, air suspension with self-levelling |  |
| Suspension, rear |  | Five-link axle in aluminium construction, steerable, with anti-squat and anti-dive control, double acoustic decoupling, air suspension with self-levelling |  |
| Brakes, front |  | Vented disc brakes, with four-piston fixed callipers |  |
| Brakes, rear |  | Vented disc brakes, with single-piston floating callipers |  |
| Driving stability systems |  | Standard: DSC incl. ABS and DTC (Dynamic Traction Control), CBC (Cornering Brake Control), DBC (Dynamic Brake Control), Dry Braking function, Fading Compensation, Start-Off Assistant, xDrive all-wheel-drive system networked with DSC, Dynamic Damper Control |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, knee airbag on the driver’s side, three-point inertia-reel seatbelts on all seats with belt latch tensioner and belt force limiter in the front, crash sensors, tyre pressure indicator |  |
| Steering |  | Electric Power Steering (EPS) with Servotronic function, optional: Integral Active Steering |  |
| Steering ratio, overall | :1 | 16.3 |  |
| Tyres, front/rear |  | 245/45 R19 102Y XL |  |
| Rims, front/rear |  | 8.5J x 18 light-alloy |  |

|  |  | BMW 750Li xDrive |  |
| --- | --- | --- | --- |
| Transmission |  |  |  |
| Type of transmission |  | Eight-speed Steptronic transmission |  |
| Gear ratios I | :1 | 5.500 |  |
| II | :1 | 3.520 |  |
| III | :1 | 2.200 |  |
| IV | :1 | 1.720 |  |
| V | :1 | 1.317 |  |
| VI | :1 | 1.000 |  |
| VII | :1 | 0.823 |  |
| VIII | :1 | 0.640 |  |
| R | :1 | 3.993 |  |
| Final drive | :1 | 2.813 |  |
| Performance |  |  |  |
| Power-to-weight ratio (DIN) | kg/kW | 5.1 |  |
| Output per litre | kW/l | 88.7 |  |
| Acceleration 0–100 km/h | s | 4.1 |  |
| Top speed | km/h | 250 |  |
| BMW EfficientDynamics |  |  |  |
| BMW EfficientDynamics standard features |  | Brake Energy Regeneration with recuperation display, Electric Power Steering, Auto Start Stop function, Optimum Shift Indicator, ECO PRO mode with coasting function and Proactive Driving Assistant, BMW EfficientLightweight, optimised aerodynamic attributes, active air flap control, on-demand operation of ancillary units, map-regulated oil pump, differential and transfer case with optimised warm-up behaviour, tyres with reduced rolling resistance |  |
| Fuel Consumption ECE |  |  |  |
| Urban | l/100 km | 13.2 – 13.1 |  |
| Extra-urban | l/100 km | 7.5 – 7.5 |  |
| Combined | l/100 km | 9.6 – 9.5 |  |
| CO2 | g/km | 218 – 218 |  |
| Emission rating |  | Euro 6d-TEMP |  |

Specifications apply to ACEA markets/data relevant to homologation applies in part only to Germany (weight)

1) Height including roof fin

2) Oil change

All figures relating to performance, fuel/electric power consumption, CO2 emissions and operating range are provisional.

The fuel consumption, CO2 emissions, electric power consumption and operating range figures are determined according to the European Regulation (EC) 715/2007 in the version applicable. The figures refer to a vehicle with basic configuration in Germany. The range shown considers the different sizes of the selected wheels/tyres and the selected items of optional equipment, and may vary during configuration.

The values are already based on the new WLTP test cycle and are translated back into NEDC-equivalent values in order to ensure comparability between the vehicles. With respect to these vehicles, for vehicle-related taxes or other duties based (at least inter alia) on CO2 emissions, the CO2 values may differ from the values stated here (depending on national legislation).

Correct as at: 16.01.2019

Further information on official fuel consumption figures and specific CO2 emission values of new passenger cars is included in the following guideline: 'Leitfaden über den Kraftstoffverbrauch, die CO2-Emissionen und den Stromverbrauch neuer Personenkraftwagen' (Guide to the fuel economy, CO2 emissions and electric power consumption of new passenger cars), which can be obtained free of charge from all dealerships, from Deutsche Automobil Treuhand GmbH (DAT), Hellmuth-Hirth-Str. 1, 73760 Ostfildern-Scharnhausen and at https://www.dat.de/co2/.

The new BMW 7 Series.
M760Li xDrive.

|  |  |  | BMW M760Li xDrive |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |  |
| No of doors/seats |  |  | 4 / 5 |  |  |  |
| Length/width/height 1) (unladen) | mm |  | 5260 / 1902 / 1479 |  |  |  |
| Wheelbase | mm |  | 3210 |  |  |  |
| Track, front/rear | mm |  | 1621 / 1618 |  |  |  |
| Ground clearance | mm |  | 135 |  |  |  |
| Turning circle | m |  | 12.9 |  |  |  |
| Fuel tank capacity | approx. l |  | 78 |  |  |  |
| Engine oil 2) | l |  | 12.0 |  |  |  |
| Weight, unladen, to DIN/EU | kg |  | 2220 / 2295 |  |  |  |
| Max load to DIN | kg |  | 620 |  |  |  |
| Max permissible weight | kg |  | 2840 |  |  |  |
| Max axle load, front/rear | kg |  | 1410 / 1490 |  |  |  |
| Max trailer load, braked (12%)/unbraked | kg |  | – / – |  |  |  |
| Max roofload/max towbar download | kg |  | 100 / – |  |  |  |
| Luggage comp capacity | l |  | 515 |  |  |  |
| Air resistance | cd x A |  | 0.30 x 2.42 |  |  |  |
| Power Unit |  |  |  |  |  |  |
| Config/No of cyls/valves |  |  | V / 12 / 4 |  |  |  |
| Engine technology |  |  | BMW TwinPower Turbo technology: two mono-scroll turbochargers, High Precision Direct Injection, Double-VANOS variable camshaft timing |  |  |  |
| Effective capacity | cc |  | 6592 |  |  |  |
| Stroke/bore | mm |  | 88.3 / 89.0 |  |  |  |
| Compression ratio | :1 |  | 10.0 |  |  |  |
| Fuel |  |  | min. RON 91 |  |  |  |
| Max output | kW/hp |  | 430 |  |  |  |
| at | rpm |  | 5250 – 5750 |  |  |  |
| Max torque | Nm |  | 850 |  |  |  |
| at | rpm |  | 1600 – 4500 |  |  |  |
| Electrical System |  |  |  |  |  |  |
| Battery/installation | Ah/– |  | 105 / luggage compartment |  |  |  |
| Driving Dynamics and Safety |  |  |  |  |  |  |
| Suspension, front |  |  | Double-wishbone axle in aluminium construction with separate lower track arm, small steering roll radius, anti-dive, air suspension with self-levelling |  |  |  |
| Suspension, rear |  |  | Five-link axle in aluminium construction, steerable, with anti-squat and anti-dive control, double acoustic decoupling, air suspension with self-levelling |  |  |  |
| Brakes, front |  |  | Vented disc brakes, with four-piston fixed callipers |  |  |  |
| Brakes, rear |  |  | Vented disc brakes, with single-piston floating callipers |  |  |  |
| Driving stability systems |  |  | Standard: DSC incl. ABS and DTC (Dynamic Traction Control), CBC (Cornering Brake Control), DBC (Dynamic Brake Control), Dry Braking function, Fading Compensation, Start-Off Assistant, xDrive all-wheel-drive system networked with DSC, Dynamic Damper Control |  |  |  |
| Safety equipment |  |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, knee airbag on the driver’s side, three-point inertia-reel seatbelts on all seats with belt latch tensioner and belt force limiter in the front, crash sensors, tyre pressure indicator |  |  |  |
| Steering |  |  | Electric Power Steering (EPS) with Servotronic function, Integral Active Steering |  |  |  |
| Steering ratio, overall | :1 |  | 16.3 |  |  |  |
| Tyres, front/rear |  |  | 245/40 ZR20 99Y XL/ 275/35 ZR20 102Y XL |  |  |  |
| Rims, front/rear |  |  | 8.5J x 20 light-alloy/ 10J x 20 light-alloy |  |  |  |
|  |  |  |  | BMW M760Li xDrive |  |  |
| Transmission |  |  |  |  |  |  |
| Type of transmission |  |  |  | Eight-speed Steptronic transmission |  |  |
| Gear ratios I |  | :1 |  | 5.000 |  |  |
| II |  | :1 |  | 3.200 |  |  |
| III |  | :1 |  | 2.143 |  |  |
| IV |  | :1 |  | 1.720 |  |  |
| V |  | :1 |  | 1.313 |  |  |
| VI |  | :1 |  | 1.000 |  |  |
| VII |  | :1 |  | 0.823 |  |  |
| VIII |  | :1 |  | 0.640 |  |  |
| R |  | :1 |  | 3.478 |  |  |
| Final drive |  | :1 |  | 2.813 |  |  |
| Performance |  |  |  |  |  |  |
| Power-to-weight ratio (DIN) |  | kg/kW |  | 5.2 |  |  |
| Output per litre |  | kW/l |  | 65.2 |  |  |
| Acceleration 0–100 km/h |  | s |  | 3.8 |  |  |
| Top speed |  | km/h |  | 250 / 305 3) |  |  |
| BMW EfficientDynamics |  |  |  |  |  |  |
| BMW EfficientDynamics standard features |  |  |  | Brake Energy Regeneration with recuperation display, Electric Power Steering, Auto Start Stop function, Optimum Shift Indicator, ECO PRO mode with coasting function and Proactive Driving Assistant, BMW EfficientLightweight, optimised aerodynamic attributes, active air flap control, on-demand operation of ancillary units, map-regulated oil pump, differential and transfer case with optimised warm-up behaviour |  |  |
| Fuel Consumption ECE |  |  |  |  |  |  |
| Urban |  | l/100 km |  | 18.3 – 18.1 |  |  |
| Extra-urban |  | l/100 km |  | 9.2 – 8.9 |  |  |
| Combined |  | l/100 km |  | 12.5 – 12.4 |  |  |
| CO2 |  | g/km |  | 285 – 282 |  |  |
| Emission rating |  |  |  | Euro 6d-TEMP |  |  |

Specifications apply to ACEA markets/data relevant to homologation applies in part only to Germany (weight)

1) Height including roof fin

2) Oil change

3) With optional M Driver’s Package

All figures relating to performance, fuel/electric power consumption, CO2 emissions and operating range are provisional.

The fuel consumption, CO2 emissions, electric power consumption and operating range figures are determined according to the European Regulation (EC) 715/2007 in the version applicable. The figures refer to a vehicle with basic configuration in Germany. The range shown considers the different sizes of the selected wheels/tyres and the selected items of optional equipment, and may vary during configuration.

The values are already based on the new WLTP test cycle and are translated back into NEDC-equivalent values in order to ensure comparability between the vehicles. With respect to these vehicles, for vehicle-related taxes or other duties based (at least inter alia) on CO2 emissions, the CO2 values may differ from the values stated here (depending on national legislation).

Correct as at: 16.01.2019

Further information on official fuel consumption figures and specific CO2 emission values of new passenger cars is included in the following guideline: 'Leitfaden über den Kraftstoffverbrauch, die CO2-Emissionen und den Stromverbrauch neuer Personenkraftwagen' (Guide to the fuel economy, CO2 emissions and electric power consumption of new passenger cars), which can be obtained free of charge from all dealerships, from Deutsche Automobil Treuhand GmbH (DAT), Hellmuth-Hirth-Str. 1, 73760 Ostfildern-Scharnhausen and at https://www.dat.de/co2/.

All figures relating to performance, fuel/electric power consumption, CO2 emissions and operating range are provisional.

The fuel consumption, CO2 emissions, electric power consumption and operating range figures are determined according to the European Regulation (EC) 715/2007 in the version applicable. The figures refer to a vehicle with basic configuration in Germany. The range shown considers the different sizes of the selected wheels/tyres and the selected items of optional equipment, and may vary during configuration.

The values are already based on the new WLTP test cycle and are translated back into NEDC-equivalent values in order to ensure comparability between the vehicles. With respect to these vehicles, for vehicle-related taxes or other duties based (at least inter alia) on CO2 emissions, the CO2 values may differ from the values stated here (depending on national legislation).

Correct as at: 16.01.2019

Further information on official fuel consumption figures and specific CO2 emission values of new passenger cars is included in the following guideline: 'Leitfaden über den Kraftstoffverbrauch, die CO2-Emissionen und den Stromverbrauch neuer Personenkraftwagen' (Guide to the fuel economy, CO2 emissions and electric power consumption of new passenger cars), which can be obtained free of charge from all dealerships, from Deutsche Automobil Treuhand GmbH (DAT), Hellmuth-Hirth-Str. 1, 73760 Ostfildern-Scharnhausen and at https://www.dat.de/co2/.

The new BMW 7 Series.
745Le xDrive.

|  |  | BMW 745Le xDrive |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |  |  |
| No of doors/seats |  |  |  |  | 4 / 5 |  |  |
| Length/width/height 1) (unladen) | mm |  |  |  | 5260 / 1902 / 1479 |  |  |
| Wheelbase | mm |  |  |  | 3210 |  |  |
| Track, front/rear | mm |  |  |  | 1611 / 1640 |  |  |
| Ground clearance | mm |  |  |  | 135 |  |  |
| Turning circle | m |  |  |  | 12.9 |  |  |
| Fuel tank capacity | approx. l |  |  |  | 46 |  |  |
| Engine oil 2) | l |  |  |  | 6.5 |  |  |
| Weight, unladen, to DIN/EU | kg |  |  |  | 2085 / 2160 |  |  |
| Max load to DIN | kg |  |  |  | 620 |  |  |
| Max permissible weight | kg |  |  |  | 2705 |  |  |
| Max axle load, front/rear | kg |  |  |  | 1300 / 1500 |  |  |
| Max roofload/max towbar download | kg |  |  |  | 100 / – |  |  |
| Luggage comp capacity | l |  |  |  | 420 |  |  |
| Air resistance | cd x A |  |  |  | 0.27 x 2.42 |  |  |
| Power Unit |  |  |  |  |  |  |  |
| Drive concept |  |  |  | Full-hybrid drive, drive torque from engine or electric motor or both transmitted to all four wheels via BMW xDrive |  |  |  |
| Max system output | kW/hp |  |  |  | 290 / 394 |  |  |
| Max system torque | Nm |  |  |  | 600 |  |  |
| System power-to-weight ratio (DIN) | kg/kW |  |  |  | 7.2 |  |  |
| Petrol Engine |  |  |  |  |  |  |  |
| Config/No of cyls/valves |  |  |  |  | In-line / 6 / 4 |  |  |
| Engine technology |  |  |  | BMW TwinPower Turbo technology: TwinScroll turbocharger, High Precision Direct Injection, VALVETRONIC fully variable valve timing, Double-VANOS variable camshaft timing |  |  |  |
| Effective capacity | cc |  |  |  | 2998 |  |  |
| Stroke/bore | mm |  |  |  | 94.6 / 82.0 |  |  |
| Compression ratio | :1 |  |  |  | 11.0 |  |  |
| Fuel |  |  |  |  | min. RON 91 |  |  |
| Max output | kW/hp |  |  |  | 210 |  |  |
| at | rpm |  |  |  | 5000 – 6000 |  |  |
| Max torque | Nm |  |  |  | 450 |  |  |
| at | rpm |  |  |  | 1500 – 3500 |  |  |
| Output per litre | kW/l |  |  |  | 70.0 |  |  |
| Electric Motor |  |  |  |  |  |  |  |
| Motor technology |  |  |  | BMW eDrive technology: synchronous electric motor integrated into eight-speed Steptronic transmission, generator function for recuperating energy for the high-voltage battery |  |  |  |
| Max output | kW/hp |  |  |  | 83 / 113 |  |  |
| at | rpm |  |  |  | 3170 |  |  |
| Max torque | Nm |  |  |  | 265 |  |  |
| at | rpm |  |  |  | 0 – 2700 |  |  |
| Recuperation output | kW |  |  |  | 20 |  |  |
| High-voltage battery |  |  |  |  |  |  |  |
| Storage technology/installation |  |  | Lithium-ion / underneath rear seats |  |  |  |  |
| Voltage | V |  |  |  | 355 |  |  |
| Energy capacity (gross) | kWh |  |  |  | 12.0 |  |  |
| Charging time for 100% charge |  |  | 4.4 h at 3.7 kW (16 A / 230 V) |  |  |  |  |

|  |  |  |  |  | BMW 745Le xDrive |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Driving Dynamics and Safety |  |  |  |  |  |  |  |  |  |  |
| Suspension, front |  |  |  |  | Double-wishbone axle in aluminium construction with separate lower track arm, small steering roll radius, anti-dive, air suspension with self-levelling |  |  |  |  |  |
| Suspension, rear |  |  |  | Five-link axle in aluminium construction, steerable, with anti-squat and anti-dive control, double acoustic decoupling, air suspension with self-levelling |  |  |  |  |  |  |
| Brakes, front |  |  |  |  | Vented disc brakes, with four-piston fixed callipers |  |  |  |  |  |
| Brakes, rear |  |  |  |  | Vented disc brakes, with single-piston floating callipers |  |  |  |  |  |
| Driving stability systems |  |  |  | Standard: DSC incl. ABS, ASC and DTC (Dynamic Traction Control), CBC (Cornering Brake Control), DBC (Dynamic Brake Control), Dry Braking function, Fading Compensation, Start-Off Assistant, DSC networked with xDrive all-wheel-drive system, Dynamic Damper Control |  |  |  |  |  |  |
| Safety equipment |  |  |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, knee airbag on the driver’s side, three-point inertia-reel seatbelts on all seats with belt latch tensioner and belt force limiter in the front, crash sensors, tyre pressure indicator |  |  |  |  |  |  |
| Steering |  |  |  |  | Electric Power Steering (EPS) with Servotronic function, optional: Integral Active Steering |  |  |  |  |  |
| Steering ratio, overall |  | :1 |  |  |  |  |  | 16.3 |  |  |
| Tyres, front/rear |  |  |  |  |  |  |  | 245/50 R18 100Y |  |  |
| Rims, front/rear |  |  |  |  |  |  |  | 8J x 18 light-alloy |  |  |
| Transmission |  |  |  |  |  |  |  |  |  |  |
| Type of transmission |  |  |  |  |  | Eight-speed Steptronic transmission |  |  |  |  |
| Gear ratios I |  | :1 |  |  |  |  |  | 4.714 |  |  |
| II |  | :1 |  |  |  |  |  | 3.143 |  |  |
| III |  | :1 |  |  |  |  |  | 2.106 |  |  |
| IV |  | :1 |  |  |  |  |  | 1.667 |  |  |
| V |  | :1 |  |  |  |  |  | 1.285 |  |  |
| VI |  | :1 |  |  |  |  |  | 1.000 |  |  |
| VII |  | :1 |  |  |  |  |  | 0.839 |  |  |
| VIII |  | :1 |  |  |  |  |  | 0.667 |  |  |
| R |  | :1 |  |  |  |  |  | 3.317 |  |  |
| Final drive |  | :1 |  |  |  |  |  | 3.231 |  |  |
| Performance |  |  |  |  |  |  |  |  |  |  |
| Acceleration 0–100 km/h |  | s |  |  |  |  |  | 5.1 |  |  |
| Top speed |  | km/h |  |  |  |  |  | 250 |  |  |
| Top speed (electric) |  | km/h |  |  |  |  |  | 140 |  |  |
| Range (electric) |  | km |  |  |  |  |  | 50 – 54 |  |  |
| BMW EfficientDynamics |  |  |  |  |  |  |  |  |  |  |
| BMW EfficientDynamics standard features |  |  | BMW eDrive technology, Electric Power Steering, hybrid-specific Auto Start Stop function, Optimum Shift Indicator, ECO PRO mode with coasting function and Proactive Driving Assistant, BMW EfficientLightweight, optimised aerodynamic attributes, active air flap control, on-demand operation of ancillary units, map-regulated oil pump, differential and transfer case with optimised warm-up behaviour, tyres with reduced rolling resistance |  |  |  |  |  |  |  |
| Fuel Consumption ECE |  |  |  |  |  |  |  |  |  |  |
| Fuel consumption combined |  | l/100 km |  |  |  |  |  | 2.6 – 2.3 |  |  |
| CO2 emissions from fuel |  | g/km |  |  |  |  |  | 59 – 52 |  |  |
| Electric power consumption combined | kWh/100 km |  |  |  |  |  |  | 16.3 – 15.8 |  |  |
| Emission rating |  |  |  |  |  |  |  | Euro 6d-TEMP |  |  |

Specifications apply to ACEA markets/data relevant to homologation applies in part only to Germany (weight)

1) Height including roof fin

2) Oil change

All figures relating to performance, fuel/electric power consumption, CO2 emissions and operating range are provisional.

The fuel consumption, CO2 emissions, electric power consumption and operating range figures are determined according to the European Regulation (EC) 715/2007 in the version applicable. The figures refer to a vehicle with basic configuration in Germany. The range shown considers the different sizes of the selected wheels/tyres and the selected items of optional equipment, and may vary during configuration.

The values are already based on the new WLTP test cycle and are translated back into NEDC-equivalent values in order to ensure comparability between the vehicles. With respect to these vehicles, for vehicle-related taxes or other duties based (at least inter alia) on CO2 emissions, the CO2 values may differ from the values stated here (depending on national legislation).

Correct as at: 16.01.2019

Further information on official fuel consumption figures and specific CO2 emission values of new passenger cars is included in the following guideline: 'Leitfaden über den Kraftstoffverbrauch, die CO2-Emissionen und den Stromverbrauch neuer Personenkraftwagen' (Guide to the fuel economy, CO2 emissions and electric power consumption of new passenger cars), which can be obtained free of charge from all dealerships, from Deutsche Automobil Treuhand GmbH (DAT), Hellmuth-Hirth-Str. 1, 73760 Ostfildern-Scharnhausen and at https://www.dat.de/co2/.

The new BMW 7 Series.
730Ld.

|  |  |  | BMW 730Ld |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |  |
| No of doors/seats |  |  | 4 / 5 |  |  |  |
| Length/width/height 1) (unladen) | mm |  | 5260 / 1902 / 1479 |  |  |  |
| Wheelbase | mm |  | 3210 |  |  |  |
| Track, front/rear | mm |  | 1618 / 1646 |  |  |  |
| Ground clearance | mm |  | 135 |  |  |  |
| Turning circle | m |  | 12.8 |  |  |  |
| Fuel tank capacity | approx. l |  | 78 |  |  |  |
| Engine oil 2) | l |  | 6.5 |  |  |  |
| Weight, unladen, to DIN/EU | kg |  | 1870 / 1945 |  |  |  |
| Max load to DIN | kg |  | 690 |  |  |  |
| Max permissible weight | kg |  | 2560 |  |  |  |
| Max axle load, front/rear | kg |  | 1225 / 1400 |  |  |  |
| Max trailer load, braked (12%)/unbraked | kg |  | 2100 / 750 |  |  |  |
| Max roofload/max towbar download | kg |  | 100 / 100 |  |  |  |
| Luggage comp capacity | l |  | 515 |  |  |  |
| Air resistance | cd x A |  | 0.24 x 2.42 |  |  |  |
| Power Unit |  |  |  |  |  |  |
| Config/No of cyls/valves |  |  | In-line / 6 / 4 |  |  |  |
| Engine technology |  |  | BMW TwinPower Turbo technology: turbocharger with variable inlet geometry, common-rail direct injection with piezo injectors (max. injection pressure: 2500 bar) |  |  |  |
| Effective capacity | cc |  | 2993 |  |  |  |
| Stroke/bore | mm |  | 90.0 / 84.0 |  |  |  |
| Compression ratio | :1 |  | 16.5 |  |  |  |
| Fuel |  |  | Diesel |  |  |  |
| Max output | kW/hp |  | 195 |  |  |  |
| at | rpm |  | 4000 |  |  |  |
| Max torque | Nm |  | 620 |  |  |  |
| at | rpm |  | 2000 – 2500 |  |  |  |
| Electrical System |  |  |  |  |  |  |
| Battery/installation | Ah/– |  | 90 / luggage compartment |  |  |  |
| Driving Dynamics and Safety |  |  |  |  |  |  |
| Suspension, front |  |  | Double-wishbone axle in aluminium construction with separate lower track arm, small steering roll radius, anti-dive, air suspension with self-levelling |  |  |  |
| Suspension, rear |  |  | Five-link axle in aluminium construction, steerable, with anti-squat and anti-dive control, double acoustic decoupling, air suspension with self-levelling |  |  |  |
| Brakes, front |  |  | Vented disc brakes, with four-piston fixed callipers |  |  |  |
| Brakes, rear |  |  | Vented disc brakes, with single-piston floating callipers |  |  |  |
| Driving stability systems |  |  | Standard: DSC incl. ABS and DTC (Dynamic Traction Control), CBC (Cornering Brake Control), DBC (Dynamic Brake Control), Dry Braking function, Fading Compensation, Start-Off Assistant, Dynamic Damper Control |  |  |  |
| Safety equipment |  |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, knee airbag on the driver’s side, three-point inertia-reel seatbelts on all seats with belt latch tensioner and belt force limiter in the front, crash sensors, tyre pressure indicator |  |  |  |
| Steering |  |  | Electric Power Steering (EPS) with Servotronic function, optional: Integral Active Steering |  |  |  |
| Steering ratio, overall | :1 |  | 16.3 |  |  |  |
| Tyres, front/rear |  |  | 225/60 R17 99Y |  |  |  |
| Rims, front/rear |  |  | 7.5J x 17 light-alloy |  |  |  |
|  |  |  |  | BMW 730Ld |  |  |
| Transmission |  |  |  |  |  |  |
| Type of transmission |  |  |  | Eight-speed Steptronic transmission |  |  |
| Gear ratios I |  | :1 |  | 5.500 |  |  |
| II |  | :1 |  | 3.520 |  |  |
| III |  | :1 |  | 2.200 |  |  |
| IV |  | :1 |  | 1.720 |  |  |
| V |  | :1 |  | 1.317 |  |  |
| VI |  | :1 |  | 1.000 |  |  |
| VII |  | :1 |  | 0.823 |  |  |
| VIII |  | :1 |  | 0.640 |  |  |
| R |  | :1 |  | 3.993 |  |  |
| Final drive |  | :1 |  | 2.563 |  |  |
| Performance |  |  |  |  |  |  |
| Power-to-weight ratio (DIN) |  | kg/kW |  | 9.6 |  |  |
| Output per litre |  | kW/l |  | 65.2 |  |  |
| Acceleration 0–100 km/h |  | s |  | 6.2 |  |  |
| Top speed |  | km/h |  | 250 |  |  |
| BMW EfficientDynamics |  |  |  |  |  |  |
| BMW EfficientDynamics standard features |  |  |  | Brake Energy Regeneration with recuperation display, Electric Power Steering, Auto Start Stop function, Optimum Shift Indicator, ECO PRO mode with coasting function and Proactive Driving Assistant, BMW EfficientLightweight, optimised aerodynamic attributes, active air flap control, on-demand operation of ancillary units, map-regulated oil pump, differential with optimised warm-up behaviour, tyres with reduced rolling resistance, BMW Blue Performance technology with SCR catalytic converter |  |  |
| Fuel Consumption ECE |  |  |  |  |  |  |
| Urban |  | l/100 km |  | 6.4 – 6.3 |  |  |
| Extra-urban |  | l/100 km |  | 5.0 – 4.8 |  |  |
| Combined |  | l/100 km |  | 5.5 – 5.3 |  |  |
| CO2 |  | g/km |  | 145 – 139 |  |  |
| Emission rating |  |  |  | Euro 6d-TEMP |  |  |

Specifications apply to ACEA markets/data relevant to homologation applies in part only to Germany (weight)

1) Height including roof fin

2) Oil change

All figures relating to performance, fuel/electric power consumption, CO2 emissions and operating range are provisional.

The fuel consumption, CO2 emissions, electric power consumption and operating range figures are determined according to the European Regulation (EC) 715/2007 in the version applicable. The figures refer to a vehicle with basic configuration in Germany. The range shown considers the different sizes of the selected wheels/tyres and the selected items of optional equipment, and may vary during configuration.

The values are already based on the new WLTP test cycle and are translated back into NEDC-equivalent values in order to ensure comparability between the vehicles. With respect to these vehicles, for vehicle-related taxes or other duties based (at least inter alia) on CO2 emissions, the CO2 values may differ from the values stated here (depending on national legislation).

Correct as at: 16.01.2019

Further information on official fuel consumption figures and specific CO2 emission values of new passenger cars is included in the following guideline: 'Leitfaden über den Kraftstoffverbrauch, die CO2-Emissionen und den Stromverbrauch neuer Personenkraftwagen' (Guide to the fuel economy, CO2 emissions and electric power consumption of new passenger cars), which can be obtained free of charge from all dealerships, from Deutsche Automobil Treuhand GmbH (DAT), Hellmuth-Hirth-Str. 1, 73760 Ostfildern-Scharnhausen and at https://www.dat.de/co2/.

The new BMW 7 Series.
730Ld xDrive.

|  |  | BMW 730Ld xDrive |  |
| --- | --- | --- | --- |
| Body |  |  |  |
| No of doors/seats |  | 4 / 5 |  |
| Length/width/height 1) (unladen) | mm | 5260 / 1902 / 1479 |  |
| Wheelbase | mm | 3210 |  |
| Track, front/rear | mm | 1617 / 1646 |  |
| Ground clearance | mm | 135 |  |
| Turning circle | m | 12.9 |  |
| Fuel tank capacity | approx. l | 78 |  |
| Engine oil 2) | l | 6.5 |  |
| Weight, unladen, to DIN/EU | kg | 1930 / 2005 |  |
| Max load to DIN | kg | 695 |  |
| Max permissible weight | kg | 2625 |  |
| Max axle load, front/rear | kg | 1275 / 1415 |  |
| Max trailer load, braked (12%)/unbraked | kg | 2100 / 750 |  |
| Max roofload/max towbar download | kg | 100 / 100 |  |
| Luggage comp capacity | l | 515 |  |
| Air resistance | cd x A | 0.25 x 2.42 |  |
| Power Unit |  |  |  |
| Config/No of cyls/valves |  | In-line / 6 / 4 |  |
| Engine technology |  | BMW TwinPower Turbo technology: turbocharger with variable inlet geometry, common-rail direct injection with piezo injectors (max. injection pressure: 2500 bar) |  |
| Effective capacity | cc | 2993 |  |
| Stroke/bore | mm | 90.0 / 84.0 |  |
| Compression ratio | :1 | 16.5 |  |
| Fuel |  | Diesel |  |
| Max output | kW/hp | 195 / 265 |  |
| at | rpm | 4000 |  |
| Max torque | Nm | 620 |  |
| at | rpm | 2000 – 2500 |  |
| Electrical System |  |  |  |
| Battery/installation | Ah/– | 90 / luggage compartment |  |
| Driving Dynamics and Safety |  |  |  |
| Suspension, front |  | Double-wishbone axle in aluminium construction with separate lower track arm, small steering roll radius, anti-dive, air suspension with self-levelling |  |
| Suspension, rear |  | Five-link axle in aluminium construction, steerable, with anti-squat and anti-dive control, double acoustic decoupling, air suspension with self-levelling |  |
| Brakes, front |  | Vented disc brakes, with four-piston fixed callipers |  |
| Brakes, rear |  | Vented disc brakes, with single-piston floating callipers |  |
| Driving stability systems |  | Standard: DSC incl. ABS and DTC (Dynamic Traction Control), CBC (Cornering Brake Control), DBC (Dynamic Brake Control), Dry Braking function, Fading Compensation, Start-Off Assistant, xDrive all-wheel-drive system networked with DSC, Dynamic Damper Control |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, knee airbag on the driver’s side, three-point inertia-reel seatbelts on all seats with belt latch tensioner and belt force limiter in the front, crash sensors, tyre pressure indicator |  |
| Steering |  | Electric Power Steering (EPS) with Servotronic function, optional: Integral Active Steering |  |
| Steering ratio, overall | :1 | 16.3 |  |
| Tyres, front/rear |  | 225/60 R17 99Y |  |
| Rims, front/rear |  | 7.5J x 17 light-alloy |  |

|  |  | BMW 730Ld xDrive |  |
| --- | --- | --- | --- |
| Transmission |  |  |  |
| Type of transmission |  | Eight-speed Steptronic transmission |  |
| Gear ratios I | :1 | 5.500 |  |
| II | :1 | 3.520 |  |
| III | :1 | 2.200 |  |
| IV | :1 | 1.720 |  |
| V | :1 | 1.317 |  |
| VI | :1 | 1.000 |  |
| VII | :1 | 0.823 |  |
| VIII | :1 | 0.640 |  |
| R | :1 | 3.993 |  |
| Final drive | :1 | 2.563 |  |
| Performance |  |  |  |
| Power-to-weight ratio (DIN) | kg/kW | 9.9 |  |
| Output per litre | kW/l | 65.2 |  |
| Acceleration 0–100 km/h | s | 5.9 |  |
| Top speed | km/h | 250 |  |
| BMW EfficientDynamics |  |  |  |
| BMW EfficientDynamics standard features |  | Brake Energy Regeneration with recuperation display, Electric Power Steering, Auto Start Stop function, Optimum Shift Indicator, ECO PRO mode with coasting function and Proactive Driving Assistant, BMW EfficientLightweight, optimised aerodynamic attributes, active air flap control, on-demand operation of ancillary units, map-regulated oil pump, differential and transfer case with optimised warm-up behaviour, tyres with reduced rolling resistance, BMW Blue Performance technology with SCR catalytic converter |  |
| Fuel Consumption ECE |  |  |  |
| Urban | l/100 km | 6.7 – 6.5 |  |
| Extra-urban | l/100 km | 5.3 – 5.1 |  |
| Combined | l/100 km | 5.8 – 5.6 |  |
| CO2 | g/km | 153 – 147 |  |
| Emission rating |  | Euro 6d-TEMP |  |

Specifications apply to ACEA markets/data relevant to homologation applies in part only to Germany (weight)

1) Height including roof fin

2) Oil change

All figures relating to performance, fuel/electric power consumption, CO2 emissions and operating range are provisional.

The fuel consumption, CO2 emissions, electric power consumption and operating range figures are determined according to the European Regulation (EC) 715/2007 in the version applicable. The figures refer to a vehicle with basic configuration in Germany. The range shown considers the different sizes of the selected wheels/tyres and the selected items of optional equipment, and may vary during configuration.

The values are already based on the new WLTP test cycle and are translated back into NEDC-equivalent values in order to ensure comparability between the vehicles. With respect to these vehicles, for vehicle-related taxes or other duties based (at least inter alia) on CO2 emissions, the CO2 values may differ from the values stated here (depending on national legislation).

Correct as at: 16.01.2019

Further information on official fuel consumption figures and specific CO2 emission values of new passenger cars is included in the following guideline: 'Leitfaden über den Kraftstoffverbrauch, die CO2-Emissionen und den Stromverbrauch neuer Personenkraftwagen' (Guide to the fuel economy, CO2 emissions and electric power consumption of new passenger cars), which can be obtained free of charge from all dealerships, from Deutsche Automobil Treuhand GmbH (DAT), Hellmuth-Hirth-Str. 1, 73760 Ostfildern-Scharnhausen and at https://www.dat.de/co2/.

Output and torque diagrams.

The new BMW 7 Series.

750Li xDrive.

The new BMW 7 Series

M760Li xDrive.

The new BMW 7 Series

745Le xDrive.

Out   [kW] Output

]

The new BMW 7 Series

730Ld.

Exterior and interior dimensions.

The new BMW 7 Series.