Technical specifications.

BMW i4.

|  |  |  | BMW i4 eDrive40 |
| --- | --- | --- | --- |
| Vehicle Category |  |  |  |
| Drive type / body style |  |  | Battery electric vehicle (BEV) / Gran Coupé |
| Body |  |  |  |
| No. of doors / seats |  |  | 5 / 5 |
| Length/width/height (unladen) |  | mm | 4783 / 1852 / 1448 |
| Wheelbase |  | mm | 2856 |
| Track, front/rear |  | mm | 1601 / 1630 |
| Ground clearance (unladen) | mm |  | 125 |
| Turning circle |  | m | 12.5 |
| Weight, unladen (DIN/EU) |  | kg | 2050 / 2125 |
| Weight distribution (unladen), front/rear |  | % / % | 45.1 / 54.9 |
| Max. load to DIN |  | kg | 555 |
| Max. permissible weight |  | kg | 2605 |
| Max. axle load, front/rear |  | kg | 1140 / 1550 |
| Max. trailer load, braked (12%)/unbraked |  | kg | 1600 / 750 |
| Max. roofload/towbar download |  | kg | 75 / 75 |
| Luggage comp. capacity |  | l | 470 – 1290 |
| Air resistance |  | cX x A | 0.24 x 2.31 |
| Power Unit |  |  |  |
| Drive concept |  |  | Electric drive, transmission of the electric motor’s drive torque to the rear wheels |
| Max. system output |  | kW/hp | 250 / 340 |
| Max. system torque |  | Nm | 430 |
| System power-to-weight ratio |  | kg/kW | 8.2 |
| Max. recuperation output |  | kW | 116 |
| Type of transmission |  |  | Automatic transmission, single-speed with fixed ratio |
| Electric Motor |  |  |  |
| Motor technology |  |  | Fifth-generation BMW eDrive technology: electrically excited synchronous motor, power electronics and single-speed transmission sharing the same housing, generator function for recuperating energy |
| Peak output to ECE R 85 |  | kW/hp | 250 / 340 |
| at |  | rpm | 8.000-17.000 |
| Continuous output to ECE R 85 |  | kW/hp | xx / xx |
| Max. torque |  | Nm | 430 |
| at |  | rpm | 0-5.000 |
| Gear ratio |  | :1 | 8.774 |
| High-voltage Battery |  |  |  |
| Storage technology |  |  | Lithium-ion |
| Installation |  |  | Underfloor |
| Voltage |  | V | 398.5 |
| Battery capacity |  | Ah | 210.6 |
| Energy capacity, gross |  | kWh | 83.9 |
| Energy capacity, net |  | kWh | 80.7 |
| Charging time, 0 – 100 % charge |  |  | < 8.5 h at 11 kW (16 A / 380 V, three-phase AC, Wallbox) |
| Charging time, 10 – 80 % charge |  |  | 31 min at 210 kW initially (DC, fast-charging station) |
| Charging Unit |  |  |  |
| Type |  |  | Combined Charging Unit (CCU) with built-in 4 kW voltage transformer for supplying power to the 12V electrical system |
| Max. charging rate AC, single-phase |  | kW | 7.4 |
| Max. charging rate AC, three-phase |  | kW | 11.0 |
| Max. charging rate DC |  | kW | 210 |

|  |  |  |  |  |  | BMW i4 eDrive40 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Driving Dynamics and Safety |  |  |  |  |  |  |  |  |
| Suspension, front |  |  |  | Double-joint spring strut axle in lightweight aluminium-steel construction, hydraulically damped torque strut bearing |  |  |  |  |
| Suspension, rear |  |  |  | Five-link axle in lightweight aluminium-steel construction; optional: air suspension with automatic self-levelling |  |  |  |  |
| Brakes, front |  |  |  | Vented disc brakes, with four-piston fixed callipers |  |  |  |  |
| Brakes, rear |  |  |  | Vented disc brakes, with single-piston floating callipers |  |  |  |  |
| Driving stability systems |  |  |  | Standard: DSC incl. ABS, ASC and DTC (Dynamic Traction Control), ARB technology (near-actuator wheel slip limitation), CBC (Cornering Brake Control), DBC (Dynamic Brake Control), Dry Braking function, fading compensation, drive-off assistant, HDC (Hill Descent Control), trailer stability control, Performance Control; optional: adaptive M suspension |  |  |  |  |
| Safety equipment |  |  |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, three-point inertia-reel seatbelts on all seats with belt latch tensioner and belt force limiter in the front, crash sensors, tyre pressure indicator |  |  |  |  |
| Steering |  |  |  |  |  |  | Electric Power Steering (EPS) with Servotronic function; optional: variable sport steering |  |
| Steering ratio, overall |  | :1 |  |  |  |  | 15.5 |  |
| Tyres, front/rear |  |  |  |  |  |  | 225/55 R17 101Y XL |  |
| Rims, front/rear |  |  |  |  |  |  | 7.5J x 17 light-alloy |  |
| Performance |  |  |  |  |  |  |  |  |
| Acceleration 0–100 km/h |  | s |  |  |  |  | 5.7 |  |
| Acceleration 0–60 km/h |  | s |  |  |  |  | xx |  |
| Acceleration 80–120 km/h |  | s |  |  |  |  | xx |  |
| Top speed |  | km/h |  |  |  |  | 190 (electronically limited) |  |
| Electric Power Consumption / Range |  |  |  |  |  |  |  |  |
| Electric power consumption combined (WLTP) |  |  | kWh/100 km |  |  |  | 20 – 16 |  |
| Range (WLTP) |  |  | km |  |  |  | up to 590 |  |
| Environmental Characteristics |  |  |  |  |  |  |  |  |
| Emission rating |  |  |  |  |  |  | Electric vehicle |  |
| Carbon Life Cycle Assessment when using green power in the use phase compared to BMW xxxxxxx |  |  |  |  |  |  | –xx % |  |
| Carbon Life Cycle Assessment when using EU28 power mix in the use phase compared to BMW xxxxxx |  |  |  |  |  |  | –xx % |  |

Provisional specifications apply to ACEA markets/data relevant to homologation applies in part only to Germany (weight)

Official fuel consumption, CO2 emissions, electric power consumption and electric range figures were determined based on the prescribed measurement procedure in accordance with European Regulation (EC) 2007/715 in the version applicable. They refer to vehicles in the German market. Where a range is shown, NEDC figures consider the different sizes of the selected wheels/tyres, while WLTP figures take into account the impact of any optional extras.

WLTP values are used for determining vehicle-related taxes or other duties based (at least inter alia) on CO2 emissions as well as eligibility for any applicable vehicle-specific subsidies. Any NEDC values that are shown were calculated based on the new WLTP measurement procedure where appropriate and translated back into equivalent NEDC measurements in order to ensure comparability between the vehicles. Only official figures based on the WLTP procedure are available for new models that have been type tested since 01.01.2021. Further information on the WLTP and NEDC measurement procedures can also be found at www.bmw.de/wltp.

Further information on official fuel consumption figures and specific CO2 emission values of new passenger cars is included in the following guideline: 'Leitfaden über den Kraftstoffverbrauch, die CO2-Emissionen und den Stromverbrauch neuer Personenkraftwagen' (Guide to the fuel economy, CO2 emissions and electric power consumption of new passenger cars), which can be obtained free of charge from all dealerships, from Deutsche Automobil Treuhand GmbH (DAT), Hellmuth-Hirth-Str. 1, 73760 Ostfildern-Scharnhausen and at https://www.dat.de/co2/.

BMW i4.

|  |  |  |  |  |  |  |  |  | BMW i4 M50 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle Category |  |  |  |  |  |  |  |  |  |  |
| Drive type / body style |  |  |  |  |  |  |  |  | Battery electric vehicle (BEV) / Gran Coupé |  |
| Body |  |  |  |  |  |  |  |  |  |  |
| No. of doors / seats |  |  |  |  |  |  |  |  | 5 / 5 |  |
| Length/width/height (unladen) |  |  |  | mm |  |  |  |  | 4783 / 1852 / 1448 |  |
| Wheelbase |  |  |  | mm |  |  |  |  | 2856 |  |
| Track, front/rear |  |  |  | mm |  |  |  |  | 1589 / 1606 |  |
| Ground clearance (unladen) |  | mm |  |  |  |  |  |  | 125 |  |
| Turning circle |  |  |  | m |  |  |  |  | 12.5 |  |
| Weight, unladen (DIN/EU) |  |  |  | kg |  |  |  |  | 2215 / 2290 |  |
| Weight distribution (unladen), front/rear |  |  |  | % / % |  |  |  |  | 48.1 / 51.9 |  |
| Max. load to DIN |  |  |  | kg |  |  |  |  | 520 |  |
| Max. permissible weight |  |  |  | kg |  |  |  |  | 2735 |  |
| Max. axle load, front/rear |  |  |  | kg |  |  |  |  | 1270 / 1550 |  |
| Max. trailer load, braked (12%)/unbraked |  |  |  | kg |  |  |  |  | 1600 / 750 |  |
| Max. roofload/towbar download |  |  |  | kg |  |  |  |  | 75 / 75 |  |
| Luggage comp. capacity |  |  |  | l |  |  |  |  | 470 – 1290 |  |
| Air resistance |  |  |  | cX x A |  |  |  |  | 0.25 x 2.33 |  |
| Power Unit |  |  |  |  |  |  |  |  |  |  |
| Drive concept |  |  |  |  |  |  |  |  | Electric drive, coordinated on-demand transmission of the drive torque from two electric motors to front and rear wheels respectively |  |
| Max. system output |  |  |  | kW/hp |  |  |  |  | 400 / 544 |  |
| Max. system torque |  |  |  | Nm |  |  |  |  | 795 |  |
| System power-to-weight ratio |  |  |  | kg/kW |  |  |  |  | 6.3 |  |
| Max. recuperation output |  |  |  | kW |  |  |  |  | 195 |  |
| Type of transmission |  |  |  |  |  |  |  |  | Automatic transmission, single-speed with fixed ratio |  |
| Electric Motors |  |  |  |  |  |  |  |  |  |  |
| Motor technology |  |  |  |  |  |  |  |  | Fifth-generation BMW eDrive technology: electrically excited synchronous motors each sharing the same housing with the power electronics and single-speed transmission, generator function for recuperating energy |  |
| Front electric motor |  |  |  |  |  |  |  |  |  |  |
| Peak output to ECE R 85 |  |  |  | kW/hp |  |  |  |  | 190 / 258 |  |
| at |  |  |  | rpm |  |  |  |  | 8.000-17.000 |  |
| Continuous output to ECE R 85 |  |  |  | kW/hp |  |  |  |  | xx / xx |  |
| Max. torque |  |  |  | Nm |  |  |  |  | xxx |  |
| at |  |  |  | rpm |  |  |  |  | 0-5.000 |  |
| Gear ratios |  |  |  | :1 |  |  |  |  | x.yyy |  |
| Rear electric motor |  |  |  |  |  |  |  |  |  |  |
| Peak output to ECE R 85 |  |  |  | kW/hp |  |  |  |  | 230 / 313 |  |
| at |  |  |  | rpm |  |  |  |  | 8.000-17.000 |  |
| Continuous output to ECE R 85 |  |  |  | kW/hp |  |  |  |  | xx / yy |  |
| Max. torque |  |  |  | Nm |  |  |  |  | xxx |  |
| at |  |  |  | rpm |  |  |  |  | 0-5.000 |  |
| Gear ratio |  |  |  | :1 |  |  |  |  | x.yyy |  |
| High-voltage Battery |  |  |  |  |  |  |  |  |  |  |
| Storage technology |  |  |  |  |  |  |  |  | Lithium-ion |  |
| Installation |  |  |  |  |  |  |  |  | Underfloor |  |
| Voltage |  |  |  | V |  |  |  |  | 398.5 |  |
| Battery capacity |  |  |  | Ah |  |  |  |  | 210.6 |  |
| Energy capacity, gross |  |  |  | kWh |  |  |  |  | 83.9 |  |
| Energy capacity, net |  |  |  | kWh |  |  |  |  | 80.7 |  |
| Charging time, 0 – 100 % charge |  |  |  |  |  |  |  |  | < 8.5 h at 11 kW (16 A / 380 V, three-phase AC, Wallbox) |  |
| Charging time, 10 – 80 % charge |  |  |  |  |  |  |  |  | 31 min at 210 kW initially (DC, fast-charging station) |  |
| Charging Unit |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  | Combined Charging Unit (CCU) with built-in 4 kW voltage transformer for supplying power to the 12V electrical system |  |
| Max. charging rate AC, single-phase |  |  |  | kW |  |  |  |  | 7.4 |  |
| Max. charging rate AC, three-phase |  |  |  | kW |  |  |  |  | 11.0 |  |
| Max. charging rate DC |  |  |  | kW |  |  |  |  | 210 |  |
|  |  |  |  |  |  |  |  | BMW i4 M50 |  |  |
| Driving Dynamics and Safety |  |  |  |  |  |  |  |  |  |  |
| Suspension, front |  |  |  |  |  | Double-joint spring strut axle in lightweight aluminium-steel construction, hydraulically damped torque strut bearing |  |  |  |  |
| Suspension, rear |  |  |  |  |  | Five-link axle in lightweight aluminium-steel construction; optional: air suspension with automatic self-levelling |  |  |  |  |
| Brakes, front |  |  |  |  |  | Vented disc brakes, with four-piston fixed callipers |  |  |  |  |
| Brakes, rear |  |  |  |  |  | Vented disc brakes, with single-piston floating callipers |  |  |  |  |
| Driving stability systems |  |  |  |  |  | Standard: DSC incl. ABS, ASC and DTC (Dynamic Traction Control), ARB technology (near-actuator wheel slip limitation), CBC (Cornering Brake Control), DBC (Dynamic Brake Control), Dry Braking function, fading compensation, drive-off assistant, HDC (Hill Descent Control), trailer stability control, Performance Control, adaptive M suspension |  |  |  |  |
| Safety equipment |  |  |  |  |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, three-point inertia-reel seatbelts on all seats with belt latch tensioner and belt force limiter in the front, crash sensors, tyre pressure indicator |  |  |  |  |
| Steering |  |  |  |  |  |  |  |  | Electric Power Steering (EPS), variable sport steering with Servotronic function |  |
| Steering ratio, overall |  |  | :1 |  |  |  |  |  | 14.1 |  |
| Tyres, front/rear |  |  |  |  |  |  |  |  | 245/45 R18 100Y XL / 255/45 R18 103Y XL |  |
| Rims, front/rear |  |  |  |  |  |  |  |  | 8.5J x 18 light-alloy / 9J x 18 light-alloy |  |
| Performance |  |  |  |  |  |  |  |  |  |  |
| Acceleration 0–100 km/h |  |  | s |  |  |  |  |  | 3.9 |  |
| Acceleration 0–60 km/h |  |  | s |  |  |  |  |  | xx |  |
| Acceleration 80–120 km/h |  |  | s |  |  |  |  |  | xx |  |
| Top speed |  |  | km/h |  |  |  |  |  | 225 (electronically limited) |  |
| Electric Power Consumption / Range |  |  |  |  |  |  |  |  |  |  |
| Electric power consumption combined (WLTP) |  |  |  |  | kWh/100 km |  |  |  | 24 – 19 |  |
| Range (WLTP) |  |  |  |  | km |  |  |  | up to 510 |  |
| Environmental Characteristics |  |  |  |  |  |  |  |  |  |  |
| Emission rating |  |  |  |  |  |  |  |  | Electric vehicle |  |
| Carbon Life Cycle Assessment when using green power in the use phase compared to BMW xxxxxxx |  |  |  |  |  |  |  |  | –xx % |  |
| Carbon Life Cycle Assessment when using EU28 power mix in the use phase compared to BMW xxxxxx |  |  |  |  |  |  |  |  | –xx % |  |

Provisional specifications apply to ACEA markets/data relevant to homologation applies in part only to Germany (weight)

Official fuel consumption, CO2 emissions, electric power consumption and electric range figures were determined based on the prescribed measurement procedure in accordance with European Regulation (EC) 2007/715 in the version applicable. They refer to vehicles in the German market. Where a range is shown, NEDC figures consider the different sizes of the selected wheels/tyres, while WLTP figures take into account the impact of any optional extras.

WLTP values are used for determining vehicle-related taxes or other duties based (at least inter alia) on CO2 emissions as well as eligibility for any applicable vehicle-specific subsidies. Any NEDC values that are shown were calculated based on the new WLTP measurement procedure where appropriate and translated back into equivalent NEDC measurements in order to ensure comparability between the vehicles. Only official figures based on the WLTP procedure are available for new models that have been type tested since 01.01.2021. Further information on the WLTP and NEDC measurement procedures can also be found at www.bmw.de/wltp.

Further information on official fuel consumption figures and specific CO2 emission values of new passenger cars is included in the following guideline: 'Leitfaden über den Kraftstoffverbrauch, die CO2-Emissionen und den Stromverbrauch neuer Personenkraftwagen' (Guide to the fuel economy, CO2 emissions and electric power consumption of new passenger cars), which can be obtained free of charge from all dealerships, from Deutsche Automobil Treuhand GmbH (DAT), Hellmuth-Hirth-Str. 1, 73760 Ostfildern-Scharnhausen and at https://www.dat.de/co2/.