[image: image1.png]Output [kW]
(to 95/1/EG)

130

125

120
115
110

105

100
95
90
85
80
79
70
65
00
55
50
45
40
39
30
29
20
15

—_—
o1 O

0

2000
Engine speed [min-1]

\\\

150
140
130
120
110
100
90
80
70
00
50
40
30
20
10
0

11000

Torque [Nm]
(to 95/1/EQ)





BMW K1200 R

Contents

1.
UK market positioning for K1200 R 
3

2.
The concept
4

3.
Technical features and design
5

4.
Range of equipment
9

5.
Engine output and torque
11

6.
Technical specifications
12

7.
The K1200 R colour range
13

For further press information please contact one of the following:

Andy Dukes / Lauren Haslehurst



08000 131 282

Tony Jakeman





01344 480185

The BMW media website, called Pressclub, contains all our press releases, press packs
and a selection of company information as well as high resolution, downloadable photographs. 
You will need to register at the site to gain full access.    

www.press.bmwgroup.com
1. UK market positioning for the new 
BMW K1200 R

The new K1200 R roadster is a muscle bike that owes its allegiance to nothing that has gone before. It is not a replacement for any other machine and represents a revolution in the unfaired bike sector. It has power and styling not previously associated with BMW Motorrad and is yet another ‘new generation’ model in the BMW range to change people’s perception of the brand.

BMW’s most powerful and aggressively-styled naked street bike, the 163 bhp K1200 R has been uncompromisingly devised for road riding. Equipped with the same engine (and digital motor electronics) as the formidable K1200 S superbike, the new roadster delivers corresponding levels of comfort, ride quality, safety, and environmental consideration. 

Visually, the K1200 R copies nothing and leads everything. The roadster’s aggressive aesthetics are a synthesis of world-leading aerodynamics and original thinking. But they are practical too. The level of performance achieved by the K1200 R would be pointless without the capability to sustain it. 

A motorcycle that delivers such outstanding performance must also be safe, and the K1200 R is one of the safest machines available. A low centre of gravity ensures remarkable manoeuvrability at slow speeds and less ‘edgy’ handling at high speeds. Sports Integral ABS and Electronic Suspension Adjustment options ensure that the roadster requires less rider input than its contemporaries. Despite its substantial power output, the K1200 R’s safety features attract favourable rates from BMW Insurance. This is backed by the industry’s most comprehensive after-market support and a vast array of options and accessories. 

The innovative roadster’s all-round capabilities have also inspired an exciting new one-make race series – the BMW Motorrad Power-Cup – which will replace the hugely successful BoxerCup on this year’s European tracks in support of MotoGP.

The K1200 R is a road bike without compromise, and is destined to attract considerable conquest sales. It will be in Motorrad dealerships from June 2005 at £9,190 on the road.

2. Concept of the new 
K1200 R

BMW Motorrad proudly presents the most powerful series production roadster currently available in the world – the new K1200 R. 

Featuring advanced technology and design previously seen on the K1200 S, the radically different muscle bike will set new standards in its market segment. With an ultra-modern 120kW (163 bhp) four-cyl​inder engine, suspension and running gear designed and built without compro​mise – for supreme riding dynamics – outstanding innovations such as the BMW Duolever and ESA, and a highly ad​vanced on-board electrical and electronic network, the K1200 R sets a highly individual benchmark for performance sports roadsters. 

Because of its extrovert and futuristic design, the K 1200 R marks a new approach beyond the conventional. Following the quantum leap (in all areas) achieved by the K1200 S, the new K series Roadster has become another standard-bearing model in the BMW motorcycle range. Combining the precision and advantages of high technology, superior power and performance with a touch of purism, the new K1200 R is truly unique – a new generation machine typical of BMW Motorrad’s current philosophy of producing lighter, faster, more agile motorcycles.

BMW Motorrad’s focus with the K1200 R – as with all BMW motorcycles – is not short term or with only ‘fashionable’ appeal to sustain it. The ob​jec​tive is to give the rider added value and practical ‘real world’ benefits as well as the latest technological developments. A comprehensive range of options and accessories, which enhance the practical value of the new street machine, complements this philosophy.

Optimum environmental compatibility is ensured by ultra-modern exhaust gas management, which incorporates a fully controlled three-way catalytic converter. Supreme active safety, as provided by the advanced Integral ABS in conjunction with a high-performance EVO brake system, is unique to BMW.

The market launch of the K1200 R is scheduled for June 2005. To underline the sporting quail​ties of the new K1200 R, a professional racing series – the Power-Cup – will commence in May 2005 featuring race machines based on the K1200 R. As with the successful BoxerCup, this new one-make series will be held during MotoGP race weekends. This is designed to create maximum awareness, in the right environment, appropriate to the high technology machine.

3. Technical features and design

Technically, the K1200 R is derived from the K1200 S high performance machine with modifications to only a few features. The drivetrain, running gear, brakes and on-board network have all been carried over from the K1200 S. The new body and equipment, and truly exceptional futuristic design, make the K1200 R a brand-new and quite different motorcycle to others in its segment. Weighing only 237 kilos or 523lb (wet weight with a full tank but without ABS) the K1200 R is the embodiment of BMW Motorrad’s new generation models that offer supe​rior handling and manoeuvrability – especially at low speeds, on country roads, and in towns.

Drivetrain

Slender and compact, the straight-four power unit displaces 1157cc, which is the same as the engine of the K1200 S. Other engine data is also similar: the new Roadster develops a slightly lower peak output of 120kW (163 bhp) at the same engine speed of 10,250 rpm, with maximum torque of 127Nm (94 lb-ft) delivered at 8,250 rpm.   

Mechanically, the engine of the K1200 R is identical to that of the K1200 S and has the same camshaft configuration and valve timing. The flow of intake air has been modified for the new machine and features just one ‘snorkel’ leading to the airbox on the right-hand side. The modified flow of intake air is the reason for the slightly lower level of maximum power and torque. The silencer and ex​haust manifold systems both come from the K1200 S.

The transmission is also unchanged from the K1200 S. However, at 2.91:1 (instead of 2.82:1 on the K1200 S) the drive​train final drive ratio has been adjusted to the reduced the top speed of the ‘naked’ motorcycle. This is because of the machine’s higher drag coefficient.    

Suspension and running gear

The suspension and running gear are slightly modified on the new model. The Duolever has a somewhat steeper angle (0.4°) and the steering head angle has been changed to 61°, with camber 11mm (0.43in) shorter at 101mm (3.98in). The change in suspension geometry has been achieved by a minor alteration in the machining of bearing points on the Duolever. 

The positive effect of the modified suspension geometry results in even better handl​ing, particularly at low speeds. Any theoretical repercussions on direc​tional stability at high speeds (versus the K1200 S) are unnoticeable and irrelevant on a motorcycle without a fairing and with a somewhat lower top speed.       

The K1200 R comes as standard with wheels of the same design as those found on the new R1200 ST. Wheel dimensions are 3.5in x 17 at the front and 5.5in x 17 at the rear when using tyres measuring 120/70-ZR17 and, respectively, 180/55-ZR17. The customer also has the option of K1200 S sports wheels measuring 3.5in x 17 at the front and 6.0 in x 17 at the rear (the rear tyre designation is 190/50-ZR17).

BMW Motorrad’s Electronic Suspension Adjustment (ESA) is also available as an optional extra. Introduced for the first time on the K1200 S and now also available on the R1200 RT, this world-first enables a rider to adjust the suspension from the handlebars – at the touch of a button – an operation that can be performed while riding. 

ESA varies inbound damping on the front wheel, and inbound and re​bound damping on the rear wheel. The rider also has the ability to adjust the rear-wheel spring base electrohydraulically while the motorcycle is at standstill (i.e. for different payloads). 

Spring travel on the K1200 R is the same as on the K1200 S sports machine, measuring 115mm (4.52in) at the front and 135mm (5.31in) at the rear. 

Brakes – high-performance EVO brake system and Integral ABS 
as an option

The K1200 R is equipped with BMW’s EVO brake system (featured on all K Series and flat-twin models). In configuration and dimensions the brake system – with disc brake diameters of 320mm (12.60in) at the front and 265mm (10.43in) at the rear – is the same as on the K1200 S. The brake hoses are steel-clad for additional safety and reliability. 

The BMW EVO brake system (EVO stands for evolution) is one of the safest and most effective brake systems in the world.

As another optional extra, the EVO brake system is available with Sports Integral ABS. This means that both brakes (front and rear) are acti​vated by the hand lever, while the footbrake lever acts only on the rear wheel.

Even when applying maximum pressure on the brakes, the rider of the K1200 R has less chance of losing control thanks to the motorcycle’s low centre of gravity and the special suspension geometry, which interacts with the kinematic configuration of the Duolever. In other words, with ABS, the rider benefits un​der virtually all conditions from maximum tyre adhesion when applying the brakes with ultimate force. This ensures best possible safety even in the most extreme situations.

Controls and instrumentation           

The handlebar and the handlebar bridge on the K1200 R are both new. 
Com​pared with the K1200 S, the handlebar is wider and higher. As appropriate for a motorcycle of this type, the rider’s seating position is more upright and further to the front. Oval rear-view mirrors are attached to the handlebar.

The instrument cluster, with speedometer and rev counter, is based on the same concept as the in​stru​ment cluster of the R1200 GS. However, the instrument dials are new. As on the K1200 S, the displays in the info-flatscreen provide ongoing infor​mation on coolant temperature, fuel level, time, and the gear currently engaged. On machines equipped with optional ESA, the info-flatscreen provides additional information on the current sus​pension set-up. Total mileage, trip mileage, and the range of the fuel remaining in the tank are all available by pressing a button.

Electrics and electronics

Boasting a wide range of innovations, the on-board network in CAN-bus 
tech​nology (Controller Area Network) has been transferred from the 
K1200 S without modification. The ad​vantages of this technology are a far wider range of available functions (compared with conventional on-board networks) and much simpler and more straightforward cable connections, absence of conventional meltdown fuses, and full-scale diagnostic management and examination options. 

A fully integrated electronic immobiliser is standard equipment and consistently exchanges coded and frequently changing data between the motorcycle key (by means of a transponder) and the on-board electronic system. This is the best and safest immobiliser currently available, ensuring that only the authorised rider has access to the machine.

Design and body 

Cosmetically, the K1200 R looks predatory, aggressive and athletic. The underlying philosophy in creating the machine was to minimise all features down to the essen​tial and achieve an emotional symbiosis. 

Such minimalist design places the focus on visible technical features and components, such as the drivetrain and suspension, and for highlighting BMW’s claim to leadership in technology and innovation. With its muscular and unashamedly masculine look, the K1200 R clearly emphasises – even at standstill – the performance and dynam​ics of what is currently the most powerful roadster in the world.

Not surprisingly, the design will provoke contradictory discussion and 
opin​ion. But, it would not have been appropriate – given the demanding standards of BMW Motorrad – to endow a motorcycle of this calibre with anything but classic beauty fused with modern concepts.

While the heritage of the K1200 R and its close ties to the K1200 S is ob​vious – both motorcycles share the same sweeping contour lines on the fuel tank and seat – the K1200 R goes much further in its styl​ing features. The machine’s contours and shapes are even more distinc​tive, making the K1200 R a truly striking, unmistakable and visually unique motorcycle. This design philosophy is continued by the side covers at the front of the fuel tank, which take up and extend a consistent line to the front end (and the oil cooler fairing beneath the striking double headlight). 

Despite its compact dimensions, the small air deflector above the headlight significantly reduces wind pressure at high speeds.

The reflectors of the dual headlight system come in free-form sur​face technology and are housed beneath extra-clear, impact- and scratch-proof, lightweight polycarbonate covers. Finished in matt chrome, the head​light housing is made of a special synthetic material to accentuate the technical character of the motorcycle.

The Duolever has plastic covers at the side to emulate the basic de​sign of the wheel bearings. At the same time these covers guide the flow of cool air to the radiator.

The K1200 R is available in a range of powerful looking colours that have character and style. Dark Graphite metallic, for example, forms a strong contrast to the drivetrain and running gear, which is finished in silver. This highlights the technical and aesthetic qualities of the new machine. White Aluminium metallic, on the other hand, combines with the drivetrain and running gear (in black) to enhance the contrasts of the K1200 R and give greater emphasis to the contours of the motorcycle and its muscular shape. 

To complement the colour combinations, the wheels are finished in Slate Grey metallic matt. Another particularly striking colour scheme is Sun Yellow non-metallic, with the drivetrain and running gear in black and the wheels in Slate Grey metallic matt. Despite the paintwork options, the seat is in black on all models. 

Fuel tank and seat

The fuel tank of the K1200 R is made of extra-light, impact-resistant 
syn​thetic material and offers a capacity of 19 litres (almost 4.2 Imp gals) includ​ing four litres in reserve. The overall configuration of the K1200 R – as with all BMW Motorrad models – serves to keep the tank par​tic​ularly slender in front of the rider, thus ensuring optimum knee contact. 

Similar in its basic design, the seat is more slender than on the K1200 S in order to create an even more forward-oriented, active riding position. The rider’s absolute geometric seat height is 820mm (32.3in) with a step-arch length of 1,810mm (71.3in). A low-level seat, which is avail​able as a no-cost option, measures 790mm (31.1in) in height with 1,750mm (68.9in) step-arch length. The lower seat can also be re​trofitted as special equipment.

4. Range of equipment

Although the K1200 R is comfortably at home in an urban environment it is – like all BMW motorcycles – perfectly suited to long distance riding and touring. 

There are also many experienced riders who prefer the classic concept of motorcycling and enjoy being in harmony with the environment; riders that want to enjoy a direct experience on their motorcycle and are happy to do so without a fairing. However, even on BMW’s new Roadster they need appropriate solutions for holiday baggage and other travel items.

This is precisely why the K1200 R is available with sports cases carried over from the K1200 S. The specially designed luggage combines superior function with sporting design and reflect the sporting character of the new high-per​formance Roadster. Combining textile and hard material, the luggage is an integrated addition to the machine and, using the racks already fitted to the motorcycle, can be mounted and removed within a matter of seconds. 

Loading space is variable. This extends from 17 litres per case in normal configuration to 25 litres when each case is fully expanded. The fully inte​grated inner bags are obviously watertight.

The sports wind deflector (fly screen) is ideal for riders that want more protection from wind and weather. The screen is extendable and has wider dimensions (compared with the standard windscreen).

The usual range of features – direct from the factory – is available for further customisation and is integrated during production at the Berlin plant. Special equipment can subsequently be fitted by the customer, or by any BMW motorcycle dealership.

Optional Extras

•
BMW Motorrad Integral ABS

•
ESA (Electronic Suspension Adjustment)

•
Sports wheels

•
Heated handlebar grips 

•
Sports windshield (fly screen)

•
White direction-indicator lights

•
Low rider’s seat (seat height 790mm or 31.1in)

•
Luggage rack

•
Anti-theft warning system

Special equipment

•
A low rider’s seat (seat height 790mm/31.1in)

•
Main stand

•
Heated grips     

•
White direction-indicator lights

•
Anti-theft warning system

•
Two power sockets
•
Luggage racks

•
Set of sports cases

•
Watertight tank rucksack

•
Watertight tank bag   

•
Watertight small sports softcase (for the luggage rack)

•
Luggage rack
•
Preparation for BMW Motorrad Navigator II

· BMW Motorrad Navigator II

5. Engine output and torque


6. Technical specifications
K1200 R

	
	
	K1200 R
	
	
	

	Engine
	
	
	
	
	

	Capacity
	cc
	1,157
	
	
	

	Bore/stroke
	mm
	79/59
	
	
	

	Output  
	kW/bhp
	120/163
	
	
	

	at
	rpm
	10,250
	
	
	

	Torque
	Nm/lb-ft
	127/94
	
	
	

	at
	rpm
	8,250
	
	
	

	Configuration 
	
	Inline
	
	
	

	No of cylinders
	
	4
	
	
	

	Compression ratio/fuel grade
	
	13:1/premium plus (98 octane)
	
	
	

	Valve/gas control  
	
	DOHC (double overhead  camshaft)
	
	
	

	Valves per cylinder 
	
	4
	
	
	

	Intake/outlet valve diameter
	mm
	32/27.5
	
	
	

	Throttle butterfly diameter   
	mm
	46
	
	
	

	Fuel supply/management
	
	BMSK
	
	
	

	Electrical system 
	
	
	
	
	

	Atlernator   
	W
	580
	
	
	

	Battery
	V/Ah
	12/14 maintenance-free
	
	
	

	Headlights  
	W
	Low beam 1x H 7/55 W
	
	
	

	
	
	High beam 1x H 7/55 W
	
	
	

	Starter
	kW
	0.7
	
	
	

	Power transmission/gearbox
	
	
	
	
	

	Clutch   
	
	Multi-plate oil bath clutch, disc dia 151 mm
	
	

	Gearbox  
	
	Dog-type six-speed gearbox
	
	
	

	Primary transmission ratio
	
	1.559
	
	
	

	Gear ratios
	
	2.521
	
	
	

	
II
	
	1.842
	
	
	

	
III
	
	1.455
	
	
	

	
IV
	
	1.287
	
	
	

	
V
	
	1.143
	
	
	

	
VI
	
	1.015
	
	
	

	Final drive 
	
	Driveshaft
	
	
	

	Final drive ratio
	
	2.91
	
	
	

	Suspension and running gear
	
	
	
	
	

	Frame      
	
	Composite aluminium frame with IHU/extrusion-pressed profiles and die-castings
	
	

	Wheel guidance, front
	
	BMW Duolever
	
	
	

	Wheel guidance, rear 
	
	BMW Paralever
	
	
	

	Spring travel, front/rear
	mm
	115/135
	
	
	

	Camber  
	mm
	101
	
	
	

	Wheelbase
	mm
	1,571
	
	
	

	Steering head angle
	°
	61
	
	
	

	Brakes 
	front
	Double disc brake, dia 320 mm
	
	
	

	
	rear 
	Single disc brake, dia 265 mm
	
	
	

	
	
	Optional BMW Motorrad Integral ABS (partly integral)
	
	
	

	Wheels
	
	Light-alloy cast wheels
	
	
	

	
	front
	3.50 x 17 MTH 2
	
	
	

	
	rear
	5.50 x 17 MTH 2 

(optional 6.00 x 17 MTH 2)
	
	
	

	Tyres 
	front
	120/70 ZR 17
	
	
	

	
	rear
	180/55 ZR 17

(190/50 ZR 17)
	
	
	

	Dimensions and weight
	
	
	
	
	

	Length, overall
	mm
	2228
	
	
	

	Width, overall, with mirrors
	mm
	856
	
	
	

	Handlebar width, without mirrors
	mm
	785
	
	
	

	Seat height
	mm
	820 (790)
	
	
	

	Weight, unladen, with full tank
	kg
	237
	
	
	

	Weight, max permissible
	kg
	450
	
	
	

	Tank capacity
	Ltr
	19
	
	
	

	Performance data
	
	
	
	
	

	Fuel consumption 
	
	
	
	
	

	90 km/h
	ltr/100 km
	4.7
	
	
	

	120 km/h
	ltr/100 km
	5.5
	
	
	

	Acceleration  
	
	
	
	
	

	0–100 km/h
	sec
	2.8
	
	
	

	Standing-start km
	sec
	–
	
	
	

	Top speed 
	km/h
	200 plus
	
	
	


7. K1200 R colour range

	
	Colour
	Drivetrain and suspension
	Wheels

	K1200 R
	Dark Graphite metallic
	Silver
	Silver

	
	White Aluminium metallic
	Black  
	Slate Grey metallic, matt 

	
	Sun Yellow non-metallic
	Black  
	Slate Grey metallic, matt 
































BMW Motorrad GB


Postal address�BMW (GB) Ltd�Ellesfield Avenue�Bracknell Berks�RG12 8TA


Tel:�08000 131 282


Fax:�01652 680 070


Website�www.worldofbmw.com



















































































BMW Motorrad Great Britain





Postal Address


BMW (GB) Ltd.


Ellesfield Avenue


Bracknell Berks


RG12 8TA





Tel:


08000 131 282





Fax:


01652 680 070





Website


www.worldofbmw.com





























