PAGE  
BMW
Media
Information

8/2005
Page 14

[image: image1.png]


1.
Highlights

A Short Look at a Great Product.

Setting the Benchmark in the Compact Class – and Beyond.

The Most Important Standard Features.

Package.

Five-door tailgate saloon with dynamic performance, elaborate, particularly dynamic chassis and suspension with rear-wheel drive, longest wheelbase 
in its segment, exterior dimensions typical of the compact class, well-balanced front-to-rear weight distribution.

Drivetrain.

The only car in its class with a straight-six power unit fitted lengthwise 
at the front and rear-wheel drive. Petrol engine with fully variable valve drive, com​fort start. 

Power Unit.
BMW 130i developing 195 kW (265 bhp), six cylinders, 2,996 cc, 
VALVE​TRONIC, double-VANOS, magnesium/aluminium composite crankcase, magnesium cylinder head cover, electrical water pump, lightest six-cylinder engine in its class weighing just 161 kg or 355 lb, maximum torque 315 Nm (232 lb-ft) from 2,500–4,000 rpm.

Acceleration from 0–100 km/h in 6.1 seconds. 
Top speed 250 km/h (155 mph) (electronically limited).
Average fuel consumption in the EU composite test cycle 9.2 litres/100 km 
(30.7 mpg Imp).

Six-speed manual gearbox.

Chassis and Suspension, Active Safety.

Truly outstanding driving dynamics not only for the compact class, combined with comfort and driving safety of an usually high standard. Aluminium 
double-joint tiebar axle at the front, five-arm rear axle in lightweight steel construction; DSC driving stability system with an enhanced range of 
func​tions: dry braking function in the wet, pre-loading of brake pads when the driver takes his foot abruptly off the gas pedal, anti-fading function at high temperatures, Set-Off Assistant preventing the car from rolling back on a 
gra​dient, Dynamic Traction Control (DTC), Automatic Stability Control (ASC), 

electronic differential lock, brake system with anti-lock function (ABS), Dynamic Brake Control (DBC), Cornering Brake Control (CBC), Electronic Brake Force Distribution (EBFD); best deceleration and stopping power in the compact class; two-stage Adaptive Brake Display, continuous, ongoing display of brake pad wear, tyre defect indicator; 17-inch light-alloy rims; foglamps.

Body Structure.

Five-door tailgate saloon body in lightweight steel structure. Use of 
high-strength steel in the interest of minimum weight.; longest wheelbase in the compact class, wide track, extremely short body overhang at the front; 
five seats with three-point inertia-reel seat belts and three headrests at the rear; luggage compartment variable in size from 330–1,150 litres 
(11.55–40.25 cu ft) with tailgate rising up to a high position for optimum loading/unloading, rear seat bench split and folding down asymmetrically, double luggage com​partment floor, partition net between luggage compartment and passenger compartment*.

Passive Safety.

Unusually stiff body-in-white for supreme driving precision and optimum 
crash safety, high-strength side impact protectors with safety hooks in all doors, all-round occupant safety with two-stage frontal airbags, side airbags at the front, curtain head airbags for the front and rear passengers, belt latch tensioners and belt force limiters at the front, safety switch interrupting the flow of power following an accident, switching off the fuel pump, opening 
the door locks, activating the hazard warning flashers and switching on the inte​rior lights.

Top marks (5 stars) for the BMW 1 Series in the Euro-NCAP crash test.

Standard Equipment.

Engine starter button with comfort start function, leather sports steering wheel, extra-short gearshift lever with leather knob, circular dials with ex​tended range of display, straight-surface, battery-free ignition key with re​mote 
control function; car recognises which key is being used to open the door and sets the radio, the air conditioning, the seats with electrical adjust​ment and 
the mirrors to the specific data and requirements of the key user; battery charge sensor ensuring easy start; on-board computer; indication of residual mileage for all parts subject to wear and tear; double exhaust sys​tem with chrome-plated tailpipes and a sporting, throaty sound; chrome-plated door entry trim; seat height adjustment for the driver and front pas​sen​ger; 
air conditioning, separate temperature control on the driver’s and front passenger’s side, BMW Business radio including CD player.

2.
Short Story

The BMW 130i:

A New Dimension in Driving Pleasure.

The dynamic performance of a thoroughbred sports car and the everyday driving qualities of a compact five-door saloon – precisely this is the 
combi​nation so many passionate motorists have been waiting for so long. 
And now their wish is coming true: The BMW 130i offers sheer driving pleasure of a standard never seen before.

Boasting a 195 kW (265 bhp) straight-six power unit, the new BMW 130i 
en​ters a world of power and performance previously to be found only in 
motor​sport. Displacing three litres, featuring VALVETRONIC valve management and revolutionary lightweight engineering, the power unit of the BMW 130i enters a new dimension in terms of superior power and all-round efficiency. This new straight-six is indeed the first production engine in 
the world with an extremely light composite magnesium/aluminium crankcase, superior en​gine technology giving the BMW 130i unique all-round qualities.

Rear-wheel drive, the most advanced and sophisticated suspension 
technol​ogy, as well as a perfect balance of weight offer all the ingredients for truly ex​ceptional agility, directional stability, and steering precision. One of 
the ad​ditional innovations featured by the BMW 130i is BMW’s revolutionary Active Steering*. Yet another new feature in this segment is DSC Dynamic Stability Control of the latest generation now also comprising an 
extended brake func​tion for enhanced comfort and safety on the road. 

The result of all these features is a driving experience quite unparalleled not only within the compact class. Indeed, the BMW 130i reigns supreme as 
the top model within BMW’s highly successful 1 Series already so popular in the market today. 

Benefiting from their drive concept conceived and designed for supreme 
driv​ing pleasure, the four-cylinder BMW 116i, 118i and 120i gasoline engine versions, as well as the BMW 118d and 120d diesel models, have already 
hit the headlines through their drive concept focusing on superior driving pleasure. And now the BMW 130i extends the successful story of the 
1 Se​ries model range by adding a particularly thrilling new version in this class.

A Genuine BMW – and a New Driving Experience in the 
Compact Class.

Powerful engines and the transmission of power to the rear wheels ensure supreme driving pleasure of the highest standard, four doors, a large tail​gate, and the highly versatile and very practical interior guarantee superior function all round. Indeed, it is precisely this concept which gives the BMW 1 Series 
its particular character as the only car in the compact class featuring rear-
wheel drive and, as a result, all the driving and handling qualities so ty​pical of a genuine BMW: superior dynamics, safe handling and optimum trac​tion in every situation. And now the BMW 130i lifts the driving pleasure provided in this way to an even higher standard than before.

The concept of combining a high-performance engine with rear-wheel drive 
in a compact car gave BMW’s engine development specialists the great 
repu​tation for the first time almost 40 years ago to have a particularly good 
feeling for driving pleasure in its purest form. And the thrill of this idea 
has re​mained unchanged to this very day, the BMW 130i now marking 
the cli​max in converting this concept into practical benefits on the road.  

With its compact exterior dimensions, an extremely short body overhang at the front, low weight, wide track and – at 2.66 metres or 104.7 inches – 
the longest wheelbase in its class, the BMW 1 Series offers the very best ingre​dients for supreme agility. Added to this, there is the ideal weight distribution of almost 50:50 front-to-rear, a harmonious balance of axle load ensuring safe handling in spontaneous steering manoeuvres, together 
with supreme directional stability in fast bends thanks to centrifugal forces acting equally on the front and rear wheels.

The newly developed five-arm rear axle ideally supports the particularly sporting overall set-up of the car refined once again for the BMW 130i, at the same time raising roll comfort to a new and even higher level. With its 
stiff structures, straight, rod-shaped arms and short levers, the innovative lightweight steel structure ensures extremely precise wheel guidance. 

The particular design and layout of the front axle also benefits the BMW 1 Series, particularly in terms of optimised driving dynamics. The double-joint spring strut tiebar axle, together with its anti-roll bar, is made largely of 
alu​minium, combining low weight with exceptional strength and stiffness.

The benefits of this innovative suspension technology come to bear in 
par​ticular in interaction with the transmission of power to the rear wheels, the BMW 1 Series combining impressive directional stability with supreme 
pre​cision also at high speeds. Another outstanding strength of the car is its 

spon​taneous response to even the smallest movements of the steering 
wheel and gas pedal. At the same time the BMW 1 Series is not affected by drive forces acting on the steering, which inevitably occur on front-wheel-drive cars. But the driver nevertheless receives exact feedback on road con​ditions 
at all times via the steering.

BMW’s Revolutionary Active Steering – now also in the 
BMW 1 Series.

Available for the first time in this class, Active Steering* in the BMW 130i 
en​sures unparalleled, ultra-smooth and precise steering qualities at all times, the steering transmission ratio and steering support automatically ad​justing 
to the current speed of the car on the road. Hence, the driver is able to guide 
the car precisely into every new bend, regardless of his road speed at the time. 

The driver also finds it a lot easier to change direction at low speeds, since he only has to turn the steering wheel slightly, for example, when parking. At high speeds, in turn, the steering transmission also ensures optimum preci​sion, maintaining a less direct reaction geared to the current road speed of the car.

The Power Unit – an Outstanding Achievement not just in the Compact Class.

The BMW 1 Series excels in particular through its unusually active character and all-round performance on the road. Through its unique concept, it now 
of​fers all the fascinating qualities of a BMW also in the compact class. And the BMW 130i, in particular, guarantees an all-new dimension in terms of 
dynamic performance, the generation and transmission of power by and from the engine going far beyond usual categories and standards.

The BMW 130i owes this supreme position in particular to a power 
unit setting a new milestone in the area of engine construction not only in the compact class: Displacing 2,996 cubic centimetres, developing maximum 
out​put of 195 kW (265 bhp), and generating peak torque of 315 Nm through​out a wide speed range from 2,500–4,000 rpm, the straight-six power 
unit featured in this unique car is among the most sophisticated, powerful, 
ef​fec​tive and, at the same time, lightest engines of its kind the world over. 
Here, therefore, the customer benefits from spontaneous power and performance combined with exemplary smoothness and economy to provide a mixture of qualities never seen before.

Weighing just 161 kilos or 355 lb, BMW’s new straight-six is the world’s 
light​est engine in its class. Inter alia, the weight of the engine has been 
optimi​sed by an innovative magnesium/aluminium composite crankcase. Another 27 per cent lighter than aluminium, the magnesium used in the crankcase reduces weight most significantly in comparison with conventional grey-cast-iron components.

BMW’s fully variable VALVETRONIC valve drive, together with infinite cam​shaft adjustment (VANOS) on both the intake and outlet valves, guarantees 
a particularly fascinating rendition of all-round efficiency. Featured in a straight-six for the first time, VALVETRONIC, as opposed to a conventional throttle butterfly power unit, ensures a significantly improved response and, at the same time, far greater efficiency in the economic use of fuel. As a re​sult, every movement of the gas pedal in the BMW 130i is translated directly and 
with supreme accuracy in the dosage of fuel into driving dynamics on the road. And at the same time the car’s fuel consumption and emission fi​gures are exceptionally good for a vehicle with this kind of performance.

A further feature deserving particular attention is the specially designed 
ex​haust system with two chrome-plated tailpipes adding exactly the right sound effect for each and every movement of the gas pedal and, 
through its sporting but rumbling sound, contributing significantly to the unique thrill of the new BMW 130i.

BMW’s three-litre straight-six is acknowledged as the symbol of superior drive​train technology – particularly in its latest configuration. Indeed, 
this en​gine is the role model and benchmark for engine development specialists the world over, at the same time offering all the ideal 
qualities appreciated so much by the sporting and ambitious driver. 

The outstanding properties of this unique power unit are featured within BMW’s model range in a number of different cars and classes: The BMW 630i Coupé, for example, receives all its impressive supremacy, motoring culture and efficiency from this outstanding straight-six, while in the BMW 530i the new power unit guarantees a perfect blend of motoring comfort and dynamic performance. And in the new BMW 330i, finally, the three-litre in-line 
six-cylinder helps the entire model series, further enhance its leadership in terms of driving pleasure.

Taking the decision to offer this power unit also beneath the hood of the 
BMW 1 Series, BMW has paved the way to a driving experience which can hardly be described in words alone, even less so by simple facts and figures. Still, the car’s performance figures and data are most certainly very impressive, 

the BMW 130i featuring a six-speed manual gearbox as stand​ard accelerating from 0–100 km/h in just 6.1 seconds. Top speed, in turn, is limited electronically to 250 km/h or 155 mph. And despite this performance truly outstanding not only in the compact class, the BMW 130i makes do 
on average with fuel consumption of just 9.2 litres/100 km or 30.7 mpg Imp.

Occupant Safety of the Highest Standard.

Not least through the use of high-strength steel, the body structure of the BMW 130i is extremely stiff and stable, providing the foundation for supreme agility and guaranteeing safe and secure driving behaviour even in a sudden lane change or turn on the road. At the same time the body lives up in full to the unique dynamic potential of the BMW 130i, offering all the safety re​serves required should the worst ever come to the worst in order to ensure a wide range of all-round occupant safety.

In the event of a collision, an elaborately conceived system of longitudinal and transverse support bars carefully and efficiently takes up all forces act​ing on the car, the passenger cell itself remaining largely unaffected by im​pact forces and the loads acting on the occupants being reduced to a mini​mum. 
The extensive safety package within the car comprises three-point inertia-reel belts on all seats, frontal airbags activated in two stages, side airbags at 
the front as well as curtain head airbags inflating around the side windows to protect both the front and rear passengers from injury.

The airbag system as well as the belt latch tensioners and belt force limiters are activated and masterminded by means of the most advanced control technology. Depending on the type and intensity of a collision, only those com​ponents are activated which are actually required to ensure optimum occupant safety. The flow of power from the battery, in turn, is automatically interrupted in the event of a collision by means of the safety battery terminal deactivating the fuel pump, opening the door locks, and switching on both 
the hazard warning flashers and the interior lights. 

To ensure safe transportation of small passengers, the BMW 1 Series offers the option to fit up to three child seats in position by means of ISOFIX 
fast​en​ers*. Where necessary, the front passenger’s airbag may be deactivated for this purpose by means of a special key switch.

The BMW 1 Series has already proven the efficiency and all-round 
protect​tion offered by its safety features in the Euro-NCAP crash test, completing this demanding trial with the maximum score of five stars.

New Technology for Active Safety.

The high standard of driving stability so characteristic of the BMW 1 Series 
is ultimately also an important safety factor. Nevertheless, even a car of this calibre may exceed the limits to its driving dynamics in an unforeseeable 
sit​uation – which is precisely why the BMW 1 Series has all the technical 
fea​tures required to significantly reduce the risk of an accident. Should, for example, a sudden manoeuvre serving to avoid an obstacle or an unexpect​ed change in road conditions lead to an extremely difficult situation, 
DSC Dynamic Stability Control serves to keep the car on course, the system applying the brakes specifically on individual wheels to prevent the car 
from swerving out of control.

The BMW 130i is the only model in the 1 Series to be fitted exclusively with the latest generation of DSC Dynamic Stability Control. This superior system offers an even wider range of sophisticated brake functions serving above 
all to minimise the car’s stopping distance and at the same time to maintain directional stability when applying the brakes in a bend by spreading out brake forces according to current requirements. A further point is that the brake 
pads are applied lightly to the brake linings at regular intervals in wet weather, to provide a “dry braking effect” and to prevent the usual delay in the response of the brakes otherwise encountered in rain and wet weather. A further advantageous feature is automatic pre-loading of the brake discs whenever the driver suddenly takes his foot off the gas pedal, which serves to significantly shorten response times.

A feature already well known and integrated in all of BMW’s DSC systems 
is DBC Dynamic Brake Control providing similar safety and superiority in the braking process. A further function preventing the car from oversteering 
in a bend when applying the brakes lightly is CBC Cornering Brake Control, while EBFD Electronic Brake Force Distribution shares out brake forces 
ac​cording to current requirements. All of these are additional functions of the DSC and ABS driving stability systems. 

ASC Automatic Stability Control helps to ensure safe traction when setting off on a slippery or rough surface, while yet a further additional function of BMW’s latest-generation DSC prevents the car from rolling back when set​ting off 
on a gradient. And last but certainly not least, DSC Dynamic Stability Control also comes with a special mode – DTC Dynamic Traction Control – providing 
a higher level of wheel slip.

Yet a further safety feature never seen before in the compact class is 
BMW’s Adaptive Headlight Control* now featured in the BMW 130i and also becoming available in all other models in the 1 Series: Following the steer​ing wheel, the headlights move in the right direction immediately when the 
car enters a bend, illuminating the road ahead in the right angle and reliably guiding the driver in the right direction. 

A Brake System also for Vehicles Following from Behind.

Reflecting its superior performance, the BMW 130i comes with a particularly powerful and efficient brake system. Extra-large brake discs both front and rear ensure the best deceleration and stopping power in this class. Absolute reliability, in turn, is guaranteed by the ongoing, continuous-action brake pad wear indicator, sensors monitoring the brake pads for their current condition.

Taking the driver’s style of motoring into account, a control unit monitors 
and calculates the remaining service life of the brake pads – a feature never seen before in the compact class. The driver is then able to check out this information at any time on the large colour monitor* at the central top posi​tion of the instrument panel or retrieve the data via the on-board computer between the speedometer and rev counter, just as he is able to call up the remaining lifetime of other parts subject to wear and tear.

Like the four-cylinder versions in the BMW 1 Series, the BMW 130i is equip​ped with BMW’s two-stage Brake Force Display quite unique in the compact class. Introducing this innovation, BMW is indeed making a significant 
con​tribution to the avoidance of rear-end collisions. When applying the brakes under normal conditions, the brake lights in the BMW 130i will come on the usual way. When applying the brakes all-out or when activating the anti-lock brake system, in turn, the brakes will automatically come on over a larger area. With their particularly brilliant effect under such conditions, the brake lights inform drivers following from behind that there is a dangerous situation ahead. The stronger light signal will then induce the driver of a vehicle fol​low​ing from behind to press down the brake pedal harder himself.

Ease of Control with Luxury Class Standards.

The ease of control ensured by BMW’s iDrive* control concept is absolutely unique in the compact class and sets the standard also for other segments in the market. BMW iDrive is automatically included in the car’s wide range 
of features whenever the customer orders his BMW 130i with a navigation system. Simply by pressing, turning and moving the Controller in the 
centre console, the driver is able to initialise and mastermind all secondary and 
com​fort functions as well as the set-up and operating modes of his car. 
Opti​mised menu guidance is presented on an extra-large Control Display 
in the upper area of the centre console.

Functions required particularly often such as the heating and air conditioning are activated by separate switches on the centre console, and buttons on 
the multifunction steering wheel serve to call up further settings and func​tions of particular importance while driving.

With the cockpit being clearly separated into a driving and comfort zone, 
and with its perfect finish to the last detail, the BMW 130i offers a standard of quality fully in line with all the demands made of even the most sophis​ticated premium car. As an option the driver is even able to control and mas​termind numerous functions of his car via voice entry*.

The Interior – more Functional than in any other Sports Car.

The driver of the BMW 130i has the great opportunity to share the unique 
thrill of sporting performance in the compact class with up to four passen​gers. Apart from offering the passengers superior seats with superior com​fort, 
the BMW 130i also comes with a most generous and, thanks to the rear seat bench tilting forwards, variable luggage compartment quite unusual for 
a sports car of this calibre: Depending on the position of the rear seat bench, luggage capacity varies from 330–1,150 litres (11.55–40.25 cu ft). 

Swivelling up 1.80 metres or almost 6 feet, the tailgate ensures simple 
and convenient loading and unloading at the rear. It unlocks either by remote control or by pressing the tailgate handle with its integrated BMW logo. 
The maximum load of 500 kilos or more than 1,100 lb again underlines the prac​tical benefits of the body concept in handling all kinds of transport 
require​ments, Numerous storage boxes and compartments, the double floor in the luggage compartment, and the power socket in the rear also contribute 
to the car’s high standard of practical value and qualities in everyday use. 
A new feature in the compact class offered by the BMW 130i is the partition net* preventing cases, baggage and other items stored in the rear from 
sliding forward into the passenger compartment when applying the brakes 
all-out, thus serving to protect the car’s occupants.

The BMW 130i combines functional, comfort-oriented and sporting features with one another in a truly unique manner. The seats, for example, offer last​ing comfort and a secure, safe seating position at all times. A wide range 
of adjustments and different seating positions also taking the requirements of particularly tall or small individuals into account, as well as the option to 
ad​just the steering column for both reach and height, helps the driver find his – or her – ideal position at the wheel at all times.


The special arrangement of the gas, brake and clutch pedals in the BMW 130i is quite unique in the compact class: With the engine being fitted length​wise, the pedals are exactly where they should be right in front of the driver’s seat, without being moved slightly to the right-hand side as is gene​rally the case in cars with a transversely mounted engine. This enables the driver to find exactly the right seating position at the steering wheel of the BMW 130i optimum 
for active motoring and avoiding any body strain or cramp even on the longest distances.

A four-way lumbar support* is available as an option. Pneumatic adjustment of backrest width* on the sports seats* is a particular feature so far only avail​able in the most sophisticated sports cars and coupés, giving the driver and 
front passenger exactly the side support they require at the simple touch of a button.

High-Tech Solutions for Comfort and Safety.

Comfort Access* enables the driver to start the engine and set off in his 
BMW 130i without even taking the key to the car out of his pocket: As soon as the driver walks up to the car, the key transmits a recognition signal and the doors are automatically unlocked when the driver pulls the door handle. 
Then all he has to do is press the starter button in order to start the engine.

Yet a further unique feature in the compact class is the outside illumination* around the doors directly linked in the BMW 130i to the interior light function: As soon as the driver or passengers unlock and open the doors, light-emitting diodes integrated in the door handles brightly illuminate the access area around the car.

The audio systems available in the BMW 1 Series are among the most powerful and sophisticated HiFi units of their kind. Particular highlights 
are the extra-large central woofers fitted as standard beneath the front seats 
as well as Logic7 studio technology*. 
The BMW 130i comes as standard with the BMW Business CD radio. 

Supplementing the car’s wide range of infotainment features, the 1 Series is available not only with sophisticated navigation systems*, but also with 
a Blue​tooth interface* for mobile phones and the BMW Assist* telematics service including access to the internet*. So here again, the BMW 1 Series ranks alone within the compact class, offering features and qualities not 
to be found on any other car in this segment.

Permanent supervision of the battery serves to guarantee ongoing and 
con​sistent reliability in the BMW 130i: As soon as the charge sensor monitoring the control unit recognises a lack or impairment of capacity, the feed of pow​er to items serving comfort purposes only is automatically reduced, thus main​taining the battery capacity required for starting the engine.

Every drive in a BMW 130i is a most enjoyable experience right from the 
start – also thanks to the memory function integrated in the key to the car storing and saving individual positions and settings for each driver: 
The BMW 130i is able to “recognise” up to three different drivers through their personal keys, automatically calling up important functions as soon as the “appropriate” driver wishes to set off in the car. So getting into the BMW 130i, the driver will always find his or her “personal” settings pre-aligned right from the start. 

The key automatically saves the last setting in the audio system (sound, volume, stations chosen), the air conditioning (temperature and air distribu​tion) as well as the seats with electrical adjustment* and the position of the mirrors.

Tailoring the BMW 130i to Your Personal Requirements:

Making Your Choice just Eight Days Prior to the Start of Production.

Given its wide range of features and qualities, the BMW 130i is most defin​ite​ly a truly unique car. And thanks to the equally wide choice of paintwork 
options, upholstery and interior equipment, as well as an unusual line-up of equipment quite exceptional in the compact class, the BMW 130i offers 
every customer exactly what he – or she – wishes to enjoy on the road.

Indeed, every BMW is a true expression of its owner’s personality. 
This also applies to the BMW 130i, which – like every BMW – is not built in a stand​ard​ised process or with a definite, pre-determined range of equipment pack​ages, but rather to the customer’s individual order, exactly as required. 
So it is virtually out of the question that two identical versions of the BMW 130i will come off the production line within the same year.

Clearly, diversity of this kind calls for sophisticated logistics – which is pre​cisely why the BMW Group has developed a production system serving to 
ensure absolute reliability as well as flexibility prior to and during production: The Customer-Oriented Sales and Production Process closely coordinates 
all steps and operations in the production of a BMW. The big advantage for the customer is that he is able to make changes to the features and equip​ment 
of a car already ordered up to just eight days prior to the start of as​sem​bly.





































