[image: image1.png]Torque [Nm]

540
520
500
480
460
440
420
400
380
360

520

500

480

460

440

420

400

380

360

340

320

300

280

260

240

220

200

180

160

140

120

100

D
S O
Output [hp]

AN
-

500 1000 1500 2000 2500 3000 3500 4000 4500 5000 5500 6000 6500 7000 7500 8000 8500

Engine speed [min-1]

— 380

— 360

— 340

— 320

— 300

— 280

— 260

— 240

— 220

— 200

— 180

— 160

— 140

— 120

— 100

— 60

— 40

Output [KW]





10  Technical specifications

	Body
	
	M6
	
	
	

	No. of doors/seats
	
	2/2+2
	
	
	

	Length/width/height (unladen)
	mm
	4,871/1,855/1,377
	
	
	

	Wheelbase
	mm
	2,781
	
	
	

	Track, front/rear
	mm
	1,567/1,584
	
	
	

	Turning circle
	m
	12.5
	
	
	

	Fuel tank capacity
	approx. ltr
	70
	
	
	

	Cooling system incl. Heating
	ltr
	15
	
	
	

	Engine oil
	ltr
	13
	
	
	

	Transmission fluid
	ltr
	1.2
	
	
	

	Final drive fluid
	ltr
	2.6
	
	
	

	Weight, unladen (EU1)
	kg
	1,785 
	
	
	

	Max. load (DIN)
	kg
	415
	
	
	

	Max. permissible weight (DIN)
	kg
	2,200
	
	
	

	Permissible axle load front/rear
	kg
	1,090/1,200
	
	
	

	Max. trailer load2
	
	
	
	
	

	braked (12%)/unbraked
	kg
	-
	
	
	

	Max. roof load/trailer nose weight
	kg
	-
	
	
	

	Luggage comp. capacity VDAKofferrauminhalt nach VDA
	ltr
	450
	
	
	

	Drag coefficient
	cX x A
	0.32 x 2.15
	
	
	

	Engine
	
	
	
	
	

	Layout/No. of cylinders/Valves Zylinder/Ventile


	
	V90/10/4
	
	
	

	Engine management
	
	 MS S65
	
	
	

	Displacement
	cc
	4,999
	
	
	

	Bore/Stroke
	mm
	92.0/75.2
	
	
	

	Compression ratio
	:1
	12.0  
	
	
	

	Fuel grade
	RON
	95-98
	
	
	

	Max. output
	kW/bhp
	373/507
	
	
	

	At engine speed
	rpm
	7,750
	
	
	

	Torque
	Nm
	520
	
	
	

	At engine speed
	rpm
	6,100
	
	
	

	Electrics
	
	
	
	
	

	Battery/Location
	Ah/–
	90/boot
	
	
	

	Alternator
	A/W
	170/2,380
	
	
	

	Chassis
	
	
	
	
	

	Front suspension
	
	Double-joint thrust rod spring-strut axle in aluminium; 

	
	
	traverse force compensation; anti-dive

	Rear suspension
	
	Integral axle (aluminium), wheel suspension with special effect

	
	
	anti-squat/anti-dive

	Brakes, front
	
	Single-piston floating-caliper Compound disc brakes

	Diameter
	mm
	        348 x 36, vented
	
	
	

	Brakes, rear
	
	Single-piston floating-caliper Compound disc brakes

	Diameter
	mm
	345 x 24, vented
	
	
	

	Driving Stability Systems
	
	ABS, CBC, DSC
	
	

	Steering
	
	Rack-and-pinion, hydraulically assisted steering and M Servotronic

	Overall ratio
	:1
	12.4
	
	
	

	Type of transmission
	
	SMG III
	

	Transmission ratio   I
	:1
	3.985
	
	
	

	                                  II
	:1
	2.652
	
	
	

	                                  III
	:1
	1.806
	
	
	

	                                  IV
	:1
	1.392
	
	
	

	                                  V
	:1
	1.159
	
	
	

	                                  VI
	:1
	1 
	
	
	

	                                  VII
	:1
	0.833
	
	
	

	                                  R
	:1
	3.985
	
	
	

	Final drive ratio
	:1
	3.620
	
	
	

	Tyres front / rear
	
	255/40 ZR 19 / 285/35 ZR 19  
	
	

	Wheels 
	
	8,5 J x 19 EH 2 IS 12 Alloy/9,5 J x 19 EH 2 IS 28 Alloy
	
	

	Performance
	
	
	
	
	

	Power to weight ratio (DIN)
	kg/kW
	 4.6
	
	
	

	Output per litre
	kW/ltr
	74.6
	
	
	

	Acceleration        
	0–62 mph
	sec
	4.6 
	
	
	

	50–75 mph in 4th gear
	sec
	4.4 
	
	
	

	Top speed3
	Mph
	155
	
	
	

	Fuel consumption (EU cycle)
	
	
	
	
	

	In town
	Mpg
	12.4
	
	
	

	Out of town
	Mpg
	27.7 
	
	
	

	Overall
	Mpg
	19.1 
	
	
	

	CO2 
	g/km
	357 
	
	
	

	Miscellaneous
	
	
	
	
	

	Emission classification
	
	EU4
	




Weight of the car in road trim (DIN) plus 75 kg for driver and luggage.
2Deviations are possible under certain conditions.
3Electronically limited.

Torque diagram





[image: image2.png]1130

Y Y

2781 |
4871
= -
~ A
| —
AN M < <t
™M O o M
<t < ANl ™

AN





[image: image3.png]



Interior and exterior dimensions






































