[image: image1.png]Output [KW]

400

350

300

250

200

150

100

50

800

700

600

500

400

300

200

100

0

0 1000 2000
Engine Speed [min—1]

3000

4000

5000

6000

7000

8000

9000

Torque [Nm]





7. Technical specifications

	Body
	
	M6 Convertible
	
	
	

	No of doors/seats

l
	 
	2+2
	
	
	

	Length/width/height (unladen)
	mm
	4,871/1,855/1,377
	
	
	

	Wheelbase
	mm
	2,781
	
	
	

	Track, front/rear

wgsdfg


	mm
	1,567/1,584
	
	
	

	Turning circle
	m
	12.5
	
	
	

	Tank capacity 
	approx. ltr
	70
	
	
	

	Cooling system including heater

l
	ltr
	15
	
	
	

	Engine oil

Getriebeöl


	ltr
	13
	
	
	

	Transmission fluid
	ltr
	2.6
	
	
	

	Final drive fluid
	ltr
	1.2
	
	
	

	Weight, unladen, to EU standard1
	kg
	2,005


	
	
	

	Max load to DIN standard
	kg
	450


	
	
	

	Max permissible weight to DIN
	kg
	2,380
	
	
	

	Max axle load, front/rear
	kg
	1,120/1,290
	
	
	

	Max trailer load2
	
	
	
	
	

	braked (12%)/unbraked


	kg
	–
	
	
	

	Max roof load/max towbar dl
	kg
	–
	
	
	

	Luggage comp to DIN
	ltr
	300–350
	
	
	

	Air resistance 
	cd x A
	0.731
	
	
	

	Power unit
	
	
	
	
	

	Config/No of cylinders/valves Zylinder/Ventile


	 
	V/10/4
	
	
	

	Engine management
	 
	MS S65
	
	
	

	Capacity
	cc
	4,999
	
	
	

	Bore/stroke 
	mm
	92.0/75.2
	
	
	

	Compression ratio


	: 1
	       12.0:1 
	
	
	

	Fuel grade 
	RON
	95–98
	
	
	

	Max output
	kW/hp
	373/507
	
	
	

	at 
	rpm
	7,750
	
	
	

	Max torque
	Nm/lb-ft
	520/383
	
	
	

	at
	rpm
	6,100
	
	
	

	Electrical system
	
	
	
	
	

	Battery/installation
	Ah/–
	90/luggage compartment
	
	
	

	Alternator
	A/W
	170/2,380
	
	
	

	Chassis and suspension
	
	
	
	
	

	Suspension, front
	 
	Two-joint spring strut axle with tiebar; small positive steering roll ratios, 

	
	
	compensation of transverse forces; reduction of brake dive

	Suspension, rear
	
	Aluminium integral axle with longitudinal arms and double track control arms,  

	
	
	anti-squat and anti-dive 

	Brakes, front 
	 
	Two-piston swing-calliper compound disc brakes

	Diameter
	mm
	374 x 36, vented and cross-drilled
	
	
	

	Brakes, rear
	 
	Single-piston swing-calliper compound disc brakes

	Diameter
	mm
	370 x 24, vented and cross-drilled
	
	
	

	Driving stability systems
	 
	ABS, CBC, DSC; Variable M Differential Lock

	Steering 


	 
	Rack-and-pinion steering with hydraulic assistance and Servotronic
	

	Steering ratio, overall
	: 1 
	13.0
	
	
	

	Transmission
	
	SMG III
	
	
	

	Gear ratios 
I

  
	: 1 
	3.985
	
	
	

	  
II
	: 1 
	2.652
	
	
	

	
III
	: 1 
	1.806
	
	
	

	
IV
	: 1 
	1.392
	
	
	

	
V
	: 1 
	1.159
	
	
	

	
VI
	: 1 
	1.000
	
	
	

	
VII
	: 1
	0.833
	
	
	

	
R
	: 1 
	3.985
	
	
	

	Final drive ratio
	: 1 
	3.620
	
	
	

	Tyres, front/rear
	 
	255/40 ZR19/285/35 ZR19 

	Rims, front/rear
	 
	8.5J x 19 EH 2 IS 12 forged aluminium/9.5J x 19 EH 2 IS 17 forged aluminium

	Performance
	
	
	
	
	

	Power-to-weight ratio, DIN
	kg/kW
	5.2
	
	
	

	Output per litre
	kW/hp
	74.6/101.5
	
	
	

	Acceleration  0–62mph
	sec
	4.8
	
	
	

	Standing-start km
	sec
	22.9
	
	
	

	In 4th gear   60–75mph
	sec
	4.7
	
	
	

	Top speed
	mph
	1553
	
	
	

	Fuel consumption in EU cycle
	
	
	
	
	

	Urban
	mpg
	12.4
	
	
	

	Extra-urban
	mpg
	26.4
	
	
	

	Composite
	mpg
	18.6
	
	
	

	CO2 
	g/km
	366
	
	
	

	Miscellaneous
	
	
	
	
	

	Emission standard
	
	EU4
	
	
	

	
	 
	
	
	
	



Weight of car in road trim (DIN) plus 75 kg for driver and luggage.

2May be increased under certain conditions. 




3Electronically limited.

8. Torque curves






































