PAGE  
BMW
Media
Information

3/2004
Page 2

Specifications BMW 5 Series Touring.
525i; 545i.

	Body

Fsdfd

d

Anzahl Türen / s
	
	525i
	545i
	
	

	No. of doors/seats

	
	5/5
	5/5
	
	

	Length/width/height (unladen)
	mm
	4,843/1,846/1,491
	4,843/1,846/1,491
	
	

	Wheelbase
	mm
	2,886
	2,886
	
	

	Track, front/rear


	mm
	1,558/1,581
	1,558/1,581
	
	

	Turning circle
	m
	11.4
	11.4
	
	

	Fuel tank capacity
	approx. ltr
	70
	70
	
	

	Cooling system incl. heating

l
	ltr
	9.8 (10.2)
	13.8 (14.2)
	
	

	Engine oil

Getriebeöl


	ltr
	6.5
	8.0
	
	

	Transmission fluid
	ltr
	Permanent filling
	Permanent filling
	
	

	Final drive fluid
	ltr
	Permanent filling
	Permanent filling
	
	

	Weight, unladen (EU1)
	kg
	1,655 (1,665) 
	1,795 (1,800)
	
	

	Max load (DIN)
	kg
	600
	560
	
	

	Max. permissible weight (DIN)
	kg
	2,180 (2,190)
	2,280 (2,285)
	
	

	Permissible axle load front/rear
	kg
	980/1,280
	1,050/1,300
	
	

	Max. trailer load2
	
	
	
	
	

	braked (12%)/unbraked


	kg
	1,800/750  
	2,000/750
	
	

	Max. roof load/trailer nose weight
	kg
	100/90
	100/90
	
	

	Luggage comp. capacity VDAKofferrauminhalt nach VDA
	ltr
	500–1,6505
	500–1,6505 
	
	

	Drag coefficient
	cX x A
	0.30 x 2.26
	0,30 x 2,26
	
	

	Engine
	
	
	
	
	

	Layout/No. of cylinders/Valves Zylinder/Ventile


	
	V/6/4
	V90/8/4
	
	

	Engine management
	
	Siemens MS45
	Bosch ME9.2.1
	
	

	Displacement
	cc
	2,494
	4,398
	
	

	Bore/Stroke
	mm
	75.0/84.0
	82.7/92.0
	
	

	Compression ratio


	:1
	10.5 
	10.0
	
	

	Fuel grade
	RON
	91–983
	91–983
	
	

	Max. output
	kW/bhp
	141/192
	245/333
	
	

	At engine speed
	rpm
	6,000
	6,100
	
	

	Torque
	Nm
	245
	450
	
	

	At engine speed
	rpm
	3,500
	3,600
	
	

	Electrics
	
	
	
	
	

	Battery/Location
	Ah/–
	80/boot
	90/boot
	
	

	Alternator
	A/W
	150/2,100
	170/2,380
	
	

	Chassis
	
	
	
	
	

	Front suspension
	
	Double-joint thrust-rod spring strut axle in aluminium; 

	
	
	compensation of transverse forces; anti-dive

	Rear suspension
	
	Integral IV multi-arm axle in aluminium; 

	
	
	wheel suspension with special effect and anti-squat/anti-dive

	Brakes, front
	
	Single-piston floating-caliper disc brakes

	Diameter
	mm
	310 x 24, vented
	348 x 30, vented vented             
	
	

	Brakes, rear
	
	Single-piston floating-caliper disc brakes

	Diameter
	mm
	320 x 20, vented
	345 x 24, vented
	
	

	Driving Stability Systems
	
	ABS+CBC, ASC; DSC, DTC, DBC; Optional: Dynamic Drive

ABS+CBC, ASC

ABS, CBC, ASC
	

	Steering
	
	Rack-and-pinion, Servotronic; Optional: Active Steering
	

	Overall ratio
	:1
	13.7
	13.7                         
	
	

	Type of transmission
	
	6-gear manual /SMG optional (6-gear Steptronic automatic transmission)

	Transmission ratio

	I
	:1
	4.350 (4.171)
	4.055 (4.171)
	
	

	

	II
	:1
	2.496 (2.340)
	2.396 (2.340)
	
	

	

	III
	:1
	1.665 (1.521)
	1.582 (1.521)
	
	

	

	IV
	:1
	1.231 (1.143)
	1.192 (1.143)
	
	

	

	V
	:1
	1.000 (0.867)
	1.000 (0.867)
	
	

	

	VI
	:1
	0.851 (0.690)
	0.872 (0.690)
	
	

	

	R
	:1
	3.926 (3.403)
	3.670 (3.403)
	
	

	Final drive ratio
	:1
	3.23 (3.73)
	2.93 (3.38)
	
	

	Tyres 
	
	225/55R16 95 W
	225/50R17 94 Y
	
	

	Wheels 
	
	7J x 16 Alloy
	7,5 J x17 Alloy
	
	

	Performance3
	
	
	
	
	

	Power to weight ratio (DIN)
	kg/kW
	11.2
	7.0
	
	

	Output per litre
	kW/ltr
	56.5
	55.7
	
	

	Acceleration        
	0–62mph
	sec
	8.2 (8.8)
	5.9 (6.0)
	
	

	
	0–1000 m
	sec
	29.0 (29.4)
	25.4 (25.6)
	
	

	50–75 mph in 4th gear
	sec
	8.9/11.9 (–)
	6.1/7.7 (–)
	
	

	Top speed
	km/h
	232 (229)
	250 (250)4
	
	

	Fuel consumption (EU cycle)
	
	
	
	
	

	In town
	ltr/100 km
	               14.1 (14.7)

16,3lllllllllllll63 (15,8)
	               16.8 (16.4)
	
	

	Out of town
	ltr/100 km
	7.5 (7.9)
	8.3 (8.2)
	
	

	Overall
	ltr/100 km
	9.9 (10.3)
	11.3 (11.2)
	
	

	CO2 
	g/km
	238 (248)
	274 (271)
	
	

	Miscellaneous
	
	
	
	
	

	Emission classification
	
	EU4
	EU4
	
	



Figures in brakets are Automatic-Gearbox figures.


Weight of the car in road trim (DIN) plus 75 kg for driver and luggage.

2Deviations are possible under certain conditions.

3Performance and fuel consumption figures relate to RON 98.


4Electronically limited.
5In case of equipment with Runflat-Tyres.

Specifications BMW 5 Series Touring.
525d; 530d.

	Body
	
	525d
	530d
	
	

	No. of doors/seats
	
	5/5
	5/5
	
	

	Length/width/height (unladen) 

)
	mm
	4,843/1,846/1,491
	 4,843/1,846/1,491
	
	

	Wheelbase
	mm
	2,886
	2,886
	
	

	Track, front/rear


	mm
	1,558/1,581
	1,558/1,581
	
	

	Turning circle
	m
	11.4
	11.4
	
	

	Fuel tank capacity
	approx. ltr
	70
	70
	
	

	Cooling system incl. heating

l
	ltr
	9.6 (9.8)
	9.6 (9.8)
	
	

	Engine oil

Getriebeöl


	ltr
	8.25
	8.25
	
	

	Transmission fluid
	ltr
	Permanent filling
	Permanent filling
	
	

	Final drive fluid
	ltr
	Permanent filling
	Permanent filling
	
	

	Weight, unladen (EU1)
	kg
	1,750 (1,760) 
	1,760 (1,775)
	
	

	Max load (DIN)
	kg
	600
	600
	
	

	Max. permissible weight (DIN)
	kg
	2,275 (2,285)
	2,285 (2,300)
	
	

	Permissible axle load front/rear
	kg
	1,050/1,300
	1,050/1,300
	
	

	Max. trailer load2
	
	
	
	
	

	braked (12%)/unbraked


	kg
	2,000/750  
	2,000/750
	
	

	Max. roof load/trailer nose weight
	kg
	100/90
	100/90
	
	

	Luggage comp. capacity VDAKofferrauminhalt nach VDA
	ltr
	500–1,6503
	500–1,6503
	
	

	Drag coefficient
	cX x A
	0.30 x 2.26
	0,30 x 2,26
	
	

	Engine
	
	
	
	
	

	Layout/No. of cylinders/Valves Zylinder/Ventile


	
	R/6/4
	R/6/4
	
	

	Engine management
	
	Bosch DDE 5 (CR)
	Bosch DDE 5 (CR)
	
	

	Displacement
	cc
	2,497
	2,993
	
	

	Bore/Stroke
	mm
	75.1/84.0
	90.0/84.0
	
	

	Compression ratio


	:1
	17.0 
	17.0
	
	

	Fuel grade
	RON
	                      Diesel
	Diesel
	
	

	Max. output
	kW/bhp
	130/177
	160/218
	
	

	At engine speed
	rpm
	4,000
	4,000
	
	

	Torque
	Nm
	400
	500
	
	

	At engine speed
	rpm
	2,000–2,750
	2,000–2,750
	
	

	Electrics
	
	
	
	
	

	Battery/Location
	Ah/–
	90/boot
	90/boot
	
	

	Alternator
	A/W
	170/2,380
	170/2,380
	
	

	Chassis
	
	
	
	
	

	Front suspension
	
	Double-joint thrust-rod spring strut axle in aluminium; 

	
	
	compensation of transverse forces; anti-dive

	Rear suspension
	
	Integral IV multi-arm axle in aluminium; 

	
	
	wheel suspension with special effect and anti-squat/anti-dive

	Brakes, front
	
	Single-piston floating-caliper disc brakes

	Diameter
	mm
	310 x 24, vented
	324 x 30, vented ventedvented             
	
	

	Brakes, rear
	
	Single-piston floating-caliper disc brakes

	Diameter
	mm
	320*20, vented
	320*20, vented
	
	

	Driving Stability Systems
	
	ABS+CBC, ASC; DSC, DTC, DBC; Optional: Dynamic Drive

ABS+CBC, ASC

ABS, CBC, ASC
	

	Steering
	
	Rack-and-pinion, Servotronic; Optional: Active Steering
	

	Overall ratio
	:1
	13.7:1
	                      13.7:1                         
	
	

	Type of transmission
	
	6-gear manual (6-gear Steptronic automatic transmission)

	Transmission ratio 
I

  
	:1
	5.080 (4.171)
	5.080 (4.171)
	
	

	
II
	:1
	2.804 (2.340)
	2.804 (2.340)
	
	

	
III
	:1
	1.783 (1.521)
	1.783 (1.521)
	
	

	
IV
	:1
	1.260 (1.143)
	1.260 (1.143)
	
	

	
V
	:1
	1.000 (0.867)
	1.000 (0.867)
	
	

	
VI
	:1
	0.835 (0.691)
	0.835 (0.691)
	
	

	
R
	:1
	4.638 (3.403)
	4.638 (3.403)
	
	

	Final drive ratio
	:1
	2.65 
	2.56 
	
	

	Tyres 
	
	225/55R16 95 W
	225/55R16 95 W
	
	

	Wheels 
	
	7J x 16 Alloy
	7J x16 Alloy
	
	

	Performance
	
	
	
	
	

	Power to weight ratio (DIN)
	kg/kW
	12.9
	10.5
	
	

	Output per litre
	kW/ltr
	52.1
	53.5
	
	

	Acceleration      
	0–62mph
	sec
	8.3 (8.5)
	7.2 (7.4)
	
	

	
	0–1000 m
	sec
	29.3(29.5)
	27.5 (27.5) 
	
	

	50–75 mph in 4th gear
	sec
	6.9/8.5 (–/–)
	5.7/6.9 (–/–)
	
	

	Top speed
	km/h
	225 (222)
	242 (240)
	
	

	Fuel consumption (EU cycle)
	
	
	
	
	

	In town
	ltr/100 km
	 9.7 (10.9) 
	9.9 (11.1)
	
	

	Out of town
	ltr/100 km
	5.6 (6.5)
	5.8 (6.4)
	
	

	Overall
	ltr/100 km
	7.0 (8.0)
	7.2 (8.0)
	
	

	CO2 
	g/km
	187 (213)
	192 (213)
	
	

	Miscellaneous
	
	
	
	
	

	Emission classification
	
	EU4 with diesel partikel filter
	EU4 with diesel partikel filter
	
	


Figures in brakets are Automatic-Gearbox figures.

Weight of the car in road trim (DIN) plus 75 kg for driver and luggage.


2Deviations are possible under certain conditions.


3In case of equipment with Runflat-Tyres. 





































