

8. Engine Output and Torque.

[image: image1.jpg]Output [Kw]

N
Q

>
S

60

207

123 kW / 10250 U/min

K 1200 S

130 Nm / 8250 U/min

r 150

r 100

r 50

2000 3000 4000 5000 6000 7000 8000 9000 10000 1100012000

Engine speed [min-1]

Torque [Nm]




9.  Specifications for BMW K1200 S

	
	
	K 1200 S
	
	
	

	Engine
	
	
	
	
	

	Capacity
	cc
	1,157
	
	
	

	Bore/stroke
	mm
	79/59
	
	
	

	Output
	kW/bhp
	123/167
	
	
	

	at
	rpm
	10,250
	
	
	

	Torque
	Nm/lb-ft
	130/96
	
	
	

	at
	rpm
	8,250
	
	
	

	Configuration
	
	In-line
	
	
	

	No of cylinders
	
	4
	
	
	

	Compression ratio/fuel grade
	
	13:1 / premium plus (98 octane)
	
	
	

	Valves/charge management
	
	DOHC (double overhead  camshaft)
	
	
	

	Valves per cylinder
	
	4
	
	
	

	Intake/outlet dia
	Mm
	32/27.5
	
	
	

	Fuel supply
	
	BMSK
	
	
	

	
	
	
	
	
	

	Electrical System
	
	
	
	
	

	Alternator
	W
	580
	
	
	

	Battery
	V/Ah
	12/14 maintenance-free
	
	
	

	Headlights
	W
	Low beam 1x H 7/55 W
	
	
	

	
	
	High beam 2x H 7/55 W
	
	
	

	Starter
	kW
	0.7
	
	
	

	
	
	
	
	
	

	Transmission
	
	
	
	
	

	Clutch
	
	Multi-plate oil bath clutch, dia 151 mm               
	
	

	Gearbox
	
	Claw-shifted six-speed gearbox
	
	
	

	Primary transmission
	
	1.559
	
	
	

	Gear ratios
I
	
	2.521
	
	
	

	
II
	
	1.842
	
	
	

	
III
	
	1.455
	
	
	

	
IV
	
	1.287
	
	
	

	
V
	
	1.143
	
	
	

	
VI
	
	1.015
	
	
	

	Rear wheel drive
	
	Driveshaft
	
	
	

	Final drive ratio
	
	2.82
	
	
	

	
	
	
	
	
	

	Suspension and Running Gear
	
	
	
	
	

	Frame
	
	
Composite aluminium frame with

IHU/extrusion-pressed profiles and die sections
	
	

	Front wheel suspension
	
	BMW Duolever
	
	
	

	Rear wheel suspension 
	
	BMW Paralever
	
	
	

	Spring travel front/rear 
	mm
	115/135
	
	
	

	Camber
	mm
	112
	
	
	

	Wheelbase
	mm
	1,571
	
	
	

	Handlebar centrepoint angle
	°
	60.6
	
	
	

	Brakes
	front
	Double-disc brake, dia 320 mm
	
	
	

	
	rear
	
Single-disc brake, dia 265 mm 
	
	
	

	
	
	BMW Motorrad Integral ABS fitted as standard (partly integral system)
	
	
	

	Wheels
	
	Light-alloy
	
	
	

	
	front
	3.50 x 17 MTH 2
	
	
	

	
	rear
	600 x 17 MTH 2
	
	
	

	Tyres
	front
	120/70 ZR 17
	
	
	

	
	rear
	190/50 ZR 17
	
	
	

	
	
	
	
	
	

	Dimensions and Weights
	
	
	
	
	

	Length, overall
	mm
	2,282
	
	
	

	Width, overall, with mirrors
	mm
	905
	
	
	

	Handlebar width, w/o mirrors
	mm
	786
	
	
	

	Seat height
	mm
	820 (790)
	
	
	

	Weight, unladen, with full tank
	kg
	248
	
	
	

	Max permissible weight
	kg
	450
	
	
	

	Fuel tank 
	ltr
	19
	
	
	

	
	
	
	
	
	

	Performance data
	
	
	
	
	

	Fuel consumption
	
	
	
	
	

	90 km/h
	ltr/100 km
	4.7
	
	
	

	120 km/h
	ltr/100 km
	5.5
	
	
	

	Acceleration
	
	
	
	
	

	0–100 km/h
	sec
	2.8
	
	
	

	Standing start/km
	sec
	–
	
	
	

	Top speed
	km/h(mph)
	200 (125) plus
	
	
	


10.  K 1200 S Colours


	
	Colour
	Seat
	

	K 1200 S
	Granite Grey Metallic
	Black
	

	
	Indigo Blue Metallic
	
	

	
	Sun Yellow Non-Metallic/White Aluminium Metallic /Dark Graphite Metallic
	Black
	

	
	Indigo Blue Metallic /Alpine White
	Black
	






































