[image: image1.png]1395

- 1513

y

- 1782

A

2760 b

1049

4580






9. Technical specifications

325i Coupé

	 
	 
	325i Coupé
	325i Coupé

	Engine platform / transmission
	 
	N52B25O1 / M6
	N52B25O1 / A6

	No of doors / seats
	 -
	2 / 4
	2 / 4

	Vehicle length / width / height -unloaded-
	mm
	4580 / 1782 / 1395
	4580 / 1782 / 1395

	Weelbase/ Turning circle
	mm / m
	2760 / 11.0
	2760 / 11.0

	Track front / rear
	mm
	1500 / 1513
	1500 / 1513

	Overhang rear
	mm
	1049
	1049

	Ground clearance -empty-
	mm
	143
	143

	Shoulder width front / rear
	mm
	1404 / 1318
	1404 / 1318

	Elbow width front / rear
	mm
	1459 / 1403
	1459 / 1403

	Eff. head room front / rear
	mm
	975 / 934
	975 / 934

	Luggage capacity
	ltr
	440
	440

	Fuel tank capacity 
	ltr
	63
	63

	Curb weight -acc DIN / EU
	kg
	1420 / 1495
	1450 / 1525

	Axle load ratio -unloaded DIN-
	%
	49.1
	48.4

	Gross vehicle weight / Payload
	kg
	1900 / 480
	1930 / 480

	Axle load limit front / rear
	kg
	905 / 1045
	905 / 1045

	Trailer load braked - 12% / 8% -
	kg
	1600 / 1700
	1600 / 1700

	Trailer load unbraked / roof load
	kg
	745 / 75
	745 / 75

	Engine type / cylinder / valves p cyl
	 -
	In line / 6 / 4
	In line / 6 / 4

	Fuel management
	 -
	MSV80
	MSV80

	Effective displacement
	cm3
	2497
	2497

	Stroke / bore
	mm
	78.8 / 82
	78.8 / 82

	Nom power output / Engine speed
	kW(hp)/min-1
	160 (218) / 6500
	160 (218) / 6500

	Max torque / Engine speed
	Nm/min-1
	250 / 2750
	250 / 2750

	RPM at max torque -range-
	1/min
	2750-4000
	2750-4000

	Compression ratio / Fuel type
	 -
	11.0 / ROZ 91-98
	11.0 / ROZ 91-98

	Exhaust emissions classification
	 - 
	EU4
	EU4

	Codification of reference gearbox
	 -
	GETRAG I
	ZF 6HP19 TÜ

	Gear ratio 1st/2nd/3rd
	 -
	4.323/2.456/1.659
	4.171/2.340/1.521

	Gear ratio 4th/5th/6th
	 -
	1.230/1.000/0.848
	1.143/0.867/0.691

	Reverse ratio
	 -
	3.938
	3.403

	Final drive ratio
	 -
	3.23
	3.73

	Steering type / ratio
	 -
	Rack-and-pinion / 16
	Rack-and-pinion / 16

	Brake front / diameter
	 - / mm
	Ventilated Disc / 330
	Ventilated Disc / 330

	Brake rear / diameter
	 - / mm
	Ventilated Disc / 300
	Ventilated Disc / 300

	Drag Cd / A / Cd x A 
	 - / m2 / m2
	0.28 / 2.08 / 0.58
	0.28 / 2.08 / 0.58

	Top Speed
	mph (km/h)
	153 (247)
	152 (245)

	Acceleration 0-62 mph (0-100 km/h) 
	s
	6.9
	7.6

	Acceleration 0-1000 metres
	s
	27.3
	28.0

	Accel 50-75 mph (80-120 km/h) 4/5 Gear
	s
	7.4 / 9.4
	 - / - 

	Weight ratio
	kg/kW 
	8.9
	9.1

	Output per litre
	kW/ltr
	64.1
	64.1

	Fuel consumption -EU urban- 
	mpg (ltr/100km)
	23.3 (12.1)
	21.9 (12.9)

	Fuel consumption -EU extra-urban-
	mpg (ltr/100km)
	45.6 (6.2)
	42.2 (6.7)

	Fuel consumption -Combined 
	mpg (ltr/100km) /km
	33.6 (8.4)
	31.4 (9.0)

	CO2-emission-EU-
	g/Km
	203
	218

	Tyre type -front-
	 -
	225/45 R17 91W RSC
	225/45 R17 91W RSC

	Tyre type -rear-
	 -
	225/45 R17 91W RSC
	225/45 R17 91W RSC

	Front rim
	 -
	8J x 17 Light alloy
	8J x 17 Light alloy

	Rear rim
	 -
	8J x 17 Light alloy
	8J x 17 Light alloy

	Radiator incl. heater / Engine oil exchange quantity
	ltr
	8.2 / 6.5
	8.5 / 6.5

	Transmission fluid -exchange quantity- 
	ltr
	Permanent filling
	Permanent filling

	Battery capacity / installed position
	Ah / -
	70 / Luggage comp.
	70 / Luggage comp.

	Alternator output rating 
	A / W
	155 / 2170
	180 / 2520

	Wheel offset
	mm
	34
	34


330i Coupé

	 
	 
	330i Coupé
	330i Coupé

	Engine platform / Transmission 
	 
	N52B30O1 / M6
	N52B30O1 / A6

	No of doors / seats
	 -
	2 / 4
	2 / 4

	Vehicle length / width / height -unloaded-
	mm
	4580 / 1782 / 1395
	4580 / 1782 / 1395

	Weelbase/ Turning circle
	mm / m
	2760 / 11.0
	2760 / 11.0

	Track front / rear
	mm
	1500 / 1513
	1500 / 1513

	Overhang rear
	mm
	1049
	1049

	Ground clearance -empty-
	mm
	143
	143

	Shoulder width front / rear
	mm
	1404 / 1318
	1404 / 1318

	Elbow width front / rear
	mm
	1459 / 1403
	1459 / 1403

	Eff. head room front / rear
	mm
	975 / 934
	975 / 934

	Luggage capacity
	ltr
	440
	440

	Fuel tank capacity 
	ltr
	63
	63

	Curb weight -acc DIN / EU
	kg
	1470 / 1545
	1485 / 1560

	Axle load ratio -unloaded DIN-
	%
	48.4
	47.9

	Gross vehicle weight / Payload
	kg
	1950 / 480
	1965 / 480

	Axle load limit front / rear
	kg
	930 / 1060
	930 / 1060

	Trailer load braked - 12% / 8% -
	kg
	1700 / 1800
	1700 / 1800

	Trailer load unbraked / roof load
	kg
	750 / 75
	750 / 75

	Engine type / cylinder / valves p cyl
	 -
	In line / 6 / 4
	In line / 6 / 4

	Fuel management
	 -
	MSV80
	MSV80

	Effective displacement
	cm3
	2996
	2996

	Stroke / bore
	mm
	88.0 / 85.0
	88.0 / 85.0

	Nom power output / Engine speed
	kW(hp)/min-1
	200 (272) / 6650
	200 (272) / 6650

	Max torque / Engine speed
	Nm/min-1
	315 / 2750
	315 / 2750

	RPM at max torque -range-
	1/min
	2750
	2750

	Compression ratio / Fuel type
	 -
	10.7 / ROZ 91-98
	10.7 / ROZ 91-98

	Exhaust emissions classification
	 - 
	EU4
	EU4

	Codification of reference gearbox
	 -
	ZF H
	GM 6

	Gear ratio 1st/2nd/3rd
	 -
	4.350/2.496/1.665
	4.171/2.371/1.551

	Gear ratio 4th/5th/6th
	 -
	1.230/1.000/0.851
	1.157/0.853/0.671

	Reverse ratio
	 -
	3.926
	3.200

	Final drive ratio
	 -
	3.15
	3.64

	Steering type / ratio
	 -
	Rack-and-pinion / 16
	Rack-and-pinion / 16

	Brake front / diameter
	 - / mm
	Ventilated Disc / 330
	Ventilated Disc / 330

	Brake rear / diameter
	 - / mm
	Ventilated Disc / 336
	Ventilated Disc / 336

	Drag Cd / A / Cd x A 
	 - / m2 / m2
	0.29 / 2.08 / 0.60
	0.29 / 2.08 / 0.60

	Top Speed
	mph (km/h)
	155 (250)
	155 (250)

	Acceleration 0-62 mph (0-100 km/h) 
	s
	6.1
	6.4

	Acceleration 0-1000 metres
	s
	26.0
	26.0

	Accel 50-75 mph (80-120 km/h) 4/5 Gear
	s
	6.5 / 8.1
	 - / - 

	Weight ratio
	kg/kW 
	7.4
	7.4

	Output per litre
	kW/ltr
	66.8
	66.8

	Fuel consumption -EU urban- 
	mpg (ltr/100km)
	22.1 (12.8)
	21.9 (12.9)

	Fuel consumption -EU extra-urban-
	mpg (ltr/100km)
	43.5 (6.5)
	41.5 (6.8)

	Fuel consumption -Combined 
	mpg (ltr/100km) /km
	32.1 (8.8)
	31.4 (9.0)

	CO2-emission-EU-
	g/Km
	212
	216

	Tyre type -front-
	 -
	225/45 R17 91W RSC
	225/45 R17 91W RSC

	Tyre type -rear-
	 -
	225/45 R17 91W RSC
	225/45 R17 91W RSC

	Front rim
	 -
	8J x 17 Light alloy
	8J x 17 Light alloy

	Rear rim
	 -
	8J x 17 Light alloy
	8J x 17 Light alloy

	Radiator incl. heater / Engine oil exchange quantity
	ltr
	8.2 / 6.5
	8.5 / 6.5

	Transmission fluid -exchange quantity- 
	ltr
	Permanent filling
	Permanent filling

	Battery capacity / installed position
	Ah / -
	70 / Luggage comp.
	70 / Luggage comp.

	Alternator output rating 
	A / W
	155 / 2170
	180 / 2520

	Wheel offset
	mm
	34
	34


335i Coupé

	 
	 
	335i Coupé
	335i Coupé

	Engine platform / Transmission
	 
	N54B30O0 / M6
	N54B30O0 / A6

	No of doors / seats
	 -
	2 / 4
	2 / 4

	Vehicle length / width / height -unloaded-
	mm
	4580 / 1782 / 1375
	4580 / 1782 / 1375

	Weelbase/ Turning circle
	mm / m
	2760 / 11.0
	2760 / 11.0

	Track front / rear
	mm
	1500 / 1507
	1500 / 1507

	Overhang rear
	mm
	1049
	1049

	Ground clearance -empty-
	mm
	 -
	 -

	Shoulder width front / rear
	mm
	1404 / 1318
	1404 / 1318

	Elbow width front / rear
	mm
	1459 / 1403
	1459 / 1403

	Eff. head room front / rear
	mm
	975 / 934
	975 / 934

	Luggage capacity
	ltr
	430
	430

	Fuel tank capacity 
	ltr
	63
	63

	Curb weight -acc DIN / EU
	kg
	1525 / 1600
	1540 / 1615

	Axle load ratio -unloaded DIN-
	%
	48.3
	47.9

	Gross vehicle weight / Payload
	kg
	2005 / 480
	2020 / 480

	Axle load limit front / rear
	kg
	960 / 1080
	960 / 1080

	Trailer load braked - 12% / 8% -
	kg
	1700 / 1800
	1700 / 1800

	Trailer load unbraked / roof load
	kg
	750 / 75
	750 / 75

	Engine type / cylinder / valves p cyl
	 -
	In line / 6 / 4
	In line / 6 / 4

	Fuel management
	 -
	MSD80
	MSD80

	Effective displacement
	cm3
	2979
	2979

	Stroke / bore
	mm
	89.6 / 84.0
	89.6 / 84.0

	Nom power output / Engine speed
	kW(hp)/min-1
	225 (306) / 5800
	225 (306) / 5800

	Max torque / Engine speed
	Nm/min-1
	400 / 1300
	400 / 1300

	RPM at max torque -range-
	1/min
	1300-5000
	1300-5000

	Compression ratio / Fuel type
	 -
	10.2 / ROZ 95-98
	10.2 / ROZ 95-98

	Exhaust emissions classification
	 - 
	EU4
	EU4

	Codification of reference gearbox
	 -
	ZF G
	ZF 6HP19 TÜ

	Gear ratio 1st/2nd/3rd
	 -
	4.055 / 2.396 / 1.582
	4.171/2.340/1.521

	Gear ratio 4th/5th/6th
	 -
	1.192 / 1.0 / 0.872
	1.143/0.867/0.691

	Reverse ratio
	 -
	3.677
	3.403

	Final drive ratio
	 -
	3.08
	3.46

	Steering type / ratio
	 -
	Rack-and-pinion / 16
	Rack-and-pinion / 16

	Brake front / diameter
	 - / mm
	Ventilated Disc / 348
	Ventilated Disc / 348

	Brake rear / diameter
	 - / mm
	Ventilated Disc / 336
	Ventilated Disc / 336

	Drag Cd / A / Cd x A 
	 - / m2 / m2
	0.30 / 2.08 / 0.62
	0.30 / 2.08 / 0.62

	Top Speed
	mph (km/h)
	155 (250)
	155 (250)

	Acceleration 0-62 mph (0-100 km/h) 
	s
	5.5
	5.7

	Acceleration 0-1000 metres
	s
	24.5
	24.6

	Accel 50-75 mph (80-120 km/h) 4/5 Gear
	s
	5.2 / 6.2
	 - / - 

	Weight ratio
	kg/kW 
	6.8
	6.8

	Output per litre
	kW/ltr
	75.5
	75.5

	Fuel consumption -EU urban- 
	mpg (ltr/100km)
	19.8 (14.3)
	20.5 (13.8)

	Fuel consumption -EU extra-urban-
	mpg (ltr/100km)
	42.2 (6.7)
	40.4 (7.0)

	Fuel consumption -Combined 
	mpg (ltr/100km) /km
	29.7 (9.5)
	29.7 (9.5)

	CO2-emission-EU-
	g/Km
	228
	228

	Tyre type -front-
	 -
	225/45 R17 91W RSC
	225/45 R17 91W RSC

	Tyre type -rear-
	 -
	255/40 R17 94W RSC
	255/40 R17 94W RSC

	Front rim
	 -
	8J x 17 Light alloy
	8J x 17 Light alloy

	Rear rim
	 -
	8,5J x 17 Light alloy
	8,5J x 17 Light alloy

	Radiator incl. heater / Engine oil exchange quantity
	ltr
	8.2 / 6.5
	8.5 / 6.5

	Transmission fluid -exchange quantity- 
	ltr
	Permanent filling
	Permanent filling

	Battery capacity / installed position
	Ah / -
	70 / Luggage comp.
	70 / Luggage comp.

	Alternator output rating 
	A / W
	180 / 2520
	180 / 2520

	Wheel offset
	mm
	34 / 37
	34 / 37


330d Coupé

	 
	 
	330d Coupé
	330d Coupé

	Engine platform / Transmission
	 
	M57D30O2 / M6
	M57D30O2 / A2

	No of doors / seats
	 -
	2 / 4
	2 / 4

	Vehicle length / width / height -unloaded-
	mm
	4580 / 1782 / 1395
	4580 / 1782 / 1395

	Weelbase/ Turning circle
	mm / m
	2760 / 11.0
	2760 / 11.0

	Track front / rear
	mm
	1500 / 1513
	1500 / 1513

	Overhang rear
	mm
	1049
	1049

	Ground clearance -empty-
	mm
	143
	143

	Shoulder width front / rear
	mm
	1404 / 1318
	1404 / 1318

	Elbow width front / rear
	mm
	1459 / 1403
	1459 / 1403

	Eff. head room front / rear
	mm
	975 / 934
	975 / 934

	Luggage capacity
	ltr
	440
	440

	Fuel tank capacity 
	ltr
	61
	61

	Curb weight -acc DIN / EU
	kg
	1525 / 1600
	1540 / 1615

	Axle load ratio -unloaded DIN-
	%
	47.7
	47.2

	Gross vehicle weight / Payload
	kg
	2005 / 480
	2020 / 480

	Axle load limit front / rear
	kg
	970 / 1075
	970 / 1075

	Trailer load braked - 12% / 8% -
	kg
	1800 / 1800
	1800 / 1800

	Trailer load unbraked / roof load
	kg
	750 / 75
	750 / 75

	Engine type / cylinder / valves p cyl
	 -
	In line / 6 / 4
	In line / 6 / 4

	Fuel management
	 -
	DDE6
	DDE6

	Effective displacement
	cm3
	2993
	2993

	Stroke / bore
	mm
	90.0 / 84.0
	90.0 / 84.0

	Nom power output / Engine speed
	kW(hp)/min-1
	170 (231) / 4000
	170 (231) / 4000

	Max torque / Engine speed
	Nm/min-1
	500 / 1750
	500 / 1750

	RPM at max torque -range-
	1/min
	1750-3000
	1750-3000

	Compression ratio / Fuel type
	 -
	17.0 / Diesel
	17.0 / Diesel

	Exhaust emissions classification
	 - 
	EU4
	EU4

	Codification of reference gearbox
	 -
	ZF G weit
	ZF 6HP26 TÜ

	Gear ratio 1st/2nd/3rd
	 -
	5.080 / 2.804 / 1.783
	4.171/2.340/1.521

	Gear ratio 4th/5th/6th
	 -
	1.260 / 1.0 / 0.835
	1.143/0.867/0.691

	Reverse ratio
	 -
	4.607
	3.403

	Final drive ratio
	 -
	2.35
	2.81

	Steering type / ratio
	 -
	Rack-and-pinion / 16
	Rack-and-pinion / 16

	Brake front / diameter
	 - / mm
	Ventilated Disc / 348
	Ventilated Disc / 348

	Brake rear / diameter
	 - / mm
	Ventilated Disc / 336
	Ventilated Disc / 336

	Drag Cd / A / Cd x A 
	 - / m2 / m2
	0.29 / 2.08 / 0.60
	0.29 / 2.08 / 0.60

	Top Speed
	mph (km/h)
	155 (250)
	155 (250)

	Acceleration 0-62 mph (0-100 km/h) 
	s
	6.6
	6.7

	Acceleration 0-1000 metres
	s
	26.5
	27.1

	Accel 50-75 mph (80-120 km/h) 4/5 Gear
	s
	4.9 / 6.3
	 - / - 

	Weight ratio
	kg/kW 
	9.0
	9.1

	Output per litre
	kW/ltr
	56.8
	56.8

	Fuel consumption -EU urban- 
	mpg (ltr/100km)
	31.7 (8.9)
	28.2 (10.0)

	Fuel consumption -EU extra-urban-
	mpg (ltr/100km)
	55.4 (5.1)
	47.9 (5.9)

	Fuel consumption -Combined 
	mpg (ltr/100km) /km
	43.5 (6.5)
	38.2 (7.4)

	CO2-emission-EU-
	g/Km
	174
	197

	Tyre type -front-
	 -
	225/45 R17 91W RSC
	225/45 R17 91W RSC

	Tyre type -rear-
	 -
	225/45 R17 91W RSC
	225/45 R17 91W RSC

	Front rim
	 -
	8J x 17 Light alloy
	8J x 17 Light alloy

	Rear rim
	 -
	8J x 17 Light alloy
	8J x 17 Light alloy

	Radiator incl. heater / Engine oil exchange quantity
	ltr
	7.9 / 7.5 
	8.2 / 7.5

	Transmission fluid -exchange quantity- 
	ltr
	Permanent filling
	Permanent filling

	Battery capacity / installed position
	Ah / -
	90 / Luggage comp.
	90 / Luggage comp.

	Alternator output rating 
	A / W
	150 / 2100
	150 / 2100

	Wheel offset
	mm
	34
	34


335d Coupé

	 
	 
	335d Coupé

	Engine platform / Transmission
	 
	M57D30T2 / A6

	No of doors / seats
	 -
	2 / 4

	Vehicle length / width / height -unloaded-
	mm
	4580 / 1782 / 1375

	Weelbase/ Turning circle
	mm / m
	2760 / 11.0

	Track front / rear
	mm
	1500 / 1507

	Overhang rear
	mm
	1049

	Ground clearance -empty-
	mm
	 -

	Shoulder width front / rear
	mm
	1404 / 1318

	Elbow width front / rear
	mm
	1459 / 1403

	Eff. head room front / rear
	mm
	975 / 934

	Luggage capacity
	ltr
	430

	Fuel tank capacity 
	ltr
	61

	Curb weight -acc DIN / EU
	kg
	1570 / 1645

	Axle load ratio -unloaded DIN-
	%
	47.3

	Gross vehicle weight / Payload
	kg
	2050 / 480

	Axle load limit front / rear
	kg
	990 / 1085

	Trailer load braked - 12% / 8% -
	kg
	1800 / 1800

	Trailer load unbraked / roof load
	kg
	750 / 75

	Engine type / cylinder / valves p cyl
	 -
	In line / 6 / 4

	Fuel management
	 -
	DDE6

	Effective displacement
	cm3
	2993

	Stroke / bore
	mm
	90.0 / 84.0

	Nom power output / Engine speed
	kW(hp)/min-1
	210 (286) / 4400

	Max torque / Engine speed
	Nm/min-1
	580 / 1750

	RPM at max torque -range-
	1/min
	1750-2250

	Compression ratio / Fuel type
	 -
	17.0 / Diesel

	Exhaust emissions classification
	 - 
	EU4

	Codification of reference gearbox
	 -
	ZF 6HP26 TÜ

	Gear ratio 1st/2nd/3rd
	 -
	4.171/2.340/1.521

	Gear ratio 4th/5th/6th
	 -
	1.143/0.867/0.691

	Reverse ratio
	 -
	3.403

	Final drive ratio
	 -
	2.81

	Steering type / ratio
	 -
	Rack-and-pinion / 16

	Brake front / diameter
	 - / mm
	Ventilated Disc / 348

	Brake rear / diameter
	 - / mm
	Ventilated Disc / 336

	Drag Cd / A / Cd x A 
	 - / m2 / m2
	0.30 / 2.08 / 0.62

	Top Speed
	mph (km/h)
	155 (250)

	Acceleration 0-62 mph (0-100 km/h) 
	s
	6.1

	Acceleration 0-1000 metres
	s
	25.3

	Accel 50-75 mph (80-120 km/h) 4/5 Gear
	s
	 - / - 

	Weight ratio
	kg/kW 
	7.5

	Output per litre
	kW/ltr
	70.2

	Fuel consumption -EU urban- 
	mpg (ltr/100km)
	27.4 (10.3)

	Fuel consumption -EU extra-urban-
	mpg (ltr/100km)
	47.9 (5.9)

	Fuel consumption -Combined 
	mpg (ltr/100km) /km
	37.7 (7.5)

	CO2-emission-EU-
	g/Km
	200

	Tyre type -front-
	 -
	225/45 R17 91W RSC

	Tyre type -rear-
	 -
	255/40 R17 94W RSC

	Front rim
	 -
	8J x 17 Light alloy

	Rear rim
	 -
	8.5J x 17 Light alloy

	Radiator incl. heater / Engine oil exchange quantity
	ltr
	8.2 / 7.5

	Transmission fluid -exchange quantity- 
	ltr
	Permanent filling

	Battery capacity / installed position
	Ah / -
	90 / Luggage comp.

	Alternator output rating 
	A / W
	170 / 2380

	Wheel offset
	mm
	34 / 37


Exterior and interior dimensions
BMW 3 Series Coupé





































