PAGE  
BMW
Media
Information

1/2007
Page 54

Specifications BMW 3 Series Convertible.
320i, 325i, 330i, 335i.

	Body

d

Amdahl Tureen / s
	
	320i
	325i
	330i
	335i

	No of doors/seats

l
	 
	2/4
	2/4
	2/4
	2/4

	Length/width/height (unladen) 

)
	mm
	4,580/1,782/1,384
	4,580/1,782/1,384
	4,580/1,782/1,384
	4,580/1,782/1,384

	Wheelbase
	mm
	2,760
	2,760
	2,760
	2,760

	Track, front/rear


	mm
	1,500/1,513
	1,500/1,513
	1,500/1,513
	1,500/1,507

	Turning circle
	m
	11.0
	11.0
	11.0
	11.0

	Tank capacity
	approx. ltr 
	63
	63
	63
	63

	Cooling system incl. heating

l
	ltr
	8.4 (9.1)
	8.2 (8.5)
	8.2 (8.5)
	8.2 (8.5)

	Engine oil


	ltr
	4.25
	6.5
	6.5
	6.5



	Transmission oil
	ltr
	lifetime
	lifetime
	lifetime
	lifetime

	Final drive fluid
	ltr
	lifetime
	lifetime
	lifetime
	lifetime

	Weight, unladen, to EU1
	kg
	1,670 (1,700)
	1,730 (1,760)
	1,780 (1,795)
	1,810 (1,825)

	Max load (DIN) 
	kg
	430
	430
	430
	430

	Max permissible weight (DIN) 
	kg
	2,025 (2,055)
	2,085 (2,115)
	2,135 (2,150)
	2,165 (2,180)

	Max axle load, front/rear
	kg
	915/1,150
	945/1,180
	960/1,200
	990/1,200

	Max trailer load2
	
	1,400/720
	
	
	

	braked (12%)/unbraked


	kg
	(1,500/720)
	1,600/750
	1,700/750
	1,700/750

	Max roofload/towbar download
	kg
	–
	–
	–
	–

	Luggage comp capacity (VDA)
	ltr
	210–350
	210–350
	210–350
	210–350

	Air drag 
	Cd x A
	0.28 x 2.08
	0.29 x 2.08
	0.29 x 2.08
	0.31 x 2.08

	Power Unit
	
	
	
	
	

	Config/No of cyls/valves


	 
	Straight/4/4
	Straight/6/4
	Straight/6/4
	Straight/6/4

	Engine management
	 
	MSD80
	MSD80
	MSD80
	MSD80

	Capacity
	cc
	1,995
	2,996
	2,996
	2,979

	Bore/stroke
	mm
	84.0/90
	85.0/88.0
	85.0/88.0
	84.0/89.6

	Compression ratio


	:1 
	12.0
	12.0
	12.0
	10.2

	Fuel grade
	RON
	91–98
	91–98
	91–98
	95–98

	Max output
	kW / hp
	125/170
	160/218
	200/272
	225/306

	at
	rpm
	6,700
	6,100
	6,700
	5,800

	Max torque
	Nm/lb-ft
	210/155
	270/199
	320/236
	400/295

	at
	rpm
	4,250
	2,400-4,200
	2,750-3,000
	1,300-5,000

	Electrical system
	
	
	
	
	

	Battery/location
	Ah/–
	90/lug comp
	90/lug comp
	90/lug comp
	90/lug comp

	Alternator
	A/W
	180/2,520 
	180/2,520
	180/2,520
	180/2,520

	Chassis and Suspension
	
	
	
	
	

	Suspension, front
	 
	Single-joint spring strut axle with displaced castor; small positive steering roll radius;

	
	
	compensation of transverse forces; anti-dive

	Suspension, rear
	
	Central-arm axle with longitudinal arms and double track control arms;

	
	
	anti-squat and anti-dive

	Brakes, front 
	 
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	312 x 24
	330 x 24
	348 x 30
	348 x 30

	Brakes, rear
	 
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	300 x 20
	300 x 20
	336 x 22
	336 x 22

	Driving stability systems
	 
	ABS, Automatic Stability Control (ASC+T), 
electronic brake power assistance specific to each wheel

	Steering


	 
	Rack-and-pinion power steering

 

	Steering transm ratio, overall
	: 1 
	
	
	
	

	Transmission, type
	 
	Six-speed manual gearbox (optional six-speed automatic)

	Gear ratios
I

  
	: 1 
	4.32 (4.17)
	4.32(4.17)
	4.35 (4.17)
	4.06 (4.17)

	
II
	: 1 
	2.45 (2.34)
	2.45 (2.34)
	2.49 (2.34)
	2.39 (2.34)

	 
III
	: 1 
	1.66 (1.52)
	1.66 (1.52)
	1.66 (1.52)
	1.58 (1.52)

	 
IV
	: 1 
	1.23 (1.14)
	1.23 (1.14)
	1.23 (1.14)
	1.19 (1.14)

	 
V
	: 1 
	1.00 (0.86)
	1.00 (0.86)
	1.00 (0.86)
	1.0 (0.86)

	 
VI
	: 1
	0.84 (0.69)
	0.84 (0.69)
	0.85 (0.69)
	0.87 (0.69)

	 
R
	: 1 
	3.93 (3.40)
	3.93 (3.40)
	3.92 (2.40)
	3.67 (3.40)

	Final drive
	: 1
	3.64 (4.10)
	3.15 (3.64)
	3.15 (3.73)
	3.08 (3.46)

	Tyres
	 
	225/45 R17 91V
	225/45 R17 91W
	225/45 R17 91Y
	F225/40 R17 94W


	Rims
	 
	8Jx17 light-alloy 
	8Jx17 light-alloy
	8Jx17 light-alloy
	f8/r8.5Jx17 light-alloy

	Performance
	
	
	
	
	

	Power-to-weight ratio (DIN)
	kg/kW
	12.8 (13.0)
	10.3 (10.5)
	8.5 (8.6)
	7.7 (7.8)

	Output per litre
	kW/hp
	62.7/85.3
	53.4/72.6
	66.8/90.8
	75.5/102.7

	Acceleration
0 –100 km/h km/h
	sec
	9.1 (9.8)
	7.6 (8.1)
	6.7 (6.9)
	5.8 (6.0)

	in fourth gear
80–120 km/h
	sec
	9.3
	8.3
	7.1
	5.9

	Top speed
	km/h
	228 (226)
	245 (243)
	250 (250)
	250 (250)

	Fuel Consumpt in EU Cycle
	
	
	
	
	

	Urban
	ltr/100 km
	9.0 (9.1)
	11.0 (11.4)
	11.1 (11.0)
	14.9 (14.4)

	Extra-urban
	ltr/100 km
	5.2 (5.3)
	6.1 (6.2)
	6.3 (6.7)
	7.0 (7.3)

	Composite/range in km
	ltr/100 km
	6.6/950 (6.7/940)
	7.9 /800 (8.1/780)
	8.1/775 (8.3/755)
	9.9/635 (9.9 /635)

	CO2 
	g/km
	157 (161)
	190 (195)
	194 (198)
	238

	Miscellaneous
	
	
	
	
	

	Emission grade
	
	EU4
	EU4
	EU4
	EU4


Figures in brackets apply to models with automatic transmission.


Weight of car in road trim (DIN) plus 75 kg for driver and luggage.



2May be increased under certain conditions.

3With roof compartment folded in.





4Performance and fuel consumption data on RON 98 fuel.

Specifications BMW 3 Series Convertible.
330d.

	Body

d


	
	330d
	
	
	

	No of doors/seats

l
	 
	2/4
	
	
	

	Length/width/height (unladen) 

)
	mm
	4,580/1,782/1,384
	
	
	

	Wheelbase
	mm
	2,760
	
	
	

	Track, front/rear


	mm
	1,500/1,513
	
	
	

	Turning circle
	m
	11.0
	
	
	

	Tank capacity
	approx. ltr
	61
	
	
	

	Cooling system incl. heating

l
	ltr
	7.9 (8.2)
	
	
	

	Engine oil


	ltr
	7.5
	
	
	

	Transmission oil
	ltr
	lifetime
	
	
	

	Final drive fluid
	ltr
	lifetime
	
	
	

	Weight, unladen, to EU1
	kg
	1,825 (1,840)
	
	
	

	Max load (DIN) 
	kg
	430
	
	
	

	Max permissible weight (DIN) 
	kg
	2,180 (2,195)
	
	
	

	Max axle load, front/rear
	kg
	1,005/1,200

/1150
	
	
	

	Max trailer load2
	
	
	
	
	

	braked (12%)/unbraked


	kg
	1,800/750
	
	
	

	Max roofload/towbar download
	kg
	–
	
	
	

	Luggage comp capacity (VDA)
	l
	210–3503
	
	
	

	Air drag 
	Cd x A
	0.29 x 2.08
	
	
	

	Power Unit
	
	
	
	
	

	Config/No of cyls/valves
	 
	Straight/6/4
	
	
	

	Engine management
	 
	CR system/DDE6.2.6
	
	
	

	Capacity
	cc
	2,993
	
	
	

	Bore/stroke
	mm
	84.0/90
	
	
	

	Compression ratio


	:1 
	17.0
	
	
	

	Fuel grade
	RON
	Diesel
	
	
	

	Max output
	kW/hp
	170 (231)
	
	
	

	at
	rpm
	4,000
	
	
	

	Max torque
	Nm/lb-ft
	500/369
	
	
	

	at
	rpm
	1,750–3,000
	
	
	

	Electrical system
	
	
	
	
	

	Battery/location
	Ah/–
	90/luggage comp
	
	
	

	Alternator
	A/W
	150/2,100 
	
	
	

	Chassis and Suspension
	
	
	
	
	

	Suspension, front
	 
	Single-joint spring strut axle with displaced castor; small positive steering roll radius;

	
	
	compensation of transverse forces; anti-dive

	Suspension, rear
	
	Central-arm axle with longitudinal arms and double track control arms;

	
	
	anti-squat and anti-dive

	Brakes, front 
	 
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	348 x 30
	
	
	

	Brakes, rear
	 
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	336 x 22
	
	
	

	Driving stability systems
	 
	ABS, Automatic Stability Control (ASC+T), 
electronic brake power assistance specific to each wheel

	Steering


	 
	Rack-and-pinion power steering
	 

	Steering transm ratio, overall
	: 1 
	16,0
	
	
	

	Transmission, type
	 
	Six-speed manual gearbox (optional six-speed automatic)

	Gear ratios 
I

  
	: 1 
	5.08 (4.17)
	
	
	

	
II
	: 1 
	2.80 (2.34)
	
	
	

	
III
	: 1 
	1.78 (1.52)
	
	
	

	
IV
	: 1 
	1.26 (1.14)
	
	
	

	
V
	: 1 
	1.0 (0.86)
	
	
	

	 
VI
	: 1
	0.84 (0.69)
	
	
	

	 
R
	: 1 
	4.60 (3.40)
	
	
	

	Final drive
	: 1
	2.47 (2.93)
	
	
	

	Tyres
	 
	225/45 R17 91Y
	
	
	

	Rims
	 
	8Jx17 light-alloy
	
	
	

	Performance4
	
	
	
	
	

	Power-to-weight ratio (DIN)
	kg/kW
	10.3 (10.4)
	
	
	

	Output per litre
	kW/hp
	56.8/77.2
	
	
	

	Acceleration
0–100 km/h
	sec
	7.1 (7.3)
	
	
	

	in fourth gear
80–120 km/h
	sec
	5.5
	
	
	

	Top speed
	km/h
	245 (243)
	
	
	

	Fuel Consumpt in EU Cycle4
	
	
	
	
	

	Urban
	ltr/100 km
	9.3 (10.0)
	
	
	

	Extra-urban
	ltr/100 km
	5.3 (5.9)
	
	
	

	Composite
	ltr/100 km
	6.8 (7.4)
	
	
	

	CO2 
	g/km
	181 (197)
	
	
	

	Miscellaneous
	
	
	
	
	

	Emission grade
	
	EU4
	
	
	


Figures in brackets apply to models with automatic transmission


Weight of car in road trim (DIN) plus 75 kg for driver and luggage.



2May be increased under certain conditions.

3With roof compartment folded in.





4Performance and fuel consumption data on RON 98 fuel.





































