PAGE  
BMW
Media
Information

7/2007
Page 49

Specifications. 
BMW M3.

	Body


	
	M3 Coupé
	
	
	

	No of doors/seats

l
	 
	2/4
	
	
	

	Length/width/height (unladen)

)
	mm
	4,615/1,804/1,418
	
	
	

	Wheelbase
	mm
	2,761
	
	
	

	Track, front/rear


	mm
	1,538/1,539
	
	
	

	Turning circle
	m
	11.7
	
	
	

	Fuel tank
	approx ltr
	63
	
	
	

	Cooling system incl heater  

l
	ltr
	11.4
	
	
	

	Engine oil


	ltr
	8.8
	
	
	

	Transmission fluid
	ltr
	2.1
	
	
	

	Final drive fluid
	ltr
	1.2
	
	
	

	Weight, unladen, to EU1
	kg
	1,655


	
	
	

	Max load to DIN
	kg
	500


	
	
	

	Max permissible weight to DIN
	kg
	2,080
	
	
	

	Max axle load, front/rear
	kg
	1,020/1,120
	
	
	

	Max trailer load2
	
	–
	
	
	

	braked (12%)/unbraked


	kg
	–
	
	
	

	Max roofload/trailer download
	kg
	75/– 
	
	
	

	Luggage compartment to DIN
	ltr
	430
	
	
	

	Air drag 
	Cd x A
	0.684
	
	
	

	Engine
	
	
	
	
	

	Config/No of cyls/valves 


	 
	V/8/4
	
	
	

	Engine management
	 
	MS S60
	
	
	

	Capacity
	cc
	3,999
	
	
	

	Bore/stroke
	mm
	92.0/75.2
	
	
	

	Compression ratio


	: 1
	       12.0 : 1 
	
	
	

	Fuel grade
	RON
	98 (95)
	
	
	

	Max output
	kW/hp
	309/420
	
	
	

	at
	rpm
	8,300
	
	
	

	Max torque
	Nm/lb-ft
	                               400/295
	
	
	

	at
	rpm
	3,900
	
	
	

	Electrical System
	
	
	
	
	

	Battery/installation
	Ah/–
	70/luggage compartment
	
	
	

	Alternator
	A/W
	180/2,520
	
	
	

	Chassis and Suspension
	
	
	
	
	

	Suspension, front
	 
	Aluminium two-joint spring strut axle with tiebar; 

	
	
	small, positive compensation of transverse forces; reduction of brake dive

	Suspension, rear
	
	Five-arm axle with anti-squat and anti-dive 

	Brakes, front
	 
	Single-piston swing-calliper compound disc brakes

	Diameter
	mm
	360 x 30, vented and cross-drilled in compound technology
	

	Brakes, rear
	 
	Single-piston swing-calliper compound disc brakes

	Diameter
	mm
	350 x 24, vented and cross-drilled in compound technology
	

	Driving stability systems
	 
	ABS, ASC, CBC, DSC; variable M differential lock

	Steering


	 
	Rack-and-pinion steering with hydraulic assistance and Servotronic

	Steering ratio, overall
	: 1 
	12.5
	
	

	Type of transmission
	
	SG 6
	
	
	

	Gear ratios
I

  
	: 1 
	4.055
	
	
	

	
II
	: 1 
	2.369
	
	
	

	
III
	: 1 
	1.582
	
	
	

	
IV
	: 1 
	1.192
	
	
	

	
V
	: 1 
	1.000
	
	
	

	
VI
	: 1 
	0.872
	
	
	

	
VII
	: 1
	–
	
	
	

	
R
	: 1 
	3.678
	
	
	

	Final drive ratio
	: 1 
	3.846
	
	
	

	Tyres, front/rear
	 
	245/40 ZR18 / 265/40 ZR18 

	Rims, front/rear
	 
	8.5J x 18 EH2+ IS 29 aluminium, forged/9.5J x 18 EH2+ IS 23 aluminium, forged

	Performance
	
	
	
	
	

	Power-to-weight ratio to DIN 
	kg/kW
	5.1
	
	
	

	Output per litre
	kW/hp
	77.3/105.1
	
	
	

	Acceleration
0–100 km/h
	sec
	4.8
	
	
	

	
standing-start km
	sec
	23.3 
	
	
	

	In 4th gear
80–120 km/h
	sec
	4.9 


	
	
	

	Top speed
	km/h
	2503
	
	
	

	Fuel Consumption in EU Cycle 
	
	
	
	
	

	Urban
	ltr/100 km
	17.9
	
	
	

	Extra-urban
	ltr/100 km
	9.2
	
	
	

	Composite
	ltr/100 km
	12.4
	
	
	

	CO2 
	g/km
	295
	
	
	

	Miscellaneous
	
	
	
	
	

	Emission standard
	
	EU4
	
	
	



Weight of car in road trim (DIN) plus 75 kg for driver and luggage.
2May be increased under certain conditions.



3Electronically limited.





































