

8. Technical specifications
M3 Convertible
	Body 
	
	
	
	
	

	No of doors/seats 
	 
	2/4
	
	
	

	Length/width/height (EU, unl)
	mm
	4,615/1,804/1,392
	
	
	

	Wheelbase
	mm
	2,761
	
	
	

	Track, front/rear 
	mm
	1,540/1,539
	
	
	

	Turning circle
	m
	11.7
	
	
	

	Tank capacity 
	approx ltr
	63
	
	
	

	Cooling system incl heater
	ltr
	11.4
	
	
	

	Engine oil
	ltr
	8.8
	
	
	

	Transmission fluid
	ltr
	2.5
	
	
	

	Final drive fluid     
	ltr
	1.2
	
	
	

	Weight, unladen, to DIN1)
	kg
	1,810
	
	
	

	Weight, unladen, to EU2)
	kg
	1,885
	
	
	

	Max load to DIN
	kg
	470
	
	
	

	Max permissible to DIN
	kg
	2,280
	
	
	

	Max axle load, front/rear    
	kg
	–/–
	
	
	

	Max trailer load1)
	
	
	
	
	

	Braked (12%)/unbraked 
	kg
	–
	
	
	

	Max roof load/max trailr download
	kg
	75/– 
	
	
	

	Luggage compartment to DIN
	ltr
	210–350
	
	
	

	Air drag       
	Cd x A
	0.688
	
	
	

	Power Unit
	
	
	
	
	

	Config/No of cyls/valves 
	 
	V/8/4
	
	
	

	Engine management
	 
	MS S60
	
	
	

	Capacity
	cc
	3,999
	
	
	

	Bore/stroke   
	mm
	92.0/75.2
	
	
	

	Compression ratio
	:1
	       12.0:1 
	
	
	

	Fuel grade
	RON
	98 (95)
	
	
	

	Max output @ 8300rpm
	kW/hp
	309/420
	
	
	

	Max torque @ 3900rpm
	Nm/lb-ft
	400/295
	
	
	

	Electrical System
	
	
	
	
	

	Battery/installation 
	Ah/–
	90/luggage comp
	
	
	

	Alternator
	A/W
	180/2,520
	
	
	

	Chassis and Suspension
	
	
	
	
	

	Suspension, front  
	 
	   Aluminium two-joint spring strut axle with tiebar; small, positive steering roll radius;     

	
	
	   Compensation lateral forces; anti-dive

	Suspension, rear  
	
	   Five-arm axle with anti-squat

	
	
	   and anti-dive

	Brakes, front
	 
	   Single-piston swing-calliper compound disc brakes

	Diameter   
	mm
	   360 x 30, vented and cross-drilled
	
	
	

	Brakes, rear   
	 
	   Single-piston swing-calliper compound disc brakes

	Diameter    
	mm
	   350 x 24, vented and cross-drilled
	
	
	

	Driving stability systems
	 
	   ABS, ASC, CBC, DSC; Variable M Differential Lock 

	Steering 
	 
	   Rack-and-pinion steering with hydraulic assistance and Servotronic
	

	Steering trans ratio:       
	:1 
	12.5
	
	
	

	Type of gearbox
	
	SG 6
	
	
	

	Gear ratios
I
	:1 
	4.055
	
	
	

	
II
	:1 
	2.369
	
	
	

	
III
	:1 
	1.582
	
	
	

	
IV
	:1 
	1.192
	
	
	

	
V
	:1 
	1.000
	
	
	

	
VI
	:1 
	0.872
	
	
	

	
VII
	:1
	–
	
	
	

	
R
	:1 
	3.678
	
	
	

	Final drive        
	:1 
	3.846
	
	
	

	Tyres, front/rear
	 
	   245/40 ZR18/265/40 ZR18

	Rims, front/rear
	 
	   8.5J x 18 EH2+ IS 29 cast aluminium/9.5J x 18 EH2+ IS 23 cast aluminium

	Performance    
	
	
	
	
	

	Power-to-weight ratio to DIN
	kg/kW
	5.9
	
	
	

	Power-to-weight ratio to DIN
	kg/hp
	4.3
	
	
	

	Output per litre
	kW/ltr
	77.3
	
	
	

	Output per litre
	hp/ltr
	105.0
	
	
	

	Acceleration 0–62mph
	sec
	5.3
	
	
	

	Top speed            
	Mph
	1554)
	
	
	

	Fuel Consumption in EU Cycle
	
	
	
	
	

	Urban   
	mpg
	15.1
	
	
	

	Extra-urban
	mpg
	29.4
	
	
	

	Composite 
	mpg
	21.9
	
	
	

	CO2, overall   
	g/km
	309
	
	
	

	Miscellaneous
	
	
	
	
	

	Emission rating     
	
	EU4
	
	
	




) Weight of vehicle in road trim (DIN)
2) Weight of vehicle in road trim (DIN) plus 75 kg for driver and luggage

3) May be increased under certain conditions              
4) electronically limited   





































