9.  Technical data

Technical Data BMW 3 Series Saloon, Touring
318i Saloon, 318i Touring
	Body

fsdfd

d

Anzahl Türen / s
	
	318i Saloon
	318i Touring
	
	

	No. of doors/seats
	
	4/5
	5/5
	
	

	Length/Width/height (unladen)
	mm
	4,471/1,739/1,415
	4,478/1,739/1,409
	
	

	Wheelbase
	mm
	2,725
	2,725
	
	

	Track, front/rear
	mm
	1,471/1,483
	1,471/1,483
	
	

	Turning circle
	m
	10.5
	10.5
	
	

	Fuel tank capacity
	approx. l
	63
	63
	
	

	Cooling system incl. heating

l
	l
	7.0
	7.0
	
	

	Engine oil
	l
	4.25
	4.25
	
	

	Transmission fluid
	l
	Permanent filling
	Permanent filling
	
	

	Final drive fluid
	l
	Permanent filling
	Permanent filling
	
	

	Weight, unladen (EU)1 
	kg
	1,395 (1,425)
	1,470 (1,485)
	
	

	Max. load (DIN)
	kg
	500
	540
	
	

	Max. permissible weight (DIN)
	kg
	1,820 (1,850)
	1,935 (1,950)
	
	

	Max. permissible axle load  fffffronfront/rear
	kg
	890/1,005
	875/1,100
	
	

	Max. trailer load2 front/rear??nb
	
	 
	
	
	

	braked (12%)/unbraked
	kg
	1,400 (1,500)/680
	1,400 (1,500)/680
	
	

	Max. roof load/trailer nose weight
	kg
	75/75
	75/75
	
	

	Luggage comp. capacity VDA
	l
	440
	435–1345
	
	

	Drag coefficient
	cX x A
	0.30 x 2.06
	0.32 x 2.06
	
	

	Engine
	
	
	
	
	

	Layout/No. of cylinders/Valves


	
	Inline/4/4
	Inline/4/4
	
	

	Engine management
	
	Bosch ME9/BMW
	Bosch ME9/BMW
	
	

	Displacement, effective
	cm³
	1,995
	1,995
	
	

	Bore/Stroke
	mm
	84.0/90.0
	84.0/90.0
	
	

	Compression ratio


	:1
	10.2 
	10.2 
	
	

	Fuel grade3
	ROZ
	91–98 3
	91–98 3
	
	

	Max. output
	kW/bhp
	105/143
	105/143
	
	

	At engine speed
	min-1
	6,000
	6,000
	
	

	Torque
	Nm/1b ft  
	200/147
	200/147
	
	

	At engine speed
	min-1
	3,750
	3,750
	
	

	Electrics
	
	
	
	
	

	Battery/Location
	Ah/–
	55/front
	55/front
	
	

	Alternator
	A/W
	120/1,680 
	120/1,680
	
	

	Chassis
	
	
	
	
	

	Front suspension
	
	Single-joint MacPherson strut suspension with displaced camber; small

	
	
	positive steering scrub radius; traverse force compensation; anti-dive

	Rear suspension
	
	Central-arm axle with longitudinal control arm and double track control

	
	
	Anti-squat, anti-dive

	Brakes, front
	
	Single-piston floating-caliper disc brakes

	Diameter
	mm
	286, ventilated
	
	286, ventilated
	

	Brakes, rear
	
	Single-piston floating-caliper disc brakes

	Diameter
	mm
	276,ventilated
	
	276, ventilated
	[image: image1.png]1483

758

988

094623

AuBen- und InnenmaRe





	Driving Stability Systems
	
	ABS, Traction Control (ASC + T), wheel-selective Electronic Braking Distribution (EBV, CBC), Dynamic Stability Control (DSC), Electronic Differential Lock (ADB), Dynamic Brake Control (DBC)
	

	Steering
	
	Rack-and-pinion, power-assisted; 3.0 revolutions from one limit stop to the other
	

	Overall ratio
	:1
	13.7
	
	13.7
	

	Type of transmission
	
	5-gear manual (5-gear-Steptronic automatic transmission)

	Transmission ratio     I

  
	:1
	4.23 (3.42)
	4.23 (3.42)
	
	

	                                  II
	:1
	2.52 (2.22)
	2.52 (2.22)
	
	

	                                  III
	:1
	1.66 (1.60)
	2.52 (2.22)
	
	

	                                  IV
	:1
	1.22 (1.00)
	1.22 (1.00)
	
	

	                                  V
	:1
	1.00 (0.75)
	1.00 (0.75)
	
	

	                                  R
	:1
	4.04 (3.03)
	4.04 (3.03)
	
	

	Final drive ratio
	:1
	3.38 (3.45)
	3.38 (3.45)
	
	

	Tyres 
	
	205/55 R 16 91 V
	205/55 R 16 91 V
	
	

	Wheels 
	
	7,0 J x 16 Steel
	7,0 J x 16 Alloy
	
	


…. / more

	Performance

fsdfd

d

Anzahl Türen / s
	
	318i Saloon
	318i Touring
	
	

	Power to weight ratio (DIN)
	kg/kW
	12.6 (12.9)
	13.3 (13.4)
	
	

	Acceleration  0–62mph
	s
	9.3 (10.2)
	9.6 (10.5)
	
	

	50 –75mph in 4th gear
	s
	9.2 (–)
	9.8 (–)
	
	

	Top speed
	mph
	135
	132
	
	

	Fuel consumption (EU cycle)
	
	
	
	
	

	In town
	mpg
	27.7
	27.2
	
	

	Out of town
	mpg
	51.4
	49.6
	
	

	Overall
	mpg
	39.2
	38.2
	
	

	CO2 (Auto)
	g/km
	175 (196)
	179 (199)
	
	

	Miscellaneous
	
	
	
	
	

	Emission classification
	
	EU3
	EU3
	
	


The values given in parenthesis apply to the automatic transmission.


 Weight, unladen, including 75 kg for driver and luggage.


2 Deviations are possible under certain conditions.


3 Information about performance relates to ROZ 98; information about fuel consumption relates to ROZ 95.

4 The value of 300 litres applies when the top is raised.

Technical Data BMW 3 Series Saloon, Touring

320d Saloon, 320d Touring.
	Body

Fsdfd

d

Anzahl Türen / s
	
	320d Saloon
	320d Touring
	
	

	No. of doors/seats

l
	
	4/5
	5/5
	
	

	Length/width/height (unladen) 

)
	mm
	4,471/1,739/1,415
	4,478/1,739/1,409
	
	

	Wheelbase
	mm
	2,725
	2,725
	
	

	Track, front/rear

wgsdfg


	mm
	1,471/1,483
	1,481/1,493
	
	

	Turning circle
	m
	10.5
	10.5
	
	

	Fuel tank capacity
	approx.
	63
	63
	
	

	Cooling system incl. heating 

l
	 l
	9.3
	9.3
	
	

	Engine oil

Getriebeöl


	l
	5.5
	5.5
	
	

	Transmission fluid
	l
	Permanent filling
	Permanent filling
	
	

	Final drive fluid
	l
	Permanent filling
	Permanent filling
	
	

	Weight, unladen (EU)1 
	kg
	1,490 (1,505)
	1,580 (1,600)
	
	

	Max. load (DIN)
	kg
	500
	500
	
	

	Max. permissible weight (DIN)
	kg
	1,915 (1,930)
	2,005 (2,025)
	
	

	Permissible axle load front/rear
	kg
	915/1,060
	920/1,130
	
	

	Max. trailer load2 
	
	
	
	
	

	braked (12%)/unbraked


	kg
	1,600/720
	1,600/720
	
	

	Max. roof load/trailer nose weight
	kg
	75/75
	75/75
	
	

	Luggage comp. capacity VDA
	l
	440
	435–1345
	
	

	Drag coefficient
	cX x A
	0.31 x 2.06
	0.32 x 2.06
	
	

	Engine
	
	
	
	
	

	Layout/No. of cylinders/Valves Zylinder/Ventile


	
	Inline/4/4
	Inline/4/4
	
	

	Engine management
	
	BOSCH DDE 5.0
	BOSCH DDE 5.0
	
	

	Displacement
	cm³
	1,995
	1,995
	
	

	Bore/Stroke
	mm
	84.0/90.0
	84.0/90.0
	
	

	Compression ratio


	:1
	17.0 
	17.0
	
	

	Fuel grade
	
	Diesel
	Diesel
	
	

	Max. output
	kW/bhp
	110/150
	110/150
	
	

	At engine speed
	min-1
	4,000
	4,000
	
	

	Torque
	Nm/1b ft
	330/243
	33/243
	
	

	At engine speed
	min-1
	2,000
	2,000
	
	

	Electrics
	
	
	
	
	

	Battery/location
	Ah/–
	80/boot
	80/boot
	
	

	Alternator
	A/W
	150/2,100
	150/2,100
	
	

	Chassis
	
	
	
	
	

	Front suspension
	
	Single-joint MacPherson strut unit with displaced camber; small

	
	
	positive steering scrub radius; traverse force compensation; anti-dive

Bremsnickreduzierung

	Rear suspension
	
	Central-arm axle with longitudinal control arm and double track control arm

	
	
	Anti-squat, anti-dive

	Brakes, front
	
	Single-piston floating-caliper disc brakes

	Diameter
	mm
	300, ventilated
	
	300 ventilated 
	

	Brakes, rear
	
	Single-piston floating-caliper disc brakes

	Diameter
	mm
	294, ventilated
	
	             294, ventilated
	

	Driving Stability Systems
	
	ABS, Traction Control (ASC + T), wheel-selective Electronic Braking Distribution (EBV, CBC), Dynamic Stability Control (DSC), Electronic Differential Lock (ADB), Dynamic Brake Control (DBC)
	

	Steering


	
	Rack-and- pinion, power-assisted; 3.0 revolutions from one limit stop to the other
	

	Overall ratio
	:1
	13.7
	
	
	13.7

	Type of transmission
	
	5-gear manual (5-gear Steptronic automatic transmission)

	Transmission ratio      I

  
	:1
	5.24 (3.42)
	5.24 (3.42)
	
	

	                                  II
	:1
	2.92 (2.22)
	2.92 (2.22)
	
	

	                                  III
	:1
	1.82 (1.60)
	1.82 (1.60)
	
	

	                                  IV
	:1
	1.27 (1.00)
	1.27 (1.00)
	
	

	                                  V
	:1
	1.00 (0.75)
	1.00 (0.75)
	
	

	                                  R
	:1
	4.72 (3.03)
	4.72 (3.03)
	
	

	Final drive ratio
	:1
	2.35 (3.07)
	2.35 (3.07)
	
	

	Tyres 
	
	205/55 R 16 91 V
	205/55 R 16 91 V
	
	

	Wheels 
	
	7 J x 16 Steel 
	7 J x 16 Alloy 
	
	

	Performance
	
	
	
	
	

	Acceleration  0–62mph
	s
	8.9 (9.1)
	9.3 (9.5)
	
	

	50–75 mph in 4th gear
	s
	7.5 (–)
	7.9 (–)
	
	

	Top speed
	mph
	134
	131
	
	


…. / more

	Fuel consumption (EU cycle
Fsdfd

d

Anzahl Türen / s
	
	320d Saloon
	320d Touring
	
	

	In town
	mpg
	37.2
	36.2
	
	

	Out of town
	mpg
	64.2
	62.8
	
	

	Overall
	mpg
	51.4
	49.6
	
	

	CO2 
	g/km
	148 (185)
	153 (190)
	
	

	
	
	
	
	
	

	Miscellaneous
	
	
	
	
	

	Emission classification
	
	EU3
	EU3
	
	


The values given in parenthesis apply to the automatic transmission.


 Weight, unladen, including 75 kg for driver and luggage.

2 Deviations are possible under certain conditions.

[image: image2.png]Gcov60 ogewusuu| pun -usgny

A

995 ———

)
(<]
] T )l
-~ 1483 ——»
1932
=
AL NN\

A ‘ ,
< - __ Lo |
gl |
| ﬁ@w (m
< 2 n_ 8% F\m j @
NS 28¢k
: D
s ——
f | /
' F Y \




Exterior and interior dimensions 

[image: image3.png]320 -

300

280

260

240

220

200 -

- 180

160 +

140

Torque [Nm]
o
-

weds
o
O .

110,0

100,0

- 265 Nm @ 1750min-1

/

/

85 kW @ 4000min-1

I

90,0

[

\

80,0

70,0

60,0

50,0

40,0

30,0

20,0

o
o
Output [kW]

0 1000 12000

Engine speed [min-1]

3000

4000

o
o

5000



BMW 3 Series Saloon

[image: image4.png]Torgue [Nm]

240

120,0
| 105 kW @ 6000min-1
200 Nm @ 3750min-1 /' \
200 — , 100,0
160 / \\ 80,0
120 60,0
80 40,0
40 20,0
23
=X,
5
Q
5
0 _ 00 3
0 4000 5000 6000 7000 |

Engine speed [min-1]



BMW 3 Series Touring

BMW 2.0-litre diesel engine

85 kW/265 Nm

BMW 2.0-litre petrol engine 

105 kW/200 Nm






















