[image: image1.png]ool

800

750

700

650

600

550

500

450

400

350

300

250

200

150

100

50

54U

320

300

280

260

\ 240

\ 200

\ 180

160

140

120

100

80

60

40

20

1000 2000 2000 4000 =000 &000 7000 2000





The new BMW 5 Series Saloon
Contents

1.
Introduction
3

2.
Market
6

3.
Drivetrain
9

4.
Chassis and Safety
14
5.
Design
19
6.
History
23
7.
Q & A
27
8.
Standard and Optional Equipment

(see separate data table)
29
9.
Technical data
30
10.
Torque Curves
36

Interior and Exterior Dimensions
43



1. Introduction
As one of the most successful cars in history with nearly 5 million models sold during a near 40-year lifetime, the BMW 5 Series Saloon has defined the executive saloon sector, offering handling and performance combined with elegance and luxury.

Now in its sixth iteration, the elements that make up those benchmark standards have been enhanced.  The near perfect 50:50 weight distribution inherent in all BMWs is still to the fore, while an advanced rear-wheel-drive chassis and developments of class-leading and award-winning engines, now offer higher power and increased efficiency.  The latest in innovative driver assistance and safety technology completes the package.

Setting new standards in the sector, the new BMW 5 Series is the first to feature Auto Start-Stop, another step in BMW’s award-winning EfficientDynamics programme, an eight-speed automatic transmission and a double-wishbone front suspension.

With its long wheelbase and short overhangs, long bonnet, dynamic lines, athletic body and the coupé-like, sleek roofline, the new model is distinctively a BMW 5 Series - designed and built to ensure the sixth generation retains the legacy created by the first five.
	Model
	Price 
OTR
	Power
Hp
	Torque   Nm
	Zero – 62mph Seconds
0 - 62
	Top Speed Mph
	Combined 
Mpg
	CO2 Emissions g/km



	BMW 520d SE Saloon
	£28,165
	184
	380
	8.1
	141 (140)
	56.5 (54.3)**
	132 (137)**

	BMW 525d SE Saloon
	£33,095
	204
	450
	7.2
	147
	45.6 (46.3)
	162 (160)

	BMW 530d SE Saloon
	£37,100
	245
	540
	6.3
	155*
	44.8 (45.6)
	166 (160)

	BMW 523i SE Saloon
	£31,235
	204
	270
	7.9
	148 (145)
	37.2
	177 (178)

	BMW 528i SE Saloon
	£32,915
	258
	310
	6.6
	155*
	36.2 (37.2)
	182 (178)

	BMW 535i SE Saloon
	£37,090
	306
	400
	6.0
	155*
	33.2 (33.6)
	199 (195)

	BMW 550i SE Saloon
	£50,520
	407
	600
	5.0
	155*
	27.2
	243


*Electronically-limited. **Figures subject to confirmation. (Figures in brackets denote automatic transmission if value different).
Drivetrain
Customers will be able to choose from three diesel powerplants and four petrol engines from launch, which utilise a range of innovative technologies including single and dual turbocharging and High Precision Direct Injection, as well as award-winning EfficientDynamics technologies.
For drivers looking for unparalleled economy the new 184hp BMW 520d delivers class-leading emission levels of 132g/km of CO2 as well as a combined fuel consumption of 56.5mpg.  It achieves this through a combination of lightweight engineering (aluminium crankcase), the latest generation of common-rail direct fuel injection and a turbocharger with variable turbine geometry, mated to its four-cylinder engine.

The new BMW 525d offers an equally impressive combination of performance and parsimony.  Powered by a 3.0-litre all-aluminium engine, it develops maximum output of 204hp yet still records a combined 45.6mpg and 162g/km CO2 emissions to set a new benchmark as the most efficient six-cylinder diesel model on the market.  The new BMW 530d shares the same engine with an output of 245hp but returns an impressive 44.8mpg on the combined cycle, with CO2 emissions of only 166g/km.
Both the BMW 525d Saloon and the 530d Saloon equipped with an automatic transmission achieve the crucial 160g/km CO2 emissions target to benefit from taxation incentives. 
Other highlights of the engine range include the new BMW 550i, which features BMW’s award-winning twin-turbocharged 4.4-litre V8 delivering 407hp.  The 3.0-litre engine in the BMW 535i is the first to combine single turbocharger technology, High Precision Direct Injection, and variable VALVETRONIC valve management. 
The naturally aspirated six-cylinder engined BMW 528i and 523i, have a version of High Precision Direct Injection with a greater focus on efficiency.  In the new 258hp BMW 528i this unit records a combined 36.2mpg and CO2 emissions of 182g/km.  Using the same technology the 523i achieves 37.2mpg on the combined cycle and CO2 emissions of 177g/km.
All models except the 550i come with six-speed manual gearbox as standard but all can be specified with the new eight-speed automatic gearbox, including the 520d, making it the first four-cylinder car in the world available with an eight-speed transmission.
EfficientDynamics
The new BMW 5 Series Saloon comes with an array of EfficientDynamics technologies. Innovations such as Optimum gearshift indicator, Brake Energy Regeneration and Active Aerodynamics all feature.  But it is the introduction of Auto Start-Stop on a manual transmission BMW 520d Saloon - a first for the sector - that provides the latest talking point. 
Chassis
The wheelbase of the new BMW 5 Series is now the longest in class, at 
2,968 millimetres, while the new model is 58mm longer than the outgoing model to provide greater interior comfort.
The new double wishbone front axle and a multilink rear axle ensure that occupants are provided with a composed ride whilst still maintaining supreme driving dynamics.  Models specified with Variable Damper Control or Adaptive Drive packages have the Drive Dynamic Control (DDC) chassis configurator to enhance ride and handling still further. DDC allows for fine tuning of the chassis with the choice of Normal, Comfort, Sport and Sport+. 

Market
It is expected that approximately 12,000 BMW 5 Series Saloons will be sold in its first full year with the fleet:retail split likely to be 70:30 and petrol:diesel share approximately 10:90.  The BMW 520d Saloon is expected to be the highest selling model with around 60 per cent of the total volume.

Enhanced standard and optional equipment
The new BMW 5 Series Saloon not only offers the driver more value, but improves on its previous incarnation’s residual values proposition too.  Standard specification now includes Dakota leather upholstery, Bluetooth telephone preparation, automatic air conditioning with two-zone control, front and rear Park Distance Control and 7-inch control display and iDrive.
A raft of intelligent new technology, including Park Assist automatic parking, Integral Active Steering, Head-up Display and Night Vision with pedestrian recognition can also be specified.
Key facts
· Sixth generation of class-leading BMW 5 Series Saloon.
· Choice of four petrol engines and three diesel engines.
· Eight-speed automatic gearbox available on all models.
· Auto Start-Stop technology debuts on biggest selling BMW 520d Saloon.
· Highest ever level of standard specification including Dakota leather 
upholstery on all models.
· On sale 20 March, 2010.

· More information is available on www.bmw-web.tv,  
www.press.bmwgroup.co.uk or www.thenewbmw5series.co.uk 
2. Market
No stone has been left unturned to ensure that the new BMW 5 Series Saloon is as competitive as it can possibly be in every facet of the sales and ownership experience, from pricing and specification to running costs and residual values.

Sean Green, Sales Director for BMW UK, said:  “We believe the new BMW 5 Series offers customers more than the competition in every area of the purchasing and ownership experience.  We have specified and priced the car extremely competitively - it’s not our strategy to introduce low-spec introductory models in order to gain what looks initially like an advantageous price point position. 

“Buyers are now more aware of the wholelife costs of vehicles, and it is this we have concentrated on.  Thanks to our excellent CO2 and fuel consumption performance, due to EfficientDynamics, the new BMW 5 Series offers lower fuel costs, VED and company car tax than most of the competition.

“We expect to sell around 12,000 BMW 5 Series Saloons in its first full year, and such is the strength of the proposition, we have some ambitious targets for conquest sales: we envisage approximately 35-40 per cent of customers coming from competitors.”
Key historical data

	Model
	2004
	2005
	2006
	2007
	2008
	2009

	BMW 5 Series Saloon
	14,227
	14,199
	16,015
	16,201
	13,967
	10,158



Market data
The 5 Series is a key car for the business and private buyer alike but the predicted fleet:retail split is 70:30 in favour corporate.  Due to fuel prices and taxation policies in the UK, diesel ownership is popular in the UK and this is reflected in the split over engine choice.  The petrol:diesel share will be approximately 10:90 in the favour of the latter with the 520d expected to be the highest selling model accounting for approximately 60 per cent of the total volume.
Deliveries will start from 20 March 2010 for six- and eight cylinder models, with 520d models coming in July due to the production ramp-up programme at the factory in Dingolfing, Germany.  While it would be logical for the UK to request that the entry-level diesel come to market first the biggest market for the 5 Series is the petrol-loving USA.
Prices start from £28,165OTR for the BMW 520d SE manual and all models come with a very high level of standard specification – among the most generous in the sector.  This includes Dakota leather upholstery, BMW Professional Radio, Bluetooth telephone preparation, Dynamic Stability Control+, Cruise Control, automatic air conditioning with two-zone control, front and rear Park Distance Control, 7-inch control display and iDrive and partial Black Panel technology.

Six-cylinder engine derivatives additionally gain USB audio interface and Ambient lighting, while the BMW 550i has the eight-speed automatic transmission as standard.
To illustrate just how competitive the new car is in terms of pricing and specification compared to the outgoing model the table below illustrates both.  The figures are even more astonishing when you take into account that the new car comes with circa £2,600 (depending on model) of standard equipment that previously would have been on the options list.
	BMW 5 Series Saloon (E60)
	Retail price 
	BMW 5 Series Saloon (New)
	Retail price
	Variance
	Approx content value

	BMW 520d SE 
	£27,150
	BMW 520d SE 
	£27,280
	+£130
	+£2,280

	BMW 525d SE 
	£30,950
	BMW 525d SE 
	£32,180
	+£1,230
	+£2,600

	BMW 530d SE 
	£34,930
	BMW 530d SE 
	£36,160
	+£1,230
	+£2,600

	BMW 523i SE 
	£28,760
	BMW 523i SE 
	£30,295
	+£1,535
	+£2,600

	BMW 525i SE 
	£30,440
	BMW 528i SE 
	£31,975
	+£1,535
	+£2,600

	BMW 530i SE 
	£34,575
	BMW 535i SE 
	£36,110
	+£1,535
	+£2,600

	BMW 550i SE 
	£45,790
	BMW 550i SE (auto only)
	£49,350
	+£3,560*
	+£2,790


*New BMW 550i Saloon comes as standard with automatic transmission.

For example, Dakota leather is standard on the new BMW 5 Series when it was an option on some models in the outgoing range, but ensuring every model was highly specified means that the customer would reap the benefits at resale time.

Martin Ward, manufacturer relationship manager at industry analyst CAP Motor Research, explains:  “BMW has been wise to include leather trim and Bluetooth as standard on all new 5 Series.  The way the used market works is that values are calculated from the lowest price achieved, which means most people would have lost out in money terms if the standard model came with cloth trim, as that would set the base price for values.  With leather as standard, that base price will be higher.
“Once on the used market, all used buyers will come to recognise that all new BMW 5 Series will have the added luxury of leather seats, and this will no doubt add to its desirability, saleability and add value to the car.”
The new BMW 5 Series is predicted to perform extremely strongly on the used market according to independent figures – far outperforming its competitors.  And thanks to its exceptional fuel economy and low CO2 levels, in-life ownership costs will be extremely competitive too.

	Model
	Price
 OTR 
	Fuel cost over 60,000 miles
	CAP RV (three years / 60,000 miles)

	BMW 520d SE auto
	£29,660
	£5,224
	£12,500 / 42 per cent

	Audi A6 2.0TDI 170 SE Multitronic
	£29,605
	£9,775
	£9,775 / 33 per cent

	Mercedes-Benz E220 CDi BlueEfficiency SE Tiptronic
	£30,130
	£5,189
	£11,850 / 40 per cent



Stephen Chater, Corporate Operations Manager at BMW UK, said:  “We have a close working relationship with our corporate customers and so can ensure that this vital business car is positioned exactly where customers want it.  That means getting the price, specification and wholelife cost proposition spot-on so that company car drivers and businesses have the strongest possible financial reasons for choosing the new BMW 5 Series, as well as the more emotive attractions of its agility, elegance, performance and high quality.”

In the UK the BMW 5 Series Saloon will be offered with four optional equipment packages:

BMW Business Advanced Media and BMW Professional Multimedia packages include BMW Navigation, BMW ConnectedDrive – Assist and Online and Voice control.  Dynamic package includes 19-inch alloy wheels, Sport seats, Anthracite headlining, Sport leather steering wheel and High-gloss exterior trim.  The Visibility package includes Adaptive headlights, Xenon headlights, High-beam Assist and headlight wash.
All four packages represent a significant saving for the customer compared to buying the options individually.  
3. Drivetrain
Buyers of the new BMW 5 Series Saloon will be able to choose from a range of three diesel powerplants and four petrol engines from launch.

Utilising a range of innovative technologies including single and dual turbocharging and High Precision Direct Injection, as well as award-winning EfficientDynamics innovations that enhance fuel economy and lower emissions, the engines in the new BMW 5 Series build on the company’s tradition of producing extremely powerful, frugal powerplants.
BMW 520d*
The UK executive sector is dominated by diesel, often to minimise company car tax or the cost of fuel due to high mileages.  In the new BMW 5 Series, three new-generation diesels will initially be offered which set the benchmark for emissions in the sector, without any compromise in performance.
Low CO2 emissions are vital for many customers in the executive sector and the new BMW 520d delivers unrivalled emission levels of 132g/km (137g/km for automatic transmission-equipped cars) as well as combined fuel consumption of 56.5mpg (54.3mpg).  Setting new records for fuel economy and emissions, the four-cylinder 1,995cc BMW 520d achieves this through a combination of composite aluminium crankcase, the latest generation of common-rail direct fuel injection and a turbocharger with variable turbine geometry.

To ensure an optimum flow of power at all speeds and under all loads, an electric adjuster sets the turbine blades in the turbocharger to the current load status of the engine in fractions of a second.  This works alongside a high pressure fuel injection system with extremely accurate magnetic valve injectors which deliver exactly the smallest amount of fuel required in any situation.

Such latest-generation technology not only delivers the optimum fuel/air mixture, but allows the engine to produce unusually high levels of power and torque when needed: 184hp at 4,000rpm, 380Nm from 1,900 to 2,750rpm.  This accelerates the new BMW 520d to 62mph in only 8.1 seconds and gives the car a top speed of 141mph (140mph). 

To enhance the efficiency of the 520d’s drivetrain, manual versions come with an Auto Start-Stop function – a first for the sector – which cuts down the amount of time idling when stopped at road junctions or in traffic congestion.

As soon as the driver moves the gearshift lever to neutral and takes his foot off the clutch pedal, the engine switches off automatically.  Then, once the driver is able to go on, all he has to do is press the clutch to start the engine.
	Model
	Power
Hp
	Torque
Nm
	Zero – 62mph seconds
	Top speed mph
	Combined mpg
	VED band
	CO2
g/km
	BIK
%

	BMW 520d Saloon
	184
	380
	8.1
	141
	56.5
	E
	132
	15

	Mercedes-Benz E220 CDI Saloon
	170
	400
	8.7
	143
	53.3
	E
	139
	16

	Audi A6 2.0 TDI Saloon
	170
	350
	8.9
	140
	49.6
	F
	149
	17


*All figures on BMW 520d Saloon are subject to confirmation. 

BMW 525d
The new BMW 525d offers an equally impressive combination of performance and parsimony.  Powered by a 2,993cc all-aluminium engine and injectors operating at up to 1,600 bar, it develops maximum output of 204hp at 4,000rpm with peak torque of 450Nm from 1,750rpm to 2,500rpm.  This helps it achieve a zero to 62mph time of 7.2 seconds before going on to a 147mph top speed.
Its 45.6mpg (46.3mpg) combined fuel efficiency figure and 162g/km (160g/km) CO2 emissions rating has set a new benchmark as the most efficient six-cylinder diesel model on the market.  Fundamentally the BMW 525d quipped with an automatic transmission achieves the crucial 160g/km threshold to attain taxation benefits. 
	Model
	Power
Hp
	Torque
Nm
	Zero – 62mph seconds
	Top speed mph
	Combined mpg
	VED band
	CO2
g/km
	BIK
%

	BMW 525d Saloon
	204
	450
	7.2
	147
	45.6
	G
	162
	20

	Mercedes-Benz E250 CDI Saloon
	204
	500
	7.7
	149
	53.3
	E
	139
	15

	Audi A6 2.7 TDI Saloon
	190
	400
	7.9
	144
	45.6
	G
	164
	20


BMW 530d
The straight-six 3.0-litre all-aluminium powerplant in the BMW 530d now delivers even more torque and power, as well as being smoother and more efficient.  With peak power of 245hp at 4,000rpm, and peak torque of 540Nm from 1,750rpm to 3,000 rpm, the BMW 530d benefits from a lightweight all-aluminium crankcase, an enhanced turbocharger system with variable turbine geometry, and the latest generation of common-rail fuel injection through upgraded piezo injectors. 

Operating at a maximum pressure of 1,800 bar and so reducing emissions through a smoother and more consistent combustion process than ever before, the BMW 530d 
returns an impressive 44.8mpg (45.6mpg) on the combined cycle, with CO2 emissions of only 166g/km (160g/km).  Like the 525d before it, the 530d also qualifies for inclusion in the 160g/km taxation club when equipped with an automatic transmission. 
	Model
	Power
Hp
	Torque
Nm
	Zero – 62mph seconds
	Top speed mph
	Combined mpg
	VED band
	CO2
g/km
	BIK
%

	BMW 530d Saloon
	245
	540
	6.3
	155*
	44.8
	H
	166
	21

	Mercedes-Benz E350 CDI Saloon
	231
	540
	6.8
	155*
	40.9
	J
	186
	25

	Audi A6 3.0 TDI Saloon
	240
	500
	6.6
	155*
	42.2
	I
	179
	23


BMW 550i
Sitting at the top of the range is the new BMW 550i, which features a twin-turbocharged engine with High Precision Direct Injection alongside a 4.4-litre V8 delivering a peak of 407hp between 5,500rpm and 6,400 rpm.

The most efficient engine of its kind, its turbochargers and catalytic converters are uniquely and conveniently housed in the vee between the two rows of cylinders, meaning that the particularly short distances between ancillaries and cylinders produce an especially direct response.

The all-aluminium unit gives the 550i the ability to accelerate in an astonishingly smooth and refined manner from standstill to 62mph in just 5.0 seconds.  Top speed is limited to 155mph, while the 550i achieves 27.2 mpg on the official combined cycle and CO2 emissions of 243g/km.
	Model
	Power
Hp
	Torque
Nm
	Zero – 62mph seconds
	Top speed mph
	Combined mpg
	VED band
	CO2
g/km
	BIK
%

	BMW 550i Saloon
	407
	600
	5.0
	155*
	27.2
	L
	243
	35

	Mercedes-Benz E500 Saloon
	382
	530
	5.2
	155*
	26.2
	L
	253
	35

	Audi S6 Quattro Saloon
	435
	540
	5.2
	155*
	22.4
	M
	299
	35


All cars are equipped with automatic transmission. *Electronically-limited.

BMW 523i, 528i and 535i
Core to the petrol range are three straight-six petrol engines - BMW 535i, BMW 528i and the BMW 523i - that offer a balance of performance and fuel economy unparalleled in this class.

The 3.0-litre engine in the BMW 535i is the first to combine BMW single turbocharger technology, High Precision Direct Injection, and VALVETRONIC, the variable valve management system, in the same package. 
Used for the first time in the six-cylinder engine, the result of the combined systems is the ability to infinitely adjust valve stroke and valve timing on the intake side, reducing throttle losses in the charge cycle to a minimum and making particularly efficient use of the energy in the fuel.  A further advantage is the response of the engine being optimised to an even higher standard. Indeed, the engine develops exceptional maximum torque of 400Nm from 1,200 to 5,000rpm, allowing for effortless acceleration at a wide range of speeds.

The new BMW 535i accelerates from standstill to 62mph in just 6.0 seconds, with a top speed limited electronically to 155mph.  On the official combined fuel consumption cycle, the BMW 535i achieves 33.2mpg (33.6mpg), while the CO2 rating is 199g/km (195g/km).
	Model
	Power
Hp
	Torque
Nm
	Zero – 62mph seconds
	Top speed mph
	Combined mpg
	VED band
	CO2
g/km
	BIK
%

	BMW 535i Saloon
	306
	400
	6.0
	155*
	33.2
	J
	199
	27

	Mercedes-Benz E350 CGI Saloon
	292
	365
	6.8
	155*
	32.8
	K
	205
	29

	Audi A6 3.0 TFSI Quattro Saloon
	290
	420
	5.9
	155*
	30.1
	K
	219
	31


*Electronically-limited.


The six-cylinder power units in the 528i and the 523i have a version of High Precision Direct Injection with a greater focus on efficiency.  This uses direct injection in a lean-burn mode, maintaining a highly efficient injection process with a reduced amount of fuel in the fuel/air mixture, while also being able to respond instantaneously to demands for more performance.

Referred to as stratified charging, this process forms various interacting layers of different fuel/air mixtures within the combustion chamber, the share of petrol in the mixture decreasing continuously with increasing distance from the spark plug.  A particularly rich and ignitable fuel/air mixture is available in the direct vicinity of the spark plug, while the leaner layers at a greater distance from the plug will burn in a clean and consistent manner.
In the new BMW 528i a maximum output is available of 258hp at 6,600rpm and peak torque of 310Nm of torque from 2,600 – 5,000rpm, allowing a zero to 62mph time of only 6.6 seconds, 36.2mpg (37.2mpg) on the combined cycle a CO2 emissions of 182g/km (178g/km).
	Model
	Power
Hp
	Torque
Nm
	Zero – 62mph seconds
	Top speed mph
	Combined mpg
	VEDband
	CO2
g/km
	BIK
%

	BMW 528i Saloon
	258
	310
	6.6
	155*
	36.2
	I
	182
	24

	Mercedes-Benz E250 CGI Saloon
	204
	310
	7.8
	148
	37.2
	I
	179
	23

	Audi A6 2.0 TFSI Saloon
	220
	280
	7.3
	149
	33.6
	J
	196
	27


All cars are equipped with automatic transmissions. *Electronically-limited.

Even more efficient, and using the same technology, is the six-cylinder 523i, which offers 37.2mpg on the combined cycle, CO2 emissions of 177g/km (178g/km) and a zero to 62mph time of 7.9seconds.
	Model
	Power
Hp
	Torque
Nm
	Zero – 62mph seconds
	Top speed mph
	Combined mpg
	VED band
	CO2
g/km
	BIK
%

	BMW 523i Saloon
	204
	270
	7.9
	148
	37.2
	I
	177
	23

	Mercedes-Benz E200 CGI Saloon
	184
	270
	8.5
	144
	38.7
	H
	169
	21

	Audi A6 2.0 TFSI Saloon
	170
	280
	8.2
	142
	37.7
	H
	174
	22



New eight-speed automatic transmission
All BMW 5 Series can be fitted with a new eight-speed automatic transmission, including the BMW 520d, making that model the first car in the world to combine a four-cylinder power unit with such a gearbox.  This new transmission comes as standard on the BMW 550i.

The advantage of eight gears is a shorter reaction time, smoother changes and the ability to match the perfect gear for any driving situation: whether trying to maximise performance or efficiency.

A further option is the new Sport Automatic derivative of the eight-speed transmission variant which allows manual selection of gears by means of shift paddles on the steering wheel.  In true Formula One style the driver is able to shift gears by selecting the right paddle to change up and the left one to change down.

In the search for even greater efficiencies, the six-speed manual gearbox on the BMW 535i, BMW 525d and BMW 520d comes with innovative dry sump lubrication, which offers significantly lower drag losses and avoids oil splash losses completely. 
To ensure sportier, more connected feel from gearshifts, the lever has been re-designed on all models and the shift bars upgraded to allow a smooth gearshift with precision and steadier than ever shift forces.
4. Chassis and Safety
Ever since the first generation was launched in 1972, the BMW 5 Series Saloon has set the benchmark for a superior driving experience in the executive sector: both in the way it handles and rides. 

This has been achieved through the utmost dedication to creating innovative, finely engineered chassis and suspension components that work in perfect harmony.  It’s a mindset that prevails today with the sixth generation, which sets new standards in an executive saloon.

And while the BMW 5 Series’ sporting qualities are an essential component of its success, so is its inherent safety. In the new model, a raft of innovative new systems and measures have been introduced that allow the driver to be fully aware of his surroundings, and more able to control what is happening around him.

Intelligent lightweight construction
Intelligent lightweight construction and a finely balanced chassis are the essential elements of the BMW 5 Series Saloon, combining to offer high levels of passive safety, dynamic sporting agility and optimised weight to reduce fuel consumptions and emissions.

For example, aluminium is used extensively in the new saloon’s bodyshell including the engine lid, the front side panels and the front spring supports on the body and for the first time, in the doors, which alone reduce the weight of the car by 23kg.

The wheelbase of the new BMW 5 Series is now the longest in class, at 2,968mm, allowing for both a high standard of occupant safety and interior space, as well as adding to the classic BMW 5 Series proportions of long, elegant body with short overhangs.  Indeed the extra wheelbase allows for even more perfect 50:50 weight distribution as the engine is located further behind the front axle than on the previous model.  As a result of the longer wheelbase, front and rear interior space has been further increased, with knee room on the rear seats up by 13mm.

Suspension
Among its many qualities, it is imperative that a BMW 5 Series must drive exquisitely: providing a compliant ride and yet offering dynamic characteristics.  To achieve this, BMW engineers have developed a technically sophisticated suspension that combines very sporting driving qualities with an even higher level of motoring comfort.
The new aluminium double track control arm on the front axle allows clear separation of wheel guidance and damping – essential in creating a pure driving experience.  While the dampers are able to respond particularly sensitively to bumps or rough areas of road, the configuration of the front axle, with the precisely matched wheel camber, keeps the tyres consistently on the road for optimum grip and a smooth ride.  This means that high lateral forces can be absorbed without a particularly firm set-up, which can be potentially detrimental to motoring comfort, as well as minimising unnecessary forces on the steering.

The newly developed Integral-V rear axle is also made of lightweight but strong aluminium.  Via the wheel supports, the rear axle subframe, the swinging arm and three track arms, the rear axle takes up drive and dynamic forces acting on the suspension with maximum efficiency. 

The innovative design of the swinging arms serves to set off longitudinal forces by direct, straight spring action to the rear.  Forces acting in radial and axial direction on the swinging arm rubber mounts are clearly separated from one another.  The rear axle ensures first-class acoustic and vibration comfort by effectively separating the suspension from the road and the drivetrain.

Bespoke chassis configurator
Drive Dynamic Control (DDC) is the overarching system that can tailor various dynamic systems within the BMW 5 Series to a driver’s tastes. DDC oversees - variously depending on which systems are fitted – throttle response, engine response, gear change characteristics, the level of steering assistance, Dynamic Stability Control thresholds, gearshift characteristics and damper settings.

Overall settings can be altered by a switch that toggles between pre-configured NORMAL, SPORT and SPORT+ modes.  In conjunction with Variable Damper Control and, respectively, Adaptive Drive, there is also the COMFORT mode for a particularly smooth style of motoring.  But through iDrive the driver can also adjust many of the systems independently to create a bespoke set-up.

Adaptive Drive is a system that allows the driver to choose the set-up of their own car, comprising Variable Damper Control and Dynamic Drive active anti-roll stability management.  It allows the car to uniquely offer a firm suspension set-up for sporty driving while softening bumps in way a car with much more comfort-orientated setting should the driver feel so inclined.  Controlled electronically, the unique and innovative new damper system, in which the inbound and rebound strokes are independent of each other, adjust both to the road surface and the driver’s style of motoring.
Dynamic Drive automatically reduces side roll of the car in bends and in sudden changes of direction, with sensors determining by how much motors in the front and rear anti-roll bars should build up counteracting forces to keep the car level.  Variable Damper Control is available as an individual option outside of the Adaptive Drive package on the BMW 520d, BMW 525d, BMW 523i, and BMW 528i.

Precise steering
Conventional mechanical/hydraulic power steering systems consume energy all the time, resulting in lower efficiency.  The new BMW 5 Series Saloon is the first car in its segment to feature Electric Power Steering (EPS) that uses no energy when the wheel is in the straight ahead position.

An electric motor is activated only when needed to provide assistance, and can be tailored so that the degree of steering force is slightly increased in accordance with the style of driving.  Another advantage of the EPS system is that it eliminates bumps and vibrations very effectively. 

EPS comes as standard with Servotronic steering assistance, which works relative to the speed of the car.  For example, when parking and steering at low speeds, assistance is enhanced so the driver hardly has to make any effort, while at higher speeds power assistance is intentionally reduced for optimum contact to the road.

Safety
The passenger cell of the new BMW 5 Series is extremely stiff. Intelligent use of high-strength multi-phase steel and hot-moulded ultra-high-strength steel give the body structure increased rigidity of approximately 55 per cent.  This rigidity keeps occupants safer in a collision and gives the dynamic elements of the suspension and chassis the perfect base on which to operate.
The latest generation of BMW ConnectedDrive in the new BMW 5 Series Saloon includes an array of industry-leading intelligent safety and driver assistance systems including traffic information, emergency call functions, online information, a telephone enquiry services, on-board camera and sensor information, comfort and infotainment.
BMW Assist, part of BMW ConnectedDrive, ensures that drivers of the new BMW 5 Series are constantly in contact with the outside world, making them feel safe and able to call on help for all manner of events.

Drivers can access news and information via the internet and SMS, have 24-7 call centre help and are able to receive locations and notes sent to the car.  This is all at the time safe in the knowledge that should they be involved in an incident, emergency services can be notified or 999 operators patched straight through to the cabin courtesy of the SOS emergency call function.
The SOS emergency function remains active whether the subscription for other services is continued or not.  The subscription is complimentary for the first three years of ownership, and £120 annually thereafter.
It works when sensors in the BMW Assist system recognise an accident has occurred.  A call is instantly and automatically put through to the emergency services, and exact co-ordinates of the car sent through too.  The 999 call centre will also know if the call was triggered manually by pushing the SOS button located in the car, or automatically through sensors.  The system also allows the operator to speak to occupants.
Among many useful features, the driver can be put in contact with BMW’s Customer Service centre, which can help with finding destinations such as hotels or restaurants.  Also RSS feeds from websites can be sent to the car, there is the ability to plan routes and send messages, and even technology where a BMW Customer Service operative can unlock the car remotely.
For further details go to www.bmwconnecteddrive.co.uk
Park Assist
Thanks to the use of EPS Steering, the new BMW 5 Series Saloon is the only car in its class available with automatic parking as an option.

Park Assist uses ultrasound sensors integrated in the side indicator surrounds to measure the length and width of possible parking spaces next to the road at a speed of up to 22mph.  By simply pressing the controller when in Park Assist mode, all the driver has to do is operate the accelerator and brake pedals and oversee the actual driving manoeuvre, while the Park Assist moves the steering wheel in order to back into the parking space.  Even when the system is deactivated it still searches for spaces, and can advise the driver when the car is put into reverse whether the space is adequate.

Other technological highlights
As well as the usual Reverse Assist camera, Surround View uses two cameras in the side mirrors to give a bird’s eye view of the car to aid slow speed manoeuvring.  The system works in conjunction with Park Distance Control to provide a true 360 degree peace of mind when parking in tight spots.  Also, Side View uses two cameras integrated in the front wings of the cars to serve as a monitor of traffic moving crosswise to the vehicle.  The feature sees two TV screen images projected on the iDrive screen so the driver can, for example, safely pull out of junctions where the view is obstructed.

In combination with Active Cruise Control, the new BMW 5 Series is available for the first time with a collision warning system incorporating a brake activation function.  
Collision warning triggers a red car symbol in the instrument cluster and, when fitted, also in the Head-Up Display.  As an impact becomes more likely an alarm sounds.  As soon as the initial symbol appears, the brakes are pre-loaded, the release thresholds on the Adaptive Brake Assistant are lowered, and the Brake Standby function on DSC Dynamic Stability Control is activated. 

The new BMW 5 Series Saloon also comes with a number of driver assistance systems, including Lane Change Warning, Lane Departure Warning, Speed Limit Info and a Speed Limit Device, the latest generation of Adaptive Headlights including Cornering Lights for variable light distribution and adaptive headlight range control, Night Vision and Head-Up Display.
While the new BMW 5 Series conforms to or exceeds all global standards for occupant safety, engineers have also worked hard to make the car perform well in the event of a pedestrian collision.

To optimise the high standard of passive pedestrian safety, an active engine compartment lid with pyrotechnic release mechanism automatically moves up in a collision with a pedestrian.  Activated at speeds between 12 - 34mph by sensors, the engine lid is lifted by 30mm at the front and 50mm at the rear to reduce the risk of injury through impact with the engine. 
5. Design
Balance is everything with the BMW 5 Series.  This is reflected in the proportions of the new model, with its distinctive long wheelbase and short overhangs, long bonnet, sharp, dynamic lines, muscular body and the coupé-like, sleek and flowing roofline.

The slight wedge shape of the body accentuates the sporting character of the new Saloon, while the new, particularly concise interpretation of the Hofmeister kink at the bottom of the C-pillar, again emphasises the dynamic looks of the car from the side. Although the metalwork and details are new, to a driver that understands the essential character of a BMW 5 Series there will be a resonance.

With nearly 40 years of BMW 5 Series, it is a design language legacy that Adrian van Hooydonk, Head of BMW Group Design, and his team were keenly aware of.
“The new BMW 5 Series is a car for people that know a lot about BMW, love BMW and they know as much as we do about the essence of BMW.  In that sense the new 5 Series has to be a 100 per cent, true BMW.”

The wheelbase of the new BMW 5 Series is longest in the segment, at 2,968mm while it is 58mm longer than its predecessor at 4,899mm, 14mm wider at 1,860 yet 4mm lower at 1,464mm.  This equates to a car whose proportions serve to emphasise the low, sleek nature of a sports salon from BMW, yet one that is roomier than the model it replaces. 
The seating position has been improved compared to the former model both front and rear, with knee room on the rear seats of the new BMW 5 Series Saloon up by 13mm.

van Hooydonk said: “We worked very hard to create a shape that to us was the perfect combination of sportiness and elegance, or, as we sometimes also refer to it, as the perfect business suit.  A BMW 5 Series has to look dynamic and lean and, yes it should be muscular, but in a well-trained athletic kind of way – not like a bodybuilder.  I think this design does all that.”

The new model is a more subtle evocation of the BMW design language, with light and shadow effects, and changing perspectives becoming more recognisable the closer you are, revealing the three-dimensional character of the convex and concave chiselled body panels. 

Four contour lines sweeping down into the kidney grille split up the engine compartment lid into individual sections.  The two middle lines come together above the kidney grille, which has been redesigned for this model to be more dynamic.
van Hooydonk said: “The kidney-shaped grille on the new BMW 5 Series is now lower and wider than on many other BMWs, and that together with the sharp lines and proportions exudes a sportiness, as well as enhancing the elegance of the car.”

As is now an integral BMW statement, the new model comes with daytime driving lights incorporating unmistakable LED light rings.  Along the side, and to reflect van Hooydonk’s view that the car should look as if it has been athletically well-trained, the famous Hofmeister kink has been honed too.  It swings strikingly back in the tightest radius ever seen on any BMW saloon, helping to stretch the silhouette of the car and give the passenger cabin a light and low appearance.

Precision is an integral element of the BMW 5 Series, in both the way it drives and the way it looks.  Sophisticated details such as design elements in chrome colour with integrated side direction indicators and the door openers embedded in the side contour line reflect that.

At the rear, the L-shaped rear light clusters again reflect the typical design of a BMW and provide that unmistakable night-time look.  Three rows of LED lights form the impression of homogeneously glowing bars, while the direction indicators and brake lights also come in LED technology.

A design informed by fuel saving.
Striking looks are not the only intention with the rear.  A vital area for aerodynamics, the designers were as keenly aware of the need for every element of the car to contribute to fuel saving and BMW’s EfficientDynamics strategy.

van Hooydonk said: “A small thing not immediately apparent to the eye, but that we have worked on very hard in the design phase was the detail that helps aerodynamic performance and reduce fuel consumption, such as the spoiler lip and trailing edges on the rear lamps.”

Both features affect the way vortices scroll off the car thus minimizing the effects of drag and improving performance. 

The new BMW 5 Series launches in SE specification first but an all-new M Sport package will be available within nine months.  This model will feature different body styling, a lower ride height and the specification enhancements to the interior design and content as before. M Sport is a model range that is very popular in the UK. Globally BMW UK is the market that orders the greatest number of such packages. On four-cylinder cars it accounts for 55 per cent of sales, while models with six or more cylinders are 85 per cent M Sport variants. 
A new BMW 5 Series Touring has just been announced too and will go on sale in September.  That model will feature a similar engine line-up and standard specification as the BMW 5 Series Saloon. 

Interior. 
High tech features, luxurious quality, comfort and sporting elegance are the characteristics of the BMW 5 Series.  This combination of sporting ability and luxurious comfort had to be reflected in the cabin.

Oliver Heilmer, interior designer of the new BMW 5 Series Saloon, said:  “For me, the interior of the BMW 5 Series has to be a contrast between a business saloon, which is calm and quiet, and a sporty car.  The top line of the dashboard describes the sporty character, with much more tension and life.”

The interior had to display the same combination of elegance and dynamism as the exterior, a challenge that led Heilmer in search of a cabin that flowed around it occupants, but retained a sporty appeal.

“I didn’t want to have a line or piece of the interior that just sticks into another surface.  They have to always flow into each other and around the cabin - for that reason door handles lean forward in the drivers direction, for example.  Each interior you design has a corner which is really challenging – and for this one the surface between the instrument cluster and monitor, as it drops and then climbs again.  I think it took about one month to get this element just right.

“The horizontal breakdown of the instrument panel underlines the generous feeling of space and continues through the door linings all the way to the rear, enabling the passengers to enjoy the motoring experience in a harmoniously balanced setting.”

The interior trim on the instrument panel and the door linings available in various colours and materials borders at the bottom on a highlight bar in Pearl Gloss Chrome adding a particular touch of style and elegance.  At the transition point from the instrument panel to the door lining further highlight bars merging up in a sweeping line add an exciting eye-catching element.

From the driving position of the new BMW 5 Series Saloon, it is clear the driver is in control: the cockpit is inclined towards the driver at an angle of seven degrees and the asymmetrically designed centre console emphasises this clear concentration of functions towards the driver.

The multifunction buttons on the newly developed steering wheels now also operate cruise control.  All control units and switches as well as the displays in the instrument cluster are finished in stylish partial Black Panel technology for clear and optimum readability. 
Information relevant to the driver as well as the control instruments are arranged on the side of the cockpit facing the driver, while the displays, control units and buttons that control comfort functions are positioned more towards the middle.

The new BMW 5 Series comes with the latest generation of BMW iDrive.  Both the standard and optional audio, navigation and communication functions are operated via the Controller on the centre console and the optional 10.2- or standard, 7-inch Control Display can be used for a multitude of functions, including leafing though the car’s handbook.
6. History
Sporting, elegant and powerful: the new BMW 5 Series is the sixth incarnation in a line of executive saloons dating back to 1972.  Since that first BMW 5 Series Saloon nearly 5 million vehicles that all embody those core principles have been sold.  (The figure stretches to 5.5 million when including Touring).  But the concept of a sporting BMW saloon actually predates the 5 Series family.  The “New Range” of BMW 1500, the BMW 1800 and the BMW 2000, were introduced from the early 1960s and were the inspiration for the current class-leading line-up. 
At the time the most successful model series BMW had ever produced, this classic series set a template for sporting but elegant, powerful and technically innovative midrange four-door saloons that the BMW 5 Series then refined and developed.

1972: an icon is launched.
The first BMW 5 Series, launched in 1972 at the Frankfurt Motor Show came with a new numbering system evocatively reminiscent of the legendary BMW 501 “Baroque Angel” and the iconic BMW 507 cars of the 1950s. 

It also introduced a whole new reinterpretation of BMW’s essential design language.  French designer Paul Bracq’s car, with its dual headlights, Hofmeister kink in the C-pillar, sleek lines, large windows and low waistline is a 1970s classic.  It was also the first BMW to be crafted using computers in significant capacity to calculate the car’s deformation zones for determining occupant safety.

The BMW 520 and the BMW 520i, with four-cylinder power units developed 115 and, respectively, 130hp, while the introduction of the 145hp BMW 525 in the second year of production offered the first six-cylinder engine in the sector and underlined buyers’ desire for increasing power levels.

As the model range expanded over its lifecycle, it reached a sporting peak when BMW Motorsport GmbH developed the BMW M535i in 1980.  This 218hp car was the forerunner of all BMW M as we know them today cars (the M1 supercar, being somewhat of a different proposition).  Namely the purest of engineering excellence distilled into a conventional production body shape.  
Such was the success of the first BMW 5 Series range it sold almost 700,000 units globally – doubling previous sales of the “New Range” models.
1981: second generation E28 brings first diesel and the mighty M5.
The second generation 1982 BMW 5 Series built on the template of the first, and was a stylistic evolution, with a number of engineering refinements that allowed it to be more spacious, safer and sportier.

As well as pioneering innovations in the sector ranging from anti-lock brakes, on-board computers and electronic fuel injection, the second generation BMW also introduced the M5 and diesel power.

Presented at the 1985 Frankfurt Motor Show, the BMW M5 used a 3.5-litre straight six engine derived from the legendary BMW M1.  It developed 286hp, offering performance previously obtainable only in thoroughbred sports cars but in a more practical package. 
Along with the birth of a performance legend, BMW also introduced diesel power for the first time in 1983, with the BMW 524td.  Using turbocharger technology, large flow cross-sections on the intake and outlet valves and a swirl-chamber combustion process in the 2.4-litre straight six powerplant, BMW’s engineers were able to increase engine power to a then substantial 115hp.  The exercise proved that efficiency, refinement and performance could be packaged in a sports saloon.
The second generation of the BMW 5 Series was replaced after seven years of production.  By this time sales had amounted to more than 722,000 units, again setting a new record.

1988: the third generation E34 sees Touring introduced. 
Longer and with more interior space, the third generation BMW 5 Series introduced technology such as Automatic Stability Control, variable VANOS camshaft management, electronically controlled dampers and speed-related Servotronic steering assistance.  A Touring variant joined the fold for the first time too.
Created under the guidance of Chief Designer Claus Luthe, the sporting elegance and flowing lines of the new car were available in a wide range of engines including 520i, 525i, 530i, 535i, 524td – all with six cylinders and electronic fuel injection, with power ranging from 115hp to 211hp.

The first BMW 5 Series Touring was revealed at the 1992 Frankfurt Motor Show, and was fitted from the start with self-levelling on the rear axle.

A new BMW M5 was also introduced unusually early on in the lifecycle, in 1990 with 315hp, further increased to 340hp in 1992.  To prove just how sporting a Touring could be, a limited run of M5 Touring models was introduced from 1992.  Sadly these were only available in LHD form and offered for sale on mainland Europe. 
Overall sales of the BMW 5 Series Touring up to 1996 amounted to approximately 125,000 units, and in total sales of the third-generation BMW 5 Series range hit more than 1.3 million units worldwide.

1995: fourth generation E39 unveiled with enhanced technical highlights.
The fourth generation of the BMW 5 Series, debuting in 1995, was an evolution of the previous version and featured the now-trademark dual round headlights and daytime driving lights behind a glass cover.

The E39 BMW 5 Series, one of the most successful cars in BMW’s history, was laden with technology including multifunction steering wheel, a navigation system, active seats and Dynamic Stability Control.

It was also the first large-scale production car worldwide made almost completely of light alloy.  Combined with the newly developed all-aluminium power units this helped significantly reduce the weight of the car, and so increase its agility and performance, while improving efficiency.  A recipe that set the tone for BMW today.

The new model entered the market with petrol and diesel straight-six power units delivering outputs from 150hp to 193hp.  Technical innovations gave both the petrol and diesel engines even more power with reduced fuel consumption, while two V8 engines were introduced in 1996.

In 1998, one of the most loved cars of the modern era was introduced: the new BMW M5 was at the time the most powerful mainstream production engine ever built by BMW - it produced 400hp from its 5.0 litre V8.

Unsurprisingly, this much admired model range set up a new sales record accounting for 1.47 million units sold (the Saloon accounted for 1,208,000 sales) up to the end of production in early 2004.
2003: E60 EfficientDynamics redefines customer expectations
The fifth generation BMW 5 Series design certainly stirred opinion but the E60 has redefined customers’ expectations of an executive car’s efficiency and economy, without compromising performance.

Introduced in 2003 and designed by Chris Bangle, it stood out for the raft of innovative technologies it introduced to the market alongside its futuristic looks. 
Aluminium construction and components ensured the E60 5 Series Saloon was balanced and as light as possible.  Engines with an aluminium or, respectively, a composite aluminium/magnesium crankcase as well as the lightweight aluminium front section of the car ensured a particularly good balance of weight front-to-rear.  Another important component newly developed at the time was the integral rear axle, again made of aluminium.
Dynamic Stability Control, Active Steering and Adaptive Drive with electronic damper adjustment and anti-roll stability management, Head-Up Display and BMW Night Vision, Active Cruise Control with Stop & Go and Lane Departure Warning were just some of advanced technologies introduced in the car.  Many of these innovations were not only firsts on the 5 Series but also in-class firsts too. 
And alongside these driver aids, in 2007 the fifth generation BMW 5 Series also brought technologies to the sector such as Brake Energy Regeneration, gearshift indicator, active aerodynamic and on-demand ancillaries as part of BMW’s award-winning EfficientDynamics programme.

Such was the class-leading BMW EfficientDynamics combination of performance and economy, that the BMW 520d returned an average fuel consumption of 55.4mpg and a CO2 emission rating of 136g/km, while offering drivers an impressive 177hp.

From 2005 to 2008, the BMW 5 Series was the best-selling car in its segment for four years in a row, with worldwide sales of the Saloon amounting to 1,096,810 by the end of 2009. 
7. Q&A
The following is a brief Q&A with Michelle Roberts, the BMW 5 Series Saloon Product Manager.  The aim of the Q&A is provide you with answers to questions that might not have already been covered and for quotes to be used in your articles.

What are the main markets for the BMW 5 Series Saloon?
The UK is actually the third largest market for the car.  The USA accounts for the biggest proportion of sales with 29 per cent of volume, Germany is next with 17 per cent of all global sales while the UK accounts for eight per cent.

Who is the typical BMW 5 Series customer?
We are lucky in that our cars appeal to a wide range of buyers.  The choice of seven engines and wide price band help satisfy that demand too.  There isn’t an average 
5 Series customer but the biggest customer group comes from 45-50 years-old males who are professionals who need their car for work but want to enjoy the driving in the process.

When will the M Sport derivative appear?
The UK is the largest market for M Sport packages in the world.  With six-cylinder engined 5 Series models M Sport is specified in approximately 95 per cent of orders.  Even on the far more price sensitive 520d model M Sport is still a popular choice accounting for around a third of orders.  Given the interest in the UK for such a derivative we would expect to be able to introduce something within the next 12 months. 
Will there be a Business Edition introduced?
This model derivative proved very popular with the last generation of BMW 5 Series Saloon and because of its high standard specification resulted in a three per cent uplift in residual value figures by third party experts.  However, there are currently no plans to do the same on the new BMW 5 Series Saloon because the standard specification is already so high. 

And what of the BMW Concept 5 Series ActiveHybrid shown at Geneva. Can we expect to see right-hand-drive versions in the near future?
Naturally such a model fits in with our efforts as part of the EfficientDynamics strategy.  But we have to be realistic as the UK is very much a diesel oriented market for the 5 Series with 95 per cent of orders being for that style of engine.  To bring a production ready version of the Concept 5 Series ActiveHybrid to the UK there would have to be a strong business case. We are currently evaluating the role of hybrid technology for BMW, but much more than that I can not say at present. 
And the obvious question, when can we expect an M5 to appear?
We obviously can’t talk of future product, however there is likely to be an M5, but should one be signed off it won’t appear for some time yet.  

One of the optional highlights is Park Assist?  You showed something similar quite some time ago, why has it taken so long to come to market?  Also what of the garage parking feature you demonstrated at an innovation day?
The truth of the matter is that BMW has had the patent for this since 2002.  But the technical features involved such as electromechanical steering assistance that were around at the time didn’t provide enough of a benefit above and beyond Park Distance Control to warrant introducing the system.  However, developments of EPS and the introduction of Surround View mean it is now a suitable time to bring the system to market.

As for the garage parking system this is something that has already passed the development cycle, but while the technical side has been accomplished there a still a number of legal and liability issues that need clarification. 

8. Standard and Optional Equipment
See separate tables.
9. Technical Data
523i Saloon
528i Saloon
	
	
	BMW 523i
	
	BMW 528i

	
	
	
	
	

	Body
	
	
	
	

	No of doors/seats
	
	4 / 5
	
	4 / 5

	Length/width/height (unladen)
	mm
	4899 / 1860 / 1464
	
	4899 / 1860 / 1464

	Wheelbase
	mm
	2968
	
	2968

	Track, front/rear
	mm
	1600 / 1627
	
	1600 / 1627

	Ground clearance
	mm
	141
	
	141

	Turning circle
	m
	11.95
	
	11.95

	Tank capacity
	app ltr
	70
	
	70

	Cooling system incl heater
	ltr
	9.0
	
	9.0

	Engine oil1)
	ltr
	6.5
	
	6.5

	Weight, unladen, to DIN/EU
	kg
	1625 / 1700
(1650 / 1725)
	
	1635 / 1710
(1655 / 1730)

	Max load to DIN
	kg
	610
	
	610

	Max permissible
	kg
	2235 (2260)
	
	2245 (2265)

	Max axle load, front/rear
	kg
	1070 / 1265
(1070 /1270)
	
	1070 / 1265
(1070 / 1270)

	Max trailer load (12%), braked/unbraked
	kg
	1900 / 750
(2000 / 750)
	
	2000 / 750

	Max roofload/max towbar
	kg
	100/90
	
	100/90

	Luggage comp capacity
	ltr
	520
	
	520

	Air resistance
	Cd  x A
	0.28 x 2.35
	
	0.28 x 2.35

	Power Unit
	
	
	
	

	Power Unit
	
	
	
	

	Config/No of cyls/valves 
	
	Straight-six / 4
	
	Straight-six / 4

	Engine technology
	
	Direct fuel injection
(High Precision Injection), lean-burn) 
	
	Direct fuel injection
(High Precision Injection), lean-burn)

	Capacity, effective
	cc
	2996
	
	2996

	Stroke / bore
	mm
	88.0 / 85.0
	
	88.0 / 85.0

	Compression ratio
	:1
	12.0
	
	12.0

	Fuel grade
	
	min RON 91
	
	min RON 91

	Max output
	kW/hp
	150 / 204
	
	190 / 258

	at 
	rpm
	6100
	
	6600

	Max torque
	Nm/lb-ft
	270/199
	
	310/228

	At
	rpm
	1500-4250
	
	2600-5000

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	80 (90) / luggage comp
	
	80 (90) / luggage comp

	Alternator
	A/W
	210 / 2940
	
	210 / 2940

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double track control arm axle with separate lower track arm level, aluminium, small steering roll radius, anti-dive

	Suspension, rear
	
	Integral-V multi-arm axle, aluminium, with steering function, anti-squat and 
anti-dive, double acoustic separation 
(air suspension with self-levelling standard on long-wheelbase version)

	Brakes, front
	
	Single-piston aluminium swing-calliper disc brakes in frame structure

	Diameter
	mm
	330 x 24 / inner-vented
	
	348 x 30 / inner-vented

	Brakes, rear
	
	Single-piston aluminium swing-calliper disc brakes

	Diameter
	mm
	330 x 20 / inner-vented
	
	330 x 20 / inner-vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking function, Fading Compensation, Start-Off Assistant; optional: Adaptive Drive

	Safety equipment
	  
	Standard: airbags for driver and front passenger, side airbags for driver 
and front passenger, head airbags for front and rear seats, three-point 
inertia-reel belts on all seats with belt stopper, belt tightener and belt force limiter at the front, crash-activated headrests at the front, crash sensors, 
Tyre Defect Indicator

	Steering
	
	Electric Power Steering (EPS) with Servotronic; 
optional: Integral Active Steering

	Steering transmission, overall
	:1
	17.1
	
	17.1

	Tyres, front/rear
	
	225/55 R17 97W
	
	225/55 R17 97W

	Rims, front/rear
	
	8J x 17 light-alloy
	
	8J x 17 light-alloy


	
	
	BMW 523i 
	
	BMW 528i

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist including Enquiry Service, remote-control functions and V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online including  Parkinfo, National Info, Google Directory Service, News, Weather Realtime and BMW Routes

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, variable light distribution and adaptive headlight range control, High-Beam Assistant, Park Distance Control, back-up camera, Surround View including Side View and Top View, BMW Night Vision with detection of persons, Head-Up Display, Parking Assistant, collision warning with brake activation function in combination with Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Speed Limit Device, Automatic/Enhanced Emergency Call 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of gearbox
	
	Six-speed manual (optional: eight-speed automatic with Steptronic)

	Gear ratios
I
	:1
	4.323 (4.714)
	
	3.498 (4.714)

	                                  
II
	:1
	2.456 (3.143)
	
	1.984 (3.143)

	                                  
III
	:1
	1.659 (2.106)
	
	1.313 (2.106)

	                                  
IV
	:1
	1.230 (1.667)
	
	1.000 (1.667)

	                                  
V
	:1
	1.000 (1.285)
	
	0.809 (1.285)

	                                   
VI
	:1
	0.848 (1.000)
	
	0.701 (1.000)

	                                   
VII
	:1
	– (0.839)
	
	– (0.839)

	                                   
VIII
	:1
	– (0.667)
	
	– (0.667)

	                                   
R
	:1
	3.938 (3.295)
	
	3.300 (3.295)

	Final drive
	:1
	3.231 (3.385)
	
	4.100 (3.385)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	10.8 (11.0)
	
	8.6 (8.7)

	Output per litre
	kW/hp
	50.1/68.1
	
	63.4/86.2

	Acceleration       0–62 mph
	sec
	7.9 (8.2)
	
	6.6 (6.7)

	                      Standing-start km
	sec
	28.5 (29.0)
	
	26.5 (26.6)

	In 4th gear        50–75 mph 
	sec
	9.0 (–)
	
	7.0 (–)

	Top speed
	Mph
	148 (145)
	
	1552))

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration with recuperation display, gearshift point indicator (on manual gearbox models), intelligent lightweight construction, on-demand operation of ancillary units, air flap control, Electric Power Steering (EPS), detachable a/c compressor, tyres with reduced roll resistance

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	Mpg
	26.9 (26.9)
	
	27.2 (27.2)

	Extra-urban
	Mpg
	47.9 (47.9)
	
	44.8 (47.1)

	Combined
	Mpg
	37.2 (37.2)
	
	36.2 (37.2)

	CO2
	g/km
	177 (178)
	
	182 (178)

	Emission rating
	
	EU5  
	
	EU5  

	
	
	
	
	



Specifications applicable to ACEA markets, data relevant to homologation applicable in part only to Germany (weight)
Figures in brackets apply to automatic transmission models
1)  Oil change
2)  Electronically-limited 
 
BMW 535i Saloon. 
BMW 550i Saloon.
	
	
	BMW 535i
	
	BMW 550i

	
	
	
	
	

	Body
	
	
	
	

	No of doors/seats
	
	4 / 5
	
	4 / 5

	Length/width/height (unladen)
	mm
	4899 / 1860 / 1464
	
	4899 / 1860 / 1464

	Wheelbase
	mm
	2968
	
	2968

	Track, front/rear
	mm
	1600 / 1627
	
	1600 / 1627

	Ground clearance
	mm
	141
	
	141

	Turning circle
	m
	11.95
	
	11.95

	Tank capacity
	app ltr
	70
	
	70

	Cooling system incl heater
	ltr
	9.3
	
	11,4

	Engine oil1)
	ltr
	6.5
	
	8,5

	Weight, unladen, to DIN/EU
	kg
	1685 / 1760
(1700 / 1775)
	
	1830 / 1905

	Max load to DIN
	kg
	610
	
	600

	Max permissible
	kg
	2295 (2310)
	
	2430

	Max axle load, front/rear
	kg
	1100 / 1275
	
	1195 / 1310

	Max trailer load (12%), braked/unbraked
	kg
	2000 / 750
	
	2000 / 750

	Max roofload/max towbar dwnl
	kg
	100/90
	
	100/90

	Luggage comp capacity
	ltr
	520
	
	520

	Air resistance
	Cd  x A
	0.29 x 2.35
	
	0.30 x 2.35

	Power Unit
	
	
	
	

	Power Unit
	
	
	
	

	Config/No of cyls/valves 
	
	Straight-six / 4
	
	V90 / 8 / 4

	Engine technology
	
	BMW TwinPower Turbo, direct fuel injection 
(High Precision Injection), fully-variable valve control (VALVETRONIC)
	
	BMW TwinPower Turbo,  direct fuel injection 
(High Precision Injection)

	Capacity, effective
	cc
	2979
	
	4395

	Stroke / bore
	mm
	89.6 / 84.0
	
	88,3 / 89,0

	Compression ratio
	:1
	10.2
	
	10.0

	Fuel grade
	
	Min RON 91
	
	min RON 91

	Max output
	kW/hp
	225 / 306
	
	300 / 407

	at 
	rpm
	5800
	
	5500-6400

	Max torque
	Nm/lb-ft
	400/295
	
	600/442

	at
	rpm
	1200-5000
	
	1750-4500

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	80 (90) / luggage comp
	
	90 / luggage comp

	Alternator
	A/W
	170 / 2380
	
	210 / 2940

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double track control arm axle with separate lower track arm level, aluminium, small steering roll radius, anti-dive

	Suspension, rear
	
	Integral-V multi-arm axle, aluminium, with steering function, anti-squat and 
anti-dive, double acoustic separation

(air suspension with self-levelling standard on long-wheelbase version)

	Brakes, front
	
	Single-piston aluminium swing-calliper disc brakes in frame structure

	Diameter
	mm
	348 x 36 / inner-vented
	
	374 x 36 / inner-vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	345 x 24 / inner-vented
	
	345 x 24 / inner-vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking function, Fading Compensation, Start-Off Assistant; optional: Adaptive Drive

	Safety equipment
	  
	Standard: airbags for driver and front passenger, side airbags for driver 
and front passenger, head airbags for front and rear seats, three-point 
inertia-reel belts on all seats with belt stopper, belt tightener and belt force limiter at the front, crash-activated headrests at the front, crash sensors, 
Tyre Defect Indicator

	Steering
	
	Electric Power Steering (EPS) with Servotronic; 
optional: Integral Active Steering

	Steering transmission, overall
	:1
	17.1
	
	17.1

	Tyres, front/rear
	
	225/55 R17 97W
	
	245/45 R18 96Y RSC

	Rims, front/rear
	
	8J x 17 light-alloy
	
	8J x 18 light-alloy

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist including Enquiry Service, remote-control functions and V-Info+, BMW TeleServices, integration of mobile terminals


	
	
	BMW 535i
	
	BMW 550i

	Infotainment
	
	Optional: access to the internet, BMW Online including Parkinfo, National Info, Google Directory Service, News, Weather Realtime and BMW Routes

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, variable light distribution and adaptive headlight range control, High-Beam Assistant, Park Distance Control, back-up camera, Surround View including Side View and Top View, BMW Night Vision with detection of persons, Head-Up Display, Parking Assistant, collision warning with brake activation function in combination with Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Speed Limit Device, Automatic/Enhanced Emergency Call 

	Transmission
	
	
	
	

	Type of gearbox
	
	Six-speed manual (optional: eight-speed automatic with Steptronic)
	
	Eight-speed automatic with Steptronic

	Gear ratios
I
	:1
	4.110 (4.714)
	
	4.714

	                                  
II
	:1
	2.315 (3.143)
	
	3.143

	                                  
III
	:1
	1.542 (2.106)
	
	2.106

	                                  
IV
	:1
	1.179 (1.667)
	
	1.667

	                                  
V
	:1
	1.000 (1.285)
	
	1.285

	                                   
VI
	:1
	0.846 (1.000)
	
	1.000 

	                                   
VII
	:1
	– (0.839)
	
	0.839

	                                   
VIII
	:1
	– (0.667)
	
	0.667

	                                   
R
	:1
	3.727 (3.295)
	
	3.317

	Final drive
	:1
	3.231 (3.077)
	
	2.813

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	7.5 (7.6)
	
	6.1

	Output per litre
	kW/hp
	75.5/102.7
	
	68.3/92.9

	Acceleration       0–62 mph
	sec
	6.0 (6.1)
	
	5.0

	                      Standing-start km
	sec
	25.2 (25.6)
	
	23.2

	In 4th gear       50–75 mph 
	sec
	5.9 (–)
	
	–

	Top speed
	Mph
	1552)
	
	1552

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration with recuperation display, gearshift point indicator (on manual gearbox models), intelligent lightweight construction, on-demand operation of ancillary units, air flap control (BMW 535i), 
Electric Power Steering (EPS), detachable a/c compressor, tyres with reduced roll resistance (BMW 535i)

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	Mpg
	23.9 (23.7)
	
	(18.3)

	Extra-urban
	Mpg
	42.8 (44.1)
	
	(37.7)

	Combined
	Mpg
	33.2 (33.6)
	
	(27.2)

	CO2
	g/km
	199 (195)
	
	243

	Emission rating
	
	EU5  
	
	EU5  

	
	
	
	
	


Specifications applicable to ACEA markets, data relevant to homologation applicable in part only to Germany (weight)
Figures in brackets apply to automatic transmission models

1) Oil change

2) Electronically-limited

BMW 520d Saloon. 
BMW 525d Saloon.
BMW 530d Saloon
	
	
	BMW 520d
	BMW 525d
	BMW 530d

	
	
	
	
	

	Body
	
	
	
	

	No of doors/seats
	
	4 / 5
	4 / 5
	4 / 5

	Length/width/height (unladen)
	mm
	4899 / 1860 / 1464
	4899 / 1860 / 1464
	4899 / 1860 / 1464

	Wheelbase
	mm
	2968
	2968
	2968

	Track, front/rear
	mm
	1600 / 1627
	1600 / 1627
	1600 / 1627

	Ground clearance
	mm
	141
	141
	141

	Turning circle
	m
	11.95
	11.95
	11.95

	Tank capacity
	app ltr
	70
	70
	70

	Cooling system incl heater
	ltr
	7.0
	8.0
	8.0

	Engine oil1)
	ltr
	5.2
	7.2
	7.2

	Weight, unladen, to DIN/EU
	kg
	1620 / 1695
(1625 / 1700)
	1690 / 1765
(1705 / 1780)
	1715 / 1790
(1720 /1795)

	Max load to DIN
	kg
	610
	610
	610

	Max permissible
	kg
	2230 (2235)
	2300 (2315)
	2325 (2330)

	Max axle load, front/rear
	kg
	1060 / 1275
	1100 / 1280
(1100 / 1285)
	1125 / 1195

	Max trailer load (12%), braked/unbraked
	kg
	2000 / 750
	2000 / 750
	2000 / 750

	Max roofload/max towbar 
	kg
	100/90
	100/90
	100/90

	Luggage comp capacity
	ltr
	520
	520
	520

	Air resistance
	Cd  x A
	0.28 x 2.35
	0.28 x 2.35
	0.28 x 2.35

	Power Unit
	
	
	
	

	Power Unit
	
	
	
	

	Config/No of cyls/valves 
	
	Straight-four / 4
	Straight-six / 4
	Straight-six / 4

	Engine technology
	
	Common-rail direct fuel injection, turbocharger with variable intake geometry
	Common-rail direct fuel injection, turbocharger with variable intake geometry
	Common-rail direct fuel injection, turbocharger with variable intake geometry

	Capacity, effective
	cc
	1995
	2993
	2993

	Stroke / bore
	mm
	90.0 / 84.0
	90.0 / 84.0
	90.0 / 84.0

	Compression ratio
	:1
	16.5
	16.5
	16.5

	Fuel grade
	
	Diesel
	Diesel
	Diesel

	Max output
	kW/hp
	135 / 184
	150 / 204
	180 / 245

	at 
	rpm
	4000
	4000 (3750)
	4000

	Max torque
	Nm/lb-ft
	380/280
	450/332
	540/398

	at
	rpm
	1900-2750
	1750-2500
	1750-3000

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	80 (90) / luggage comp
	90 / luggage comp
	90 / luggage comp

	Alternator
	A/W
	180 / 2520
	180 / 2520
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double track control arm axle with separate lower track arm level, aluminium, small steering roll radius, anti-dive

	Suspension, rear
	
	Integral-V multi-arm axle, aluminium, with steering function, anti-squat and 
anti-dive, double acoustic separation
(air suspension with self-levelling standard on long-wheelbase version)

	Brakes, front
	
	Single-piston aluminium swing-calliper disc brakes in frame structure

	Diameter
	mm
	330 x 24 / inner-vented
	330 x 24 / inner-vented
	330 x 24 / inner-vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	330 x 20 / inner-vented
	330 x 20 / inner-vented
	330 x 20 / inner-vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking function, Fading Compensation, Start-Off Assistant; optional: Adaptive Drive

	Safety equipment
	  
	Standard: airbags for driver and front passenger, side airbags for driver 
and front passenger, head airbags for front and rear seats, three-point 
inertia-reel belts on all seats with belt stopper, belt tightener and belt force limiter at the front, crash-activated headrests at the front, crash sensors, 
Tyre Defect Indicator

	Steering
	
	Electric Power Steering (EPS) with Servotronic; 
optional: Integral Active Steering

	Steering transmission, overall
	:1
	17.1
	17.1
	17.1

	Tyres, front/rear
	
	225/55 R17 97W
	225/55 R17 97W
	225/55 R17 97W

	Rims, front/rear
	
	8J x 17 light-alloy
	8J x 17 light-alloy
	8J x 17 light-alloy

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist including Enquiry Service, remote-control functions and V-Info+, BMW TeleServices, integration of mobile terminals


	
	
	BMW 520d
	BMW 525d
	BMW 530d

	
	
	
	
	

	
	
	
	
	

	Infotainment
	
	Optional: access to the internet, BMW Online including  Parkinfo, National Info, Google Directory Service, News, Weather Realtime and BMW Routes

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, variable light distribution and adaptive headlight range control, High-Beam Assistant, Park Distance Control, back-up camera, Surround View including Side View and Top View, BMW Night Vision with detection of persons, Head-Up Display, Parking Assistant, collision warning with brake activation function in combination with Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Speed Limit Device, Automatic/Enhanced Emergency Call 

	Transmission
	
	
	
	

	Type of gearbox
	
	Six-speed manual (optional: eight-speed automatic with Steptronic)

	Gear ratios
I
	:1
	4.110 (4.714)
	4.110 (4.714)
	5.080 (4.714)

	                                  
II
	:1
	2.248 (3.143)
	2.248 (3.143)
	2.804 (3.143)

	                                  
III
	:1
	1.403 (2.106)
	1.403 (2.106)
	1.783 (2.106)

	                                  
IV
	:1
	1.000 (1.667)
	1.000 (1.667)
	1.260 (1.667)

	                                  
V
	:1
	0.802 (1.285)
	0.802 (1.285)
	1.000 (1.285)

	                                   
VI
	:1
	0.659 (1.000)
	0.659 (1.000)
	0.835 (1.000)

	                                   
VII
	:1
	– (0.839)
	– (0.839)
	– (0.839)

	                                   
VIII
	:1
	– (0.667)
	– (0.667)
	– (0.667)

	                                   
R
	:1
	3.727 (3.295)
	3.727 (3.295)
	4.607 (3.317)

	Final drive
	:1
	3.385 (2.929)
	3.154 (2.471)
	2.563 (2.471)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	12.1 (12.2)
	11.3 (11.4)
	9.5 (9.6)

	Output per litre
	kW/hp
	67.7/92.1
	50.1/68.1
	60.1/81.7

	Acceleration       0–62 mph
	sec
	8.1 (8.1)2)
	7.2 (7.2)
	6.3 (6.3)

	                      Standing-start km
	sec
	29.0 (29.0)2)
	27.7 (27.8)
	26.5 (26.5)

	In 4th gear        50–75 mph 
	sec
	6.9 (–)2)
	6.3 (–)
	5.0 (–)

	Top speed
	Mph
	141 (140)2))
	147 (147)
	155 (155) 5)

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration with recuperation display, Auto Start Stop 
(BMW 520d with manual gearbox) gearshift point indicator (on manual gearbox models), intelligent lightweight construction, on-demand operation of ancillary units, air flap control, Electric Power Steering (EPS), detachable a/c compressor, tyres with reduced roll resistance

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	Mpg
	3)
	34.9 (36.2)
	35.3 (36.2)

	Extra-urban
	Mpg
	3)
	55.4 (55.4)
	53.3 (53.3)

	Combined
	Mpg
	56.5 (54.3)2)
	45.6 (46.3)
	44.8 (45.6)

	CO2
	g/km
	132 (137)2)
	162 (160)
	166 (160)

	Emission rating
	
	EU5 
	EU5  
	EU54)  

	
	
	
	
	


Specifications applicable to ACEA markets, data relevant to homologation applicable in part only to Germany (weight)
Figures in brackets apply to automatic transmission models
1)  Oil change
2)  Provisional figures
3)  Not yet available
4)  EU6 with optional BMW BluePerformance technology
5)  Not yet available

10. Torque Curves and Dimensions
[image: image2.png]ool

500

450

400

350

300

250

200

1000

2000

2000

4000

=000

&000

7000

2000

20U

240

230

220

210

200

190

180

170

160

150

140

130

120

110

100

90

80

70

60

50

40

30

20



BMW 550i Saloon

[image: image3.png]Luu

475

450

425

400

375

350

325

300

275

250

225

200

175

150

100 /
75

50

25

0 1000 2000 2000 4000 =000 &000 7000

220

210

200

190

180

170

160

150

140

130

120

110

100

90

80

70

60

50

40

30

20

2000




[image: image4.png]4uu

350

300

250

200

150

100

50

16U

150

140

130

120

110

100

90

80

70

60

50

40

30

20

1000

2000

2000

4000

=000

&000

7000

2000




[image: image5.png]ool

800

750

700

650

600

550

500

450

400

350

300

250

200

150

100

50

1000

2000

2000

4000

=000

200

190

180

170

160

150

140

130

120

110

100

90

80

70

60

50

40

30

20



[image: image6.png]cuu

750

700

650

600

550

500

450

400

350

300

250

200

150

100

50

16U

150

140

130

120

110

100

90

80

70

60

50

40

30

20

1000

2000

2000

4000

=000




[image: image7.png]cuu

550

500

450

400

350

300

250

200

150

100

50

12U

100

[/

50

25

1000 2000

2000 4000 =000




[image: image8.png]



BMW 535i Saloon







BMW 528i Saloon








BMW 523i Saloon







BMW 530d Saloon






BMW 525d Saloon





BMW 520d Saloon







Exterior and Interior Dimensions


Engine speed [min-1]





  0                                  1000                              2000                              3000                              4000                             5000 





Torque  [Nm]





Output [kW]








380 Nm @ 1750-2750 min -1





135 kW @ 4000 min -1





Engine speed [min-1]





  0                                  1000                              2000                             3000                             4000                              5000 





Torque [Nm]





Output [kW]








450 Nm @ 1750-2500 min -1





150 kW @ 4000 min -1





Engine speed [min-1]





      0                              1000                           2000                          3000                           4000                           5000 





Torque [Nm]





Output [kW]








540 Nm @ 1750-3000 min -1





180 kW @ 4000 min -1





Engine speed [min-1]





  0                   1000               2000               3000               4000               5000               6000               7000               8000








Torque [Nm]





Output [kW]








270 Nm @ 1500-4250 min -1





150 kW @ 6100 min -1





Engine speed [min-1]





 0                   1000               2000               3000               4000                5000               6000               7000               8000





Torque [Nm]





Output [kW]








310 Nm @ 2600-5000 min -1





190 kW @ 6600 min -1





Engine speed [min-1]





  0                   1000                2000               3000               4000                5000                6000               7000               8000





Torque [Nm]





Output [kW]








400 Nm @ 1200-5000 min -1





225 kW @ 5800 min -1





Engine speed [min-1]





  0                   1000                2000               3000               4000                5000                6000               7000               8000





Torque [Nm]





Output [kW]








600 Nm @ 1750-4500 min -1





300 kW @ 5500-6400 min -1








