MINI Cooper Countryman, MINI Cooper Countryman Automatic. (July 2010)

| Body |  |  | MINI Cooper Countryman |  | MINI Cooper Countryman Automatic |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No of doors/seats |  |  | 5 / 4 (5) |  | 5 / 4 (5) |  |  |
| Length/width/height (unladen) | mm |  | 4097 / 1789 / 1561 |  | 4097 / 1789 / 1561 |  |  |
| Wheelbase | mm |  | 2595 |  | 2595 |  |  |
| Track, front/rear | mm |  | 1534 / 1559 |  | 1534 / 1559 |  |  |
| Turning circle | m |  | 11.6 |  | 11.6 |  |  |
| Tank capacity | ca. l |  | 47 |  | 47 |  |  |
| Cooling system incl. heater | l |  | 5.5 |  | 6.0 |  |  |
| Engine oil | l |  | 4.2 |  | 4.2 |  |  |
| # Transmission oil incl. drive train | l |  | lifetime |  | lifetime |  |  |
| Weight, unladen to EU (DIN)1 | kg |  | 1265 / 1340 |  | 1295 / 1370 |  |  |
| Max load to DIN | kg |  | 470 |  | 470 |  |  |
| Max permissible load | kg |  | 1735 |  | 1765 |  |  |
| Max axle load, front/rear | kg |  | 930 / 855 |  | 960 / 855 |  |  |
| Max trailer load2 braked (12%) / unbraked | kg |  | - /- |  | 1000/ 500 |  |  |
| Max roofload/max download | kg |  | 75 / - |  | 75 / 75 |  |  |
| Luggage comp | l |  | 350 / 450 / 1170 |  | 350 / 450 / 1170 |  |  |
| Air drag cx / A / cx x A | - / m2 / m2 |  | 0.35/ 2.36 / 0.83 |  | 0.35/ 2.36 / 0.83 |  |  |
| Engine |  |  |  |  |  |  |  |
| Config/No of cyls/valves |  |  | Inline / 4 / 4 |  | Inline / 4 / 4 |  |  |
| Engine management |  |  | MEV 17.2.2 |  | MEV 17.2.2 |  |  |
| Capacity | cm³ |  | 1598 |  | 1598 |  |  |
| Bore/stroke | mm |  | 77/ 85.8 |  | 77/ 85.8 |  |  |
| Compression ratio | :1 |  | 11.0 |  | 11.0 |  |  |
| Fuel grade | ROZ |  | 91-98 |  | 91-98 |  |  |
| Max output | kW / PS |  | 90 / 122 |  | 90 / 122 |  |  |
| at | min–1 |  | 6000 |  | 6000 |  |  |
| Max torque (with overboost) | Nm |  | 160 |  | 160 |  |  |
| at | min–1 |  | 4250 |  | 4250 |  |  |
| Electrical System |  |  |  |  |  |  |  |
| Battery/installation | Ah / – |  | 60 / Engine compartment |  | 55 / Engine compartment |  |  |
| Alternator | A |  | 150 |  | 120 |  |  |
| Chassis |  |  |  |  |  |  |  |
| Suspension, front |  |  | # Single-joint MacPherson spring strut axle with anti-dive control |  |  |  |  |
| Suspension, rear |  |  | # Multi-link axle with trailing arms with aluminium lightweight construction |  |  |  |  |
| Front brakes |  |  | Disc vented |  | Disc vented |  |  |
| Diameter | mm |  | 294 x 22 |  | 294 x 22 |  |  |
| Rear brakes |  |  | Disc |  | Disc |  |  |
| Diameter | mm |  | 280 x 10 |  | 280 x 10 |  |  |
| Driving stability systems | # Hydraulic 2-circuit brake system with Antilock Brake System (ABS), electronic brake force distribution (EBD) and Cornering Brake Control (CBC), Dynamic Stability Control (DSC) with Brake Assist and Hill Assist, optional: Dynamic Traction Control (DTC) and Electronic Differential Lock Control (EDLC). Parking brake acts mechanically on rear wheels |  |  |  |  |  |  |
| Steering | # Electric power steering (EPS); 2.4 rotations in total |  |  |  |  |  |  |
| Steering transmission, overall | :1 |  | 14.1 |  | 14.1 |  |  |
| # Tires |  |  | 205/60 R16 92H |  | 205/60 R16 92H |  |  |
| # Wheels |  |  | 6,5J x 16 LM |  | 6,5J x 16 LM |  |  |
| Transmission |  |  |  |  |  |  |  |  |
| Type of gearbox |  |  | Six-speed manual transmission |  | Six-speed automatic |  |  |
| Gear ratios I |  | :1 | 3.214 |  | 4.148 |  |  |
| II |  | :1 | 1.792 |  | 2.370 |  |  |
| III |  | :1 | 1.194 |  | 1.556 |  |  |
| IV |  | :1 | 0.914 |  | 1.155 |  |  |
| V |  | :1 | 0.784 |  | 0.859 |  |  |
| VI |  | :1 | 0.683 |  | 0.686 |  |  |
| # Reverse gear |  | :1 | 3.143 |  | 3.394 |  |  |
| # Final drive ratio |  | :1 | 4.722 |  | 4.643 |  |  |

| Performance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Power-to-weight ratio to DIN | kg / kW | 14.1 | 14.4 |  |
| Output per litre | kW / l | 56.3 | 56.3 |  |
| Acceleration 0–100 km/h | s | 10.5 | 11.6 |  |
| 0–1000 m | s | 32.2 | 33.3 |  |
| In 4th/5th gear 80–120 km/h | s | 11.6 / 14.9 | – / – |  |
| Top speed | km / h | 190 | 182 |  |
| Fuel Consumption in EU Cycle |  |  |  |  |
| Urban | l / 100 km | 7.4 | 9.3 |  |
| Extra-urban | l / 100 km | 5.2 | 6.0 |  |
| Composite | l / 100 km | 6.0 | 7.2 |  |
| CO2 | g / km | 140 | 168 |  |
| Miscellaneous |  |  |  |  |
| Emission rating |  | EU5 | EU5 |  |
| # Ground clearance (empty) | mm | 149 | 149 |  |

1 Weight of the car in road trim (DIN) plus 75 kg for driver and luggage.

2 Deviations are possible under certain circumstances.