MINI Cooper D Countryman. (July 2010)

| Body |  |  |  |  | MINI Cooper D Countryman |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No of doors/seats |  |  |  |  | 5 / 4 (5) |  |  |
| Length/width/height (unladen) | mm |  |  |  | 4097 / 1789 / 1561 |  |  |
| Wheelbase | mm |  |  |  | 2595 |  |  |
| Track, front/rear | mm |  |  |  | 1534 / 1559 |  |  |
| Turning circle | m |  |  |  | 11.6 |  |  |
| Tank capacity | ca. l |  |  |  | 47 |  |  |
| Cooling system incl. heater | l |  |  |  | 5.4 |  |  |
| Engine oil | l |  |  |  | 5.2 |  |  |
| # Transmission oil incl. drive train | l |  |  |  | lifetime |  |  |
| Weight, unladen to EU (DIN)1 | kg |  |  |  | 1310 / 1385 |  |  |
| Max load to DIN | kg |  |  |  | 470 |  |  |
| Max permissible load | kg |  |  |  | 1780 |  |  |
| Max axle load, front/rear | kg |  |  |  | 985 / 850 |  |  |
| Max trailer load2 braked (12%) / unbraked | kg |  |  |  | 750 / 500 |  |  |
| Max roofload/max download | kg |  |  |  | 75 / 75 |  |  |
| Luggage comp | l |  |  |  | 350 / 450 / 1170 |  |  |
| Air drag cx / A / cx x A | - / m2 / m2 |  |  |  | 0.35/ 2.36 / 0.83 |  |  |
| Engine |  |  |  |  |  |  |
| Config/No of cyls/valves |  |  |  |  | Inline / 4 / 4 |  |  |
| Engine management |  |  |  |  | DDE 7.0 |  |  |
| Capacity | cm³ |  |  |  | 1598 |  |  |
| Bore/stroke | mm |  |  |  | 78/ 83.6 |  |  |
| Compression ratio | :1 |  |  |  | 16.5 |  |  |
| Fuel grade | ROZ |  |  |  | Diesel |  |  |
| Max output | kW / PS |  |  |  | 82 / 112 |  |  |
| at | min–1 |  |  |  | 4000 |  |  |
| Max torque (with overboost) | Nm |  |  |  | 270 |  |  |
| at | min–1 |  |  |  | 1750-2250 |  |  |
| Electrical System |  |  |  |  |  |  |
| Battery/installation | Ah / – |  |  |  | 70 / Engine compartment |  |  |
| Alternator | A |  |  |  | 150 |  |  |
| Chassis |  |  |  |  |  |  |
| Suspension, front |  |  |  | # Single-joint MacPherson spring strut axle with anti-dive control |  |  |
| Suspension, rear |  |  |  | # Multi-link axle with trailing arms with aluminium lightweight construction |  |  |
| Front brakes |  |  |  |  | Disc vented |
| Diameter |  | mm |  |  | 294 x 22 |  |  |
| Rear brakes |  |  |  |  | Disc |
| Diameter |  | mm |  |  | 280 x 10 |  |  |
| Driving stability systems |  | # Hydraulic 2-circuit brake system with Antilock Brake System (ABS), electronic brake force distribution (EBD) and Cornering Brake Control (CBC), Dynamic Stability Control (DSC) with Brake Assist and Hill Assist, optional: Dynamic Traction Control (DTC) and Electronic Differential Lock Control (EDLC). Parking brake acts mechanically on rear wheels |  |  |  |  |  |
| Steering |  | # Electric power steering (EPS); 2.4 rotations in total |  |  |  |  |  |
| Steering transmission, overall |  | :1 |  |  | 14.1 |  |  |
| # Tires |  |  |  |  | 205/60 R16 92H |  |  |
| # Wheels |  |  |  |  | 6,5J x 16 LM |  |  |
| Transmission |  |  |  |  |  |  |
| Type of gearbox |  |  |  |  | # Six-speed manual transmission |  |  |
| Gear ratios I |  | :1 |  |  | 3.308 |  |  |
| II |  | :1 |  |  | 1.870 |  |  |
| III |  | :1 |  |  | 1.194 |  |  |
| IV |  | :1 |  |  | 0.872 |  |  |
| V |  | :1 |  |  | 0.721 |  |  |
| VI |  | :1 |  |  | 0.596 |  |  |
| # Reverse gear |  | :1 |  |  | 3.231 |  |  |
| # Final drive ratio |  | :1 |  |  | 3.706 |  |  |

| Performance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Power-to-weight ratio to DIN | kg / kW |  | 16.0 |  |
| Output per litre | kW / l |  | 51.3 |  |
| Acceleration 0–100 km/h | s |  | 10.9 |  |
| 0–1000 m | s |  | 32.6 |  |
| In 4th/5th gear 80–120 km/h | s |  | 9.7 / 11.9 |  |
| Top speed | km / h |  | 185 |  |
| Fuel Consumption in EU Cycle |  |  |  |  |
| Urban | l / 100 km |  | 4.7 |  |
| Extra-urban | l / 100 km |  | 4.2 |  |
| Composite | l / 100 km |  | 4.4 |  |
| CO2 | g / km |  | 115 |  |
| Miscellaneous |  |  |  |  |
| Emission rating |  |  | EU5 |  |
| # Ground clearance (empty) | mm |  | 149 |  |

1 Weight of the car in road trim (DIN) plus 75 kg for driver and luggage.

2 Deviations are possible under certain circumstances.