	A subsidiary

 of BMW AG
	[image: image1.wmf]Printed on Recycled Paper

BMW

US Press Information


–7–

	For Release:
	December 15, 2005

	
	

	Contact:
	David J. Buchko

Product Communications Manager

201-307-3789 / dave.buchko@bmwna.com
William Scully

BMW Product Communications Specialist

201-307-3790 / william.scully@bmwna.com


	
	


2006 BMW 5 SERIES
Technical Specifications

Bold within table indicates new specification for 2006.

General
525i
525xi
530i
530xi
530xi
550i

[image: image2.png]



Sedan
Sedan
Sedan
Sedan
Sports
Sedan






Wagon

Curb weight, lb.:


Manual transmission
3396
3593
3450
3670
3847
3902

Sequential Manual Gearbox


   (SMG)
NA
NA
3450
NA
NA
3909


Automatic transmission
3439
3638
3472
3704
3880
3924

Weight distribution, front/rear, %:


Manual transmission
50.6/49.4
51.2/48.8
51.1/48.9
52.1/47.9
48.7/51.3
51.4/48.6

Sequential Manual Gearbox
NA
NA
51.1/48.9
NA
NA
51.4/48.6
Automatic transmission
51.0/49.0
51.5/48.5
51.4/48.6
52.1/47.9
49.1/50.9
51.7/48.3

NA – Not available or not applicable

General (cont.)
525i
525xi
530i
530xi
530xi
550i


Sedan
Sedan
Sedan
Sedan
Sports
Sedan






Wagon

Wheelbase, in.
113.7 




113.6

Track, front/rear, in.


(w/standard wheels)
61.3/62.3 1



61.3/62.2

Length, front/rear, in.
191.1 1



191.2

Width, in.
72.7 

Height, in.
57.8
58.3
57.8
58.3
58.7
57.8

Body

Type
4-door sedan 1


5-door wagon

Aerodynamic drag coefficient
0.28
0.30
0.28
0.30
0.33
0.29

EPA size classification
Midsize 1



Midsize







station wagon

Accommodations

Seating capacity, persons
5 2
Shoulder room, front/rear, in.
57.3/57.2 2
Head room, front/rear, in.
37.7/37.0 1



37.6/37.6
Leg room, front/rear, in.
41.5/36.0 2
EPA passenger-compartment


volume, cu ft.
99.1 1



96.5
EPA cargo volume, cu ft.
14.01, 




17.6/33.6/58.3 

Engine & electrical
525i
525xi
530i
530xi
530xi
550i



Sedan
Sedan
Sedan
Sedan
Sports
Sedan







Wagon

Engine type
DOHC inline 24-valve 6-cylinder, magnesium/aluminum
DOHC (4-cam)



composite construction, Valvetronic variable intake-
32-valve V-8,



valve lift & Double VANOS 
 variable intake- & exhaust-

Valvetronic variable



valve timing 




intake-valve lift &








Double VANOS 5







variable intake-








& exhaust-valve








timing

Bore x stroke, mm/in.
85.0 x 88.0/3.35 x 3.46 6



93.0 x 88.3/







3.66x3.48
Displacement, cc/cu in.
2996/183 6




4799/293
Compression ratio
10.7:1 6   




10.51
Power @ rpm, hp
215 @ 6250 


255 @6600 

360@6300
Torque @ rpm, lb-ft.
185 @ 2750 7

220 @ 2750 6
360@3400
Engine-management system
MSV70 (525i/xi & 530i/xi) or ME9 (550i) with knock control; Valvetronic, 


variable valve timing, engine cooling & other functions included in 


control strategy 2
Induction  system
Single-stage 5

3-stage 6

2-stage
Fuel requirement
Premium unleaded 2
Fuel capacity, U.S. gal.
18.5 2
Battery capacity, amp-hr.
110 2
Alternator output rating, amp./W
185/2590 6




170/2380
Drivetrain
525i
525xi
530i
530xi
530xi
550i



Sedan
Sedan
Sedan
Sedan
Sports 
Sedan







Wagon

Drive system, engine location/
Front/
Front/
Front/
Front/
Front/
Front/


drive wheels
RWD
AWD
RWD
AWD
AWD
RWD

Manual transmission
Getrag I,
Getrag H,



ZF G,



6-speed
6-speed 




6-speed

Ratios: 1st
4.32:1
4.35:1 9



4.06:1



2nd
2.46:1
2.50:1 9



2.40:1



3rd
1.66:1
1.66:1 9



1.58:1



4th
1.23:1
1.23:1 9



1.19:1



5th
1.00:1
1.00:1 9



1.00:1



6th
0.85:1
0.85:1 9



0.87:1

Final drive ratio
3.38:1
3.46:1
3.23:1
3.23:1
3.38:1
2.93:1

Sequential Manual Gearbox


530i & 550i Sedans only: Electrohydraulic/


(SMG)


electronic controls applied to 6-speed manual






transmission; ratios & final drive ratio as for






6-speed manual transmission; includes Sport mode






affecting shift characteristics & engine response






to accelerator pedal

Drivetrain (cont.)
525i
525xi
530i
530xi
530xi
550i



Sedan
Sedan
Sedan
Sedan
Sports 
Sedan








Wagon

Automatic transmission
6-speed STEPTRONIC with Adaptive Transmission Control, selectable




Manual & Sport modes:




ZF 6 HP 19 6




ZF 6 HP 26

Ratios: 1st
4.17:1 2


2nd
2.34:1 2


3rd
1.52:1 2


4th
1.14:1 2


5th
0.87:1 2


6th
0.69:1 2
Final drive ratio
3.91:1
4.10:1
3:64:1
3.73:1
3.73:1
3.38:1

Chassis

Body/frame construction
Unitized; aluminum front-end structure & skin, otherwise steel

Front suspension
Struts, double-pivot lower arms, coil springs, twin-tube gas-pressure




shock absorbers, anti-roll bar, subframe 2 




(525i, 530i & 550i Sedans: aluminum lower arms, steering knuckles, strut tubes,




subframe & thrust plate)




(525i, 530i & 550i Sedans ZSP: sport calibration, Active Roll Stabilization)

Rear suspension
4-link Integral suspension; coil springs, twin-tube gas-pressure shock absorbers,




anti-roll bar; aluminum suspension system & subframe 2




(525i, 530i & 550i Sedans ZSP: sport calibration, Active Roll Stabilization)

ZSP– Sport Package

Chassis (cont.) 
525i
525xi
530i
530xi
530xi
550i



Sedan
Sedan
Sedan
Sedan
Sports 
Sedan








Wagon

Steering type: standard
Rack & pinion, Servotronic vehicle-speed-sensitive power assist 2



optional
Rack & pinion, Active Steering with Servotronic vehicle-speed-sensitive





power assist


Overall ratio, standard steering
Mechanically variable; mean ratio is 13.7:1 2

Overall ratio, Active Steering
Electrically/electronically variable over wide range of 10.8:1 – 18.0:1


Turning circle, ft.
37.5
39.0
37.5
39.0
39.0
37.5

4-wheel ventilated disc brakes


with aluminum calipers:


Front, dia. x thickness, mm/in.
324 x 30/12.8 x 1.18 6



348 x 30/









13.7 x 1.18



Rear, dia. x thickness, mm/in.
320 x 20/12.6 x 0.79 6



345 x 24/










13.6 x 0.94

Wheels: standard
Cast alloy, 17 x 7.5 2


optional
Cast alloy, 17 x 8.0 7
Cast alloy, 18 x 8.0 8

18 x 8.0 front/









18 x 9.0 rear

Tires:
standard
225/50R-17 V-rated all-season 2


optional
245/45R-17 W-rated
245/40R-18 W-rated

245/40R-18 front/




run-flat performance 7, 

run-flat performance 8,10

275/35R-18 rear









W-rated run-flat









performance 10
Chassis (cont.) 
525i
525xi
530i
530xi
530xi
550i



Sedan
Sedan
Sedan
Sedan
Sports 
Sedan








Wagon

Stability-enhancement system
Dynamic Stability Control (DSC), including Dynamic Traction Control,




electronic brake proportioning, antilock braking (ABS), cornering/braking




stability enhancement, Dynamic Brake Control, Brake Fade Compensation,




Brake Standby, Brake Drying, Comfort Stop & Start-off Assistant




(525/530xi models: delete Dynamic Traction Control & Comfort Stop,




add Hill Descent Control)

Performance data

Acceleration, 0-60 mph, sec. 
:


Manual transmission or SMG
7.3
7.9
6.4
6.6
6.8
5.4


Automatic transmission
7.6
8.2
6.6
6.9
7.3
5.5

Top speed, mph:


Manual transmission or SMG
150
147
150
150
150
150


Automatic transmission
150
144
150
150
150
150

Fuel economy, EPA est. MPG,


city/highway:


Manual transmission
20/30
19/28
20/30
19/28
19/28
15/23


Sequential Manual Gearbox
NA
NA
19/27
NA
NA
16/22

Automatic transmission
21/29
20/27
21/29
20/27
20/27
17/25
NA – Not available or not applicable

#      #      #
Company


BMW of North America, LLC





BMW Group Company





Mailing address


PO Box 1227


Westwood, NJ


07675-1227





Office address


300 Chestnut Ridge Road 


Woodcliff Lake, NJ


07677-7731





Telephone


(201)  307-3606





Fax


307-3607





Internet


bmwusa.com


 �








� – Specification applies to all Sedan models.


� – Specification applies to all models.


� – Can be expanded via available folding rear seats.


� – Rear seats upright, loaded to top of seats / rear seats upright, loaded to ceiling / rear seats folded, loaded to ceiling.


� – VANOS = VAriable NOckenwellen Steuerung = variable camshaft control, or variable valve timing.


� – Specification applies to 525i/xi & 530i/xi models.


� – Specification applies to 525i & 525xi models.


� – Specification applies to 530i & 530xi models.


� – Specification applies to 525xi, 530i & 530xi models.


� – Due to low-profile tires, please note: Wheels, tires and suspension parts are more susceptible to road hazard and consequential damages. Performance tires are not recommended for driving in snow and ice conditions.


� – BMW AG test results. Actual acceleration results may vary depending on specification of vehicle; road and environmental conditions; testing procedures and driving style. These results should be used for comparison only, and verification should not be attempted on public roads. BMW urges you to obey all posted speed limits and to please wear your safety belt at all times.





–more–

–more–


