	A subsidiary

 of BMW AG
	[image: image1.wmf]Printed on Recycled Paper

BMW

US Press Information

	Company
BMW of North America, LLC

BMW Group Company

Mailing address

PO Box 1227

Westwood, NJ

07675-1227
Office address

300 Chestnut Ridge Road 

Woodcliff Lake, NJ

07677-7731
Telephone

(201)  307-3606
Fax

(201)  307-3607

Internet

bmwusa.com
 [image: image1.wmf]



PAGE  
- 4 -


	For Release:
	March 5, 2007

	
	

	Contact:
	Thomas J. Plucinsky
Product and Technology Communications Manager

201-307-3783 / thomas.plucinsky@bmwna.com
William Scully

BMW Product Communications Specialist

201-307-3790 / william.scully@bmwna.com


	
	


2007 BMW Z4 ROADSTERS AND COUPE
Technical Specifications
General
Z4
Z4
Z4


Roadster
Roadster
Coupe



3.0i
3.0si
3.0si
Curb weight, lb.:


Manual transmission
3020
3086

3108

Automatic transmission
3086
3131

3156
Weight distribution, front/rear, %:


Manual transmission
49.9/50.1
49.9/50.1
49.7/50.3

Automatic transmission
50.6/49.4 
50.6/49.4
50.3/49.7
Wheelbase, in.
98.2 


[image: image2.wmf]Printed on Recycled Paper

Track, front/rear, in.
58.0/60.0 1
Length, in.
161.1 1
Width, in.
70.1 1
Height, in. (with softtop up)
51.1
51.1

49.9
Body

Type
2-seat Roadster 

2-seat Coupe
Aerodynamic drag coefficient
0.36 2 (with softtop up) 
0.34
EPA size classification
2-Seater 1
Accommodations
Z4
Z4
Z4


Roadster
Roadster
Coupe



3.0i
3.0si
3.0si
Seating capacity, persons
2 1
Shoulder room, in.
52.5 1
Head room, in.
37.3 2  (with softtop up)
37.3
Leg room, in.
42.0 1
EPA interior volume, cu ft.
47.6 2 (with softtop up)
47.6
Cargo volume, cu ft.
8.5/9.2 2, 


10.1/12.0 

Engine & electrical


Engine type
DOHC inline 24-valve 6-cylinder, 





magnesium/aluminum composite 
construction, 




Valvetronic variable intake-valve lift & Double 




VANOS 
 steplessly  variable intake- &



exhaust-valve timing 1
Bore x stroke, mm/in.
85.0 x 88.0/3.35 x 3.46 1

Displacement, cc/cu in.
2996/183 1

Compression ratio
10.7:1 1
Power @ rpm, hp
215 @ 6250
255 @ 6600 


Torque @ rpm, lb-ft.
185 @ 2750
220 @ 2750 6

Engine-management system 
MSV70 with knock control (2 sensors); 




Valvetronic, variable valve timing, electronic 




throttle system, engine cooling & other functions 




included in control strategy 1
Induction system
Single-stage
3-stage 6


Fuel requirement
Premium unleaded 1
Fuel capacity, U.S. gal.
14.5 1
Battery capacity, amp-hr.
55 1
Alternator output, amp./W
120/1680 2


155/2170
Drivetrain
Z4

Z4

Z4




Roadster
Roadster
Coupe




3.0i
3.0si
3.0si
Drive system
Front engine/rear drive 1
Manual transmission
Getrag I,
ZF Type H,
Getrag H,



6-speed
6-speed

6-speed
Ratios:
1st
4.32:1
4.35:1 6




2nd
2.46:1
2.50:1 6 




3rd
1.66:1
1.66:1 6 




4th
1.23:1
1.23:1 6 




5th
1.00:1
1.00:1 6 




6th
0.85:1
0.85:1 6 




Reverse
3.94:1
3.93:1 6 


Final drive ratio
3.38:1
3.23:1

3.46:1
Automatic transmission
ZF 6  HP 19, 6-speed 1

Ratios:
1st
4.17:1 1


2nd
2.34:1 1


3rd
1.52:1 1


4th
1.14:1 1


5th
0.87:1 1



6th
0.69:1 1


Reverse
3.40:1 1
Final drive ratio
3.73:1
3.64:1 6


Chassis

Body/frame construction
Unitized steel structure with aluminum



hood 1
Front suspension
Struts, arc-shaped forged-aluminum

 

lower arms with hydraulic cushions,



coil springs, twin-tube gas-pressure



shock absorbers, anti-roll bar; alumi-



num thrust plate and strut-tower braces 1
Rear suspension
Multi-link system with Central Links,



upper & lower lateral links (upper link of



cast aluminum), coil  springs, twin-tube



gas-pressure shock absorbers, anti-roll bar, 



steel thrust plate & V-brace 1
Chassis (cont.)
Z4

Z4

Z4



Roadster
Roadster
Coupe




3.0i
3.0si
3.0si
Steering type
Rack & pinion, vehicle-speed-sensitive



electric power assist 1

Overall ratio
14.2:1 1

Turns, lock-to-lock
3.0 1

Turning circle, ft.
32.2 1

4-wheel ventilated disc brakes,  

vacuum-assisted:


Diameter, front, mm/in.
300/11.8 
325/12.8 
325/12.8

Diameter, rear, mm/in.
294/11.6 1


Wheels (standard)
Cast alloy, 17 x 8.0 1
Wheels (optional), front/rear
Cast alloy 1,




17 x 8.0/
18 x 8.0/ 

18 x 8.0/


17 x 8.5
18 x 8.5

18 x 8.5 


(ZSP)
(ZSP)

(ZSP)
Run-flat performance tires (standard)
225/45R-17 1,

 

Run-flat performance tires (optional), 
front/rear
225/45R-17/
225/40R-18/



245/40R-17 
255/35R-18



(ZSP) 7
(ZSP) 6,7



Stability-enhancement system
Dynamic Stability Control (DSC), 
including all-




speed traction control, Dynamic Traction 




Control, electronic brake proportioning, antilock 




braking (ABS), cornering/braking stability



enhancement, Dynamic Brake Control, Brake 




Fade Compensation, Brake Standby, Brake 




Drying & Start-off Assistant 1
ZSP – Sport Package
erformance data
Z4

Z4

Z4




Roadster
Roadster
Coupe




3.0i
3.0si
3.0si
Acceleration, 0-60 mph, sec. 
:


Manual transmission
6.2

5.6 6



Automatic transmission
6.6

5.7 6


Top speed, mph 


Manual transmission
149
155 6



Automatic transmission
149
155 6


Fuel economy, EPA est. MPG, 

city/highway 
:


Manual transmission
20/30 1


Automatic transmission
21/30 1


#      #      #
Company


BMW of North America, LLC





BMW Group Company





Mailing address


PO Box 1227


Westwood, NJ


07675-1227





Office address


300 Chestnut Ridge Road 


Woodcliff Lake, NJ


07677-7731





Telephone


(201)  307-3606





Fax


307-3607





Internet


bmwusa.com


 �











� – Specification applies to all three models


� – Specification applies to both Roadster models


� – Softtop lowered/raised; German DIN measurement.


� – Beneath cargo cover/cover retracted; German DIN measurement


� – VANOS = VAriable NOckenwellen Steuerung = variable camshaft control, or variable valve timing.


� – Applies to Z4 Roadster 3.0si and Coupe 3.0si


� – Due to low-profile tires, please note: Wheels, tires and suspension parts are more susceptible to road hazard and consequential damages. Z4 models are not equipped with a spare tire and wheel. Performance tires are not recommended for driving in snow and ice conditions. 


� – BMW AG test results. Actual acceleration results may vary depending on specification of vehicle; road and environmental conditions; testing procedures and driving style. These results should be used for comparison only, and verification should not be attempted on public roads. BMW urges you to obey all posted speed limits and to please wear your safety belt at all times.


� – Electronically limited.


� – Preliminary data





- more -


- more -

[image: image3.png]


