	A subsidiary

 of BMW AG
	[image: image1.wmf]Printed on Recycled Paper

BMW

US Press Information

	Company
BMW of North America, LLC

BMW Group Company

Mailing address

PO Box 1227

Westwood, NJ

07675-1227
Office address

300 Chestnut Ridge Road 

Woodcliff Lake, NJ

07677-7731
Telephone

(201)  307-3606
Fax

(201)  307-3607

Internet

bmwusa.com
 [image: image1.wmf]



PAGE  
- 4 -


	For Release:
	September 1, 2007

	
	

	Contact:
	Thomas Plucinsky
Product and Technology Communications Manager

201-307-3783 / thomas.plucinsky@bmwna.com
William Scully

BMW Product Communications Specialist

201-307-3790 / william.scully@bmwna.com


	
	


2008 BMW M5
Technical Specifications
General

Curb weight, lb.
4012

Weight distribution, front/rear, %
52.2/47.8

Wheelbase, in.
113.7

Track, front/rear, in.
62.2/61.7

Length, in.
191.5

Width, in.
72.7

Height, in.
57.8

Body

Type
4-door sedan

Aerodynamic drag coefficient
0.31

EPA size classification
Midsize

Accommodations

Seating capacity, persons
5

Shoulder room, front/rear, in.
57.3/57.2

Head room, front/rear, in.
39.1/38.1
Leg room, front/rear, in.
41.5/36.0

EPA passenger-compartment volume, cu ft.
99.1

EPA cargo volume, cu ft.
14.0 

Engine & electrical

Engine type
DOHC (4-cam) 40-valve V-10, high-



Pressure Double VANOS 
 steplessly



variable valve timing, 10 individual



electronically controlled throttles with



P400, P500 & P500 S settings

Bore x stroke, mm/in.
92.0 x 75.2/3.62 x 2.96

Displacement, cc/cu in.
4999/305

Compression ratio
12.0:1

Power @ rpm, hp
500 @ 7750

Torque @ rpm, lb-ft.
383 @ 6100

Engine-management system
MS S65 with ionic-current knock control,



variable valve timing, electronic throttles,



variable maximum power level, g-sensitive



lubrication system, oil-level/temperature



sender, variable tachometer warning 


segment, catalyst protection & other 


functions included in control strategy

Fuel requirement
Premium unleaded

Fuel capacity, U.S. gal.
18.5

Battery capacity, amp-hr.
90

Alternator output rating, amp./W
170/2380

Drivetrain
Drive system
Front engine/rear-wheel drive

Sequential Manual Gearbox (SMG III 247)
Electrohydraulically actuated,



electronically controlled 7-speed,



with 6 Sequential & 5 Automated shift



programs; twin-disc clutch

Ratios: 1st
3.99:1



2nd
2.65:1



3rd
1.81:1



4th
1.39:1



5th
1.16:1



6th
1.00:1



7th
0.83:1



Reverse
3.99:1

Final drive ratio
3.62:1

Drivetrain (cont.)
6-speed manual transmission (ZF G)
Ratios: 1st
4.06:1



2nd
2.40:1



3rd
1.58:1



4th
1.191



5th
1.00:1



6th
0.87:1



Reverse
3.68:1

Final drive ratio
3.62:1

Chassis

Body/frame construction
Unitized; aluminum front end,



otherwise steel

Front suspension
BMW M sport suspension in



aluminum; struts, double-pivot lower



arms, coil springs, electronically



controlled twin-tube gas-pressure



shock absorbers with Normal,



Comfort & Sport modes, anti-roll



bar, subframe, thrust plate

Rear suspension
BMW M sport suspension in



aluminum; 4-link Integral system, coil



springs, electronically controlled twin-



tube gas-pressure shock absorbers



with Normal, Comfort & Sport modes,



anti-roll bar

Steering type
Rack &  pinion, Servotronic vehicle-



speed- sensitive power assist



with Comfort & Sport assist levels

Overall ratio
Variable; mean ratio is 12.4:1

Turning circle, ft.
40.7

4-wheel compound, ventilated &


cross-ventilated disc brakes:


Front, diameter x thickness, mm/in.
374 x 36/14.7 x 1.42


Front calipers
Aluminum


Rear, diameter x thickness, mm/in.
370 x 24/14.6 x 0.94


Rear calipers
Cast iron

Wheels
Light cast alloy, 19 x 8.5 front/19 x 9.5 rear
Chassis (cont.)
Tires
255/40ZR-19 front /



285/35ZR-19 rear



special performance 

Stability-enhancement system
Dynamic Stability Control (DSC),



including all-speed traction control,



electronic brake proportioning,



antilock braking (ABS), cornering/



braking stability enhancement,

Dynamic Brake Control, Brake Standby, Brake Drying, Start-off Assistant



& M Dynamic mode;



M Variable Differential Lock

Performance data

Acceleration, 0-60 mph, sec. 

4.5

Top speed, mph
155 

Fuel economy, EPA est.. MPG,


city/highway
11/17 

#     #     #

� – Can be expanded via available folding rear seats.


� – VANOS = VAriable NOckenwellen Steuerung = variable camshaft control, or variable valve timing.


� – Due to low-profile tires, please note: Wheels, tires and suspension parts are more susceptible to road hazard and consequential damages. The M5 is not equipped with a spare tire and wheel. Performance tires are not recommended for driving in snow and ice conditions.


� – BMW AG test results. Actual acceleration results may vary depending on specification of vehicle; road and environmental conditions; testing procedures and driving style. These results should be used for comparison only, and verification should not be attempted on public roads. BMW urges you to obey all posted speed limits and to please wear your safety belt at all times.


� – Electronically limited.





- more -


- more -

[image: image2.png]


