	A subsidiary

 of BMW AG
	[image: image1.wmf]Printed on Recycled Paper

BMW

US Press Information

	Company
BMW of North America, LLC

BMW Group Company

Mailing address

PO Box 1227

Westwood, NJ

07675-1227
Office address

300 Chestnut Ridge Road 

Woodcliff Lake, NJ

07677-7731
Telephone

(201)  307-3606
Fax

(201)  307-3607

Internet

bmwusa.com
 [image: image1.wmf]



PAGE  
- 2 -


	For Release:
	October 4, 2007

	
	

	Contact:
	Thomas J. Plucinsky
Product and Technology Communications Manager

201-307-3783 / thomas.plucinsky@bmwna.com
William Scully

BMW Product Communications Specialist

201-307-3790 / william.scully@bmwna.com


	
	


2008 BMW X3 SAV®
Technical Specifications

Bold within table indicates new specification for 2008
General
X3 3.0si

Curb weight, lb.:


Manual transmission
4012

Automatic transmission
4067

Weight distribution, front/rear %



Manual transmission
50.8/49.2


Automatic transmission
50.9/49.1

Gross vehicle weight rating


(GVWR), lb.:


Manual transmission
5049


Automatic transmission
5104

Wheelbase, in.
110.1

Track, front/rear, in.
60.0/60.7
Length, in.
179.9
Width, in.
73.0
Height, in.
65.9 
Ground clearance, in.
8.0
Approach/departure angle, deg.
24.0/21.9 
Ramp breakover angle, deg.
18.0 

Body 
X3 3.0si
Type
5-door Sports Activity Vehicle®
Aerodynamic drag coefficient
0.35
EPA size classification
Special Purpose Vehicle
Accommodations

Seating capacity, persons
5
Shoulder room, front/rear, in.
55.6/55.0
Head room, front/rear, in.
39.3/39.4

Leg room, front/rear, in.
40.2/35.8
SAE interior volume, cu ft.
90.1
SAE cargo volume, cu ft.
30.0/71.0 
 

Engine & electrical

Engine type
DOHC inline 24-valve 6-cylinder, 



magnesium/aluminum composite construction,



Valvetronic variable intake-valve lift & Double



VANOS 
 steplessly variable intake & exhaust



valve timing
Bore x stroke, mm/in.
85.0 x 88.0/3.35 x 3.46
Displacement, cc/cu in.
2996/182.8 

Compression ratio
10.7:1 

Power @ rpm, hp
260 @ 6600

Torque @ rpm, lb-ft.
225 @ 2750

Engine-management system
Siemens MSV80 with knock control (2 sensors),



variable valve timing, electronic throttle system,



engine cooling & other functions included in 


control strategy

Fuel requirement
Premium unleaded
Fuel capacity, U.S. gal.
17.7
Battery capacity, amp-hr.
70
Alternator output rating, amp./W
150/2100
Drivetrain
X3 3.0si

Drive system
Front engine/all-wheel drive; xDrive system with



electronically controlled multi-disc clutch for



variable front/rear torque split depending upon



road & driving conditions, all-wheel electronic



traction control
Manual transmission
ZF Type H, 6-speed
Ratios: 1st
4.35:1



2nd
2.50:1



3rd
1.66:1



4th
1.23:1



5th
1.00:1



6th
0.85:1



Reverse
3.93:1

Final drive ratio
3.73:1

Automatic transmission
GM6, 6-speed

Ratios:
1st
4.07:1



2nd
2.37:1



3rd
1.55:1



4th
1.16:1



5th
0.85:1



6th
0.67:1



Reverse
3.20:1

Final drive ratio
4.44:1

Chassis

Body/frame construction
Unitized steel

Front suspension
Struts, double-pivot lower arms, twin-tube



gas-pressure shock absorbers, anti-roll bar



(Sport Package: sport calibration)

Rear suspension
Multi-link system with Central Links, upper &



lower lateral links, coil springs, twin-tube gas-



pressure shock absorbers, anti-roll bar



(Sport Package: sport calibration)

Steering type
Rack & pinion, engine-speed-sensitive



power assist



(optional: vehicle-speed-sensitive power assist)

Overall ratio
18.9:1


Turning circle, ft.
38.4

Chassis (cont.)
X3 3.0si

4-wheel ventilated disc brakes:


Diameter, front, mm/in.
325/12.8 


Diameter, rear, mm/in.
320/12.6 

Alloy wheels:


Standard
17 x 8.0


Sport Package, Sports Activity


Package or optional on Standard
18 x 8.0


Opt. (Require Sport Package)
19 x 8.5  front/19 x 9.0 rear
Tires:

 
Standard
235/55R-17 all-season


Sport Package or optional
235/50R-18 all-season


Opt. (require Sport Package)
235/45R-19 front/255/40R-19 rear


performance 

Stability-enhancement system
Dynamic Stability Control (DSC); includes electronic



brake proportioning, antilock braking (ABS),



Dynamic Brake Control, cornering/braking stability



enhancement & Hill Descent Control, Brake Fade 


Compensation, Brake Standby, Brake 



Drying & Start-off Assistant

Performance data

Acceleration, 0-60 mph, sec. 


Manual transmission
6.9


Automatic transmission
7.1

Top speed, mph:


Std. or Sport Package 


All-season tires
130 


Sport Package with 


optional performance tires



Manual transmission
143



Automatic transmission
142

Performance data (cont.)
X3 3.0si

Towing capacity, lb.
3500
Tongue weight, lb.
350
Payload, lb. (man./auto.)
1036 / 1036

EPA estimated MPG, city/highway:


Manual transmission
16/23

Automatic transmission
17/24
SAV and Sports Activity Vehicle are the registered trademarks for BMW light trucks.  

#      #      #

�− Rear seats upright/folded, loaded to ceiling.


� – VANOS = VAriable NOckenwellen Steuerung = variable camshaft control, or variable valve timing.


� – Due to low-profile tires, please note: wheels, tires and suspension parts are more susceptible to road hazard and consequential damages. Performance tires are not recommended for driving in snow and ice conditions.


� – BMW AG test results. Actual acceleration results may vary depending on specification of vehicle; road and environmental conditions; testing procedures and driving style. These results should be used for comparison only, and verification should not be attempted on public roads. BMW urges you to obey all posted speed limits and to please wear your safety belt at all times.


� – Electronically limited.





- more -


- more -

[image: image2.png]


