| For Release: | September 1, 2007 |
| --- | --- |
| Contact: | Thomas Plucinsky Product and Technology Communications Manager 201-307-3783 / thomas.plucinsky@bmwna.com Vincent Kung BMW Product Communications Specialist 201-307-3825 / vincent.kung@bmwna.com |

2008 BMW 6 SERIES

Technical Specifications

Bold within table indicates new specification for 2008.

General	650i	650i
		Coupe	Convertible

Curb weight, lb.:

Manual transmission	3814	4277

Automatic transmission	3814	4277

Weight distribution, front/rear, %:

Manual transmission	52.6/47.4	49.0/51.0

Automatic transmission	52.6/47.4	49.0/51.0

Wheelbase, in.	109.4

Track, front/rear, in.	61.3/62.8 1,

Length, in.	190.2 1

Width, in.	73.0 1

Height, in.	54.1 1

Body

Type	2-door coupe	2-door convertible

Aerodynamic drag coefficient	0.30	0.31

EPA size classification	Subcompact	Subcompact

Accommodations	650i	650i
		Coupe	Convertible

Seating capacity, persons	4	4

Shoulder room, front/rear, in.	56.4/49.4 1

Head room, front/rear, in.	37.6/35.6	37.8/36.5

Leg room, front/rear, in.	42.0/30.1	42.0/29.0

EPA interior volume, cu ft.	81.0	82.0

EPA cargo volume, cu ft.	13.0	12.4/10.6

Engine & electrical

Engine type	DOHC 32-valve (4-cam) V-8, Valvetronic variable

intake-valve lift & Double VANOS  steplessly

variable intake- & exhaust-valve timing 1

Bore x stroke, mm/in.	93.0 x 88.3/3.66 x 3.48 1

Displacement, cc/cu in.	4799/293 1

Compression ratio	10.5:1 1

Power @ rpm, hp	360 @ 6300 1

Torque @ rpm, lb-ft.	360 @ 3400 1

Engine-management system	Motronic ME9 with knock control; variable

valve lift & timing, electronic throttle system,

engine cooling & other functions included in

control strategy 1

Fuel requirement	Premium unleaded 1

Fuel capacity, U.S. gal.	18.5 1

Battery capacity, amp-hr.	110 1

Alternator output rating, amp./W	170/2380 1

Drivetrain

Drive system	Front engine/rear-wheel drive 1

Manual transmission	ZF Type G, 6-speed 1

Ratios:	1st	4.06:1 1

2nd	2.40:1 1

3rd	1.58:1 1

4th	1.19:1 1

Drivetrain (cont.)	650i	650i
		Coupe	Convertible

Ratios:	5th	1.00:1 1

6th	0.87:1 1

Reverse	3.77:1 1

Final drive ratio	3.46:1	3.46:1

Automatic transmission	ZF 6 HP 26 TU, 6-speed Technically Updated Sport

automatic, with Adaptive Transmission Control,

steering-wheel mounted shift paddles, selectable

Sport & Manual modes, rpm-matching on manual

downshifts 1

Ratios:	1st	4.17:1 1

2nd	2.34:1 1

3rd	1.52:1 1

4th	1.14:1 1

5th	0.87:1 1

6th	0.69:1 1

Reverse	3.40:1 1

Final drive ratio	3.46:1 1

Chassis

Body/frame construction	Unitized; aluminum front-end structure, aluminum

hood & doors, thermoplastic front fenders,

Sheet Molding Compound (SMC) trunklid 1

Front suspension	Struts, double-pivot lower arms, coil springs,

twin-tube gas-pressure shock absorbers, Active

Roll Stabilization; aluminum suspension system,

thrust plate & subframe 1 (Convertible: additional

diagonal braces)

Rear suspension	4-link Integral Link system, coil springs, twin-

tube gas-pressure shock absorbers, Active Roll

Stabilization; aluminum suspension system & sub-

frame 1 (Convertible: additional diagonal braces)

Steering type	Rack & pinion, Servotronic vehicle-speed-sensitive

variable power assist 1

Overall ratio	Mechanically variable; mean ratio is 14.4:1 1

Overall ratio, Active Steering	Electrically/electronically variable over wide range

(optional)	of 10:1 – 18:1 1

Turning circle, ft.	37.4 1

Chassis (cont.)	650i	650i
		Coupe	Convertible

Brakes	4-wheel ventilated discs with lightweight aluminum/

cast-iron rotors, aluminum front/cast-iron rear

calipers 1

Diameter, front, mm/in.	348/13.7 1

Diameter, rear, mm/in.	345/13.6 1

Wheels (standard)	Cast alloy, 18 x 8.0 front/18 x 9.0 rear 1

Wheels (optional)	Cast alloy, 19 x 8.5 front/19 x 9.0 rear	1

Wheels (Sport Package)	Forged alloy, 19 x 8.5 front/19 x 9.0 rear 1

Tires (standard)	245/45R-18 front / 275/40R-18 rear

run-flat performance 1,

Tires (optional)	245/40R-19 front / 275/35R-19 rear

run-flat performance (Sport Package) 1, 6

Stability-enhancement system	Dynamic Stability Control (DSC); includes Dynamic

Traction Control, electronic brake proportioning,

antilock braking (ABS), Dynamic Brake Control,

Brake Fade Compensation, Brake Standby,

Brake Drying, Comfort Stop, Start-off Assistant

& cornering/braking stability enhancement 1

Performance data

Acceleration, 0-60 mph, sec. :

Manual transmission	5.3	5.6

Automatic transmission	5.4	5.7

EPA estimated MPG, city/highway:

Manual transmission	15/22	14/21

Automatic transmission	15/23	15/23

– Specification applies to both models.

– With standard wheels.

– With softtop raised.

– Softtop raised/lowered, variable softtop storage compartment adjusted accordingly.

– VANOS = VAriable NOckenwellen Steuerung = variable camshaft control, or variable valve timing.

– Due to low-profile tires, please note: wheels, tires and suspension parts are more susceptible to road hazard and consequential damages. 6 Series models are not equipped with a spare tire and wheel. Performance tires are not recommended for driving in snow and ice conditions.

– BMW AG test results. Actual acceleration results may vary depending on specification of vehicle; road and environmental conditions; testing procedures and driving style. These results should be used for comparison only, and verification should not be attempted on public roads. BMW urges you to obey all posted speed limits and to please wear your safety belt at all times.