	A subsidiary

 of BMW AG
	[image: image1.wmf]Printed on Recycled Paper

BMW

US Press Information

	Company
BMW of North America, LLC

BMW Group Company

Mailing address

PO Box 1227

Westwood, NJ

07675-1227
Office address

300 Chestnut Ridge Road 

Woodcliff Lake, NJ

07677-7731
Telephone

(201)  307-3606
Fax

(201)  307-3607

Internet

bmwusa.com
 [image: image1.wmf]



PAGE  
- 2 -


	.For Release:
	September 1, 2007

	
	

	Contact:
	Thomas J. Plucinsky
Product and Technology Communications Manager

201-307-3783 / thomas.plucinsky@bmwna.com
William Scully

BMW Product Communications Specialist

201-307-3790 / william.scully@bmwna.com


	
	


2008 BMW M6 COUPE AND CONVERTIBLE
Technical Specifications

General
Curb weight, lb. 

M6 Coupe
3909

M6 Convertible
4398
Weight distribution, front/rear, % 


M6 Coupe
53.5/46.5

M6 Convertible
49.9/50.1
Wheelbase, in.
109.5
Track, front/rear, in.
61.7/62.4
Length, in.
191.8
Width, in.
73.0
Height , in.


M6 Coupe
54.0

M6 Convertible
54.2
Body

Type 


M6 Coupe
2-door coupe

M6 Convertible
2-door convertible
Aerodynamic drag coefficient  

M6 Coupe
0.32
[image: image2.wmf]Printed on Recycled Paper


M6 Convertible
0.34
EPA size classification
Subcompact

Accommodations

Seating capacity, persons
4

Shoulder room, front/rear, in.
56.4/49.4

Head room, front/rear, in.


M6 Coupe
37.6/35.6

M6 Convertible
37.8/36.5
Leg room, front/rear, in.

M6 Coupe
42.0/30.1

M6 Convertible
42/29
EPA passenger-compartment


volume, cu ft.


M6 Coupe
81.0


M6 Convertible
82.0
EPA cargo volume, cu ft. 


M6 Coupe
13.0

M6 Convertible 
12.4/10.6 
 
Engine & electrical




Engine type
DOHC (4-cam) 40-valve V-10, high-



Pressure Double VANOS 
 steplessly



variable valve timing, 10 individual



electronically controlled throttles with



P400, P500 & P500 S settings

Bore x stroke, mm/in.
92.0 x 75.2/3.62 x 2.96

Displacement, cc/cu in.
4999/305

Compression ratio
12.0:1

Power @ rpm, hp
500 @ 7750

Torque @ rpm, lb-ft.
383 @ 6100

Engine-management system
MS S65 with ionic-current knock control,


variable valve timing,electronic throttles,


variable maximum power level, g-sensitive 


lubrication system, oil-level/temperature 


sender, variable tachometer warning 


segment, catalyst protection & other 


functions included in control strategy

Fuel requirement
Premium unleaded

Engine & electrical (cont.)
Fuel capacity, U.S. gal.
18.5

Battery capacity, amp-hr.
90
Alternator output rating, amp./W
170/2380

Drivetrain
Drive system
Front engine/rear-wheel drive

Sequential Manual Gearbox  (SMG)
Electrohydraulically actuated,



electronically controlled 7-speed, 



with 6 Sequential & 5 Automated shift



programs; twin-disc clutch 
Ratios: 1st
3.99:1



2nd
2.65:1



3rd
1.81:1



4th
1.39:1



5th
1.16:1



6th
1.00:1



7th
0.83:1



Reverse
3.99:1

Manual transmission 

ZF Type G, 6-speed

Ratios: 1st
4.06:1



2nd
2.40:1



3rd
1.58:1



4th
1.19:1



5th
1.00:1



6th
0.87:1



Reverse
3.68:1

Final drive ratio
3.62:1

Chassis





Body/frame construction
Unitized; aluminum front-end



structure, aluminum hood & doors,



thermoplastic front fenders, carbon-



fiber roof & Sheet Molding Compound



(SMC) trunklid

Chassis (cont.)

Front suspension
BMW M sport suspension in



aluminum; struts, double-pivot lower



arms, coil springs, electronically



controlled twin-tube gas-pressure



shock absorbers with Normal,



Comfort & Sport modes, anti-roll



bar, subframe, thrust plate

Rear suspension
BMW M sport suspension in



aluminum; 4-link Integral system, coil



springs, electronically controlled twin-



tube gas-pressure shock absorbers



with Normal, Comfort & Sport modes,



anti-roll bar

Steering type
Rack &  pinion, Servotronic vehicle-



speed- sensitive power assist



with Comfort & Sport assist levels

Steering ratio
Variable; mean ratio is 13.0:1

Turning circle, ft.
41.0

4-wheel compound, ventilated &


cross-ventilated disc brakes:


Front, diameter x thickness, mm/in.
374 x 36/14.7 x 1.42


Front calipers
Aluminum


Rear, diameter x thickness, mm/in.
370 x 24/14.6 x 0.94


Rear calipers
Cast iron

Wheels
Cast alloy, 19 x 8.5 front/



19 x 9.5 rear
Tires
255/40ZR-19 front/



285/35ZR-19 rear



special performance 

Stability-enhancement system
Dynamic Stability Control (DSC),



including all-speed traction control,



electronic brake proportioning,



antilock braking (ABS), cornering/



braking stability enhancement,



Dynamic Brake Control, Brake


 

Standby, Brake Drying, Start-off 



Assistant & M Dynamic mode;



M Variable Differential Lock

Performance data

Acceleration, 0-60 mph, sec. 
 


M6 Coupe
4.5

M6 Convertible
4.6
Top speed, mph
155 

Fuel economy, EPA est. MPG,


city/highway


M6 Coupe (SMG and Manual)
11/17

M6 Convertible (SMG and Manual)
11/17
#      #      #
Company


BMW of North America, LLC





BMW Group Company


Mailing address


PO Box 1227


Westwood, NJ


07675-1227





Office address


300 Chestnut Ridge Road 


Woodcliff Lake, NJ


07677-7731





Telephone


(201)  307-3606





Fax


307-3607





Internet


bmwusa.com


 �











�– Specifications for the M6 Coupe and M6 Convertible are the same, except where noted.


� – Softtop raised/lowered, variable softtop storage compartment adjusted accordingly


� – VANOS = VAriable NOckenwellen Steuerung = variable camshaft control, or variable valve timing.


� – Available as a no-cost option


� – Due to low-profile tires, please note: Wheels, tires and suspension parts are more susceptible to road hazard and consequential damages. The M6 is not equipped with a spare tire and wheel. Performance tires are not recommended for driving in snow and ice conditions.


� – BMW AG test results. Actual acceleration results may vary depending on specification of vehicle; road and environmental conditions; testing procedures and driving style. These results should be used for comparison only, and verification should not be attempted on public roads. BMW urges you to obey all posted speed limits and to please wear your safety belt at all times.


� – Electronically limited.





- more -


- more -

[image: image3.png]


