	A subsidiary

 of BMW AG
	[image: image1.png]


BMW

US Press Information


–4–

	For Release:
	October 1, 2007

	
	

	Contact:
	Thomas Plucinsky
Product & Technical Communications Manager

201-307-3783 / thomas.plucinsky@bmwna.com
William Scully

BMW Product Communications Specialist

201-307-3790 / william.scully@bmwna.com


	
	


2008 BMW X5 SAV®
Technical Specifications

General
X5 3.0si
X5 4.8i


Curb weight, lb.
4982
5335
Weight distribution, front/rear, %
47.6/52.4
49.6/50.4
Wheelbase, in.
115.5 

Track, front/rear, in.
64.7/65.0 1,

Length, in.
191.1 1
Width, in.
76.1 1

Height, in.
69.5 1
SAV®-specific 




Gross Vehicle Weight Rating


(GVWR), lb.
6327 

6537
Payload, lb.
1345
1202
Onroad towing capacity, lb.:


Maximum trailer gross weight
6000 1

Suggested max. tongue weight
600 1
Ramp angle, deg.
19.1 1

Front overhang angle, deg.
24.5 1
Rear overhang angle, deg.
22.8 1
Ground clearance, unloaded, in.
8.3 1
Maximum climbing ability, %
50 1
Maximum lateral tilt, %
50 1
Fording depth @ 4 mph, in.
19.7 1
Body 

X5 3.0si
X5 4.8i

Type

5-door Sports Activity Vehicle® 1
Aerodynamic drag coefficient

0.34
0.35
EPA size classification

Special Purpose Vehicle1

Accommodations 



Seating capacity, persons
5 
 / 7 

Shoulder room, front/rear, in.
60.0/58.0 1, 3

Head room, front/rear, in.:


Without moonroof
39.3/39.0 1, 3

With moonroof
38.4/38.3 1, 3
Leg room, front/rear, in.
40.0/36.6 1, 3

EPA passenger volume, cu ft.
102.4 1, 3
Cargo volume, cu ft.
21.9/61.8 1, 

Cargo volume with 3rd-row


seating, cu ft.
7.1/18.5 

Engine & electrical 



Engine type
DOHC 24-valve inline

DOHC (4-cam) 32-valve 


6-cylinder, magnesium/
V-8, aluminum block & 



aluminum composite
heads, Valvetronic variable


construction, Valvetronic

intake-valve lift & Double 


variable intake-valve
VANOS 6 steplessly variable


lift & Double VANOS 

intake-& exhaust-valve 


steplessly variable intake-
timing


& exhaust-valve timing




Bore x stroke, mm/in.
85.0 x 88.0/3.35 x 3.46
93.0 x 88.3/3.66 x 3.48
Displacement, cc/cu in.
2996/183
4799/293
Compression ratio
10.7:1
10.5:1
Power @ rpm, hp
260 @ 6600
350 @ 6300
Torque @ rpm, lb-ft.
225 @ 2750

350 @ 3400-3800
Engine-management system
MS V80 with knock
Motronic ME 9.2.3 with



control; variable valve
knock control; variable




lift & timing, 3-stage
valve lift & timing, 2-stage



induction system, engine
induction system, engine
cooling & other functions
cooling & other functions


included in control strategy
included in control strategy
Engine & electrical (cont.)
X5 3.0si

X5 4.8i

Fuel requirement
Premium unleaded 1

Fuel capacity, U.S. gal.
22.5 1

Battery capacity, amp-hr.
70
90
Alternator output, amp./W
180/2520 1

Drivetrain 

Drive system
Front engine/all-wheel drive; xDrive system with



electronically controlled multi-disc clutch for variable


front/rear torque split; all-wheel electronic traction 

control1

Automatic transmission
ZF 6 HP 19 TU, 6-speed

ZF 6 HP 26 TU, 6-speed

Ratios:
1st
4.17:1 1



2nd
2.34:1 1



3rd
1.52:1 1



4th
1.14:1 1



5th
0.87:1 1



6th
0.69:1 1



Reverse
3.40:1 1

Final drive ratio
4.44:1
3.91:1

Chassis 



Body/frame construction
Unitized steel structure 1

Front suspension
Upper & lower lateral links with aluminum upper links,



steel double-pivot lower links, coil springs, twin-tube



gas-pressure shock absorbers, anti-roll bar;



aluminum thrust plate 1


ZSP: AdaptiveDrive, consisting of Active Roll Stabilization 


& Electronic Damping Control
Rear suspension
4-link Integral suspension with aluminum upper & lower



arms & wheel carriers; coil springs (3.0si base); air 


springs (3.0si w/self-leveling, 4.8i standard)


ZSP: Adaptive Drive, consisting of Active Roll Stabilization 


& Electronic Damping Control

Steering type:


Standard
Rack & pinion, power-assisted


Optional Active Steering
Rack & pinion, Servotronic vehicle-speed-sensitive



power assist 1


Overall ratio, standard steering
Mechanically variable; mean ratio is 15.2:1 1

Overall ratio, Active Steering
Electronically variable; mean ratio is 15.2:1 1

Chassis (cont.)
X5 3.0si

X5 4.8i


Turns, lock-to-lock
3.1 1

Turning circle, ft.
42.0 1
Brakes
4-wheel ventilated discs


Diameter x thickness, mm/in.:


  Front
332 x 30/13.1 x 1.18
348 x 30/13.7 x 0.1.18


  Rear
320 x 20/12.6 x 0.79
345 x 24/13.6 x 0.94


Assist
Vacuum 1

Cast-alloy wheels:


Standard
18 x 8.5 1

Sport Package
19 x 9.0 1


Optional
NA
20 x 10.0 front/20 x 11.0 rear

Tires:


Standard
All-season, 255/55R-18 1

Sport Package
All-season, 255/50R19 


Optional
Performance, 275/40R-20 front / 315/35R-20 rear 
Stability-enhancement system
Dynamic Stability Control (DSC), including all-speed 


traction (ADB-X) & stability (ASC-X) control via engine 


&/or brake intervention, Dynamic Traction Control, 


electronic brake proportioning, antilock braking (ABS), 


Cornering Brake Control, cornering stability 


enhancement, Dynamic Brake Control, Hill Descent 


Control, Trailer-towing stabilization, Brake Fade 


Compensation, Brake Standby, Brake Drying & 


Automatic Hold

Performance data 




Acceleration, 0-60 mph, sec. 

7.8
6.4
Top speed, mph 
:


Standard or Sport Package
130 1

With optional W-rated tires
NA
150

Fuel economy, EPA est. MPG, 

city/highway
15/21
14/19
SAV and Sports Activity Vehicle are the registered trademarks for BMW light trucks.
#     #     #






� – Specification applies to both models.


� – With standard wheels.


� – With standard front & 2nd-row seating; 3rd row optional.


� – With optional 3rd row.


� – DIN measurement; aft and to top of 2nd-row seats / aft of front seats, 2nd-row seating folded flush,


     cargo floor to ceiling.


� – 3rd-row seating upright/folded flush into cargo floor.


� – VANOS = VAriable NOckenwellen Steuerung = variable camshaft control, or variable valve timing.


� – BMW AG test results. Actual acceleration results may vary depending on specification of vehicle; road and environmental conditions; testing procedures and driving style. These results should be used for comparison only, and verification should not be attempted on public roads. BMW urges you to obey all posted speed limits and to please wear your safety belt at all times.


� – Electronically limited.





–more–

- more -


