	A subsidiary

 of BMW AG
	[image: image1.wmf]Printed on Recycled Paper

BMW

US Press Information

	Company
BMW of North America, LLC

BMW Group Company

Mailing address

PO Box 1227

Westwood, NJ

07675-1227
Office address

300 Chestnut Ridge Road 

Woodcliff Lake, NJ

07677-7731
Telephone

(201) 307-3606
Fax

(201)  307-3607

Internet

bmwusa.com
 [image: image1.wmf]



PAGE  
- 7 -


	For Release:
	December 15, 2005

	
	

	Contact:
	David J. Buchko

Product Communications Manager

201-307-3789 / dave.buchko@bmwna.com
William Scully

BMW Product Communications Specialist

201-307-3790 / william.scully@bmwna.com


	
	


2006 BMW 3 SERIES SEDANS & SPORTS WAGON
Technical Specifications

General
325i
325xi
325xi
330i
330xi


Sedan
Sedan
Sports
Sedan
Sedan



Wagon
Curb weight, lb.:


Manual transmission
3285
3560
3737
3417
3627

Automatic transmission
3351
3605
3782
3450
3671
Weight distribution, front/rear, %:


Manual transmission
50.3/49.7
52.6/47.4
50.4/49.6
50.6/49.4
52.6/47.4


Automatic transmission
50.7/49.3
52.9/47.1
50.7/49.3
51.1/48.9
52.9/47.1

Wheelbase, in.
108.7 

Track, front/rear, in. (with standard wheels)
59.1/59.6 1
Length, in.
178.2 1
Width, in.
71.5 1
Height, in.
55.9 1
Body
325i
325xi
325xi
330i
330xi


Sedan
Sedan
Sports
Sedan
Sedan



Wagon

Type
4-door sedan 

5-door wagon

Aerodynamic drag coefficient
0.30
0.31
0.31
0.30
0.32
EPA size classification
Compact
Compact
Small
Compact
Compact





Station





Wagon

Accommodations
Seating capacity, persons
5 1
Shoulder room, front/rear, in.
55.4/55.1 1
Head room, front/rear
37.4/37.1 2

37.3/36.9

Leg room, front/rear
41.5/34.6 1
EPA passenger-compartment


volume, cu ft.
93.0 2

92.6

EPA cargo volume, cu ft.
12.0 2, 


16.2/24.8/60.9 



Engine & electrical
325i
325xi
325xi
330i
330xi


Sedan
Sedan
Sports
Sedan
Sedan



Wagon

Engine type
DOHC inline 24-valve 6-cylinder, magnesium/aluminum composite



construction, Valvetronic variable intake-valve lift & Double



VANOS 
 steplessly variable intake- & exhaust-valve timing 1
Bore x stroke, mm/in.
85.0 x 88.0 1
Displacement, cc/cu in.
2996/183 1
Compression ratio
10.7 1
Power @ rpm, hp
215 @ 6250 


255 @ 6600 

Torque @ rpm, lb-ft.
185 @ 2750 6

220 @ 2750 7
Engine-management system
MSV70 with knock control (2 sensors); Valvetronic, variable valve



timing, engine cooling & other functions included in control strategy 1
Induction system
Single-stage 6

3-stage 7
Fuel requirement
Premium unleaded 1
Fuel capacity, U.S. gal. 
15.9 1
Battery capacity, amp-hr.
70 1
Alternator output rating, amp./W
180/2520 1

Drivetrain
325i
325xi
325xi
330i
330xi


Sedan
Sedan
Sports
Sedan
Sedan



Wagon

Drive system, engine/drive wheels
Front/
Front/
Front/
Front/
Front/



RWD 
AWD
AWD
RWD
RWD

Manual transmission
Getrag I,
Getrag H,



6-speed
6-speed 

Ratios:1st
4.32:1
4.35:1 8



2nd
2.46:1
2.50:1 8



3rd
1.66:1
1.66:1 8


4th
1.23:1
1.23:1 8


5th
1.00:1
1.00:1 8


6th
0.85:1
0.85:1 8
Final drive ratio
3.23:1
3.38:1
3.38:1
3.15:1
3.23:1

Automatic transmission
ZF 6 HP 19, 6-speed 1
Ratios: 1st
4.17:1 1


2nd
2.34:1 1


3rd
1.52:1 1


4th
1.14:1 1


5th
0.87:1 1


6th
0.69:1 1
Final drive ratio
3.73:1
3.91:1
3.91:1
3.64:1
3.64:1

RWD – Rear-wheel drive     AWD – All-wheel drive

Chassis
325i
325xi
325xi
330i
330xi


Sedan
Sedan
Sports
Sedan
Sedan



Wagon

Body/frame construction
Unitized steel 1
Front suspension
Struts, double-pivot lower arms, coil springs, twin-tube gas-pressure




shock absorbers, tubular anti-roll bar, subframe 1




(325i/330i models: aluminum lower arms,




steering knuckle & subframe)




(325i/330i ZSP: sport suspension calibration)

Rear suspension
5-link system, coil springs, twin-tube gas-pressure shock absorbers,




tubular anti-roll bar 1




(325i/330i ZSP: sport suspension calibration)

Rack-&-pinion steering:

Standard system
Engine-speed-sensitive power assist


Overall ratio
16.0 1

Turns lock-to-lock
3.0 1


Optional Active Steering
Vehicle-speed-sensitive ratio & power assist


Overall ratio
Electrically/electronically variable over wide range of 10.8:1 – 18.0:1


Turns lock-to-lock
1.8-3.0

Turning circle, ft.
36.1 

38.7 

ZSP – Sport Package

Chassis
325i
325xi
325xi
330i
330xi


Sedan
Sedan
Sports
Sedan
Sedan



Wagon

4-wheel ventilated disc brakes, vacuum-assisted:


Front, diameter x thickness, mm/in.
300 x 24/11.8 x 0.94 6

330 x 24/13.0 x 0.94 7

Caliper material
Aluminum


Rear, diameter x thickness, mm/in.
300 x 20/11.8 x 0.79 6

336 x 22/13.2 x 0.87 7

Caliper material
Cast iron

Cast-alloy wheels: standard
16 x 7.0 6


17 x 8.0 7


optional (Sport Package)
17 x 8.0 F/
17 x 8.0
17 x 8.0
18 x 8.0 F/
17 x 8.0




17 x 8.5 R


18 x 8.5 R



optional “xi” models,
NA
17 x 8.0 F/17 x 8.5 R 

NA
18 x 8.0 F/



require Sport Package




18 x 8.5 R

Run-flat tires:
standard
205/55R-16


225/45R-17



H-rated all-season 6

V-rated all-season 7, 



optional (Sport Package
225/45R-17 F/

225/40R-18 F/




“i” models; stand-alone
255/40R-17 R

255/35R-18 R



“xi” models, require
W-rated performance 12

W-rated performance 12


Sport Package)







optional (Sport Package,

225/45R-17
H-rated

Same as std.



“xi” models)

all-season 12




F – Front     R – Rear
Chassis
325i
325xi
325xi
330i
330xi


Sedan
Sedan
Sports
Sedan
Sedan



Wagon

Stability-enhancement system
Dynamic Stability Control, including electronic brake proportioning,



antilock braking (ABS), cornering/braking stability enhancement,



Dynamic Brake Control, Brake Fade Compensation, Brake Standby,



Brake Drying & Start-up Assistant, plus –


Dynamic Traction Control, Comfort Stop (325i/330i models)



Hill Descent Control (325xi/330xi models)

Performance data

Acceleration, 0-60 mph, sec.




Manual transmission
6.7
7.1
7.3
6.1
6.3


Automatic transmission
7.2
7.7
7.9
6.3
6.6

Top speed, mph:


Standard
130 1,


Sport Package, MT/AT
150/150 14
130/13014
130/13014
150/150 14
130/130 14


Sport Package + optional tires
NA
146/145
146/145
NA
150/150 14

EPA estimated MPG, city/highway:


Manual transmission
20/30
19/28
19/28
20/30
19/28


Automatic transmission
21/29
20/27
20/27
21/29
20/27

#      #      #
� – Specification applies to all models.


� – Specification applies to all Sedan models.


� – Does not include compartment under trunk floor, which encloses about 1.8 cu ft.; can be expanded via available folding rear seats.


� – Rear seats upright, loaded to top of seats/rear seats upright, loaded to ceiling/ rear seats folded, loaded to ceiling.


� – VANOS = VAriable NOckenwellen Steuerung = variable camshaft control, or variable valve timing.


� – Specification applies to 325i & 325xi models.


� – Specification applies to 330i & 330xi models.


� – Specification applies to 325xi, 330i & 330xi models.


� – Specification applies to 325i & 330i models (RWD).


� – Specification applies to 325xi & 330xi models (AWD).


� – Specification applies to 325xi models only.


� – Due to low-profile tires, please note: Wheels, tires and suspension parts are more susceptible to road hazard and consequential damages. These models are not equipped with a spare tire and wheel. Performance tires are not recommended for driving in snow and ice conditions.


� − BMW AG test results. Actual acceleration results may vary depending on specification of vehicle; road and environmental conditions; testing procedures and driving style. These results should be used for comparison only, and verification should not be attempted on public roads. BMW urges you to obey all posted speed limits and to please wear your safety belt at all times.


� – Electronically limited





- more -


- more -

[image: image2.png]


