	A subsidiary

 of BMW AG
	[image: image1.wmf]Printed on Recycled Paper

BMW

US Press Information


–5–

	For Release:
	December 15, 2005

	
	

	Contact:
	David J. Buchko

Product Communications Manager

201-307-3789 / dave.buchko@bmwna.com
William Scully

BMW Product Communications Specialist

201-307-3790 / william.scully@bmwna.com


	
	


2006 BMW X5 SERIES
Technical Specifications

General

X5 3.0i
X5 4.4i
X5 4.8is

Curb weight, lb.:


Manual transmission
4652
NA  

NA
Automatic transmission              
4696
4927

5016

Weight distribution, front/rear, %:




Manual transmission
48.1/51.9
NA
NA


Automatic transmission
48.6/51.4
49.7/50.3
49.5/50.5

Gross Vehicle Weight Rating


(GVWR), lb.



6008

      6008

6008             

Wheelbase, in.




111.0

      111.0

111.0

Length, in.




183.7

      183.7

183.7

Width, in.




 73.7

       73.7

73.7

[image: image2.png]


Height, in.




 67.5
 
       67.2

67.2

Accommodations 

Seating capacity, person
5 1
Shoulder room, front/rear, in.


58.0/57.2  1
Head room, front/rear, in.:


without moonroof



39.9/38.5
39.9/38.5

–


with moonroof



38.9/38.5
38.9/38.5

38.9/38.5

Leg room, front/rear, in.


39.3/35.4 1 

EPA interior volume, cu ft.


97.6 1 

Max SAE cargo volume,





rear seats folded, cu ft.


69.0 
  

Body 





X5 3.0i

X5 4.4i
X5 4.8is

Type





Unitized all-steel structure 1
Aerodynamic drag coefficient (CD)
0.35

       0.36

0.38

EPA size classification



Special Purpose Vehicle1
Engine & electrical

Engine type
DOHC 24-valve
DOHC 32-valve (4-cam)



inline 6-cylinder,
V-8, Valvetronic variable



Double VANOS 

intake-valve lift, Double



variable intake-
VANOS 3 variable intake-



& exhaust-valve
& exhaust-timing,



timing, 2-stage
steplessly variable



induction system
induction system 

Bore x stroke, mm/in.                           
84.0 x 89.6/
92.0 x 82.7/
93.0 x 88.3



3.31 x 3.53
3.62 x 3.26
3.66 x 3.48

Displacement, cc/cu in.
2979/182
4398/268
4799/293

Compression ratio
10.2:1
10.5:1
10.5:1

Power @ rpm, SAE net
225@5900
315@5400
355@6200

Torque @ rpm, lb-ft.
214@3500
324@3600
369@3500

Fuel requirement
Premium unleaded 1  

Fuel capacity, U.S. gal
24.6 1  

Engine management system
Siemens MS 43
Bosch Motronic ME 9.2.1

Alternator output, amp./W
140/1960
170/2380 (liquid-cooled




& fully encapsulated)

Battery capacity, amp-hr.
80
90

90 

Drivetrain
X5 3.0i
X5 4.4i

X5 4.8is

Drive system




Front engine/all-wheel drive; xDrive system








with electronically controlled multi-disc








clutch for variable front/rear torque split








depending upon road & driving conditions,








all-wheel electronic traction control 

Manual transmission
Wide-ratio
      NA

NA



ZF Type H, 6-speed

Ratios:


1st
5.14:1


2nd
2.83:1


3rd
1.79:1


4th
1.26:1


5th
1.00:1


6th
0.83:1


Reverse
4.64:1

Final drive ratio
4.10:1

Automatic transmission
STEPTRONIC with Adaptive Transmission


Control, Sport & Manual modes 1
Transmission model/type
GM5 / 5-speed
ZF Type HP / 6-speed 4

Ratios:


1st
3.42:1
3.17:1 4


2nd
2.22:1
2.34:1 4


3rd
1.60:1
1.52:1 4


4th
1.00:1
1.14:1 4


5th
0.75:1
0.87:1 4


6th
–
0.69:1 4


Reverse
3.03:1
3.40:1 4

Final drive ratio
4.10:1
4.10:1
3.91:1

Chassis
X5 3.0i
X5 4.4i
X5 4.8is



Front suspension



Struts, double-pivot lower arms, twin-tube








gas-pressure shock absorbers, anti-roll bar; 








coil springs (standard 3.0i & 4.4i);








air springs (3.0i & 4.4i with adjustable ride








height, standard 4.8is)








(Sport Package & 4.8is: sport calibration) 1
Rear suspension



4-link Integral suspension with aluminum








Integral Link, forward upper lateral arms








& wheel carrier; steel lower & rear upper








lateral arms; coil springs (3.0i base); air








springs (3.0i w/ self-leveling or adjustable








ride height, 4.4i & 4.8is standard) 1







(Sport Package & 4.8is: sport calibration) 1
Steering type:
Rack & pinion 1


Variable power assist
Engine-speed-sensitive 4
Vehicle-




speed-




sensitive


Overall ratio
21.1:1 1


Turns, lock-to-lock
3.5 1

Turning circle, ft.
39.7 1
4-wheel disc brakes:



 
Front (mm/in.), ventilated)
332/13.1
332/13.1
356/14.0
Rear (mm/in.)
324/12.8
324/12.8
324/12.8



solid
ventilated
ventilated

Alloy wheels: 


Standard
17 x 7.5
18 x 8.5
20 x 9.5 F/





20 x 10.5 R


Sport Package
18 x 8.5
18 x 8.5
NA




Optional (require Sport Package)
NA
19 x 9.0 F/
NA




19 x10.0 rear

Chassis (cont.)
X5 3.0i
X5 4.4i
X5 4.8is

Tires:


Standard
235/65R-17
255/55R-18
275/40R-20/





315/35R-20





front/rear 



H-rated
H-rated
W-rated



all-season
all-season
performce.


Sport Package
255/55R-18
255/55R-18
NA



H-rated
V-rated



all-season
all-season


Optional with Sport Pkg.
NA
255/50R-19/




285/45R-19




front/rear 5
                                                                   

V-rated per-




formance

Stability-enhancement system
Dynamic Stability Control (DSC); includes



electronic brake proportioning, antilock



braking (ABS), Dynamic Brake Control,



cornering/braking stability enhancement



& Hill Descent Control) 1
Performance data
Acceleration, 0-60 mph, sec.: 


Manual transmission

7.8
NA

NA


Automatic transmission

8.3
6.8
5.9

Top speed, mph
130
130
153





(150 w/





Sport Package )


Towing capacity, braked, lb.

6000
6000
6000

EPA estimated MPG, city/highway:


Manual transmission
15/21
NA

NA

   
Automatic transmission
16/21
16/22

16/21

NA – Not applicable                                             #      #      #              
Company


BMW of North America, LLC





BMW Group Company





Mailing address


PO Box 1227


Westwood, NJ


07675-1227





Office address


300 Chestnut Ridge Road 


Woodcliff Lake, NJ


07677-7731





Telephone


(201)  307-3606





Fax


307-3607





Internet


bmwusa.com


 �








1 - Specification applies to all models.


2 - 16.1-cu-ft. capacity to top of upright rear-seat backrests; 23.8 cu ft. when cargo is loaded to ceiling.





� – VANOS = VAriable NOckenwellen Steuerung = variable camshaft control, or variable valve timing.


� – Specification applies to X5 4.4i & 4.8is.


� – Due to low-profile tires, please note: wheels, tires and suspension parts are more susceptible to road hazard and consequential damages. Performance tires are not recommended for driving in snow and ice conditions.


� – BMW AG test results. Actual acceleration results may vary depending on specification of vehicle; road and environmental conditions; testing procedures and driving style. These results should be used for comparison only, and verification should not be attempted on public roads. BMW urges you to obey all posted speed limits and to please wear your safety belt at all times.





–more–

- more -


