# 2010 S 1000 RR SPECIFICATIONS

Engine

Type	Water-cooled 4-stroke in-line four-cylinder-engine, two camshafts, four valves per cylinder

Bore x stroke	80.0 mm x 49.7 mm

Displacement	999 cc

Horsepower	142 kW (193 hp) at 13,000 rpm

Redline	14,200 rpm

Torque	83 lb/ft @ 9,750 rpm

Compression ratio	13.0 : 1

Engine configuration	Straight-four

No of cylinders	4

Valve operation	DOHC (Double Overhead Camshaft), valves operated by single cam followers beneath engine

Valves per cylinder	4 titanium valves

Intake / exhaust valve diameter	2 x 33.5 mm intake / 2 x 27.2 mm exhaust

Throttle	E-gas fly-by-wire throttle with four power modes: Rain, Sport, Rain or Slick

Throttle butterfly diameter	48 mm (1.89 inches)

Intake	Variable length intake manifold

Mixture control / engine management	Variable pressure electronic intake pipe injection/digital engine management including knock sensor (BMS-K-P)

Emission control	Catalytic-2 Closed-loop 3-way catalytic converter

Fuel requirement	Unleaded premium, octane 98 (RON), automatic knock control permits operation with minimum octane 95 (RON).

# Charging system	Three-phase alternator 350 W

Battery	12 V / 10 Ah, maintenance-free

Power Transmission

# Clutch	Multi-disc, anti-hopping oil bath clutch operated mechanically

# HP Gearshift Assistant	Optional clutch-less up-shifts

Gearbox	Constant mesh 6-speed gearbox

Primary transmission ratio	1:1.652

Gear ratios	I	1:2.6471
II	1:2.091
III	1:1.727
IV	1:1.500
V	1:1.360
VI	1:1.261

Final drive system	Chain

Final drive ratio	1:2.588

Frame and Suspension

# Frame	Bridge-type frame, cast aluminum, load-bearing engine

# Sub frame	Bolt-on, aluminum

Front suspension	46 mm upside-down fork, rebound and compression adjustable

Rear suspension	Adjustable rear shock with separate high and low speed damping adjustments for rebound and compression

# Swing arm	Cast aluminum double  swing arm with adjustable pivots and central spring strut

Suspension travel, front / rear	4.72 in / 5.12 in

Wheelbase	56.38 in

Trail	3.76 in

Steering head angle	66.1°

Wheels / tires

Front wheel	Ten-spoke 3.5 x 17" pressure-cast aluminum wheel

Rear wheel	Ten-spoke 6.0 x 17" pressure-cast aluminum wheel

Front tire	120/70 ZR 17

Rear tire	190/55 ZR 17

Brakes

Brakes, front	Twin floating discs, 12.6” (320 mm) diameter , radial 4-piston fixed calipers

Brakes, rear	Single disc, 8.7” (220 mm) diameter, single-piston floating caliper

Brake lines	Braided steel

Race ABS	Optional, Race ABS (party integral, on-demand) with four settings: Rain, Sport, Race or Slick

DTC*	Dynamic Traction Control with four settings: Rain, Sport, Race or Slick
*(only available in combination with optional Race ABS, on-demand)

Dimensions / weights

Seat height	32.3 in

Inner leg curve	71.3 in

Weight, unladen, with full tank	450 lbs (455 lbs with Race ABS)

Weight - dry	404 lbs

Permitted total weight	860 lbs

Payload (with standard equipment)	412 lbs

Fuel tank capacity	Aluminum fuel tank, 4.6 gallons

Reserve	Approx. 1.1 gallons

Length	81.0 in

Height (excl. mirrors)	44.8 in

Width (incl. mirrors)	35.5 in

Colors

N44 Thunder Gray Metallic	Black seat, gray wheels		NC

N45 Alpine white / blue metallic / magma red	Black seat, black wheels		$750

N46 Acid green metallic	Black seat, gray wheels		NC

N47 Mineral silver metallic	Black seat, black wheels		NC

Price / options

MSRP	$13,800

129 Race ABS with DTC	$1,480

177 Gearshift assistant	$450

603 Alarm	$395